3GPP TSG RAN WG1 #110bis-e		     R1-2208887

e-Meeting, October 10th – 19th, 2022

Source:	vivo

Title:	Discussion on HARQ-ACK multiplexing of unicast SPS PDSCHs and multicast DG PDSCHs

Agenda Item:	8.12

Document for:	Discussion and Decision

- Introduction

In this contribution, we share our view on HARQ-ACK multiplexing of unicast SPS PDSCHs and multicast DG PDSCHs.

- HARQ-ACK multiplexing of unicast SPS PDSCHs and multicast DG PDSCHs

For the multiplexing of HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback with the same priority, it was agreed to multiplex these HARQ-ACK on the PUCCH configured by SPS-PUCCH-AN-List configured for unicast in previous meetings.

Agreement (RAN1#107bis)

When HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, down-select from:

- Option 1: the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast.

- Option 2: the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

Agreement (RAN1#108)

When HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, the following option 1 (from the previous agreement) is adopted:

- Option 1: the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast.

- Option 2: the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast

However, SPS-PUCCH-AN-List configured for unicast is based on the capability of supporting multiple of unicast SPS PDSCHs, i.e., FG12-2, which should be independent from R17 multiplexing of unicast HARQ-ACK and multicast HARQ-ACK with the same priority. There is no reason to always bundle these two features. Considering at least R17 UEs without capability of SPS-PUCCH-AN-List for unicast, the relevant UE behavior needs to be specified. Otherwise, gNB wouldn’t be able to schedule multicast PDSCHs to transmit HARQ-ACK in the same slot with unicast SPS HARQ-ACK, which implies great restrictions on gNB’s scheduling, especially when unicast SPS PDSCH is configured with short periodicity.

When SPS-PUCCH-AN-List configured for unicast is not configured, one way is to multiplex HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback on one of the PUCCH resources configured for multicast. i.e., adopting option 2 in the above agreements in this case. Another simpler way is to drop one of these two kinds of HARQ-ACK. Either dropping unicast SPS HARQ-ACK, or dropping multicast DG HARQ-ACK is workable. Considering gNB may get HARQ-ACK information from other UEs in the same MBS group and use PTM for retransmission of multicast PDSCHs, it is preferred to drop HARQ-ACK for multicast in this case.

Proposal: When HARQ-ACK for unicast SPS PDSCHs only and HARQ-ACK for multicast dynamic grant PDSCHs with ACK/NACK based feedback with the same priority are indicated to be transmitted in the same PUCCH slot,

- If SPS-PUCCH-AN-List for unicast is not configured, HARQ-ACK for multicast dynamic grant PDSCHs with ACK/NACK based feedback is dropped.

- Conclusion

In this contribution, we discuss HARQ-ACK multiplexing of unicast SPS PDSCHs and multicast DG PDSCHs, and the following proposal is made.

Proposal: When HARQ-ACK for unicast SPS PDSCHs only and multicast dynamic grant PDSCHs with ACK/NACK based feedback with the same priority are indicated to be transmitted in the same PUCCH slot

- If SPS-PUCCH-AN-List for unicast is not configured, HARQ-ACK for multicast dynamic grant PDSCHs with ACK/NACK based feedback is dropped.

References

[1]	RAN WG1 #107bis e-meeting Chairman’s Notes

[2]	RAN WG1 #108 e-meeting Chairman’s Notes