3GPP TSG RAN WG1 Meeting #110bis-e	R1-2208994

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Type-2 HARQ-ACK codebook in PUSCH for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Langbo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE has to always multiplex HARQ-ACK information for PDSCH receptions with disabled HARQ-ACK information in PUSCH even if UL DAI value VT-DAIUL=4 when the UE has received a PDCCH scheduling PDSCH receptions with disabled HARQ-ACK information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the condition for disabling multiplexing HARQ-ACK information in PUSCH transmission in case UL DAI value VT-DAIUL=4 by excluding the PDCCH scheduling PDSCH receptions with disabled HARQ-ACK information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL DAI value VT-DAIUL=4 cannot disable the multiplexing of HARQ-ACK information for PDSCH receptions with disabled HARQ-ACK information in PUSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 9.1.3.2	Type-2 HARQ-ACK codebook in physical uplink shared channel

<omit>

If a UE is not provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for a DCI format scheduling PDSCH receptions providing transport blocks for HARQ processes with enabled HARQ-ACK information or having associated HARQ-ACK information without scheduling PDSCH receptions on any serving cell c, and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH as described in clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission.

If a UE is provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with first value VT-DAIUL=4 or with second value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for a DCI format scheduling PDSCH reception providing a transport block for HARQ process with enabled HARQ-ACK information or having associated HARQ-ACK information without scheduling PDSCH reception on any serving cell c, and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in clause 9.1.3.1, the UE does not multiplex HARQ-ACK information for the first sub-codebook or for the second sub-codebook, respectively, in the PUSCH transmission.

<omit>

END OF CHANGE