3GPP TSG RAN WG1 #110bis-e		R1-2209162

e-Meeting, October 10th –19th, 2022

Source:	Panasonic

Title: 	Discussion on dynamic waveform switching

Agenda Item:		9.14.3

Document for:	Discussion

Introduction

A new work item on further NR coverage enhancements has been approved for Rel.18 [1]. One of objectives is to specify enhancements to support dynamic switching between DFT-s-OFDM and CP-OFDM. This document provides our view on dynamic switching between DFT-s-OFDM and CP-OFDM.

Discussion

DFT-s-OFDM waveform is beneficial for UL coverage limited scenario because of its lower PAPR compared with CP-OFDM waveform. While CP-OFDM is beneficial for higher SINR scenario because of more friendly realization of MIMO (= more than rank 1 transmission). In the current NR specification, more than rank 1 transmission is supported only for CP-OFDM and UL waveform is configured via RRC. Therefore, to use more than rank 1 in cell-edge UEs is difficult and such restriction is inefficient. Instead of supporting more than rank 1 for DFT-s-OFDM waveform, which requires more complexity in the standardization and implementation, RAN decided to support dynamic switching between DFT-s-OFDM and CP-OFDM [1].

Background (Rel.15 ~ Rel.17 behavior)

For a PUSCH scheduled by RAR UL grant, or for a PUSCH scheduled by fallbackRAR UL grant, (i.e., for Msg.3 PUSCH initial transmission), or for a PUSCH scheduled by DCI format 0-0 with CRC scrambled by TC-RNTI (i.e., for Msg.3 PUSCH retransmission), the UE shall use the waveform either DFT-s-OFDM or CP-OFDM according to the higher layer parameter msg3-transformPrecoder. For a Msg.A PUSCH, the UE shall use the waveform either DFT-s-OFDM or CP-OFDM according to the higher layer configured parameter msgA-TransformPrecoder. If higher layer parameter msgA-TransformPrecoder is not configured, the UE shall use the waveform either DFT-s-OFDM or CP-OFDM according to the higher layer configured parameter msg3-transformPrecoder.

For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI = 1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI (e.g., DG-PUSCH)

- In case DCI format 0-0, the UE shall use the waveform according to the higher layer configured parameter msg3-transformPrecoder.

- In case DCI format 0-1/0-2

  - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall use the waveform according to this parameter.

  - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall use the waveform according to the higher layer configured parameter msg3-transformPrecoder.

For PUSCH transmission with a configured grant (i.e., CG-PUSCH)

- If the UE is configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall use the waveform according to this parameter.

- If the UE is not configured with the higher layer parameter transformPrecoder in confifugredGrantConfig, the UE shall use the waveform according to higher layer configured parameter msg3-transformPrecoder.

The above Rel.15 ~ Rel.17 waveform determination behavior can be summarized as follows.

|  | If higher layer parameter is configured | If higher layer parameter is not configured |
| --- | --- | --- |
| Msg.3 PUSCH | Based on msg3-transformPrecoder | N/A |
| Msg.A PUSCH | Based on msgA-TransformPrecoder | Based on msg3-transformPrecoder |
| DG-PUSCH scheduled by DCI format 0-0 | Based on msg3-transformPrecoder |  |
| DG-PUSCH scheduled by DCI format 0-1/0-2 | Based on transformPrecoder in pusch-Config | Based on msg3-transformPrecoder |
| CG-PUSCH | Based on transformPrecoder in configuredGrantConfig | Based on msg3-transformPrecoder |

In the current specification, UL waveform is configured via RRC as described above. On the other hand, dynamic waveform switching depending on which DCI format is used is supported, for example, DFT-s-OFDM for DCI format 0-0 and CP-OFDM for DCI format 0-1. However, in this case, PUSCH using DFT-s-OFDM is always scheduled by DCI format 0-0, and then, PUSCH using DFT-s-OFDM transmission will be restrictive from the scheduling flexibility point of view, e.g., no carrier indicator, no BWP indicator, no precoding / antenna port indication, no CBG transmission, etc. Therefore, the necessity function should be that both DFT-s-OFDM and CP-OFDM are supported for a single DCI format such as DCI format 0-1 and 0-2.

Observation 1: In the current specification, dynamic switching between DFT-s-OFDM and CP-OFDM depending on which DCI format is used is supported. For example, DFT-s-OFDM for DCI format 0-0 and CP-OFDM for DCI format 0-1 is already possible.

Observation 2: The necessity function is that both DFT-s-OFDM and CP-OFDM are supported for a single DCI format such as DCI format 0-1 and 0-2.

What kind of PUSCH transmission should support the dynamic waveform switching?

As summarized above, there are several types of PUSCH transmissions (Msg.3 PUSCH, Msg.A PUSCH, DG-PUSCH scheduled by DCI format 0-0, DG-PUSCH scheduled by DCI format 0-1/0-2, and CG-PUSCH) in NR. Firstly, whether dynamic waveform switching is supported or not to each PUSCH transmission type should be discussed.

In our view, at least DG-PUSCH scheduled by DCI format 0-1/0-2 should support dynamic waveform switching. Then, the discussion point would be whether other PUSCH transmission types support dynamic waveform switching or not.

For DG-PUSCH scheduled by DCI format 0-0, in order to utilize DCI format 0-0 regardless of RRC reconfiguration and to keep the same payload size in common search space, no change should be applied. Therefore, our view is that dynamic waveform switching is not supported for DG-PUSCH scheduled by DCI format 0-0.

For CG-PUSCH, for Type 1 CG-PUSCH, since there is no dynamic indication, it is not necessary to support dynamic waveform switching. For Type 2 CG-PUSCH, if dynamic switching is supported for DCI format 0-1/0-2, to support dynamic waveform switching via activation DCI with the unified indication mechanism could be considered.

For Msg.3 or Msg.A PUSCH, the suitable waveform could be different among UEs, for example CP-OFDM for cell-center UEs while DFT-s-OFDM for cell-edge UEs. Therefore, to have flexibility for the waveform might be beneficial even for UL transmissions for initial access at least for Msg.3 PUSCH. Potential specification impact would be signaling design, especially indication of waveform shall not impact the legacy UE interpretation of the RAR or DCI format 0-0 with CRC scrambled by RA-RNTI respectively. However, it would be lower priority discussion compared to DG-PUSCH scheduled by DCI format 0-1/0-2.

Proposal 1: Dynamic switching between DFT-s-OFDM and CP-OFDM is supported for DG-PUSCH scheduled by DCI format 0-1/0-2.

Proposal 2: Dynamic switching between DFT-s-OFDM and CP-OFDM is not supported for DG-PUSCH scheduled by DCI format 0-0.

Proposal 3: Dynamic switching between DFT-s-OFDM and CP-OFDM is not supported for Type 1 CG-PUSCH.

Proposal 4: Further study whether dynamic switching between DFT-s-OFDM and CP-OFDM is supported or not for Type 2 CG-PUSCH.

Proposal 5: Whether dynamic switching between DFT-s-OFDM and CP-OFDM is supported or not for Msg.3 PUSCH can be discussed, but the discussion should be lower priority.

Dynamic waveform indication mechanism

To enabled or disabled dynamic waveform switching should be configured via RRC. Following two options are considered.

- Option 1: transformPrecoder in pusch-Config has new value like “dynamicSwitch’.

- Option 2: New RRC parameter transformPrecoderDynamicis introduced in pusch-Config.

In Option 1, if higher layer parameter transformPrecoder in pusch-Config is set to ‘dynamicSwitch’, the UE shall determine whether DFT-s-OFDM or CP-OFDM is used as defined by the dynamic manner. In Option 2, if higher layer parameter transformPrecoderDynamic is configured in pusch-Config, transformPrecoder in pusch-Config is not valid (i.e., UE ignore this parameter) and the UE shall determine whether DFT-s-OFDM or CP-OFDM is used as defined by dynamic manner. Option 1 and Option 2 are no difference of physical layer behavior but only how RRC signaling is designed. In our view, since Option 2 does not require the change for the existing parameter transformPrecoder, Optio 2 is more reasonable approach for backward compatibility signaling reason. However, RRC design could be up to RAN2 discussion.

Proposal 6: Dynamic waveform switching is per-UE indication.

Proposal 7: Dynamic switching between DFT-s-OFDM and CP-OFDM is enabled or disabled via RRC.

For DCI format 0-1/0-2, the field size of some DCI fields depends on the applied waveform for PUSCH (e.g., FDRA, frequency hopping flag, precoding information and number of layers, antenna ports, PTRS-DMRS association, and DMRS sequence initialization). The payload size should be deterministic before the detection of the DCI is important as multiple DCI sizes increases the number of blind detections. Basically, the field size for CP-OFDM is larger than the case for DFT-s-OFDM, therefore, the simplest way is that the field size should be based on CP-OFDM, which is larger or equal payload size with DFT-s-OFDM.

Proposal 8: When dynamic switching between DFT-s-OFDM and CP-OFDM is enabled, the DCI field size is determined based on CP-OFDM which is larger or equal payload size with DFT-s-OFDM.

For the dynamic waveform indication, following options could be considered.

- Option 1: Explicit indication, i.e., new DCI field (1 bit) for dynamic waveform switching is introduced.

- Option 2: Implicit indication, i.e., dynamic waveform switching is indicated without additional DCI field

  - Option 2-1: Joint indication with FDRA

  - Option 2-2: Joint indication with TDRA

  - Option 2-3: Joint indication with MCS index

  - Option 2-4: Joint indication with the number of layers

Option 1 is straightforward way and flexible waveform switching with other scheduling parameters is possible. On the other hand, the potential concern is DCI overhead increase. Especially, DCI format 0-2 is intended for URLLC usage and smaller payload size (result in higher reliability) might be desirable.

Option 2 has the merit that dynamic waveform switching without increasing DCI payload size is possible. Implicit indication examples are summarized as follows.

|  | Waveform determination factor | Indication example |
| --- | --- | --- |
| Option 2-1: Joint indication with FDRA | Resource allocation type | DFT-s-OFDM is applied if resource allocation type 1 is used, while CP-OFDM is applied if resource allocation type 0 is used. |
| Option 2-2: Joint indication with TDRA | TDRA field value | Waveform indication is jointly coded with SLIV in TDRA table, by adding an additional column for the waveform. |
| Option 2-3: Joint indication with MCS index | MCS index value | The UE is configured with threshold. If the index indicated by MCS field is lower than the threshold, DFT-s-OFDM is applied; otherwise, CP-OFDM is applied. |
| Option 2-4: Joint indication with the number of layers | Number of layers | 1-layer transmission is always based on DFT-s-OFDM, while 2 or more layers transmission is based on CP-OFDM. |

On Option 2-1, DFT-s-OFDM only supports resource allocation type 1. Therefore, implicit indication using FDRA such as, DFT-s-OFDM is applied if resource allocation type 1 is used, while CP-OFDM is applied if resource allocation type 0 is used, is possibility. On Option 2-2, similar to dynamic indication of repetition, waveform indication can also be jointly coded with SLIV in TDRA table, by adding an additional column for the waveform. On Option 2-3, the UE is configured with threshold. In low MCS, DFT-s-OFDM would be more suitable, while in high MCS (and multiple rank transmission), CP-OFDM is more suitable. On Option 2-4, waveform is determined based on the number of layers. Since the main reason to support dynamic switching between DFT-s-OFDM and CP-OFDM is that higher rank operation in good SINR is limited when semi-static DFT-s-OFDM configuration, the necessary functionality should be to support dynamic switch between 1-layer transmission with DFT-s-OFDM and 2 or more layers transmission with CP-OFDM. Then, just to support the implicit indication based on number of layers would be the most straightforward design.

Proposal 9: Discuss the indication mechanism for dynamic switching between DFT-s-OFDM and CP-OFDM. Following options are further discussed.

- Option 1: Explicit indication, i.e., new DCI field (1 bit) for dynamic waveform switching is introduced.

- Option 2: Implicit indication

  - Option 2-1: Joint indication with FDRA

  - Option 2-2: Joint indication with TDRA

  - Option 2-3: Joint indication with MCS index

  - Option 2-4: Joint indication with the number of layers

Conclusion

In this contribution, we provide our view on dynamic switching between DFT-s-OFDM and CP-OFDM for NR coverage enhancement. We made following observations and proposals.

Observation 1: In the current specification, dynamic switching between DFT-s-OFDM and CP-OFDM depending on which DCI format is used is supported. For example, DFT-s-OFDM for DCI format 0-0 and CP-OFDM for DCI format 0-1 is already possible.

Observation 2: The necessity function is that both DFT-s-OFDM and CP-OFDM are supported for a single DCI format such as DCI format 0-1 and 0-2.

Proposal 1: Dynamic switching between DFT-s-OFDM and CP-OFDM is supported for DG-PUSCH scheduled by DCI format 0-1/0-2.

Proposal 2: Dynamic switching between DFT-s-OFDM and CP-OFDM is not supported for DG-PUSCH scheduled by DCI format 0-0.

Proposal 3: Dynamic switching between DFT-s-OFDM and CP-OFDM is not supported for Type 1 CG-PUSCH.

Proposal 4: Further study whether dynamic switching between DFT-s-OFDM and CP-OFDM is supported or not for Type 2 CG-PUSCH.

Proposal 5: Whether dynamic switching between DFT-s-OFDM and CP-OFDM is supported or not for Msg.3 PUSCH can be discussed, but the discussion should be lower priority.

Proposal 6: Dynamic waveform switching is per-UE indication.

Proposal 7: Dynamic switching between DFT-s-OFDM and CP-OFDM is enabled or disabled via RRC.

Proposal 8: When dynamic switching between DFT-s-OFDM and CP-OFDM is enabled, the DCI field size is determined based on CP-OFDM which is larger or equal payload size with DFT-s-OFDM.

Proposal 9: Discuss the indication mechanism for dynamic switching between DFT-s-OFDM and CP-OFDM. Following options are further discussed.

- Option 1: Explicit indication, i.e., new DCI field (1 bit) for dynamic waveform switching is introduced.

- Option 2: Implicit indication

  - Option 2-1: Joint indication with FDRA

  - Option 2-2: Joint indication with TDRA

  - Option 2-3: Joint indication with MCS index

  - Option 2-4: Joint indication with the number of layers

- Reference

[1]	RP-221858, “Revised WID on further NR coverage enhancements,” China Telecom, RAN#96.