3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209178

# e-Meeting, October 10th – 19th, 2022

Agenda Item:	8.2

Source:	Ericsson

Title:	Discussion on Group 2 search space configuration

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss a correction under "Maintenance on Supporting NR from 52.6GHz to 71 GHz." The corresponding draft CR is in [1].

# 2	Discussion

In RAN2#119-e, the following CR to 38.331 was agreed which refers back to 38.213 Section 10 for a description on the number of slots configured for multi-slot PDCCH monitoring. This CR is captured in v17.2.0 of 38.331 (currently in draft form).

monitoringSlotsWithinSlotGroup Indicates which slot(s) within a slot group are configured for multi-slot PDCCH monitoring. The first (leftmost, most significant) bit represents the first slot in the slot group, the second bit represents the second slot in the slot group, and so on. A bit set to '1' indicates that the corresponding slot is configured for multi-slot PDCCH monitoring (see TS 38.213 [13], clause 10). The number of slots for multi-slot PDCCH monitoring is configured according to clause 10 in TS 38.213 [13].

-	a bitmap, by monitoringSlotsWithinSlotGroup, that applies per group of slots and provides a PDCCH monitoring pattern indicating slots in a group of slots for PDCCH monitoring

-	a size of the group of slots is same as a size of monitoringSlotsWithinSlotGroup

-	for a Type1-PDCCH CSS set provided by ra-SearchSpace in dedicated RRC signaling, or for a Type3-PDCCH CSS set, or for a USS set, the PDCCH monitoring pattern indicates only consecutive slots in the group of slots for PDCCH monitoring and, at least for one combination Xs,Ys indicated by the UE as a capability, a number of the consecutive slots is not larger than Ys

Currently 38.213 Section 10 contains the following description of the parameter monitoringSlotsWithinSlotGroup

The yellow highlighted text captures the number of slots configured for multi-slot PDCCH monitoring for so-called Group (1) search spaces, i.e., Type1 with dedicated RRC, Type3 CSS, and USS. However, the number of slots configured for Group (2) search spaces is missing, i.e., for Type0/0A/2 CSS and Type1 CSS without dedicated RRC. The number of slots for Group (2) was agreed in RAN1#109-e as follows:

Agreement

For search space set configuration of multi-slot PDCCH monitoring, for Group (2) SSs

- For Type0/0A/2 CSS

  - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive

  - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L

- For Type1 CSS without dedicated RRC

  - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per group of L slots should be no larger than 1

Hence, our proposal is to update 38.213 Section 10 according to the above agreement. We provide a draft CR for this [1]. The text of the CR is copied into the Appendix of this document for convenience.

Adopt draft CR in [1]

# Conclusion

In this paper we propose the following:

Proposal 1	Adopt draft CR in [1]

# References

R1-2209179, “Draft CR on Group 2 search space configuration”, Ericsson, RAN1#110bis-e, October 2022.

# Appendix

10.1	UE procedure for determining physical downlink control channel assignment

*** Unchanged text omitted ***

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10 search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace:

-	a search space set index s, 0<s<40 , by searchSpaceId

-	an association between the search space set  s and a CORESET p by controlResourceSetId or by controlResourceSetId-v1610

-	a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset or by monitoringSlotPeriodicityAndOffset-r17

-	a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET for PDCCH monitoring within each slot where the UE monitors PDCCH, by monitoringSymbolsWithinSlot

-	a duration of Ts<ks indicating a number of slots that the search space set s exists by duration, or a number of slots in consecutive groups of slots where the search space set s can exist by duration-r17

-	a bitmap of length L, by monitoringSlotsWithinSlotGroup, that applies per group of slots and provides a PDCCH monitoring pattern indicating slots in a group of slots for PDCCH monitoring

-	a size of the group of slots is same as a size of monitoringSlotsWithinSlotGroup

-	for a Type1-PDCCH CSS set provided by ra-SearchSpace in dedicated RRC signaling, or for a Type3-PDCCH CSS set, or for a USS set, the PDCCH monitoring pattern indicates only consecutive slots in the group of slots for PDCCH monitoring and, at least for one combination Xs,Ys indicated by the UE as a capability, a number of the consecutive slots is not larger than Ys

-	for a Type0/0A/2-PDCCH CSS set, the PDCCH monitoring pattern can indicate consecutive or non-consecutive slots in the group of slots for PDCCH monitoring, and the number of slots can be up to L

-	for a Type1-PDCCH CSS set provided by ra-SearchSpace in PDCCH-ConfigCommon, the PDCCH monitoring pattern can indicate one slot in the group of slots for PDCCH monitoring

*** Unchanged text omitted ***