3GPP TSG RAN WG1#110-bis		R1-2209387

Online, August 10th – 19th, 2022

Source:	Panasonic

Title: 	Discussion on XR specific power saving techniques

Agenda Item:		9.10.1

Document for:	Discussion

# Introduction

The study of XR in NR started in Rel. 17 and continues in Rel. 18 following by a working item. The objectives of study item on XR in Rel. 18 were defined [1]. The enhancements will be studied to achieve improved power consumption and network capacity considering the characteristics of the XR traffics. In the RAN1#109-e meeting, it was agreed to study power saving techniques for CDRX and PDCCH monitoring operations [2]. In this contribution, we present several power saving techniques to support XR in NR more efficiently.

# Characteristics of XR traffics

The traffics for XR applications were studied extensively during the study item phase in Rel. 17 [3]. XR applications impose high data rates both in downlink and uplink. The high data rates are originated from video and pose/control streams. The video streams are considered as quasi-periodic traffics. The video streams typically generate video frames at the rate of 60/90/120 fps. Each frame could arrive over a jitter window. The jitter could span over a [-4,4] ms window or even a larger window. In addition, the frame sizes could be large and vary. In order to carry the video streams in the network, each frame should be divided into smaller IP packets. All the IP packets should be received successfully to reconstruct the frame out of them. For typical XR applications, such as AR/VR, the packet delay budget (PDB) could be 10 ms, while typical cloud gaming applications require PDB of 15 ms [2]. The pose/control streams generate less amount of traffic (e.g., 0.2 Mbps) but with more frequent packets, e.g., 250 fps and the PDB of 10 ms. Moreover, the XR applications would require supporting multi-stream traffics in uplink (UL) and downlink (DL).

# Discussions

The XR traffics cannot be supported efficiently with the current NR specifications.  For instance, the supported periodicities for DRX configurations do not match well with the video stream periodicities, i.e., 16.66667, 11.11111, and 8.33333 ms (which are corresponding to 60/90/120 fps). In addition, the duration of a jitter window, in which a frame would arrive, is comparable with the tolerable PDB, which brings scheduling complexity. Moreover, the support of simultaneous data streams in UL and DL imposes high transmissions for carrying the data and control information, increasing the power consumption. The reset of this section presents different solutions to address the mentioned issues.

## Support of non-integer periodicities

The video stream should be supported in both UL and DL. Due to varying packet sizes of the frames, a combination of configured and dynamic scheduling schemes would be reasonable. In this regard, the DRX configurations should be able to support periodicities corresponding to the video stream frame arrivals. Here, DRX configuration means the periodicity of the start timing of drxStartOffset.

Proposal 1: DRX should efficiently handle the non-integer periodicity transmissions, including the video stream frame periodicities like 16.66667, 11.11111, and 8.33333 ms.

The non-integer periodicities could be supported in a semi-static manner (e.g., through RRC configurations) or/and dynamic manners (e.g., through a DCI). Here we discuss some methods to handle non-integer periodicities.

Approach 1: Rounding the non-integer transmission instances

To support video streams, new non-integer periodicity values could be defined for DRX cycle. The beginning of the DRX ON duration is calculated according to the non-integer periodicity value and then matched to the next radio resources, e.g., slot, mini-slot, or symbol. The granularity of matching radio resource (slot, mini-slot, or symbol) can be part of DRX configurations.

Figure 1 illustrates a configured DRX with the periodicity of 16.6667 to support a video stream with 60 fps. The granularity of 1 ms is set for matching the non-integer value instances to the radio resources. The DRX ON duration begin at 16.6667*n, where n is a non-negative integer value. The ceiling scheme ensures matching the non-integer periodicity to the next earliest radio resource for each instance.

Figure 1. Matching non-integer DRX cycle to radio resources.

Approach 2: Alternating periodicities

The second approach for handling the non-integer periodicity transmissions is the use of alternating periodicities for a DRX configuration. A DRX can be configured with two integer periodicities associated with the number of occasions. Let’s assume that the DRX is configured with the integer periodicities of p1 and p2, which are associated with number of occasions of n1 and n2. The configuration applies the periodicity of p1 for n1 occasions and then it switches to the periodicity of p2 for n2 occasions.

Figure 2 illustrates a scenario that the UE is configured with a DRX to support a video stream with 60 fps. The configurations apply the periodicity of 17 ms for two occasions and then the periodicity of 16 ms for one occasion. Supporting the alternating periodicities with the number of occasions can be used to adjust the PDCCH monitoring occasions to the traffics with non-integer periodicities.

Figure 2. Supporting alternating periodicities.

Approach 3: A DCI to reconfigure DRX

In the third approach, a DCI can be sent to reconfigure some of DRX configuration parameters, such as drxStartOffset. Figure 3 illustrates a scenario that a UE is configured with a DRX cycle with the periodicity of 17 ms. The gNB sends a DCI to the UE after delivering the Frame 2 in order to decrease drxStartOffset by 1 ms. The DCI reconfiguration aligns the DRX ON duration to the jitter window of the frame arrivals.

Figure 3. A DCI for DRX reconfiguration.

Comparison of the approaches

In the following table, we compare the approaches discussed above. A dynamic DRX reconfiguration is always necessary to compensate the TA change caused by the movement of UE position and non-aligned oscillator clocks between the network and UE. In addition, always to use a DCI would increase the DCI overhead. Therefore, we see the combination between DCI based (approach 3) and semi-static based (approach 1, 2) would be reasonable. When the combination between DCI based and semi-static based are used, alternating periodicity (approach 2) would be the simplest among semi-static as it does not require the accurate alignment of non-integer periodicity.

Table 1. Approaches to efficiently handle the non-integer periodicity transmissions

| Approach | Pros | Cons |
| --- | --- | --- |
| 1. Rounding the non-integer transmission instances | - Only semi-static configurations. | - the accurate alignment of non-integer periodicity is required. |
| 3. Alternating periodicities | - Only semi-static configurations. - the alignment of non-integer periodicity is on the order of alternating periodicity. | - Depending on the combination of alternative periodicities, it may not fully be aligned with the target non-integer periodicity. This can be corrected by DCI based scheme. |
| 4. DCI to reconfigure DRX | - The adjustment of the periodicity other than caused by the fractional periodicity are possible like the accumulation of UE-gNB frequency error, the adjustment caused by the autonomous TA adjustment and so on. | - A DCI is required to adjust the periodicity. - The handling of miss/false detection of DCI. |

Proposal 2: The combination between DCI based (approach 3) and alternating periodicities (approach 2) should be taken.

## DRX and jitter handling

As described in section 2, each frame could arrive over a jitter window. The jitter could span over a [-4,4] ms window or even a larger window. The starting time of OnDuration can be set to the earliest time of the jitter window. If the UE is kept wake-up over the actual frame arrival, the power consumption is increased by the PDCCH reception. In the last RAN1 meeting, following was concluded.

Conclusion

RAN1 does not assume instantaneous jitter value for a frame is predictable for Rel-18 XR SI power saving study before further input is provided by SA.

Therefore, we think following would be the most reasonable operation for DL.

- OnDuration is set as the earliest time of the jitter variation. The periodicity of OnDuration is based on non-integer periodicities.

- If there is immediate data in DL buffer at gNB, PDCCH skipping and/or search space set switching are applied for the space PDCCH reception until the data is available at gNB buffer.

- When data is available at gNB buffer, PDCCH skipping and/or search space set switching are adjusted to frequent PDCCH monitoring.

Based on the above operation, Release 17 function of UE Power Saving Enhancements can sufficiently cover the necessary operation for jitter handling.

Observation 1: The combination of PDCCH skipping/search space set switching and non-integer periodicity based OnDuration can manage the jitter handling for DL traffic.

# Conclusion

In this contribution, we provide our view on techniques for supporting XR applications efficiently. We made following proposals and observation.

Proposal 1: DRX should efficiently handle the non-integer periodicity transmissions, including the video stream frame periodicities like 16.66667, 11.11111, and 8.33333 ms.

Proposal 2: The combination between DCI based (approach 3) and alternating periodicities (approach 2) should be taken.

Observation 1: The combination of PDCCH skipping/search space set switching and non-integer periodicity based OnDuration can manage the jitter handling for DL traffic.

# Reference

[1] 	RP-213558, “Study on XR Enhancements for NR,” RAN#94-e.

[2] 	R1-2205412, “Final Moderator Summary on XR specific power saving techniques,” RAN#109-e.

[3] 	TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17),” V17.0.0.

[4]	TS 38.321, “Medium Access Control (MAC) protocol specification”