3GPP TSG RAN WG1 #110bis			R1-2209700

e-Meeting, October 10th – 19th, 2022

Agenda Item:		8.3

Source:		Samsung

Title:		Discussion on the timeline of determining SPS HARQ-ACK deferral

Document for:		Discussion and decision

Introduction

The following agreements were made in RAN1#106e-bis regarding SPS HARQ-ACK deferral.

Conclusion For SPS HARQ-ACK deferral, the operation in the ‘initial’ slot is further clarified as: - The UE performs first the (Rel-16) UCI multiplexing operation. If after the UCI multiplexing operation into a PUCCH or PUSCH if any, and if the UE would be transmitting SPS HARQ-ACK using the PUCCH SPS-PUCCH-AN-List-r16 or n1PUCCH-AN which is not valid, the SPS HARQ-ACK configured for deferral is deferred. Agreement The RAN1#106-e agreement on the target slot definition is updated as follows (in RED): Agreement (from RAN1#106-e) For SPS HARQ-ACK deferral, the target PUCCH slot is defined as the next PUCCH slot, where after performing the (Rel-16) UCI multiplexing operation into a PUCCH or PUSCH if any, the UE would be either (i) transmitting HARQ-ACK using a PUCCH/PUSCH other than the PUCCH determined from PUCCH SPS-PUCCH-AN-List-r16 or n1PUCCH-AN or (ii) would be transmitting HARQ-ACK using a PUCCH resource configured in PUCCH SPS-PUCCH-AN-List-r16 or n1PUCCH-AN being regarded as valid. sps-PUCCH-AN-List-r16 or n1PUCCH-AN PUCCH resource is regarded as valid, or a PUCCH resource (from PUCCH-ResourceSet, i.e. DG PDSCH HARQ multiplexed) is dynamically indicated - The target PUCCH slot determination is based on the total HARQ-ACK payload size including deferred SPS HARQ-ACK information and non-deferred HARQ-ACK information (if any) of a candidate target PUCCH slot - The final PUCCH resource selection in the target PUCCH slot in terms of PUCCH resource set and PUCCH resource ID follows the Rel-16 procedures.. Conclusion: If the UE is not configured with Rel-17 Intra-UE multiplexing, SPS HARQ for deferral of different PHY priorities can be separately deferred with the target PUCCHs separately determinated according to their respective PHY priorities. · FFS on the PHY priority handling for SPS HARQ deferral if the UE configured with Rel-17 Intra-UE multiplexing

When determining the SPS HARQ-ACK deferral after resolving the overlapping between the PUCCH with SPS HARQ-ACK and other PUCCH/PUSCH, the timeline issue was not considered in the previous discussion. This contribution discusses the timeline issue for determining SPS HARQ-ACK deferral.

Discussion

For UCI multiplexing in a PUCCH or PUSCH, the UCI multiplexing timeline is ensured by gNB. For a deferred SPS HARQ-ACK multiplexing in a PUCCH or PUSCH in a target slot, the multiplexing timeline should also be satisfied.

For an activated SPS PDSCH, a PUCCH with the SPS HARQ-ACK only can be determined semi-statically and whether the PUCCH with the SPS HARQ-ACK only collides with semi-static DL symbols can be determined semi-statically as well. If there is no overlapping PUCCH or PUSCH, the timeline for multiplexing the deferred SPS HARQ-ACK in the target slot can always be satisfied. However, when considering the overlapping PUCCH or PUSCH, the timeline becomes an issue.

Consider a simple example in Figure 1, whether the SPS HARQ-ACK is deferred depends on the status of HP SR. For a positive SR, the SPS HARQ-ACK is deferred. However, for a negative SR, the SPS HARQ-ACK is not deferred. If UE determines the SPS HARQ-ACK deferral early, for example, at the start of slot 0, the SPS HARQ-ACK should be deferred since the UL traffic has not arrived yet and the SR is negative. However, if the UE determines the SPS HARQ-ACK deferral late, for example, at the end of slot 0 the SPS HARQ-ACK should not be deferred because the SR becomes positive. For deferred SPS HARQ-ACK, it takes time for a UE to generate the HARQ-ACK codebook and encode the UCI bits when multiplexing the UCI in a PUCCH, for example, Tproc is the processing time as shown in Figure 1. If the determination is performed after T0, there will be no enough time to ensure the multiplexing procedure.

Figure 1

The timeline for multiplexing the deferred SPS HARQ-ACK in a PUSCH should also be considered as well. Consider another example in Figure 2, for multiplexing in a PUSCH, a UE should first determine the HARQ-ACK codebook and then determines the PUCCH for carrying the HARQ-ACK and finally the UE determines whether the HARQ-ACK is multiplexed in an overlapping PUSCH.

Figure 2

To address the issue, the time when a UE determines the SPS HARQ-ACK deferral should be specified. A simple solution could be that the UE determines the SPS HARQ-ACK deferral N symbols before the end of the current PUCCH slot. The value of N can reuse the values for PUSCH preparation time N2 as defined in TS 38.214.

Proposal: For SPS HARQ-ACK deferral, a UE determines the SPS HARQ-ACK deferral N2 symbols as defined in TS 38.214 before the end of the slot of the PUCCH with SPS HARQ-ACK.

Conclusion

This contribution discusses the timeline issue for determining SPS HARQ-ACK deferral. The proposal is summarized below.

Proposal: For SPS HARQ-ACK deferral, a UE determines the SPS HARQ-ACK deferral N2 symbols as defined in TS 38.214 before the end of the slot of the PUCCH with SPS HARQ-ACK.