3GPP TSG RAN WG1 #110bis-e			R1-2209725

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.2.3.2

Source:	Samsung

Title:	Representative sub use cases for beam management

Document for:	Discussion and Decision

- Introduction

In RAN#94-e, Rel-18 new study item on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” is endorsed. The objective of the study item is as follows.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we will provide our view on AI/ML for beam management, including the discussion on the representative sub use cases for beam management and the corresponding specification impact.

- General discussion

In NR, FR2 beam management is challenging. In order to identify the optimal beam pair between a gNB and a UE, a large number of beam measurements and reports are required. This results in significant overhead and latency. The situation is even worse when UE mobility is considered. In a high-speed scenario, due to the latency of beam management procedure, it is almost impossible to maintain a desired narrow beam pair between a gNB and a UE.

In order to address the latency and overhead issue above, AI/ML-based solution has drawn a great deal of interest. Excellent in prediction and compression, AI/ML is expected to be a promising tool for beam prediction and beam measurement feedback compression. In the following, several AI/ML representative sub use cases for beam prediction and beam measurement feedback compression are discussed.

- Representative sub use cases for beam prediction in spatial domain

Agreement For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations - BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams - BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams - FFS: details of BM-Case1 and BM-Case2 - FFS: other sub use cases Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range Agreement For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side Conclusion: For the sub use case BM-Case1, consider the following alternatives for further study: - Alt.1: Set B is a subset of Set A o FFS: the number of beams in Set A and B o FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …) - Alt.2: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams) o FFS: the number of beams in Set A and B o FFS: QCL relation between beams in Set A and beams in Set B o FFS: construction of Set B (e.g., regular pre-defined codebook, codebook other than regular pre-defined one) - Note1: Set A is for DL beam prediction and Set B is for DL beam measurement. - Note2: The narrow and wide beam terminology is for SI discussion only and have no specification impact - Note3: The codebook constructions of Set A and Set B can be clarified by the companies. Conclusion Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input: - Alt.1: Only L1-RSRP measurement based on Set B - Alt.2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companions in the discussion: Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: CIR based on Set B - Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives Note2: All the inputs are “nominal” and only for discussion purpose.

As mentioned in SID [1], the representative sub use cases for beam prediction includes the beam prediction in spatial domain and beam prediction in time domain. In this section, the representative sub use cases for beam prediction in spatial domain is discussed.

According to the discussion in previous RAN1 meeting, it is common understanding that one-sided AI model is considered for the use case of beam management. In the following, AI/ML inference at NW side and AI/ML inference at UE side are discussed, respectively.

3.1 AI/ML inference at NW side

In the following, the sub use cases of AI/ML inference at NW side are discussed.

BM-Case1

According to the discussion in last RAN1 meeting, one sub case, BM-Case1, is defined for DL beam predication in spatial-domain. In this case, two beam sets, Set A and Set B, are considered, where Set A is the beam set for prediction and Set B is the beam set for measurement.

In terms of AI/ML inference at NW, due to the payload restriction of CSI report, not all measurement results for the beams in Set B is visible by gNB. For better discussion the cases for NW side AI/ML inference, Set C can be defined in addition to Set A and Set B. Here, Set C consists of the beams reported by UE from Set B. For example, according to the current NR beam management procedure, UE reports the best N beams that corresponds to the best L1-RSRP out of a set of measurement beams. Hence, in this example, Set C is the N beams reported by UE.

Proposal 1: For the sub use case BM-Case1, consider to define Set C for AI/ML inference at NW side.

- Set C consists of the beams reported by UE from Set B.

For BM-Case1, two alternatives are provided for the relationship between Set A and Set B:

- Alt.1. Set B is a subset of Set A;

- Alt.2. Set A and Set B are different (e.g., Set A consists of narrow beams and Set B consists of wide beams).

With the consideration of Alt.1, a typical procedure for BM-Case1 is as follows:

- Step#1. gNB transmits RSs corresponding to the beams from Set B.

  - Here, Set B is a subset of Set A. Typically, the beams in Set B can be obtained from spatial domain down-sampling of Set A.

- Step#2. UE measures the RSs and provides corresponding L1-RSRP report.

  - UE measures Set B, and provides the measurement results of Set C to gNB. In current spec, Set C is determined according to the L1-RSRP measured for Set B.

- Step#3. gNB predicts the best beam(s) in set A on the basis of the L1-RSRP report.

  - For the final step, gNB obtains Set C and the corresponding L1-RSRP and use them as the input of an AI model. Correspondingly, for example, the L1-RSRP corresponding to Set A can be provided by the AI model as output. With those predicted L1-RSRP results of Set A, gNB can choose the one(s) that correspond(s) to the highest L1-RSRP(s) as the predicted best beam(s).

One benefit of this case is overhead reduction. As mentioned above, gNB can transmit less RSs in comparison with legacy. Also, from the perspective of UE, less RSs are required to be measured. Another benefit of this case is latency reduction, especially the time period for gNB Tx beam sweeping can be reduced since only a subset of beams are needed for beam sweeping.

For this case, there are potentially two specification impacts. One potential specification impact is the enhancement to the existing beam measurement/reporting mechanism to assist gNB prediction. For example, the quantization step size of L1-RSRP may not be adequate for proper beam prediction at gNB-side. The other potential specification impact is from the perspective of UE-side. For example, a gNB may provide assistance information for a UE to identify the predicted gNB Tx beam (for the better refinement of UE Rx beam).

With the consideration of Alt.2, a typical procedure for BM-Case1 is as follows:

- Step#1. gNB transmits RSs according to the beams from Set B.

  - Here, Set B is different from Set A. For example, Set B is for wide beam and Set A is for narrow beam. Typically, Set B consists of SSBs which corresponds to P1 procedure and Set B consists of CSI-RSs which corresponds to P2 procedure.

- Step#2. UE measures the RSs and provides corresponding L1-RSRP report.

  - UE measures Set B, and provides the measurement results of Set C to gNB. In current spec, Set C is determined according to the L1-RSRP measured for Set B.

- Step#3. gNB predicts the best beam(s) in set A on the basis of the L1-RSRP report.

  - #1. For the final step, gNB obtains the best beam from A. gNB obtains Set C and the corresponding L1-RSRP and use them as the input of an AI model. Correspondingly, for example, the L1-RSRP of Set A can be provided by the AI model as output. With those predicted L1-RSRP results of Set A, gNB can choose the one(s) that correspond(s) to the highest L1-RSRP(s) as the predicted best beam.

  - #2. For the final step, gNB obtains a set of predicted beams containing the best beam. gNB obtains Set C and the corresponding L1-RSRP and use them as the input of an AI model. Correspondingly, for example, the predicted beam indexes can be provided by the AI model as output. With predicted beam indexes, gNB can further perform P2 procedure with higher accuracy.

One benefit of this case is to reduce the measurement overhead of P2 procedure. With the measurement of wide beams, one or a set of narrow beams can be predicted without any measurement. Also, from the perspective of UE, less RSs are required to be measured. Another benefit of this case is latency reduction, especially the time period for P2 procedure can be reduced.

For Alt-2, similar aspects of specification impact are expected as for Alt-1.

Proposal 2: For BM-Case1, further study the specification impacts for AI/ML inference at NW side considering the following aspects.

- Enhancement on L1 beam report mechanism

- Assistance information for beam prediction

Case-1b

This case is similar to BM-Case1 but is for UL beam prediction. The procedure of this case is as follows:

- Step#1. UE transmits SRSs on a subset beam that corresponds to a full beam set.

  - For example, a UE intends to transmit 8 beams for different directions. Conventional method is to transmit 8 corresponding SRSs for those 8 beams for the purpose of UE beam sweeping. However, for this case, the UE only transmits SRSs on a subset of those beams (e.g., 4 SRSs for 4 beams).

- Step#2. gNB measures the L1-RSRP of the SRSs.

  - For this step, the gNB measures the 4 SRSs in terms of L1-RSRP.

- Step#3. gNB predicts the best beams within the full beam set.

  - For the final step, the gNB collects the SRS indexes and the corresponding measurement result of those four RS and use them as the input of an AI model. Correspondingly, for example, the L1-RSRP of the 8 beams can be provided by the AI model as output. With those predicted RSRP results of full beam set, gNB choose the one(s) that correspond(s) to the highest L1-RSRP(s) as the predict best beam(s) for UE.

One benefit of this case is overhead reduction. Similar as BM-Case1, a UE can transmit less RSs in comparison with the legacy. Also, from the perspective of a gNB, less RSs are required to be measured. Another benefit of this case is latency reduction, especially the time period for UE Tx beam sweeping can be reduced since only a subset of beams are needed for beam sweeping.

For this case, one potential specification impact is that a UE is required to provide additional information to assist gNB prediction. For example, the gNB does not aware of the full Tx beam set from UE. Hence, at least the corresponding information (e.g., the spatial information of UE Tx beam) is needed for gNB to perform the beam prediction.

3.2 AI/ML inference at UE side

In the following, the sub use cases of UE-side prediction are discussed.

BM-Case1

As discussed in the previous section, Set A and Set B are defined for BM-Case1. A typical procedure for this case is as follows:

- Step#1. gNB transmits RSs corresponding to the beams from Set B.

  - #1. Set B is a subset of Set A. Typically, the beams in Set B can be obtained from spatial domain down-sampling of Set A.

  - #2. Set B is different from Set A. For example, Set B is for wide beam and Set A is for narrow beam. Typically, Set B consists of SSBs which corresponds to P1 procedure and Set B consists of CSI-RSs which corresponds to P2 procedure.

- Step#2. UE measures the RSs.

  - UE measures Set B.

- Step#3. UE reports the best predicted beams within Set A.

  - For the final step, UE predicts the best beam in Set A according to the measurement results from Set B and assistance information. Here, assistance information should teach the association between Set A and Set B; otherwise, AI/ML inference at UE side is not possible.

The benefit of this case is similar to BM-Case1. Apart from that, since both RS measurement and beam prediction are at UE-side, measurement result of Set B instead of Set C can be used for beam prediction. This means that more measurement information can be exploited. However, in comparison with BM-Case1, this case requires AI model inference at UE-side. This may increase the UE complexity.

For this case, one potential specification impact is that the gNB needs to provide the assistance information for UE beam prediction. A straightforward way is to use Tx structure parameters (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction) as assistance information. However, this type of assistance information may disclose gNB hardware which is very unlikely to be acceptable by network vendor. Hence, the virtualization of the Tx structure parameter is needed. Another potential specification impact is L1 beam report mechanism. For example, the method to indicate Set A for L1 beam report.

Proposal 3: For BM-Case1, further study the specification impacts for AI/ML inference at UE side considering the following aspects.

- Assistance information for AI/ML inference at UE side

- Enhancement on L1 beam report mechanism

Case-2b

This is another case for DL beam prediction. The procedure of this case is as follows:

- Step#1. gNB transmits SSBs or CSI-RSs with repetition on.

  - For example, a UE consists of a full Rx beam set with 8 beams. Conventional method is to receive 8 SSB bursts or CSI-RSs with repetition ‘on’ for UE beam sweeping. However, for this case, the UE can measure less than 8 SSB bursts or the gNB can transmit less than CSI-RSs (e.g., 4 SSB bursts or CSI-RSs with repetition on).

- Step#2. UE performs RX beam prediction within the full UE Rx beam set.

  - For this step, UE measures the 4 SSB bursts or CSI-RSs with repetition on and perform Rx beam prediction for the purpose of Rx beam determination/refinement. Here, the measurement result of the SSBs or CSI-RSs can be used as the input for an AI model and the predicted L1-RSRP for the full Rx beam set can be regarded as output. By using this method, the best Rx beam within the full UE Rx beam set can be determined.

- [Step#3. UE perform UL transmission using the same beam as the predicted Rx beam]

  - For this step, gNB can further perform UL transmission using the predicted Rx beam.

One benefit of this case is overhead reduction. As mentioned above, the gNB can transmit less RSs in comparison with legacy. Also, from the perspective of UE, less RSs are required to be measured. Another benefit of this case is latency reduction, especially the time period for UE Rx beam determination or refinement can be reduced.

For this case, one potential specification impact is UE capability, where an advanced UE capability for Rx beam sweeping may be needed. Another potential specification impact is that UE may provide additional information to assist gNB for the reception of the UL transmission.

- Representative sub use cases for beam prediction in time domain

Agreement Regarding the sub use case BM-Case2, the measurement results of K (K>=1) latest measurement instances are used for AI/ML model input: - The value of K is up to companies Agreement Regarding the sub use case BM-Case2, AI/ML model output should be F predictions for F future time instances, where each prediction is for each time instance. - At least F = 1 - The other value(s) of F is up to companies Agreement For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side Conclusion For the sub use case BM-Case2, further study the following alternatives with potential down-selection: - Alt.1: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams) - FFS: QCL relation between beams in Set A and beams in Set B - Alt.2: Set B is a subset of Set A (Set A and Set B are not the same) - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …) - Alt.3: Set A and Set B are the same - Note1: Predicted beam(s) are selected from Set A and measured beams used as input are selected from Set B. - Note2: It is up to companies to provide other alternative(s) - Note3: The narrow and wide beam terminology is for SI discussion only and have no specification impact Conclusion Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance): - Alt.1: Only L1-RSRP measurement based on Set B - Alt 2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam pointing angles beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives Note2: All the inputs are “nominal” and only for discussion purpose.

In this section, both time domain beam prediction at NW side and time domain beam prediction at UE side are discussed, respectively.

4.1 AI/ML inference at NW side

In the following, the sub use case of gNB-side prediction is discussed.

BM-Case2

According to the discussion in last RAN1 meeting, one sub case, BM-Case2, is defined for DL temporal beam predication. In this case, two beam sets, Set A and Set B, are considered, where Set A is the beam set for prediction and Set B is the beam set for measurement.

In terms of AI/ML inference at NW, due to the payload restriction of CSI report, not all measurement results for the beams in Set B is visible by gNB. For better discussion the cases for NW side AI/ML inference, Set C can be defined in addition to Set A and Set B. Here, Set C consists of the beams reported by UE from Set B. For example, according to the current NR beam management procedure, UE reports the best N beams that corresponds to the best L1-RSRP out of a set of measurement beams. Hence, in this example, Set C is the N beams reported by UE.

Proposal 4: For the sub use case BM-Case2, consider to define Set C for AI/ML inference at NW side.

- Set C consists of the beams reported by UE from Set B.

For BM-Case2, two alternatives are provided for the relationship between Set A and Set B:

- Alt.1. Set B is a subset of Set A;

- Alt.2. Set A and Set B are different (e.g., Set A consists of narrow beams and Set B consists of wide beams).

- Alt.3: Set A and Set B are the same

A typical procedure of this case is as follows:

- Step#1. gNB transmits RSs corresponding to the beams from Set B.

  - #1. Set B is a subset of Set A. Typically, the beams in Set B can be obtained from spatial domain down-sampling of Set A.

  - #2. Set B is different from Set A. For example, Set B is for wide beam and Set A is for narrow beam. Typically, Set B consists of SSBs which corresponds to P1 procedure and Set B consists of CSI-RSs which corresponds to P2 procedure.

  - #3. Set B is the same as Set A. This can be regarded as purely time domain prediction (without any spatial domain predication).

- Step#2. UE measures the RS and provides corresponding L1-RSRP report for each measurement time instance for the RS.

  - UE measures Set B and provides the measurement results of Set C to gNB. In current spec, Set C is determined according to the L1-RSRP measured for Set B.

- Step#3. gNB predicts the future best beam(s) in set A to the historical L1-RSRP reporting.

  - For the final step, gNB obtains K latest measurement results of Set C and use them as the input of an AI model. Correspondingly, for example, the L1-RSRP of F future instances of the beams from Set A can be provided by the AI model as output. With those predicted L1-RSRP results of Set A, gNB can choose the one(s) that correspond(s) to the highest L1-RSRP(s) for each of the F instance as the predicted best beam(s).

This case is beneficial to the link adaptation of gNB. For example, with the knowledge of the future L1-RSRP, gNB can perform beam switch or MCS adjustment in advance to avoid potential link failure or retransmission. Another benefit of this case is latency reduction, especially the time period for gNB Tx beam sweeping and/or indication/activation of the future beam can be reduced from the beam switch in advance.

One potential specification impact is the enhancement to the existing beam measurement/reporting mechanism to assist gNB prediction. This is because the existing beam reporting mechanism may not be accurate enough for gNB-side beam prediction. For example, the quantization step size of L1-RSRP report can be improved similar as mentioned in BM-Case1. Another potential specification impact is the enhancement to the existing TCI indication/activation to reduce the time period for future TX beam sweeping. For example, gNB can indicate/activate the future TCI of the CSI-RS or the RS for receiving the scheduled PDSCH from pre-configured future TCIs that are output from the AI model.

Proposal 5: For BM-Case2, further study the specification impacts for AI/ML inference at NW side considering the following aspects.

- Enhancement on L1 beam report mechanism

4.2 AI/ML inference at UE side

In the following, the sub use case of UE-side prediction is discussed.

BM-Case2

As discussed in the previous section, Set A and Set B are defined for BM-Case2. A typical procedure for this case is as follows:

- Step#1. gNB transmits RSs corresponding to the beams from Set B.

  - #1. Set B is a subset of Set A. Typically, the beams in Set B can be obtained from spatial domain down-sampling of Set A.

  - #2. Set B is different from Set A. For example, Set B is for wide beam and Set A is for narrow beam. Typically, Set B consists of SSBs which corresponds to P1 procedure and Set B consists of CSI-RSs which corresponds to P2 procedure.

  - #3. Set B is the same as Set A. This can be regarded as purely time domain prediction (without any spatial domain predication).

- Step#2. UE measures the RSs corresponding to the beams from Set B and provides the predicted/future L1-RSRP of the RSs.

  - For this step, the UE collects K latest measurement results of Set B and use them as the input of an AI model. Accordingly, the L1-RSRP for the RS in a future F time instances can be provided by the AI model as output.

The benefit of this case is similar to the gNB-side case. Apart from that, since both RS measurement and beam prediction are at UE-side, there is no quantization penalty in terms of RS measurement result. However, in comparison with gNB-side case, this may increase the UE complexity but offload gNB complexity for multiple UEs’ beam prediction

For this case, one potential specification impact is the definition of future L1-RSRP for a RS measurement. Another potential specification impact is the UE-side case/events that can leverage the predicted/future L1-RSRP.

Proposal 6: For BM-Case2, further study the specification impacts for AI/ML inference at UE side considering the following aspects.

- Enhancement on L1 beam report mechanism

- UE-side case/events that can leverage the predicted/future L1-RSRP

- Considerations on PHY layer aspects for beam prediction

Agreement Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output: - Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure) - E.g., N predicted beams can be the top-N predicted beams - Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - FFS: details of Beam angle(s) - FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s)) - Note1: It is up to companies to provide other alternative(s) - Note2: Beam ID is only used for discussion purpose - Note3: All the outputs are “nominal” and only for discussion purpose - Note4: Values of N is up to each company. - Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side. - Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output Agreement For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) - Note1: DL Rx beam prediction may or may not have spec impact Agreement For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded Agreement Regarding the model monitoring for BM-Case1 and BM-Case2, to investigate specification impacts from the following aspects - Performance metric(s) - Benchmark/reference for the performance comparison - Signaling/configuration/measurement/report for model monitoring, e.g., signaling aspects related to assistance information (if supported), Reference signals - Other aspect(s) is not precluded Agreement In order to facilitate the AI/ML model inference, study the following aspects as a starting point: - Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting - Enhanced or new signaling for measurement configuration/triggering - Signaling of assistance information (if applicable) - Other aspect(s) is not precluded

In this section, PHY layer aspects in common for both BM-Case1 and BM-Case2 are discussed.

5.1 AI/ML output

According to the discussion in previous RAN1 meeting, three alternatives for the AI/ML output of BM-Case1 and BM-Case2 were agreed.

- Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

- Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information

- Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

Among the three alternatives, Alt 1 is our preference. This is because, the model output in Alt 1 is aligned with the measurement quantity in the existing beam management framework. Also, the model output in Alt 1 can be beneficial for gNB side AI/ML inference and UE side AI/ML inference. In comparison, for Alt 2, the necessity of adding extra information/output other than the output in Alt 1 is questionable. If this information/output is expected to be exchanged over air interface, the interpretation for the information/output from the other side may require huge specification impact. Also, the introduction of the information/output prohibits legacy UEs/gNBs to be involved in the procedure of data collection. Hence, the introduction of the information/output is acceptable only if its benefit is well justified by simulation results. For Alt 3, beam angle is used as output instead of beam ID. Similar arguments for Alt-2 still holds for Alt-3. Besides, both the beam angle and beam ID can be regarded as a representative of a beam. If the number of training data is enough, the association between beams can be obtained by AI/ML model. In this sense, beam ID (Alt-1) is more suitable as AI/ML output since it is easier to exchange over air interface and consistent with legacy design.

Proposal 7: For AI/ML output for beam prediction, Alt 1 (beam ID + L1-RSRP) is preferred.

Also, according to the discussion in previous RAN1 meeting, three alternatives for the predicted beams of BM-Case1 and BM-Case2 were agreed.

- Alt.1: DL Tx beam prediction

- Alt.2: DL Rx beam prediction

- Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam)

Among the three alternatives, Alt 1 is our preference since it can be considered for AI/ML model inference either at gNB-side or UE-side. For Alt 2, it can be considered for AI/ML model inference at UE-side. In the current design of NR system, pure Rx beam selection or prediction can also be UE implementation. This alternative can be deprioritized in this study in comparison with other alternatives. For Alt3, it can be considered for AI/ML model inference at UE-side since it includes DL Rx beam prediction similar to Alt 2. For Alt.3, training or inference of the AI/ML model without the additional Tx information (e.g., Tx beam angle) from gNB would be difficult. This may somehow mandate gNB disclose of its beam-book related information, which is not preferable. Therefore, we have the following proposal.

Proposal 8: For predicted beams, Alt 1 (DL Tx beam prediction) is preferred.

5.2 Data collection

For AI/ML-based beam management, it is common understanding in the group that one-sided AI/ML model is used for beam prediction. In comparison with two-sided model, it is expected that one-sided model is easier subject to training and/or fine-tuning. This means that it is feasible to perform model training/refinement based on the data collected over air interface. In order to facilitate the procedure of data collection, the following aspects are discussed.

Content/type of the collected data

The first aspect is content/type of the collected data. For the prediction of DL beam, data collection can only be performed at UE side. The key to data collection is that UE shall obtain the quasi-ground truth of RS measurement for model training or refinement. This is highly dependent on the capability of a UE, i.e., what kind of RS feature a UE can measure. For example, if gNB would collect data from legacy UEs for model training/refinement, the only feature of a RS can be measured is L1-RSRP or L1-SINR. If some advance features of RS measurement are needed for AI/ML model input, the data collection for the model is only possible by the advanced UEs (e.g., the UEs that can obtain beam angle/width according RS measurement).

Apart from the RS measurement, the label(s) to the RS measurement is also needed for data collection. The label(s) is dependent on the required input of a particular AI/ML model. One of the meaningful labels could be the time stamp/information that corresponds to a measurement result.

Signaling/configuration/measurement/report for data collection

Another aspect is the procedure/signaling related to the data collection. For this aspect, AI/ML model inference at gNB-side and AI/ML model inference at UE-side are discussed, respectively.

For the case that AI/ML model inference at gNB-side, gNB should provide a measurement and/or report configuration to UE for data collection. The report configuration may include the configuration of RS and measurement quantity and/or other related information. As a baseline, legacy mechanism for beam measurement and report can be reused as much as possible. However, legacy mechanism has some drawbacks. The drawbacks and the corresponding solutions are discussed in the following bullets:

- One drawback of legacy mechanism is that, if the number of beams for measurement in a group is large, only the beams with top-N (N<5) L1-RSRP result can be reported to gNB. This means that not all measurement results at UE side can be conveyed to gNB, which impacts the flexibility of data collection. To address that, a more flexible RS measurement and report mechanism is desired. For example, a measurement and report mechanism that can provide full measurement results (e.g., for set A) without UE-side filtering.

- Another drawback of legacy mechanism is that it is restrictive to provide time stamp for RS measurement result. For example, the time stamp of a L1-RSRP measurement can only be implicitly derived by the time of the report. This is very restrictive since it mandates the coupling between UE report and time stamp. In the case of data collection, frequent UE report is unnecessary since those measurement results will not be easily outdated. To address that, one potential method is to report RS measurement results in addition with the corresponding explicit/implicit time stamp. By this way, one report can carry more than one measurement results corresponding to a single RS.

Another issue for further study is how to handle/buffer the collected data before the report to gNB. From the perspective of UE, it is not preferred to buffer the collected data with huge size.

Proposal 9. For the data collection for AI/ML model training, in the case that AI/ML model is at gNB-side, the following aspects can be further study:

- Potential enhancement for the measurement and report for data collection

- The handling/buffering of the collected data

For the case that AI/ML model inference at UE-side, UE is intended to measure the RS transmitted by gNB. As a baseline, legacy mechanism for RS measurement can be reused as much as possible. On the top of that, to accommodate the data format required by the UE-side AI/ML model, a report is necessary from UE to provide its preference of data collection towards gNB. The preference of data collection could be intended/preferred RS transmission for UE measurement and intended/preferred time domain pattern of the RS transmission. After the reception of the preference of data collection from UE, gNB can provide RS measurement configuration towards the UE based on the preferred RSs and the preferred time domain pattern.

Proposal 10. For the data collection for AI/ML model training, in the case that AI/ML model is at UE-side, the following aspects can be further study:

- UE report for the preference of data collection, e.g., intended/preferred RS transmission for UE measurement, intended/preferred time domain pattern of the RS transmission

- RS measurement configuration for data collection

5.3 Model monitoring

For AI/ML-based beam management, model monitoring is an important way to guarantee the performance of beam prediction. The key idea of model monitoring is to supervise the performance gap between AI/ML model-based method and measurement-based baseline method. In the following, different aspects for the performance monitoring of AI/ML model are discussed.

Performance metric(s)

The performance metric of AI/ML model has been widely discussion in 9.2.3.1. Specifically, different options for beam prediction accuracy related KPIs were agreed. To avoid duplicated discussion, those options can be considered for the performance metric of model monitoring.

Proposal 11. For the performance metric(s) of AI/ML model monitoring, beam prediction accuracy related KPIs agreed in 9.2.3.1 can be considered.

Benchmark/reference for the performance comparison

Similarly, the benchmark/reference for performance comparison for AI/ML based method is also discussed in in 9.2.3.1. Specifically, two options were agreed for spatial-domain beam prediction and temporal beam prediction, respectively. One option is for upper-bound benchmark and the other option is for lower-bound benchmark. To avoid duplicated discussion, those options can be considered as starting point for benchmark/reference for the performance comparison.

Proposal 12. For benchmark/reference for the performance comparison of the AI/ML model monitoring, baseline performance options for spatial-domain beam prediction and temporal beam prediction agreed in 9.2.3.1 can be considered.

Signaling/configuration/measurement/report for model monitoring

With the consideration of performance metric and benchmark/reference as above, the general procedure of model monitoring is as follows. For example, if the upper-bound benchmark is used and the performance gap between AI/ML model-based method and the benchmark is acceptable (e.g., within a threshold), AI/ML model-based method can be used; otherwise, AI/ML model-based method is disabled and measurement-based method is used. For another example, if the lower-bound benchmark is used and the performance gap between AI/ML model-based method and the benchmark is good enough (e.g., greater than a threshold), AI/ML model-based method can be used; otherwise, AI/ML model-based method is disabled and measurement-based method is used.

In the following, the procedure/signaling related to model monitoring is discussed based on the assumption of AI/ML model inference at gNB-side and the assumption of AI/ML model inference at UE-side, respectively.

For the case that AI/ML model inference at gNB-side, gNB should provide indication to UE for model monitoring. For the case that upper-bound benchmark is selected for model monitoring, the requirement from gNB for model monitoring is very similar to the requirement for data collection. Similar measurement and report enhancement for data collection can be considered for model monitoring.

Proposal 13. For the AI/ML model monitoring, in the case that AI/ML model is at gNB-side, the following aspect can be further study:

- Potential enhancement for the measurement and report for model monitoring

For the case that AI/ML model inference at UE-side, UE is intended to measure the RS for benchmark. As a baseline, legacy mechanism for RS measurement can be reused as much as possible. It is notable that UE should report the result of model monitoring. That is, UE should generate a report for the performance/validation of its AI/ML model. With this knowledge, gNB can make the decision on whether AI/ML model can be used for beam prediction or not.

Proposal 14. For the AI/ML model monitoring, in the case that AI/ML model is at UE-side, the following aspects can be further study:

- UE report for the performance/validation of AI/ML model

- Conclusion

The observations and proposals made in this contribution are summarized below.

Proposal 1: For the sub use case BM-Case1, consider to define Set C for AI/ML inference at NW side.

- Set C consists of the beams reported by UE from Set B.

Proposal 2: For BM-Case1, further study the specification impacts for AI/ML inference at NW side considering the following aspects.

- Enhancement on L1 beam report mechanism

- Assistance information for beam prediction

Proposal 3: For BM-Case1, further study the specification impacts for AI/ML inference at UE side considering the following aspects.

- Assistance information for AI/ML inference at UE side

- Enhancement on L1 beam report mechanism

Proposal 4: For the sub use case BM-Case2, consider to define Set C for AI/ML inference at NW side.

- Set C consists of the beams reported by UE from Set B.

Proposal 5: For BM-Case2, further study the specification impacts for AI/ML inference at NW side considering the following aspects.

- Enhancement on L1 beam report mechanism

Proposal 6: For BM-Case2, further study the specification impacts for AI/ML inference at UE side considering the following aspects.

- Enhancement on L1 beam report mechanism

- UE-side case/events that can leverage the predicted/future L1-RSRP

Proposal 7: For AI/ML output for beam prediction, Alt 1 (beam ID + L1-RSRP) is preferred.

Proposal 8: For predicted beams, Alt 1 (DL Tx beam prediction) is preferred.

Proposal 9. For the data collection for AI/ML model training, in the case that AI/ML model is at gNB-side, the following aspects can be further study:

- Potential enhancement for the measurement and report for data collection

- The handling/buffering of the collected data

Proposal 10. For the data collection for AI/ML model training, in the case that AI/ML model is at UE-side, the following aspects can be further study:

- UE report for the preference of data collection, e.g., intended/preferred RS transmission for UE measurement, intended/preferred time domain pattern of the RS transmission

- RS measurement configuration for data collection

Proposal 11. For the performance metric(s) of AI/ML model monitoring, beam prediction accuracy related KPIs agreed in 9.2.3.1 can be considered.

Proposal 12. For benchmark/reference for the performance comparison of the AI/ML model monitoring, baseline performance options for spatial-domain beam prediction and temporal beam prediction agreed in 9.2.3.1 can be considered.

Proposal 13. For the AI/ML model monitoring, in the case that AI/ML model is at gNB-side, the following aspect can be further study:

- Potential enhancement for the measurement and report for model monitoring

Proposal 14. For the AI/ML model monitoring, in the case that AI/ML model is at UE-side, the following aspects can be further study:

- UE report for the performance/validation of AI/ML model

References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm.