3GPP TSG RAN WG1 #110bis-e			R1-2209912

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.6.1

Source: 	NTT DOCOMO, INC.

Title: 	Discussion on further UE complexity reduction for eRedCap

Document for:	Discussion and Decision

- Introduction

A new WID on further NR RedCap UE complexity reduction (FS_NR_redcap_enh) was approved at the RAN#97-e meeting [1]. The objectives of the WI are shown below.

The objective is to specify support for the following enhancements: Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2 and CT1 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#98-e regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone - Whether or not/how a separate early indication can be supported - Other restrictions of the WI (e.g., connectivity restrictions, band, etc.)

In this contribution, we discuss on further UE complexity reduction for Rel-18 eRedCap.

- Discussion

At the last two RAN1 meetings, the UE complexity reduction features for Rel-18 eRedCap including further UE bandwidth reduction, further UE peak rate reduction, relaxed UE processing timeline and combinations of UE complexity reduction features are studied, and finally the study outcomes for further UE complexity reduction were captured in TR38.865 [2].

In the following sections, we provide the discussion for bandwidth reduction and peak rate reduction for further UE complexity reduction.

  - Further UE bandwidth reduction

In Rel-17, UE bandwidth reduction to 20MHz in FR1 was discussed and some new features, e.g., separate initial DL/UL BWP for Rel-17 RedCap etc., were specified. In Rel-18, now we discuss for further UE complexity reduction and one feature is further UE BB bandwidth reduction to 5MHz as captured in the objectives of the WID.

    - Clarification on the definition of UE BB bandwidth reduction

At the RAN#97-e meeting, it was discussed which of the bandwidth reduction option BW3 and PR3 in the SI should be selected as the WI scope, and it was concluded to select BW3, i.e., the definition of further UE BB bandwidth reduction captured in WID is based on the bandwidth reduction option BW3. However, according to the discussion during RAN#97-e meeting, the definition of the BW3 and PR3 seems not crystal clear among the companies, and hence we provide our understanding on the definition of BW3 and PR3 at first.

The agreements at the RAN1#109-e meeting related to the definition of BW3 and PR3 are excerpted below.

Agreement: - The following options for further UE bandwidth reduction can be studied: - … - Option BW3: 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH with 20 MHz RF bandwidth for UL and DL. The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - … - At least the following cases are studied: - The resource allocation spans a bandwidth of maximum 5 MHz (Maximum UE channel bandwidth). - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - It is FFS whether to study other cases. - … Agreement: - … - For Options BW3, - For 15 kHz SCS, 25 contiguous RBs are assumed to fit within the 5 MHz. - For 30 kHz SCS, 11 contiguous RBs are assumed to fit within the 5 MHz. - Larger number of RBs that fit within 5 MHz can optionally be studied. Agreement: - The following options for further UE peak rate reduction can be studied: - … - Option PR3: Restriction of maximum number of PRBs for PDSCH and PUSCH. - At least the following cases are studied: - The studied peak rate reduction applies to both UE-specific (unicast) and common (broadcast) channels. - The resource allocation spans a bandwidth of maximum 20 MHz (maximum UE channel bandwidth). - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - It is FFS whether to study other cases. - … Agreement: - The restricted number of PRBs in Option PR3 is a hardcoded limit. Agreement: - … - For Option PR3, - For 15 kHz SCS, the maximum number of RBs is 25. - For 30 kHz SCS, the maximum number of RBs is 11. - Other number of RBs that meet the 10-Mbps peak rate target can optionally be studied. - …

As explicitly described in the agreement, it should be common understanding that available RBs for allocation are confined within contiguous 5MHz for BW3 while available RBs for allocation can be non-contiguous 5MHz for PR3. On top of that, according to the discussion at RAN1#110 and RAN#97-e meeting, the companies are not on the same page regarding the assumption of post-FFT data buffering for BW3 and PR3.

Based on the complexity reduction estimation captured in TR38.865, the post-FFT data buffering block is captured as a contributor for further complexity reduction for BW3, but not for PR3. Thus, it should be assumed that post-FFT data buffering is restricted to 5MHz for BW3 but 20MHz for PR3. Furthermore, considering the 5MHz post-FFT data buffering for BW3, the sufficient time separation between the scheduling PDCCH and the scheduled PDSCH is required, e.g., via cross-slot scheduling. Alternatively, FDRA needs to be restricted within a pre-defined 5MHz for BW3 to avoid the time separation between PDCCH and PDSCH.

On the other hand, such scheduling restrictions are not required for PR3 with 20MHz post-FFT data buffering. Furthermore, some companies argued that if UE can buffer 20MHz, the UE can receive the channels exceeding 5MHz bandwidth without any performance loss, i.e., the UE can receive the channel up to 20MHz. However, if such assumption is applied for eRedCap, we are not sure whether the peak data rate is reduced to the target 10Mbps via restriction on maximum number of PRBs. It is also captured in the TR that peak data rate reduction would contribute not only for the complexity reduction for UE base band processing block but also for memory size reduction. Therefore, if what can be done with PR3 is expanded, the difference of complexity reduction gain between BW3 and PR3 would be larger.

At least peak data rate should be reduced to 10Mbps as described in the justification of the WID, therefore, such assumption, i.e., eRedCap UE can receive the channel up to 20MHz with 20MHz post-FFT buffer, should not be supported as Rel-18 eRedCap.

As mentioned above, the current WID objective is based on the definition of BW3, and hence, we assume that UE can receive data channels which does not exceed 5MHz.

Proposal 1: Rel-18 eRedCap UE can receive data channels which does not exceed 5MHz.

    - Impacts on initial access and random access

Common PDSCH reception

As captured in the TR38.865 section 7.2, some impacts can be expected regarding common data channels when the UE BB bandwidth for data channels is restricted to 5MHz.

First of all, Rel-18 eRedCap UE can receive SSB and corresponding MIB-configured CORESET#0 same as legacy UEs, however, it should be discussed how to receive SIB1, OSI, Msg2/4/B and/or paging PDSCH with 5MHz bandwidth restriction.

As discussed in the section 2.1.1, we assume that eRedCap UE can buffer only 5MHz for data channel, and hence the sufficient time separation between the scheduling PDCCH and the scheduled PDSCH is required, e.g., via cross-slot scheduling, or FDRA needs to be restricted within a pre-defined 5MHz. However, according to the current specification, cross-slot scheduling is not supported especially for SIB1 scheduling when the SSB and CORESET#0 multiplexing pattern is 1.

One possible option for common PDSCH scheduling for Rel-18 eRedCap UE is introducing cross-slot scheduling even for SSB and CORESET#0 multiplexing pattern 1 to ensure the sufficient gap between PDCCH decoding and PDSCH reception. This option can ensure the time between PDCCH and PDSCH, however, NW overhead would be increased since the common PDSCH resource(s) may not be able to be shared between Rel-18 eRedCap UE and legacy UEs.

To avoid NW overhead increase, we think some other options can also be considered.

Second possible option is that Rel-18 eRedCap UE would change the data buffering bandwidthwithin the gap between common PDSCH resources which is repeatedly transmitted and soft-combining to receive whole bandwidth of the PDSCH. For this option, the PDSCH transmitted for legacy UEs can be reused by Rel-18 eRedCap UEs and the eRedCap UE can receive whole bandwidth of the PDSCH as legacy UEs. Hence, the NW overhead would not be increased and also the coverage would not be affected.

Another possible option is that Rel-18 eRedCap UE would puncture the bits outside its BB bandwidth when the bandwidth of the common PDSCH exceeds 5MHz. For this option, the PDSCH transmitted for legacy UEs can be reused by Rel-18 eRedCap UEs, however, it was observed at the SI phase that the link performance would be largely degraded especially for SIB1. Therefore, this option is not preferable from coverage degradation perspective.

Proposal 2: It should be discussed how to receive SIB1, OSI, Msg2/4/B and/or paging PDSCH for Rel-18 eRedCap considering following alternatives when the PDSCH resource(s) for legacy UE exceeds 5MHz;

Alt.1: Rel-18 eRedCap dedicated PDSCH for SIB1, OSI, Msg2/4/B and/or paging resource is scheduled

- Opt.1-1: Support cross-slot scheduling for the PDSCH for Rel-18 eRedCap.

Alt.2: SIB1, OSI, Msg2/4/B and/or paging PDSCH resource is shared between Rel-18 eRedCap and legacy UEs

- Opt.2-1: Rel-18 eRedCap UE changes the data buffering bandwidth and soft-combining for the whole PDSCH reception when the bandwidth of the PDSCH exceeds 5MHz.

- Opt.2-2: Rel-18 eRedCap UE performs puncturing the bits outside its BB bandwidth when the bandwidth of the PDSCH exceeds 5MHz.

For Rel-17 RedCap UEs, separate initial DL/UL BWP configuration was supported when the initial DL/UL BWP for legacy UEs span larger bandwidth than 20MHz. As described in WID objective, Rel-18 eRedCap has 20MHz RF bandwidth and can receive/transmit the channels/signals which exceeds 5MHz BW except for data channels. Therefore, we think it is not necessary to introduce another Rel-18 eRedCap specific separate initial DL/UL BWP and Initial DL/UL BWP operation framework for Rel-17 RedCap can be reused for Rel-18 eRedCap.

Proposal 3: Initial DL/UL BWP operation framework for Rel-17 RedCap can be reused for Rel-18 eRedCap.

Random access and Early indication of Rel-18 eRedCap

In Rel-17, the early indication of RedCap during random access, i.e., Msg1, Msg3 or MsgA, was supported to perform random access procedure appropriately within 20MHz bandwidth. On top of that, it was discussed at the RAN#97-e meeting whether separate early indication from Rel-17 RedCap is required for Rel-18 eRedCap, but no consensus was achieved.

In our understanding, whether the separate early indication for eRedCap is required or not depends on whether the scheduling/resource of data channels needs to be optimized for eRedCap during random access. More specifically, separate early indication of Rel-18 eRedCap is not required if all the data channels during random access can be scheduled for Rel-18 eRedCap UE based on the scheduling/resource allocation mechanisms in Rel-17 RedCap, otherwise, separate early indication is required.

Proposal 4: Whether the separate early indication from Rel-17 RedCap is required or not should be discussed based on the discussion progress of common PDSCH reception during random access procedure for Rel-18 eRedCap UE.

  - Further UE peak rate reduction

In addition to further UE bandwidth reduction to 5MHz in FR1, another complexity reduction feature, further UE peak data rate reduction, is captured in WID objective. For Rel-17 RedCap, the target peak data rate was set as 150 Mbps. In Rel-18, the target peak data rate is reduced to 10 Mbps for further UE complexity reduction.

During the SI for Rel-18 eRedCap, three options of peak rate reduction was studied and it was concluded at the RAN1#110 and RAN#97-e meeting that peak rate reduction option PR1 in SI, i.e., relaxation of the constraint (vLayers·Qm·f ≥ 4), is captured as the discussion scope for peak rate reduction in the WID.

    - Relation between bandwidth reduction

At the RAN#97-e meeting, it was discussed whether the peak rate reduction feature can be supported as standalone feature for Rel-18 eRedCap or only as add-on feature with bandwidth reduction feature, but no consensus was achieved.

According to the TR38.865, the complexity reduction gain for peak rate reduction option PR1 was observed as only 3.73~5.36% if it is supported as standalone feature. Therefore, majority of companies supported the PR1 as an add-on feature with bandwidth reduction feature to ensure the complexity reduction gain while some company prefer to allow to support PR1 as standalone feature.

However, if PR1 is supported as standalone feature, some Rel-18 eRedCap UE may support bandwidth reduction feature but some other eRedCap UE may support only peak data rate reduction feature. It implies that eRedCap UE can be distinguished into more than one UE types and may lead the market fragmentation. Considering that it is also captured in the WID objective that “Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction”, peak data rate reduction should be supported as only add-on feature for bandwidth reduction.

Proposal 5: Peak rate reduction feature should be supported as add-on feature with bandwidth reduction feature for Rel-18 eRedCap.

    - Constraint relaxation on peak rate calculation

As captured in TR38.865 section 7.3, significant impacts for PR1 are not expected, however, at least the exact value of relaxed constraint for peak rate reduction should be discussed.

Regarding the data rate for NR, it is calculated by the following equation according to TS38.306 [3];

wherein

J is the number of aggregated component carriers in a band or band combination

Rmax = 948/1024

For the j-th CC,

is the maximum number of layers

is the maximum modulation order

is the scaling factor

is the numerology

is the average OFDM symbol duration in a subframe for numerology

is the maximum RB allocation in bandwidth  with numerology

is the overhead and takes the following values. [0.14], for frequency range FR1 for DL

In the current specification, the constraint on vLayers·Qm·f is specified as vLayers·Qm·f ≥ 4. As explained above, this constraint would be relaxed for further peak data reduction, i.e., vLayers·Qm·f can be smaller than 4. In addition, as discussed in section 2.2.1, we believe that the peak rate reduction feature should be supported on top of bandwidth reduction feature. Based on the assumption, we calculate the peak data rate with 5MHz bandwidth and the peak data rate with relaxed constraints are shown in the table 1 below.

According to the table 1, the target peak data rate of 10 Mbps can be achieved when vLayers·Qm·f is 3. In addition, vLayers·Qm·f cannot be one with the scaling factor value currently supported, and hence the new smaller scaling factor, e.g., 0.5, can be introduced if the constraint needs to be relaxed as vLayers·Qm·f ≥ 1.

Therefore, the constraint would not need to be too small such as 1, while 1 had been assumed as the relaxed constraint during SI. However, it was captured in the TR that the memory size would be reduced linearly according to the peak data rate, thus smaller value of the constraint may be beneficial in terms of further complexity reduction.

Proposal 6: The constraint on the lower limit of vLayers·Qm·f should be equal to or smaller than 3.

- New scaling factor value, e,g, 0.5, can be considered if constraint on vLayers·Qm·f  is 1.

Table 1: Peak data rate [Mbps] for option PR1

| Number of RB, SCS and v∙Q∙f | Peak rate |
| --- | --- |
| 25 RB, 15 kHz SCS, v∙Q∙f=1 | 3.3 |
| 25 RB, 15 kHz SCS, v∙Q∙f=2 | 6.7 |
| 25 RB, 15 kHz SCS, v∙Q∙f=3 | 10.0 |
| 11 RB, 30 kHz SCS, v∙Q∙f=1 | 2.9 |
| 11 RB, 30 kHz SCS, v∙Q∙f=2 | 5.9 |
| 11 RB, 30 kHz SCS, v∙Q∙f=3 | 8.8 |

- Conclusion

In this contribution, we discussed bandwidth reduction and peak data rate reduction for further UE complexity reduction. Based on the discussion, we made following proposals.

Proposal 1: Rel-18 eRedCap UE can receive data channels which does not exceed 5MHz.

Proposal 2: It should be discussed how to receive SIB1, OSI, Msg2/4/B and/or paging PDSCH for Rel-18 eRedCap considering following alternatives when the PDSCH resource(s) for legacy UE exceeds 5MHz;

Alt.1: Rel-18 eRedCap dedicated PDSCH for SIB1, OSI, Msg2/4/B and/or paging resource is scheduled

- Opt.1-1: Support cross-slot scheduling for the PDSCH for Rel-18 eRedCap.

Alt.2: SIB1, OSI, Msg2/4/B and/or paging PDSCH resource is shared between Rel-18 eRedCap and legacy UEs

- Opt.2-1: Rel-18 eRedCap UE changes the data buffering bandwidth and soft-combining for the PDSCH reception when the bandwidth of the PDSCH exceeds 5MHz.

- Opt.2-2: Rel-18 eRedCap UE performs puncturing the bits outside its BB bandwidth when the bandwidth of the PDSCH exceeds 5MHz.

Proposal 3: Initial DL/UL BWP operation framework for Rel-17 RedCap can be reused for Rel-18 eRedCap.

Proposal 4: Whether the separate early indication from Rel-17 RedCap is required or not should be discussed based on the discussion progress of common PDSCH reception during random access procedure for Rel-18 eRedCap UE.

Proposal 5: Peak rate reduction feature should be supported as add-on feature with bandwidth reduction feature for Rel-18 eRedCap.

Proposal 6: The constraint on the lower limit of vLayers·Qm·f should be equal to or smaller than 3.

- New scaling factor value, e,g, 0.5, can be considered if constraint on vLayers·Qm·f  is 1.

References

[1] RP-222675, Enhanced support of reduced capability NR devices, Ericsson.

[2] RR38.865, Study on further NR RedCap UE complexity reduction (Release 18).

[3] TS38.306, User Equipment (UE) radio access capabilities.