3GPP TSG RAN WG1 #110bis-e	R1-2210098

e-meeting, 10 – 19 October, 2022

Agenda item:		8.16.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues for UE features topics 1

Document for:		Discussion and Decision

# Introduction

In RAN1#110 the RAN1 UE features have been further developed [1]. In this contribution we present our views on the UE features for NR coverage enhancement for list of topics 1.

# Discussion

Detailed comments on UE features are listed below.

## eIIoT & URLLC

- 25-3a:

- Confirm 25-3 and 30-5 as pre-requisites.

- 25-3b:

- Missing pre-requisite FG: FG 25-3

- 25-6:

- Remove the yellow marked ’type 3 or’ from the FG components, as Type 3 CB is a Rel-16 feature already.

- 25-12:

- Confirm the FG components (i.e. remove yellow highlight)

- 25-16:

- Confirm the prerequisite FGs (i.e. remove yellow highlight)

- 25-18:

- Pre-requisite feature groups: 6-6 (UL CA), 11-3, 12-1

## Coverage enhancement

- 30-4a/b/c/d:

- Per band and per BC indication, given that UL CA without simultaneous transmission is also supposed to be supported here.

- Add the following note: UE supporting this capability only supports it on transmissions with a single UL carrier, even when UE is configured with UL CA.

- 30-4e:

- Per UE is sufficient if prerequisites are agreed per band/BC already. No differentiation needed.

- 30-4g:

- Description of "Feature group" should be (changes are highlighted in red):

- "Restart DM-RS bundling after the dynamic events that violate power consistency and phase continuity"

- Description of "Components" should be (changes are highlighted in red):

- "Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity, except events which are triggered by DCI or MAC CE but regarded as semi-static events"

- Description of "Consequence if the feature is not supported by the UE" should be (changes are highlighted in red):

- "UE does not support restarting DM-RS bundling after the dynamic events that violate power consistency and phase continuity"

- Per UE is sufficient if prerequisites are agreed per band/BC already. No differentiation needed.

- 30-4h:

- Per UE is sufficient if prerequisites are agreed per band/BC already.

## MBS

- 33-3-3a:

- Per UE

- 33-3-3b:

- Per UE

- 33-3-4:

- Per UE

- 33-3-5:

- Per UE or per band, motivation for finer granularity is not clear. If limitations on level of support is needed, then it is preferrable to add those restrictions explicitly in the component description.

- 33-5-1e/f/g/i:

- Per UE

- 33-5-2:

- Per UE

- 33-6-1/1a:

- Per UE

- 33-6-2:

- Per UE

- 33-8-1:

- Per band as a compromise

- 33-9:

- Per UE

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for list of topics 1, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- R1-2207923, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #110 Thursday”, Moderators (AT&T, NTT DOCOMO, INC.), Aug. 2022