# 3GPP TSG RAN WG1 110-bis-e							R1-2210309

# e-meeting, October 10th – October 19th, 2022

Agenda item:	8.14

Source: 	Qualcomm Incorporated

Title: 	Maintenance on NB-IoT/eMTC support for Non-Terrestrial Network

Document for:	Discussion and Decision

# Processing time for downlink reception

In terrestrial networks, eMTC/NB-IoT UEs typically require a certain amount of “minimum processing time” to process a downlink reception, before transmitting an associated uplink that may be triggered by the downlink reception. Examples include:

- (N)PDSCH triggering HARQ-ACK

- (N)PDCCH triggering (N)PUSCH

- (N)PDCCH triggering PDCCH-ordered (N)PRACH

For purposes of illustration, we will take the example of NPDSCH triggering HARQ-ACK in NB-IoT.

Below is the standards text (from TS 36.213) for transmitting HARQ-ACK:

16.4.2   UE procedure for reporting ACK/NACK

The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of

-    n+k0'+Koffset-1 DL subframe for FDD,

-    k0'-1 NB-IoT UL subframes following the end of n+12 subframe for TDD,

transmission of the NPUSCH carrying ACK/NACK response…

In FDD (relevant to NTN), for terrestrial networks, Koffset=0, and hence, the “processing time” that a UE has, to generate the HARQ ACK, is given by:

  - k0'-1-[TTA,terrestrial] (Terms normalized to same time units, e.g., a time unit of slots/subframes)

For legacy terrestrial, however, this TTA is typically very small. Hence, the “minimum” processing time that a UE needs (in terrestrial) is given by:

  - Tproc,terrestrialmin=k0',min-1-[TTA,terrestrialmax] (Terms normalized to same time units, e.g., a time unit of slots/subframes)

Observation 1: In terrestrial networks, the minimum processing time that a UE needs to process an NPDSCH before it can transmit a corresponding HARQ-ACK is given by k0',min-1-[TTA,terrestrialmax].

For NTN, there is a very large TA, which is reflected by the following two additional terms (w.r.t terrestrial) in the expression for TA in TS 36.211:

  - TTA,NTN=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs=TTA,terrestrial+NTA,adjcommon+NTA,adjUETs.

Further, due to the (non-zero) Koffset term in NTN, the “processing time” that the UE has in NTN, to generate the HARQ ACK is given by:

  - k0'-1+Koffset-[TTA,NTN] (Terms normalized to same time units, e.g., a time unit of slots/subframes)

Hence, if Koffset< [(NTA,adjcommon+NTA,adjUE)Ts] for a UE, the “processing time” to generate the HARQ ACK is smaller than terrestrial for a given value of k0'. Such a scenario may occur, e.g., for certain UEs at the cell-edge, in a cell where the Koffset is not configured overly conservatively

We want to avoid such a situation, since certain legacy UEs cannot meet a “tighter than terrestrial” processing time (in this running example, from NPDSCH to HARQ-ACK)

Observation 2: Under certain circumstances—e.g., for the same value of k0', if Koffset< [(NTA,adjcommon+NTA,adjUE)Ts]—a  NB-IoT UE over NTN may have to process a NPDSCH in less than the minimum processing time afforded to it in terrestrial networks

- This may prevent a legacy NB-IoT UE from operating seamlessly in an NTN network

To this end, we propose the following limitation in the specifications (for this running example of NPDSCH triggering HARQ-ACK), such that a NB-IoT UE is not required to meet a tighter processing timeline than terrestrial in NTN.

Proposal 1: For NB-IoT over NTN, for a NPDSCH triggering a HARQ-ACK, the following condition must be satisfied, for the UE to be required to process the NPDSCH:

- k0'-1+Koffset-[TTA,NTN]≥Tproc,terrestrialmin  =k0',min-1-[TTA,terrestrialmax],

where k0', min, TTA,terrestrialmax and  TTA,NTN are defined in TS 36.211, and [.] denotes a quantization to appropriate slot/subframe/RU units.

While our running example has been for NPDSCH triggering HARQ-ACK, the same principle applies to other downlink-triggering-uplink processing times, which we analogously propose to amend, in the proposals below:

Proposal 2: For NB-IoT over NTN, for a NPDCCH triggering a NPUSCH, the following condition must be satisfied, for the UE to be required to transmit the NPUSCH:

- k0-1+Koffset-[TTA,NTN]≥Tproc,terrestrialmin  =k0min-1-[TTA,terrestrialmax],

where k0min, TTA,terrestrialmax and  TTA,NTN are defined in the specifications, and [.] denotes a quantization to appropriate slot/subframe/RU units.

Proposal 3: For NB-IoT over NTN, for a NPDCCH triggering a “PDCCH order” based NPRACH, the following condition must be satisfied, for the UE to be required to transmit the NPRACH:

- k2-1+Koffset-[TTA,NTN]≥Tproc,terrestrialmin  =k2min-1-[TTA,terrestrialmax],

where k2min, TTA,terrestrialmax and  TTA,NTN are defined in the specifications, and [.] denotes a quantization to appropriate slot/subframe/RU units.

While the above proposals are made for NB-IoT, similar amendments to the specifications are necessary to maintain adequate processing time for eMTC UEs over NTN.

Proposal 4: Specify similar conditions for processing times for “downlink triggering uplink” settings for eMTC over NTN, as described in Proposals 1 through 3 for NB-IoT over NTN in this contribution.

# Conclusion

In this contribution, we express our views regarding remaining issues (specifically to do with processing times for downlink receptions) in Release 17 IoT-NTN. Our observations and proposals are summarized below:

Observation 1: In terrestrial networks, the minimum processing time that a UE needs to process an NPDSCH before it can transmit a corresponding HARQ-ACK is given by k0',min-1-[TTA,terrestrialmax].

Observation 2: Under certain circumstances—e.g., for the same value of k0', if Koffset< [(NTA,adjcommon+NTA,adjUE)Ts]—a  NB-IoT UE over NTN may have to process a NPDSCH in less than the minimum processing time afforded to it in terrestrial networks

- This may prevent a legacy NB-IoT UE from operating seamlessly in an NTN network

Proposal 1: For NB-IoT over NTN, for a NPDSCH triggering a HARQ-ACK, the following condition must be satisfied, for the UE to be required to process the NPDSCH:

- k0'-1+Koffset-[TTA,NTN]≥Tproc,terrestrialmin  =k0',min-1-[TTA,terrestrialmax],

where k0', min, TTA,terrestrialmax and  TTA,NTN are defined in TS 36.211, and [.] denotes a quantization to appropriate slot/subframe/RU units.

Proposal 2: For NB-IoT over NTN, for a NPDCCH triggering a NPUSCH, the following condition must be satisfied, for the UE to be required to transmit the NPUSCH:

- k0-1+Koffset-[TTA,NTN]≥Tproc,terrestrialmin  =k0min-1-[TTA,terrestrialmax],

where k0min, TTA,terrestrialmax and  TTA,NTN are defined in the specifications, and [.] denotes a quantization to appropriate slot/subframe/RU units.

Proposal 3: For NB-IoT over NTN, for a NPDCCH triggering a “PDCCH order” based NPRACH, the following condition must be satisfied, for the UE to be required to transmit the NPRACH:

- k2-1+Koffset-[TTA,NTN]≥Tproc,terrestrialmin  =k2min-1-[TTA,terrestrialmax],

where k2min, TTA,terrestrialmax and  TTA,NTN are defined in the specifications, and [.] denotes a quantization to appropriate slot/subframe/RU units.

Proposal 4: Specify similar conditions for processing times for “downlink triggering uplink” settings for eMTC over NTN, as described in Proposals 1 through 3 for NB-IoT over NTN in this contribution.