3GPP TSG RAN WG1 #110bis-e			R1-2210495

Electronic Meeting, October 10th – 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the description for the minimum number of Y and Y’ candidate slots in sidelink partial sensing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to agreements in RAN1#106-e and RAN1#107-e, the selection of Y should correspond to PBPS and CPS for periodic transmission (Prsvp_TX≠0), rather than only the PBPS. Similarly, the selection of Y’ candidate slots should correspond PBPS and/or CPS results (if available) for aperiodic transmission (Prsvp_TX=0), rather than only the CPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the parameter description for sl-MinNumCandidateSlotsPeriodic to include CPS and periodic transmission (Prsvp_TX≠0). Correct the parameter description for sl-MinNumCandidateSlotsAPeriodic to include PBPS and aperiodic transmission (Prsvp_TX=0). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The descriptions for sl-MinNumCandidateSlotsPeriodic and sl-MinNumCandidateSlotsAperiodic are not aligned with the agreements and remained incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts omitted>

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

-	Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources corresponding to periodic-based partial sensing and contiguous partial sensing for resource (re)selection triggered by periodic transmission (Prsvp_TX≠0).

-	Optionally, minimum number of Y' slots as Y'min   (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to periodic-based partial sensing and/or contiguous partial sensing results (if available) for resource (re)selection triggered by aperiodic transmission (Prsvp_TX=0).

<Unchanged parts omitted>