3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2210509

# e-Meeting, October 10-19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on SS0 availability for scheduling MBS to TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (CMCC), Huawei, HiSilicon, CBN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 38.331 v17.2.0, the explaination of searchSpaceMCCH and searchSpaceMTCH are as the following: searchSpaceMCCH - ID of the search space for MCCH. If the field is absent, the UE does not receive MCCH in this BWP (see TS 38.213 [13], clause 10). searchSpaceMTCH - ID of the search space for MTCH of MBS broadcast. If the field is absent, the UE applies searchSpaceMCCH also for MTCH, (see TS 38.213 [13], clause 10). In TS 38.213 v17.3.0: - searchSpaceZero in PDCCH-ConfigCommon when searchSpaceMCCH and searchSpaceMTCH are not provided, for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI. Hence, UE behaviors are different according to TS 38.331 and TS 38.213 regarding whether UE monitors DCI format 4_0 in searchSpaceZero. |  |  |  |  |  |  |  |  |
| Summary of change: |  | searchSpaceZero is used for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI for MTCH when it is configured by searchSpaceMCCH and searchSpaceMTCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When searchSpaceMCCH and searchSpaceMTCH are not configured, UE behaviors are different according to TS 38.331 and TS 38.213. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.1	UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set on the primary cell of the MCG configured by

-	pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI, or

-	searchSpaceZero by configuring searchSpaceID=0 for searchSpaceMCCH or searchSpaceMTCH for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI for broadcast

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0B-PDCCH CSS set configured by searchSpaceMCCH and searchSpaceMTCH for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI for MTCH, on the primary cell of the MCG

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-	a Type1A-PDCCH CSS set configured by sdt-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a C-RNTI or a CS-RNTI on the primary cell as described in clause 19.1

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type2A-PDCCH CSS set configured by peiSearchSpace in DownlinkConfigCommonSIB for a DCI format 2_7 with CRC scrambled by a PEI-RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by

-	SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, or CI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI, or

-	SearchSpace in PDCCH-ConfigMulticast for DCI formats with CRC scrambled by G-RNTI, or G-CS-RNTI, or

-	searchSpaceMCCH and searchSpaceMTCH on a secondary cell for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI for broadcast, and

-	a USS set configured by

-	SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, or SL Semi-Persistent Scheduling V-RNTI

< Unchanged parts are omitted >