3GPP TSG RAN WG1 Meeting # 110bis-e	R1-2210530

# E-meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on enhanced Type 3 HARQ-ACK codebook and HARQ-ACK re-transmission triggering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Nokia), OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As part of the Rel-17 HARQ-ACK feedback enhancements, the MCS field in the triggering DCI is used to: - For the enhanced Type-3 HARQ-ACK codebook triggering to indicated one of the M configured enh. Type 3 CBs (if the new field is not configured), based on the following agreement: Agreement If more than one (M>1) enhanced Type 3 HARQ-ACK codebook is configured and the triggering DCI with the ‘one-shot HARQ-ACK request’ set to ‘1’, - If the FDRA field is not valid, i.e. all “1s” or all “0s” as per Rel-16, then PDSCH is not scheduled: - If a new field with N= log2 (M) bits is configured in the triggering DCI, the UE uses this new field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the new field is not configured in the triggering DCI, the UE uses the MCS field to indicate one of M configured e-Type 3 HARQ-ACK CBs …... - For the HARQ-ACK re-transmission triggering to indicate the HARQ-ACK re-transmission slot offset of the PUCCH, based on the following agreement: Agreement For one-shot triggering of HARQ-ACK re-transmission, the HARQ_retx_offset is indicated by the bits of the MCS field for transport block 1.. For both cases, the current specification is not clear if the MCS field of transport block 1 or transport block 2 in DCI format 1_1 is to be used for the related indication. |
| Summary of change: |  | Clarify that the MCS field of transport block 1 in DCI format 1_1 is used for - indicating the enh. Type-3 HARQ-ACK codebook for enhanced Type-3 HARQ-ACK codebook triggering and - the HARQ re-tx offset indication for HARQ-ACK re-transmission triggering. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behavior for enh. Type-3 HARQ-ACK codebook triggering and HARQ-ACK re-transmission triggering using DCI format 1_1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4, 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

### 9.1.4	Type-3 HARQ-ACK codebook determination

< Unchanged parts are omitted >

If

-	a UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, and

-	the CRC of the DCI is scrambled by a C-RNTI or an MCS-C-RNTI, and

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in the DCI format are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in the DCI format are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in the DCI format are equal to 0 or 1

the DCI format provides a request for a Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception. If the UE is provided pdsch-HARQ-ACK-EnhType3List and the DCI format includes an enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-EnhType3Index, the UE determines a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-EnhType3List corresponding to the pdsch-HARQ-ACK-EnhType3Index value. If the DCI format does not include the enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-EnhType3Index value is provided by the value of the MCS field for transport block 1 in the DCI format 1_1 or the MCS field in the DCI format 1_2. The UE is expected to provide HARQ-ACK information in response to the request for the Type-3 HARQ-ACK codebook after N symbols from the last symbol of a PDCCH providing the DCI format, where the value of N for μ=0,1,2 is provided in clause 10.2 by replacing "SPS PDSCH release" with "DCI format".

< Unchanged parts are omitted >

### 9.1.5	HARQ-ACK codebook retransmission

With reference to slots of PUCCH transmissions on the primary cell and for Type-1 or Type-2 HARQ-ACK codebooks, a UE that transmitted or would transmit a PUCCH or a PUSCH with a first HARQ-ACK codebook in slot m can be indicated by a DCI format with CRC scrambled by a C-RNTI or a MCS-C-RNTI that does not schedule a PDSCH reception [4, TS 38.212] and is received in a PDCCH ending in slot n, to transmit a PUCCH with the first HARQ-ACK codebook in slot n+k, where slot n+k is after slot m. The UE determines k and a resource for the PUCCH transmission as described in clauses 9.2.3 and 9.2.5. If the UE is provided a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern, the UE further determines a corresponding cell based on the periodic cell switching pattern as described in clause 9.A.

If the HARQ-ACK retransmission indicator field value in the DCI format 1_1 or 1_2 is '1', the UE determines slot m as m=n-l where l is determined by a one-to-one mapping in ascending order among the values of the MCS field for transport block 1 in the DCI format 1_1 or the MCS field in the DCI format 1_2 and the values from -7 to 24.

< Unchanged parts are omitted >