3GPP TSG RAN WG1 #110bis-e	        		   R1-2210610

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0127 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TDRA for multiple PUSCH scheduling in TS 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (LG Electronics), Huawei, HiSilicon, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS38.331, the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH-r17 is removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete pusch-TimeDomainAllocationListForMultiPUSCH-r17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong RRC parameter to configure multiple PUSCH scheduling by single DCI in Rel-17 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.1.2	Format 0_1

<Unchanged parts omitted>

-	Time domain resource assignment – 0, 1, 2, 3, 4, 5, or 6 bits

If the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 is not configured and if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is not configured and if the higher layer parameter pusch-TimeDomainAllocationList is configured, 0, 1, 2, 3, or 4 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationList;

If the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 is configured or if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured , 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 or pusch-TimeDomainAllocationListForMultiPUSCH;

-	otherwise the bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the default table.

-	Frequency hopping flag – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured, or if the higher layer parameter frequencyHopping is not configured and the higher layer parameter pusch-RepTypeIndicatorDCI-0-1 is not configured to pusch-RepTypeB, or if the higher layer parameter frequencyHoppingDCI-0-1 is not configured and pusch-RepTypeIndicatorDCI-0-1 is configured to pusch-RepTypeB, or if only resource allocation type 2 is configured;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

-	Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PUSCH among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214].

-	Redundancy version – – number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PUSCHs among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

<Unchanged parts omitted>