3GPP TSG-RAN WG1 Meeting #111	R1-2210901

Toulouse, France, November 14 – 18, 2022

Agenda Item:	9.5.1.2

Source:	Huawei, HiSilicon

Title:	Remaining issues for SL positioning solutions

Document for:	Discussion and Decision

Introduction

In RAN1#110bis-e [1], progress was made on the potential solutions to support SL positioning.

In this paper, we further provide our views on the remaining issues on solutions to support SL positioning in the study item. In order to reach conclusions in SI phase, we prefer to simplify the discussion and focus only on the issues that were not identified in the last meeting.

SL positioning methods

In RAN1#110bis-e [1], positioning methods have been discussed and we have the following agreement.

Agreement - From the potential candidate Positioning methods using at least SL measurements, at least the following should be introduced: - RTT-type solution(s) using SL - SL-AoA - SL-TDOA - FFS: SL-AoD

SL positioning methods

RTT based positioning methods

In RAN1#110bis-e [1], RTT-type solutions using SL have been discussed and we have the following agreement.

Agreement - With regards to the RTT-type solutions using SL, down-select between the following 2 alternatives: - Alt. 1: it corresponds to a single-sided RTT method - Alt. 2: it may correspond to either a single-sided or double-sided RTT method - With regards to the double-sided RTT, - companies are encouraged to analyze and evaluate the effect in performance for the single-sided SL RTT due to clock drift - Study the order of the SL-PRS transmissions for double-sided RTT - Study the impact of UE mobility - FFS study whether there is or what is the spec impact of double-sided RTT method - Note: the above may correspond to RTT with one or multiple devices

RTT based positioning methods have the well-known advantage of robustness to the synchronization error. In addition to the synchronization error, timing drift between two UEs generally exists due to the oscillator imperfection. To mitigate timing drift, the double-sided RTT can be adopted. The typical double-side RTT can be illustrated by the following Figure 1:

t23

t14

UE-A

UE-B

t45

t36

t1

t2

t3

t4

t5

t6

Figure 1: Legacy double-sided RTT

The RTT between UE-A and UE-B can be calculated by the following equation:

RTT=t14×t36-t23×t45t14+t45

However, this requires that UE-B transmits the SL-PRS between the two SL-PRS transmitted by the UE-A. Actually, for the double-sided RTT, this limitation is not necessary. Considering the following sequential SL-PRS transmission shown in Figure 2:

t23

t14

UE-A

UE-B

t45

t36

t1

t2

t3

t4

t5

t6

Figure 2: General double-sided RTT without transmission order restriction

Under this transmission scheme, the RTT between UE-A and UE-B can be given by:

RTT=t45-t14t23t36

Where t14t23 can be used to calibrate the timing drift in order to estimated Rx-Tx timing difference t36.

The derivation error of the above RTT calculation method is analyzed in the Appendix.

- Proposal 1: For RTT-type solutions using SL, we support Alt.2 (from the previous agreement): it may correspond to either a single-sided or double-sided RTT method.

- For double-sided RTT based method, support not to restrict the SL-PRS transmission order.

SL-AoD based positioning method

Different from the RSRP/RSRPP based AoD method in Uu, the phase difference based AoD used by Bluetooth exploits the phase difference between signals transmitted from different antennas. Since the UE has capability of coherent reception for obtaining AoA measurement, it should also be possible to consider the coherent transmission for obtaining the AoD measurement.

Observation 1: There is no blocking issue on the feasibility of AoD since AoA already requires coherent capability at UE side.

Enabling AoD allows measuring angle from some anchor UEs with only transmission capability, e.g. RSU, while the measurement results can be directly calculated by the target UE side to reduce positioning latency.

Observation 2: Enabling SL-AoD allows a deployment of anchor UE with transmission capabilities, when the target UE can use locally estimated AoD for UE-based positioning, which benefits low latency.

We think that phase difference based AoD should be supported at least for FR1 AoD positioning where beamforming is not likely performed.

- Proposal 2: Phase difference based AoD positioning should be supported for SL positioning.

Report free RTT

The positioning latency and signaling overhead can be reduced if Rx-Tx time difference is not reported in SL-RTT. To support such report free RTT, the reply-time should be known in advance. To achieve this goal, the transmission time of SL-PRS can be adjusted based on the actual time of reception of corresponding SL-PRS, so that the reply time can be a fixed value. For example, in Figure 3, UE1 is configured to transmit PRS1 to UE2 at time t1, UE2 is configured to transmit PRS2 to UE1 at time t2. When UE2 obtains the exact time of reception t3 after measuring PRS1, UE2 can adjust the transmission time of PRS2 to t4 = t2+ (t3-t1). Hence, the reply time of UE2 is t4-t3 = t2-t1. Since t1 and t2 are known at UE1, UE1 can calculate the time of flight without measurement report from UE2. The timing adjustment is t3-t1, which is usually 10~100ns level and much shorter than cyclic prefix. Hence, the introduced interference caused by timing adjustment is in control.

t3-t1

t3

UE1

UE2

t1

t4

t5

t2

t3-t1

Figure 3: Example of report free RTT method

Proposal 3: Support report free RTT based on adjusting the transmission time of SL-PRS.

SL positioning Measurements

In RAN1#110bis-e [1], SL positioning measurements have been discussed and we have the followingagreement.

Agreement With regards to the Positioning methods supported using SL-PRS measurements - at least the following measurements are considered: - SL Rx-Tx measurement - SL RSTD measurement - SL RSRP measurement - SL RSRPP measurement - SL RTOA measurement - SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement - Companies are encouraged to study other measurements (e.g., time difference of arrival of 2 SL-PRS transmitted at 2 different times from the same anchor) and provide their analysis into why they are needed in light of the above measurements. - Companies are encouraged to study potential enhancements, such as SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement - FFS any additional measurements

Sidelink positioning measurements can be determined according to the positioning methods. To support the SL-AoD positioning method, we have the following proposal:

- Proposal 4: To support the SL-AoD positioning method, the following measurement should be introduced:

- SL Phase measurement of the first path

SL-PRS pattern design

In RAN1#110bis-e [1], SL-PRS structure have been discussed and we have the following agreement.

Agreement With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics: - With regards to the value N (comb size) and the number M of SL-PRS symbols within a slot excluding the symbol(s) used for AGC training / RxTx Turnaround: - At least the following values are considered as potential candidate values: N = {1,2,4,6,8,12} - FFS: the values considered as potential candidate values for M - FFS1: Whether to consider N>12 as a potential candidate value(s) - The symbols of a SL-PRS resource within a slot are consecutive symbols - FFS: consecutive and/or non-consecutive symbols for shared resource pool (if supported) - FFS: RE-Offset sequence within a SL-PRS resource, including whether to have in the end of the SL-PRS pattern a symbol with the same RE-offset as the first symbol, for phase-tracking purpose

Doppler frequency shift estimation is necessary especially for the V2X scenario where the vehicular UE maybe move at the speed up to 250km/h. It requires comparing the phase difference of the same frequency in different symbols of the received SL-PRS. So, a repetition SL-PRS pattern would be beneficial for the estimation, shown in Figure 4. The value M = n+1, with N = K×n for partial staggering pattern and n = K×N for full staggering pattern where (K≥1) should be supported where M is the number of symbols and N is the comb size.

AGC

GAP

M=5, N=4

H1(k)

H5(k)

AGC

GAP

H1(k)

H3(k)

M=3, N=2

AGC

GAP

H1(k)

H7(k)

M=7, N=6

Figure 4: Example of M=n+1, N=n comb pattern

As shown in the Figure 4, by comparing the phase difference of H1k and Hn+1k of the same RE, the Doppler frequency shift can be estimated. Therefore, we have the following proposal:

Proposal 5: Support to introduce a repetition structure with the last symbol being the same RE offset as the first symbol.

Dedicated resource pool for SL-PRS

Slot structure

In RAN1#110bis-e [1], slot structure for the dedicated resource pool have been discussed and we have the following agreement.

Agreement For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included - FFS: Details - FFS: definition of PSSCH associated with SL-PRS transmission(s) - Note: Companies are encouraged to provide their analysis and views on the above

- For dedicated resource pool, it provides the possibility of lager bandwidth and lager multi-user capacity due to more flexible configuration. Considering the convenience of SL-PRS reservation, the slot structure should at least contain PSCCH and SL-PRS. Figure 5 shows an example of potential slot structure. To support the reservation scheme similar to sidelink communication, a part of time/frequency resource, such as 2 or 3 symbols, can be allocated to PSCCH which carry the control and reservation information of the SL-PRS. Therefore, we prefer Opt.2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included.

- For Opt.1, it provides an extremely simplified design. However, according to the previous agreement, the reservation information associated with the SL-PRS should be transmitted, and preferably via SCI. With Opt.1, the reservation SCI is expected to be transmitted in a separate pool. Regarding such details of Opt.1, we prefer to leave it for WI phase.

- For Opt.3, there was one remaining FFS on the definition of PSSCH associated with SL-PRS transmission. We do not think any SL-SCH contained within PSSCH should be treated as something that is associated with SL-PRS transmission, because it is up to the RAN2 protocol design, and payload in the MAC PDU should be transparent to the SL-PRS transmission at all. if the transmission of SL-PRS and the potential measurement report are included in the same resource pool, regardless whether they are in the same slot or in different slot, it becomes a de-fact shared resource pool, and backward compatibility with legacy UE should be ensured according to the agreement made in RAN1#110bis-e. Then the only logical explanation could be the 2nd-stage SCI as the only payload in the PSSCH. However, having a PSSCH structure to only contain the 2nd-stage SCI does not seem quite necessary especially for a new pool that is not backward compatible with legacy UEs.

AGC

PSCCH

SL-PRS

GAP

AGC

2-3 symbol

2-9 symbol

Figure 5: Example of potential slot structure of SL-PRS resource pool

- Proposal 6: For channel structure of the dedicated resource pool, do not support Opt. 3, and leave other details of Opt.1 and Opt.2 for the WI phase.

Bandwidth of SL-PRS transmission

In RAN1#110bis-e [1], bandwidth of SL-PRS transmission for the dedicated resource pool have been discussed and we have the following agreement.

Agreement At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

- In sidelink positioning, there are multiple scenarios e.g. V2X, IIOT, public safety and commercial, with different requirements respectively. Consequently, we believe the flexible configuration of bandwidth for SL-PRS transmission is more suitable for dedicated resource pool. If so, only the necessary bandwidth could be selected for SL-PRS transmission based on different positioning accuracy requirement. Therefore, we have the following proposal:

- Proposal 7: For bandwidth of SL-PRS transmission in the dedicated resource pool, support Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool.

Resource configuration and reservation

In RAN1#110bis-e [1], SL-PRS configuration/activation/triggering have been discussed and we have the following agreements.

Agreement From RAN1 perspective, the following cast types of SL-PRS transmission can be introduced for SL positioning: Unicast, Groupcast (not including many to one) - Broadcast (as a working assumption). - FFS: Applicability of the above cast types Agreement With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication

SL-PRS resource configuration and reservation

Before transmission of SL-PRS, configuration of SL-PRS is necessary, hence the receiver can correctly decode the physical signals and perform the measurement procedures. For the configuration or pre-configuration of SL-PRS, we prefer the high-layer approach. For example, SL-PRS resources can be configured by gNB when UE is in coverage or pre-configured when UE is out of coverage. The configuration of SL-PRS resource could be flexible. Different SL-PRS resource can be configured with different bandwidth, different comb size, different offset, and different symbol to meet different positioning requirement.

With the configured or pre-configured SL-PRS resources, SCI could be used to carry the reservation information of the SL-PRS within the same slot and for the future slots. When a UE receives the SCI for it, upon decoding the reservation information, it can start the reception and measurement procedures of the SL-PRS. The latency between the reservation and SL-PRS transmission can be constrained within one slot. It is observed that by low layer activation/triggering, low latency could be achieved. In addition, the reservation information is broadcast in nature due to being detectable for all other UEs performing sensing, which is more suited for SCI as does for the communication. So, we believe SL-MAC-CE or higher-layer signaling is not necessary for SL-PRS reservation/indication.

- Proposal 8: For SL signaling of the reservation/indication of SL-PRS resource(s), do not further support SL-MAC-CE or higher-layer signaling.

Cast type of SL-PRS transmission

For SL positioning, the potential solutions of relative positioning, ranging and absolute positioning need to be studied. The cast type of SL-PRS transmission can be determined based on the requirement of the targeted positioning scenarios.

Unicast can be used at least for one-to-one relative positioning or ranging. Groupcast can be applied to relative or absolute positioning within a group which is established in higher layers. The group may contain for example one target UE and one or more anchor UEs, for which the absolute positioning can be accomplished if the anchor UEs’ locations are known. Such two cast types are sufficient to cover all of the SL positioning scenarios.

If the broadcast for SL-PRS in discussion corresponds to periodic SL-PRS transmission without the indication of broadcast destination ID or corresponds to SL-PRS with the indication of a broadcast destination ID for the case of RSU, such mechanism of broadcast SL-PRS depends on other design in general and needs more study about the feasibility. Therefore, we have the following proposal:

- Proposal 9: For broadcast of SL-PRS transmission, no need to discuss or confirm the working assumption in SI phase and it is consequently not supported for the normative work.

Resource allocation for SL-PRS

Resource allocation in Scheme 1

In RAN1#110bis-e [1], scheme 1 of SL-PRS resource allocation has been discussed and we have the following agreement.

Agreement Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options: - Opt. 1: through higher layers from the LMF - Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB - Up to further discussion which one or more of these shall be applicable

In Scheme 1, gNB controls the resource allocation for SL-PRS transmission. The general procedure can be described as follows. When a UE wants to transmit SL-PRS according to a certain positioning service, it first sends the SL-PRS transmission request to the gNB to request resources. Upon the request of the UE, gNB grants certain resources for transmitting the SL-PRS. The granted resource can be configured by gNB following the similar mechanism like mode 1 for SL communication, e.g. dynamic grant, configured grant type1/type 2.

We do not think LMF based resource allocation/indication is necessary, since technically LMF cannot decide the RAN-level resource allocation especially when the resource is used for UE-to-UE SL-PRS transmission. Even for Uu positioning, LMF serves as the information centre to collect the assistance data and forward the configuration to the UE without determining the detailed configuration at all. From this point of view, SL-PRS is more like UL-SRS, which is transmitted by the UE, and should be granted by gNB, instead of LMF.

Therefore, we have the following proposal:

- Proposal 10: For Scheme 1 SL-PRS resource allocation, support Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB.

Resource allocation in Scheme 2

For the study of the SL-PRS resource allocation scheme 2, to save the standard effort, it should reuse the legacy design in sidelink communication as much as possible.

Observation 3: The legacy design for UE autonomous resource allocation in sidelink R16/R17 should be reused as much as possible.

There are a couple of positioning methods can be used for sidelink, like: RTT, TDOA, AoA, AoD, etc. Different sidelink position methods may require different operations among target UE and anchor UEs, but it’s better to give a unified and transparent Scheme 2 design for different sidelink positioning/ranging methods to avoid the diverse design.

Observation 4: A unified Scheme 2 mechanism and related signaling framework should be considered to support different sidelink positioning methods.

Sensing

With regards to sensing of SL-PRS, we think that the following procedure for sensing in sidelink communication can be reused:

- The resource selection UE-1 detects and decodes other UE-0’s SCI to obtain reserved resources, the reservation period and priority;

- The UE-1 determine the UE-0’s RSRP, e.g., according to PSCCH DMRS configuration;

- UE-1 compares the UE-0’s RSRP to the RSRP threshold ThprioRX,prioTX;

- UE-1 does resource exclusion from the resource selection window.

From the above understanding, we have the following observations and proposal:

Observation 5: To support the sensing-based Scheme 2 resource allocation for SL-PRS, at least the following information is needed:

- Reservation period;

- Priority.

Observation 6: To support the sensing-based Scheme 2 resource allocation for SL-PRS, RSRP measurement should be defined.

Proposal 11: The SL-PRS is transmitted together with the scheduling PSCCH in the dedicated resource pool, and at least the following information is carried by the SCI:

- Reservation period;

- Priority.

Random resource selection

In the last meeting, feature lead organized some email discussion on the potential resource selection mechanism. The last proposal for discussion is copy as following from [3]:

[CLOSED] Feature Lead Proposal 5.2.2.A-v3 For Scheme 2, with regards to Resource selection mechanism for SL-PRS, pick one or more of the following options: - Option 1: A sensing based resource selection should be introduced - FFS: whether the above sensing is based on SCI or based on SL-PRS, or based on both SCI and SL-PRS. - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs (including IUC scheme) for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed. - FFS: Option 3: whether an additional option where Higher-layer resource selection could be included

Sidelink positioning/ranging can be used for V2X, IIoT, public safety and commercial use cases. In some cases, the UE participating sidelink positioning may be sensitive to power consumption. And furthermore, in V2X use cases, some VRU may or may not have the receive capability, but its location information is also important to the cars in some cases. This issue has been heavily discussed in sidelink enhancement in Rel-17. To support this kind of UE communication, random selection is supported for UE autonomous resource allocation. In Rel-16, random selection is also supported to be configured in the exceptional transmission resource pool. Hence option 1 and option 2 is supported.

Proposal 12: Both sensing based and random selection in Scheme 2 are supported according to UE capability or configuration.

For whether or not support option 3, we understand that the current resource selection is performed by the UE higher layers based on the measurement provided by the physical layer. It is still not clear what Option 3 entails. In general, we do not think that it is needed to define the higher layer signaling between UEs for the purpose of resource selection, but it could be further investigated whether intra-UE cross-layer interaction is needed for resource selection.

If random selection is supported, we think that is should be enabled by higher layers. As already specified for SL communication, the random selection is allowed in ‘sl-AllowedResourceSelectionConfig’, and it is the UE implementation to decide whether perform random selection for the legacy Rel-17 UE.  For random selection of SL-PRS, we can reuse the legacy design above. Hence, we have proposal as:

Proposal 13: Whether sensing-based or random selection in Scheme 2 is used should base on the (pre-)configuration and UE implementation.

Time domain behaviour

In RAN1#110bis-e [1], SL-PRS time domain behavior has been discussed and we have the following agreement.

Agreement With regards to the SL-PRS time domain behavior, at least study the following behaviors from Tx UE perspective: - Periodic SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity - FFS: any additional details, including whether or not higher layers can start/stop transmission. - Semi-persistent SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation - FFS: any additional details - Aperiodic SL-PRS - SL-PRS is transmitted at least once after [triggering/request] - Note: the brackets in the above means that companies are encouraged to study further whether “triggering” and/or “request” should be used and provide their definitions. - FFS: any additional details - FFS: Applicability of the above time behaviors for scheme 1 & scheme 2 - FFS: Rx UE behavior is separately discussed. - FFS: What mechanism(s) are used for activation/deactivation/triggering is part of the study

For sidelink communication, the resource reservation interval will be delivered from UE’s higher layer to PHY. Then the UE’s PHY will indicate the given resource reservation interval in the SCI. Furthermore, the resource reservation interval value can be switched to 0 if there are some resource selection limitation, e.g. resource reselection happened. From this point, any UE’s higher layer can change or switch the delivered resource reservation interval value according to its traffic characteristic or its own implementation.

Observation 7: Different resource reservation interval values can be used by SCI which associates with different SL-PRS time domain behavior (periodic/semi-persistent/aperiodic SL-PRS).

Proposal 14: From scheme 2 perspective, all the SL-PRS time domain behavior (periodic/semi-persistent/aperiodic SL-PRS) can be supported.

Multiple SL-PRS transmission for RTT positioning

For RTT based positioning, typically it needs two UEs take turns to transmit SL-PRS and measure the signal as shown in Figure 6. In order to ensure the RTT functionality for the SL-PRS transmission, the transmitting UE-A should request UE-B for SL-PRS reverse transmission during an RTT session. For SL-PRS transmission triggering, lower layer signaling (i.e. SCI/MAC-CE) from other UE can also be used. Hence the SL-PRS transmission request can be at least carried by SL MAC-CE or SCI, while whether the request itself and/or the detailed time/frequency resource can be conveyed in the corresponding signaling can be left in the work item phase.

PRS 3

PRS 2

PRS 1

UE-A

UE-B

One RTT session

Figure 6 One RTT session includes multiple SL-PRS resources

Observation 8: For RTT-based SL positioning, there are several associated SL-PRS transmissions.

Proposal 15: Support to request SL-PRS transmission from UE-A to UE-B via SL MAC-CE or SCI.

The request of SL-PRS transmissions can be done jointly with the transmission or reservation by UE-A via inter-UE coordination, which can make all the associated SL-PRS transmission resources within an expected time window. By this way, the transmission delay can be reduced compared to separate resource selection from every one-shot transmission.

Proposal 16: For one RTT session, the inter-UE coordination mechanism can be used to indicate the associated SL-PRS transmission among different UEs, e.g.

- The transmitting UE can reserve the next associated SL-PRS resources for itself within the reserved period;

- The transmitting UE can allocate/recommend the associated SL-PRS resources for the responding UE within the reserved period.

Power Control for SL-PRS transmission

In RAN1#109e [2], SL-PRS power control scheme has been discussed and we have the following agreement.

Agreement Study power control mechanisms for SL-PRS transmission, including whether it is necessary.

In NR sidelink, the power control is open-loop which can be based on DL pathloss or SL pathloss or both. For SL PRS, the interference with UL transmission could also be considered like SL communication. So the DL pathloss-based power control can be enabled for SL PRS. In addition, the distance between Tx UE and Rx UE is an important factor in determining the transmit power. In order to decrease the interference to other UEs and satisfy the transmission requirement, the SL pathloss-based power control can also be considered in SL-PRS. The power control mechanism of PSSCH can be reused for SL-PRS. So, we have the following proposals.

Proposal 17: Support to design SL-PRS power control scheme based on DL pathloss and SL pathloss.

For the SL pathloss-based power control, the Tx UE requires the feedback of the Rx UE for the estimation of SL pathloss. In SL communication, the feedback is the average RSRP based on the PSSCH DMRS transmitted by Tx UE. Considering SL-PRS could be in a different resource pool than for SL data, we think SL-PRS could also be utilized as pathloss reference signal. Further, the type of pathloss reference signal and related information like PRS resource ID or resource set ID could be indicated to UE in SL power control configuration information.

Proposal 18: Support to use SL PRS as the pathloss reference signal.

Identified specification impact

We believe the summary of specification impact of SL positioning solution should consider the following items:

A new SL positioning reference signal with comb-based structure and Gold sequence should be supported. Details can be further investigated in the normative phase, including

  - Number of symbols, the RE offset sequence, AGC symbol, RxTx turnaround time

SL-TDOA, SL-AoA, SL-AoD, and SL RTT-like positioning methods and measurements to support the positioning methods should be supported.

The physical layer procedure to support the SL positioning methods should be supported.

  - The resource configuration/indication/reservation is based on higher layer signaling and lower layer signaling

  - Dedicated resource pool for SL-PRS transmission should be supported. The presence of PSCCH in the dedicated resource pool is can be further investigated in the normative phase.

  - Shared resource pool for SL-PRS transmission with Rel-16/Rel-17/Rel-18 SL communication should be further investigated in the normative phase, where the backward compatibility with Rel-16/Rel-17 UE should be ensured.

  - Resource allocation scheme 1 should be supported

  - Resource allocation scheme 2 should be supported, where both sensing based and random selection based modes should be supported, and inter-UE coordination and congestion control can be further investigated in the normative phase.

  - SL-PRS transmission request between two UEs should be supported, where at least SL MAC-CE and SCI-based scheme should be considered.

  - SL-PRS power control mechanism should be further investigated in the normative phase.

Conclusion

In this contribution, we provided our views on the solutions to support SL positioning. Based on the discussion, we have the following observations and proposals.

Observation 1: There is no blocking issue on the feasibility of AoD since AoA already requires coherent capability at UE side.

Observation 2: Enabling SL-AoD allows a deployment of anchor UE with transmission capabilities, when the target UE can use locally estimated AoD for UE-based positioning, which benefits low latency.

Observation 3: The legacy design for UE autonomous resource allocation in sidelink R16/R17 should be reused as much as possible.

Observation 4: A unified Scheme 2 mechanism and related signaling framework should be considered to support different sidelink positioning methods.

Observation 5: To support the sensing-based Scheme 2 resource allocation for SL-PRS, at least the following information is needed:

- Reservation period;

- Priority.

Observation 6: To support the sensing-based Scheme 2 resource allocation for SL-PRS, RSRP measurement should be defined.

Observation 7: Different resource reservation interval values can be used by SCI which associates with different SL-PRS time domain behavior (periodic/semi-persistent/aperiodic SL-PRS).

Observation 8: For RTT-based SL positioning, there are several associated SL-PRS transmissions.

- Proposal 1: For RTT-type solutions using SL, we support Alt.2 (from the previous agreement): it may correspond to either a single-sided or double-sided RTT method.

- For double-sided RTT based method, support not to restrict the SL-PRS transmission order.

- Proposal 2: Phase difference based AoD positioning should be supported for SL positioning.

Proposal 3: Support report free RTT based on adjusting the transmission time of SL-PRS.

- Proposal 4: To support the SL-AoD positioning method, the following measurement should be introduced:

- SL Phase measurement of the first path

Proposal 5: Support to introduce a repetition structure with the last symbol being the same RE offset as the first symbol.

- Proposal 6: For channel structure of the dedicated resource pool, do not support Opt. 3, and leave other details of Opt.1 and Opt.2 for the WI phase.

- Proposal 7: For bandwidth of SL-PRS transmission in the dedicated resource pool, support Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool.

- Proposal 8: For SL signaling of the reservation/indication of SL-PRS resource(s), do not further support SL-MAC-CE or higher-layer signaling.

- Proposal 9: For broadcast of SL-PRS transmission, no need to discuss or confirm the working assumption in SI phase and it is consequently not supported for the normative work.

- Proposal 10: For Scheme 1 SL-PRS resource allocation, support Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB.

Proposal 11: The SL-PRS is transmitted together with the scheduling PSCCH in the dedicated resource pool, and at least the following information is carried by the SCI:

- Reservation period;

- Priority.

Proposal 12: Both sensing based and random selection in Scheme 2 are supported according to UE capability or configuration.

Proposal 13: Whether sensing-based or random selection in Scheme 2 is used should base on the (pre-)configuration and UE implementation.

Proposal 14: From scheme 2 perspective, all the SL-PRS time domain behavior (periodic/semi-persistent/aperiodic SL-PRS) can be supported.

Proposal 15: Support to request SL-PRS transmission from UE-A to UE-B via SL MAC-CE or SCI.

Proposal 16: For one RTT session, the inter-UE coordination mechanism can be used to indicate the associated SL-PRS transmission among different UEs, e.g.

- The transmitting UE can reserve the next associated SL-PRS resources for itself within the reserved period;

- The transmitting UE can allocate/recommend the associated SL-PRS resources for the responding UE within the reserved period.

Proposal 17: Support to design SL-PRS power control scheme based on DL pathloss and SL pathloss.

Proposal 18: Support to use SL PRS as the pathloss reference signal.

- Reference

- RAN1, Chair’s Notes RAN1#110 v17, RAN1#110bis-e, e-Meeting, 10th – 19th October, 2022.

- RAN1, Chair’s Notes RAN1#109e v19, RAN1#109-e, e-Meeting, 9th – 20th  May 2022.

- R1-2210598, Moderator Summary #3 on potential solutions for SL positioning, RAN1#110bis-e, e-Meeting, 10nd – 19th October, 2022.

- TS 38.331, NR Radio Resource Control (RRC) protocol specification (Release 17).

- Appendix

For the transmission scheme as shown in Figure 2, it is noted that UE-B transmits backward the SL-PRS after the two SL-PRS transmitted by UE-A. Timing drift can be modeled as

t14=1+eat14t23=1+ebt23t45=1+eat45t36=1+ebt36

First, we can observe that t14 should be equal to t23(without timing drift), i.e., the receiving time difference is equal to transmitting time difference. Based on this, we have

1+ea1+eb=t14t23

The RTT can be calculated as

RTT=t45-t14t23t36

=t45-1+ea1+ebt36

=1+eat45-1+ea1+eb(1+eb)t36

=1+ea(t45-t36)

=1+eaRTT

≈RTT

- Where RTT denotes real round trip time and the last step comes from that the timing drift, i.e. ea or eb, is typically at level of 10-6.  The timing error of RTT is eaRTT, which is less than 10-12 and could be neglected.