3GPP TSG RAN WG1 #111			R1-2211999

Toulouse, France, November 14th – 18th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on Multi-carrier UL Tx switching scheme

Agenda Item:	9.9.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#110bis-e meeting, some details on Rel-18 Multi-carrier UL Tx switching scheme including complexity reduction options, conditions where the switching gap is required, ambiguity issues, UE capabilities and RRC configurations were discussed and several agreements/working assumptions were made [1, 2]. In addition, it was agreed as working assumption that UL Tx switching schemes across up to 4 bands are specified in Rel-18.

In this contribution, we discuss on remaining details on Rel-18 Multi-carrier UL Tx switching schemes to complete RAN1 work on this objective.

- Discussion on supported switching cases for each scenario

In Rel-16 and Rel-17, following switching cases are supported. Although the supported switching cases are not explicitly captured in the specifications, TS38.214 section 6.1.6 specifies the conditions where the switching gap is required based on the supported switching cases.

- 1Tx-2Tx switching

  - Switched UL, SUL

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2)) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P | 1P+(0P+0P) |
| Case 2 | 0T+2T | 0P+2P, 0P+1P | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |

  - Dual UL

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2)) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, 0P+1P | 1P+(0P+0P), 1P+(1P+0P), 1P+(0P+1P), 1P+(1P+1P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P) |
| Case 2 | 0T+2T | 0P+2P, 0P+1P | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |

- 2Tx-2Tx switching

  - Switched UL, SUL

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2)) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- | --- |
| Case 2 | 0T+2T | 0P+2P, 0P+1P | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |
| Case 3 | 2T+0T | 2P+0P, 1P+0P | 2P+(0P+0P), 1P+(0P+0P) |

  - Dual UL

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2)) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, 0P+1P | 1P+(0P+0P), 1P+(1P+0P), 1P+(0P+1P), 1P+(1P+1P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P) |
| Case 2 | 0T+2T | 0P+2P, 0P+1P | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |
| Case 3 | 2T+0T | 2P+0P, 1P+0P | 2P+(0P+0P), 1P+(0P+0P) |

For Rel-18 UL Tx switching, following agreement regarding the supported switching cases was made at the last meeting.

Agreement Consider following alternatives on the supported switching cases (Tx chain states) for each scenario - Scenario#1: For switched UL, if UE supports up to 2 ports UL transmission on all the bands in the band combination, - Alt.1-1: only switching cases (Tx chain states) with 2T are assumed - In case of 3 bands, 3 switching cases ({2T,0T,0T}, {0T,2T,0T}, {0T,0T,2T}) are assumed - In case of 4 bands, 4 switching cases ({2T,0T,0T,0T}, {0T,2T,0T,0T}, {0T,0T,2T,0T}, {0T,0T,0T,2T}) are assumed - Alt.1-2: switching cases (Tx chain states) with 1T-1T can also be assumed - FFS: detailed switching cases to be assumed - Scenario#2: For switched UL, if UE supports up to 2 ports UL transmission only on some of the bands, - Alt.2-1: for the band where 2 ports UL transmission is not supported, switching cases (Tx chain states) with 1T-1T can be assumed - FFS: detailed switching cases to be assumed with different number of bands supporting up to 2 ports UL transmission - Alt.2-2: only switching cases (Tx chain states) with 2T are assumed - Assumed switching cases are same as Scenario#1 - Alt.2-3: switching cases (Tx chain states) with 1T-1T can also be assumed - FFS: detailed switching cases to be assumed - FFS: Scenario#3: For dual UL, if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission on all the bands in the band combination, - Alt.3-1: corresponding switching case(s) with 1T-1T for the band pair(s) are not assumed - FFS: if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission only on some of the bands - Alt.3-2: corresponding switching case(s) with 1T-1T for the band pair(s) are assumed - Assumed switching cases are same as the case where UE supports dual UL for all band pairs in the band combination

For scenario#1, we think it can be considered as extension of 2Tx-2Tx switched UL scenario with 3 or 4 bands. Even in Rel-17 2Tx-2Tx switched UL scenario with 2 bands, switching case with 1T-1T could be considered as it would be beneficial to avoid the switching gap when 1 port transmission on band A and 1 port transmission on band B are alternately performed. However, it was not supported. Therefore, for scenario#1, Alt.1-1 should be selected and RAN1 should avoid repeating same discussion already done in Rel-17.

Proposal 1: For switched UL, if UE supports up to 2 ports UL transmission on all the bands in the band combination, only switching cases (Tx chain states) with 2T are assumed.

For scenario#2, we think it can be considered as similar scenario with 1Tx-2Tx switched UL scenario since up to 2 ports UL transmission is not supported at least on some of the bands. As summarized above, in case of 1Tx-2Tx switched UL with 2 bands, switching case with 1T-1T is assumed and switching case with 2T for a band where up to 2 ports UL transmission is not supported is not assumed. In that sense, Alt.2-1 would be aligned with such principle and supported switching cases can be as below.

- Ex1: For UL Tx switching across 3 bands (A, B, C), if band A supports up to 2 ports UL transmission while band B and C do not support up to 2 ports UL transmission, supported switching cases are ({2T, 0T, 0T} and {0T, 1T, 1T}).

In above example, supported switching cases can be simply derived and it would be beneficial as number of switching cases is minimized. On the other hand, supported switching cases for following other examples may be ambiguous if Alt.2-1 principle is applied.

- Ex2: For UL Tx switching across 3 bands (A, B, C), if all the bands do not support up to 2 ports UL transmission, supported switching cases are ({1T, 1T, 0T}, {1T, 0T, 1T} and {0T, 1T, 1T}) although it causes ambiguous switching state issue? Or, supported switching cases are ({1T, 1T, 0T} and {0T, 0T, 2T}) although band A/B and C are treated differently?

- Ex3: For UL Tx switching across 3 bands (A, B, C), if band A and B support up to 2 ports UL transmission while band C does not support up to 2 ports UL transmission, supported switching cases are ({2T, 0T, 0T}, {0T, 2T, 0T}, {0T, 1T, 1T} and {1T, 0T, 1T}) although it also causes ambiguous switching state issue? Or, supported switching cases are ({2T, 0T, 0T}, {0T, 2T, 0T} and {0T, 1T, 1T}) although band A and B are treated differently?

On the other hand, Alt.2-2 is quite simple that for all above examples, supported switching cases are ({2T, 0T, 0T}, {0T, 2T, 0T} and {0T, 0T, 2T}) same as scenario#1 (Alt.1-1). In addition, it may be possible for some implementation that 2 Tx chains are used even for 1 port transmission in one band e.g., for high power transmission, and hence even if up to 2 ports UL transmission is not supported for a band, switching case with 2T on the band may be beneficial for such implementation.

Therefore, in terms of the simplicity and implementation flexibility, Alt.2-2 would be better choice for scenario#2. Alternatively, it would be possible to consider UE capability to differentiate Alt.2-1 and Alt.2-2 so that UE can minimize the number of supported switching cases as in Ex1 or UE can achieve the implementation flexibility such as using 2 Tx chains for 1 port transmission.

Proposal 2: For switched UL, if UE supports up to 2 ports UL transmission only on some of the bands in the band combination, following alternatives can be considered.

- Alt.1: only switching cases (Tx chain states) with 2T are assumed.

- Alt.2: UE capability to differentiate following assumptions is introduced.

  - Assumption 1: only switching cases (Tx chain states) with 2T are assumed

  - Assumption 2: for the band where 2 ports UL transmission is not supported, switching cases (Tx chain states) with 1T-1T can be assumed while switching case with 2T on the band is not assumed

    - If there is only one band where 2 ports UL transmission is not supported, supported switching cases (Tx chain states) are ({1T, 1T, 0T}, {0T, 2T, 0T}, {0T, 0T, 2T}) in case of 3 bands and ({1T, 1T, 0T, 0T}, {0T, 2T, 0T, 0T}, {0T, 0T, 2T, 0T}, {0T, 0T, 0T, 2T}) in case of 4 bands

    - If there are 2 bands where 2 ports UL transmission is not supported, supported switching cases (Tx chain states) are ({1T, 1T, 0T}, {0T, 0T, 2T}) in case of 3 bands and ({1T, 1T, 0T, 0T}, {0T, 0T, 2T, 0T}, {0T, 0T, 0T, 2T}) in case of 4 bands

    - If there are 3 bands where 2 ports UL transmission is not supported, supported switching cases (Tx chain states) are ({1T, 1T, 0T}, {0T, 0T, 2T}) in case of 3 bands and ({1T, 1T, 0T, 0T}, {0T, 0T, 2T, 0T}, {0T, 0T, 0T, 2T}) in case of 4 bands

    - If there are 4 bands where 2 ports UL transmission is not supported, supported switching cases (Tx chain states) are ({1T, 1T, 0T, 0T}, {0T, 0T, 1T, 1T}) in case of 4 bands

For scenario#3, it should be common understanding that this scenario is to be supported according to the following agreement made at the last meeting.

Agreement If Rel-18 UL Tx switching for 3 or 4 bands with dual UL is supported, UE is allowed to support only some of band pairs for concurrent UL transmission based on UE capability - The supported band pair for concurrent transmission requires the support of UL CA on the corresponding band pair(s) by the UE - Details on the UE capability such as how to report the support of dual UL and the supported band pair(s) for concurrent UL transmission are further discussed - Details on the gNB configuration/indication such as how to indicate the band pair(s) UE should expect for concurrent UL transmission are further discussed - Note: UE is also allowed to support all band pairs for concurrent transmission, and the design of Rel-18 UL Tx switching for 3 or 4 bands with dual UL does not impose any restriction

In addition, for the band pair that UE does not support concurrent UL transmission, the UE shall support switching between the bands in the band pair according to the following agreement.

Agreement Confirm the following working assumption made at the RAN1#110 meeting. Working Assumption If Rel-18 UL Tx switching is supported, following switching mechanism is considered as baseline for the Rel-18 UL Tx switching across 3 or 4 bands - Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via dynamic grant and/or RRC configuration for UL transmission

Based on above understanding, Alt.3-1 and Alt.3-2 for scenario#3 can be discussed. In case that UE supports concurrent transmission on all band pairs in the band combination, supported switching cases are as below.

- In case of 3 bands, supported switching cases are ({2T, 0T, 0T}, {0T, 2T, 0T}, {0T, 0T, 2T}, {1T, 1T, 0T}, {1T, 0T, 1T}, {0T, 1T, 1T})

- In case of 4 bands, supported switching cases are ({2T, 0T, 0T, 0T}, {0T, 2T, 0T, 0T}, {0T, 0T, 2T, 0T}, {0T, 0T, 0T, 2T}, {1T, 1T, 0T, 0T}, {1T, 0T, 1T, 0T}, {1T, 0T, 0T, 1T}, {0T, 1T, 1T, 0T}, {0T, 1T, 0T, 1T}, {0T, 0T, 1T, 1T})

In case of Alt.3-2, supported switching cases are same as above irrespective of band pair(s) where concurrent transmission is not supported. It would be simple and 1T-1T for the band pair would be anyway useful to avoid the switching gap when 1 port transmission on each band in the band pair is alternately performed.

On the other hand, Alt.3-1 can reduce the number of supported switching cases, and it can be a part of solution for the ambiguous switching state issue.

Therefore, similar to scenario#2, Alt.3-2 would be better choice in terms of simplicity while Alt.3-1 may be beneficial for some UE implementation where the number of supported switching cases need to be reduced as much as possible. It would be possible to consider UE capability to differentiate Alt.3-1 and Alt.3-2.

Proposal 3: For dual UL, if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission on all the bands in the band combination, following alternatives can be considered.

- Alt.1: all possible switching cases (Tx chain states) are assumed irrespective of band pair(s) where concurrent transmission is not supported.

- Alt.2: UE capability to differentiate following assumptions is introduced.

  - Assumption 1: all possible switching cases (Tx chain states) are assumed irrespective of band pair(s) where concurrent transmission is not supported

  - Assumption 2: switching case(s) with 1T-1T for the band pair(s) where concurrent transmission is not supported are not assumed

There is a FFS on the case where UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission only on some of the bands. It is a mixed scenario between scenario#2 and #3. Therefore, combination of solutions in Proposal 2 and 3 can be considered for such mixed scenario.

Proposal 4: For dual UL, if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission only on some of the bands in the band combination, following alternatives can be considered.

- Alt.1: all possible switching cases (Tx chain states) are assumed irrespective of band pair(s) where concurrent transmission is not supported and band(s) where up to 2 ports UL transmission is not supported.

- Alt.2: UE capabilities to differentiate following assumptions are introduced

  - Assumption 1: all possible switching cases (Tx chain states) are assumed irrespective of band pair(s) where concurrent transmission is not supported and band(s) where up to 2 ports UL transmission is not supported

  - Assumption 2: switching case(s) with 1T-1T for the band pair(s) where concurrent transmission is not supported are not assumed, while switching cases with 2T on each band in the band pair are assumed even if up to 2 ports UL transmission is not supported for the band(s)

  - Assumption 3: switching case(s) with 1T-1T even for the band pair(s) where concurrent transmission is not supported are assumed, while switching case with 2T on a band in the band pair is not assumed if up to 2 ports UL transmission is not supported for the band

- Discussion on applicable scenarios for the new switching gap conditions

As mentioned in the previous section, conditions where the switching gap is required are specified in TS38.214 section 6.1.6. At the last RAN1 meeting, following agreement was made regarding the switching gap conditions.

Agreement If Rel-18 UL Tx switching for 3 or 4 bands is supported, following is considered as baseline. - Existing conditions where the switching period is required can be reused for Rel-18 UL Tx switching with 3 or 4 bands when only two bands are involved in a switching - New conditions where the switching period is required should be introduced for Rel-18 UL Tx switching with 3 or 4 bands when more than two bands are involved in a switching - For dual UL, following new conditions are considered - When the UE is to transmit a 1-port or 2-port transmission on one uplink carrier on one band (1st band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (2nd and 3rd band) - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 2T on a carrier on another band (3rd band) - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on one of the bands and another different band (1st or 2nd band, and 3rd band) - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (3rd and 4th band) - FFS for switched UL and/or for the case with complexity reduction option 1 or 2 - FFS the same or different switch period for existing conditions and new conditions

In the above agreement, it is FFS whether the agreed new conditions are applicable to switched UL scenarios and/or scenarios with complexity reduction option 1/2. The agreed new conditions are about switching between switching case with 1T-1T associated with two bands and switching case with 1T-1T or 2T associated with different band(s) from the two bands. Therefore, new conditions are applicable to scenario(s) where switching case(s) with 1T-1T is/are supported.

Based on the discussion in the previous section, we propose that scenario#1(switched UL, all bands support up to 2 ports) assumes only switching cases with 2T, and hence the new conditions are not applicable to the scenario#1. For other scenarios, we propose two alternatives as in Proposal 2/3/4. In case of Alt.1 or Assumption 1 in Alt.2, the new conditions are applicable to dual UL scenarios and are not applicable to switched UL scenarios. In case of Assumption 2/3 in Alt.2, some of new conditions are applicable to specific switched UL scenarios and some of new conditions may not be applicable to specific dual UL scenarios. Therefore, applicable scenarios for the new switching gap conditions should be discussed once supported switching cases for each scenario are determined.

Proposal 5: Applicable scenarios for the new conditions where the switching gap is required should be discussed once supported switching cases for each scenario are determined.

- In scenario#1 (switched UL, all bands support up to 2 ports), new conditions are not applicable

- In case of Alt.1 or Assumption 1 in Alt.2 in Proposal 2 for scenario#2 (switched UL, only some of the bands support up to 2 ports), new conditions are not applicable

- In case of Assumption 2 in Alt.2 in Proposal 2 for scenario#2 (switched UL, only some of the bands support up to 2 ports), some or all of new conditions may be applicable

- In case of Alt.1 or Assumption 1 in Alt.2 in Proposal 3 for scenario#3 (dual UL, only some of the band pairs support concurrent transmission, all bands support up to 2 ports), new conditions are applicable

- In case of Assumption 2 in Alt.2 in Proposal 3 for scenario#3 (dual UL, only some of the band pairs support concurrent transmission, all bands support up to 2 ports), some of new conditions may not be applicable

- In case of Alt.1 or Assumption 1 in Alt.2 in Proposal 4 for mixed scenario#2/3 (dual UL, only some of the band pairs support concurrent transmission, only some of the bands support up to 2 ports), new conditions are applicable

- In case of Assumption 2 or 3 in Alt.2 in Proposal 4 for mixed scenario#2/3 (dual UL, only some of the band pairs support concurrent transmission, only some of the bands support up to 2 ports), some of new conditions may not be applicable

There is another FFS on whether same or different switch period is applied for existing conditions and new conditions. We think it should be based on the reported switching period for each band pair according to the RAN4 LS below [3]. Anyway, this FFS point should be discussed in RAN4 as well as other points regarding same or different switching period.

Agreement: On the length of switching period: - For UL switching period with Tx switching across 3 or 4 bands, RAN4 agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL. - The length of switching period is applied per band pair for each band combination. - For each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting, which is similar as in Rel-17. - Note: For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured. - For the same band pair, RAN4 has not concluded on whether the same or a different value can be reported for the specific band pair supporting Tx switching across 3 or 4 bands in Rel-18 compared to Tx switching across 2 bands specified in Rel-16/17.

- Discussion on solution for potential ambiguous state issue

Following working assumption was made at the last RAN1 meeting regarding the potential ambiguous state issue.

Working Assumption At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases - Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported - if twoT is indicated, both of two Tx chains are switched to band B - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A - Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported - if twoT is indicated, both of two Tx chains are switched to band C - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS - Alt.1: based on gNB’s configuration/indication e.g., new RRC parameter - Alt.2: based on predefined rule - Other alternative is not precluded - FFS for other potential cases

As described in the working assumption, existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState in CellGroupConfig can solve at least the ambiguous state issue in Case#1 where only two bands are involved in a switching. Therefore, even for Rel-18 UL Tx switching with 3 or 4 bands, it should be reused, and the working assumption should be confirmed.

In Case#2 where three bands are involved in a switching, as described in the working assumption, existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState in CellGroupConfig is not sufficient to solve the ambiguous state issue as oneT would have multiple possible switching cases (Tx chain states). There are two alternatives in the working assumption to cope with such issue on oneT in Case#2, and we think Alt.1 (how to determine the associated band for another Tx chain is based on gNB’s configuration/indication via new RRC parameter) is preferable in terms of the flexibility. For example, two prioritized bands (serving cells) can be configured in case of oneT so that UE can determine the associated band for another Tx chain as below.

- If next transmission is 1 port transmission on the first prioritized band, another Tx chain is associated with second prioritized band.

- If next transmission is 1 port transmission on the band other than the first prioritized band, another Tx chain is associated with first prioritized band.

There is a FFS on other potential cases. However, we don’t find any other case than Case#1 and #2. On the other hand, whether each of Case#1 and #2 is only applicable to dual UL scenarios or also applicable to switched UL scenarios is dependent on the supported switching cases for switched UL scenarios as discussed in the section 2. In case of Alt.1 or Assumption 1 in Alt.2 in Proposal 2, there is no ambiguous state issue for switched UL scenarios. In case of Assumption 2 in Alt.2 in Proposal 2, there may be some case where the ambiguous state issue exists, e.g., if there is only one band where 2 ports UL transmission is not supported, supported switching cases (Tx chain states) are ({1T, 1T, 0T}, {0T, 2T, 0T}, {0T, 0T, 2T}) in case of 3 bands and ({1T, 1T, 0T, 0T}, {0T, 2T, 0T, 0T}, {0T, 0T, 2T, 0T}, {0T, 0T, 0T, 2T}) in case of 4 bands, and hence 1 port transmission on band B can be realized by multiple switching cases.

Proposal 6: Following working assumption can be confirmed with updates.

Working Assumption

At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases

- Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported

  - if twoT is indicated, both of two Tx chains are switched to band B

  - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A

- Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported

  - if twoT is indicated, both of two Tx chains are switched to band C

  - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS

    - Alt.1: determined based on gNB’s configuration/indication of two prioritized bands (serving cells)e.g., new RRC parameter

      - If next transmission is 1 port transmission on the first prioritized band, another Tx chain is associated with second prioritized band.

      - If next transmission is 1 port transmission on the band other than the first prioritized band, another Tx chain is associated with first prioritized band.

    - Alt.2: based on predefined rule

    - Other alternative is not precluded

FFS for other potential cases

- Discussion on potential restriction on number of supported bands for up to 2 ports transmission

Following agreement was made at the last RAN1 meeting regarding complexity reduction option 2.

Agreement If Rel-18 UL Tx switching for 3 or 4 bands is supported, UE is allowed to support only some of band(s) for up to 2 ports UL transmission based on UE capability - Further down-select from the following alternatives - Alt.1: no restriction for both switched UL and dual UL and for both 3 bands and 4 bands - Alt.2: at least one band should support up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands - Alt.3: at least two bands should support up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands - Details on the UE capability such as whether existing per-FS UL-MIMO capability can be reused or not are further discussed - Details on the gNB configuration/indication such as whether/how to additionally indicate 2 ports UL transmission mode for a band/cell are further discussed - Existing MIMO mechanism for MIMO mode indication should be reused - Note: UE is also allowed to support all bands for up to 2 ports UL transmission, and the design of Rel-18 UL Tx switching for 3 or 4 bands does not impose any restriction

In the agreement, three alternatives on the potential restriction on number of supported bands for up to 2 ports transmission are captured. Based on the further discussion, it was proposed to take Alt.1 as compromised way forward, but it was not agreed as some company had concern on the impact of performance if UE only supports up to 1 port transmission.

Although it can be discussed as part of UE feature discussion, we would like to provide our view on this issue. Basically, we also have the concern on the performance impact of Alt.1. In Rel-16/17, at least one band supports up to 2 ports transmission in both switched UL and dual UL scenarios. Therefore, we considered Alt.2 would be reasonable assumption for Rel-18 UL Tx switching across 3 or 4 bands so that performance gain over Rel-16/17 UL Tx switching can be ensured. However, there are several companies strongly prefer Alt.1 based on the concern on complexity increase in Rel-18 UL Tx switching due to increased number of bands. Therefore, we would like to propose another potential compromise that Alt.2 is applied to switched UL with 3 bands while Alt.1 is applied to all other scenarios as complexity increase would be concerned.

Proposal 7: Following potential restriction on number of supported bands for up to 2 ports transmission is applied.

- In case of switched UL with 3 bands, at least one band should support up to 2 ports UL transmission.

- In case of switched UL with 4 bands and dual UL with 3 or 4 bands, there is no restriction on the number of bands supporting up to 2 ports UL transmission.

In addition, above agreement describes that details on UE capability and gNB configuration for this complexity reduction mechanism are further discussed. We think the details can be discussed in RAN2.

- Discussion on potential minimum separation time between two UL Tx switchings

Following working assumption was made at the last RAN1 meeting regarding potential minimum separation time between two UL Tx switchings as additional complexity reduction mechanism.

Working assumption Study the following alternatives for the minimum separation time between two UL Tx switchings for Rel-18 UL Tx switching schemes across up to 3 or 4 bands, and decide in RAN1#111 whether/which of the following alternatives is needed - Alt.1: define 14 symbols based on a SCS (FFS on SCS) as minimum separation time between two UL Tx switchings - Alt.2: define that no more than one uplink Tx switching within a reference slot based on a SCS (FFS on SCS) - Alt.3: define X slots as minimum separation time between two UL Tx switchings where 3 bands are involved in total, and define Y slots as minimum separation time between two UL Tx switchings where 4 bands are involved in total, where X and/or Y is no less than 1 (FFS on X,Y, FFS reference SCS for the slots in case of multiple SCSs across carriers or expressed in unit of micro second) - Alt.4: report the minimum separation time for different switching cases - Other alternative is not precluded - FFS: Applicable cases for the restriction - Note: Companies are encouraged to provide detailed numbers of minimum separation time

In Rel-16/17, the UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL, 1, µUL, 2). It seems straightforward to adapt at least this general restriction to Rel-18 UL Tx switching across 3 or 4 bands as well, i.e., Alt.2 in above working assumption. Regarding the SCS for the reference slot, the following extended definition based on Rel-16/17 definition for 2 bands case can be considered.

- µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, where µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active bandwidth parts of the bands in the band combination

However, there are proposals to define “minimum separation time” so that the minimum separation between two UL Tx switchings is ensured. Based on the above general restriction like Alt.2, although the number of UL Tx switchings within a reference slot is limited to up to one, two UL Tx switchings in consecutive slots can be occurred within a short time such as within 14 symbols. Therefore, in some specific cases, the introduction of minimum separation time would be beneficial to address potential UE complexity concern e.g., when more than two bands are involved for a switching, sufficient time separation from the previous UL Tx switching would allow UE to perform some preparation. In that sense, we are open to consider additional restriction based on the minimum separation time only for specific cases where such minimum separation time is really needed.

Proposal 8: Following restriction is applied for Rel-18 UL Tx switching across 3 or 4 bands.

- The UE does not expect to perform more than one uplink switching in a reference slot based on µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, where µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active bandwidth parts of the bands in the band combination

- Additionally, when the UE is to perform uplink switching where more than two bands are involved, the UE does not expect to perform the uplink switching unless it is equal to or larger than [14] symbols based on µUL  separated from the last uplink switching, where µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, and µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active bandwidth parts of the bands in the band combination

- Discussion on potential ambiguity issue on switching period location

Following proposal was discussed at the last RAN1 meeting regarding potential ambiguity issue on switching period location, but it was not agreed mainly due to the bracket part [2].

- Study on how to reuse Rel-16/17 approach to determine the switching period location [when the scheduled gap between two transmissions is smaller than the reported switching gap] i.e., semi-static configuration of switching period location on one of the bands for each switching band pair, and consider following options to solve the potential ambiguity issue on the switching period location for decision in RAN1#111 - Opt.0: Switching period location can be determined based on the indication of switching period location per band pair - Opt.1: Switching period location can be determined based on predefined rule such as switch-from or switch-to - Opt.2: Switching period location can be determined or configured based on specific band(s) - Opt.3: Switching period location can be determined based on the indication of switching period location {switch-from, switch-to} - Opt.4: Switching period location can be determined based on the priority list of bands configured to the UE - Opt.5: Switching period location can be determined based on the indication of switching period location {switch-from, switch-to} per band pair - Other option is not precluded - FFS whether RAN1 spec impact is needed

Regarding this issue, RAN4 informed RAN1 below in their LS [4].

Issue 4: Location of switching period - For single-TAG case, RAN4 agreed to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1. - Meanwhile, RAN4 has not concluded on the switching period location for 2-TAG case, with further discussions ongoing.

Therefore, further details on how to reuse Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) should be discussed in RAN1.

In Rel-16/17, semi-static configuration of switching period location (i.e., uplinkTxSwitchingPeriodLocation-r16) is provided in ServingCellConfig of one of the band between two bands, so that switching period is located on the band/serving cell carrier where uplinkTxSwitchingPeriodLocation-r16 is configured (i.e., as TRUE). In case of 3 or 4 bands, there would be the case where both bands involved for a switching has uplinkTxSwitchingPeriodLocation-r16 TRUE or both have uplinkTxSwitchingPeriodLocation-r16 FALSE, so that UE cannot determine on which band/serving cell carrier the switching period is located. In addition, when more than 2 bands are involved for the switching, it is unclear how to determine the switching period location. Hence, in addition to the existing RRC parameter such as uplinkTxSwitchingPeriodLocation-r16 in ServingCellConfig, some additional rule or configuration would be necessary.

We think that Opt.4 in the above proposal e.g., configuring the priority list can solve above issues so that the UE can determine the switching period location.

Regarding the bracket part in above proposal, our understanding is that when the scheduled gap between two transmissions (between the end of the transmission before the switching and the start of another transmission after the switching) is longer than the reported switching period, switching period location would not be a matter i.e., anyway UE does not perform UL transmission within the scheduled gap.

Proposal 9: For potential ambiguity issue on the switching period location, existing semi-static configuration of switching period location (uplinkTxSwitchingPeriodLocation-r16 in ServingCellConfig) is reused, and additional semi-static configuration of priority list is also used to determine the switching period location.

- Further details on the configurations can be discussed and decided in RAN2

- Conclusion

In this contribution, we presented our views on remaining details on Rel-18 Multi-carrier UL Tx switching schemes. Based on the discussion in this contribution, following proposals were made.

Proposal 1: For switched UL, if UE supports up to 2 ports UL transmission on all the bands in the band combination, only switching cases (Tx chain states) with 2T are assumed.

Proposal 2: For switched UL, if UE supports up to 2 ports UL transmission only on some of the bands in the band combination, following alternatives can be considered.

- Alt.1: only switching cases (Tx chain states) with 2T are assumed.

- Alt.2: UE capability to differentiate following assumptions is introduced.

  - Assumption 1: only switching cases (Tx chain states) with 2T are assumed

  - Assumption 2: for the band where 2 ports UL transmission is not supported, switching cases (Tx chain states) with 1T-1T can be assumed while switching case with 2T on the band is not assumed

    - If there is only one band where 2 ports UL transmission is not supported, supported switching cases (Tx chain states) are ({1T, 1T, 0T}, {0T, 2T, 0T}, {0T, 0T, 2T}) in case of 3 bands and ({1T, 1T, 0T, 0T}, {0T, 2T, 0T, 0T}, {0T, 0T, 2T, 0T}, {0T, 0T, 0T, 2T}) in case of 4 bands

    - If there are 2 bands where 2 ports UL transmission is not supported, supported switching cases (Tx chain states) are ({1T, 1T, 0T}, {0T, 0T, 2T}) in case of 3 bands and ({1T, 1T, 0T, 0T}, {0T, 0T, 2T, 0T}, {0T, 0T, 0T, 2T}) in case of 4 bands

    - If there are 3 bands where 2 ports UL transmission is not supported, supported switching cases (Tx chain states) are ({1T, 1T, 0T}, {0T, 0T, 2T}) in case of 3 bands and ({1T, 1T, 0T, 0T}, {0T, 0T, 2T, 0T}, {0T, 0T, 0T, 2T}) in case of 4 bands

    - If there are 4 bands where 2 ports UL transmission is not supported, supported switching cases (Tx chain states) are ({1T, 1T, 0T, 0T}, {0T, 0T, 1T, 1T}) in case of 4 bands

Proposal 3: For dual UL, if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission on all the bands in the band combination, following alternatives can be considered.

- Alt.1: all possible switching cases (Tx chain states) are assumed irrespective of band pair(s) where concurrent transmission is not supported.

- Alt.2: UE capability to differentiate following assumptions is introduced.

  - Assumption 1: all possible switching cases (Tx chain states) are assumed irrespective of band pair(s) where concurrent transmission is not supported

  - Assumption 2: switching case(s) with 1T-1T for the band pair(s) where concurrent transmission is not supported are not assumed

Proposal 4: For dual UL, if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission only on some of the bands in the band combination, following alternatives can be considered.

- Alt.1: all possible switching cases (Tx chain states) are assumed irrespective of band pair(s) where concurrent transmission is not supported and band(s) where up to 2 ports UL transmission is not supported.

- Alt.2: UE capabilities to differentiate following assumptions are introduced

  - Assumption 1: all possible switching cases (Tx chain states) are assumed irrespective of band pair(s) where concurrent transmission is not supported and band(s) where up to 2 ports UL transmission is not supported

  - Assumption 2: switching case(s) with 1T-1T for the band pair(s) where concurrent transmission is not supported are not assumed, while switching cases with 2T on each band in the band pair are assumed even if up to 2 ports UL transmission is not supported for the band(s)

  - Assumption 3: switching case(s) with 1T-1T even for the band pair(s) where concurrent transmission is not supported are assumed, while switching case with 2T on a band in the band pair is not assumed if up to 2 ports UL transmission is not supported for the band

Proposal 5: Applicable scenarios for the new conditions where the switching gap is required should be discussed once supported switching cases for each scenario are determined.

- In scenario#1 (switched UL, all bands support up to 2 ports), new conditions are not applicable

- In case of Alt.1 or Assumption 1 in Alt.2 in Proposal 2 for scenario#2 (switched UL, only some of the bands support up to 2 ports), new conditions are not applicable

- In case of Assumption 2 in Alt.2 in Proposal 2 for scenario#2 (switched UL, only some of the bands support up to 2 ports), some or all of new conditions may be applicable

- In case of Alt.1 or Assumption 1 in Alt.2 in Proposal 3 for scenario#3 (dual UL, only some of the band pairs support concurrent transmission, all bands support up to 2 ports), new conditions are applicable

- In case of Assumption 2 in Alt.2 in Proposal 3 for scenario#3 (dual UL, only some of the band pairs support concurrent transmission, all bands support up to 2 ports), some of new conditions may not be applicable

- In case of Alt.1 or Assumption 1 in Alt.2 in Proposal 4 for mixed scenario#2/3 (dual UL, only some of the band pairs support concurrent transmission, only some of the bands support up to 2 ports), new conditions are applicable

- In case of Assumption 2 or 3 in Alt.2 in Proposal 4 for mixed scenario#2/3 (dual UL, only some of the band pairs support concurrent transmission, only some of the bands support up to 2 ports), some of new conditions may not be applicable

Proposal 6: Following working assumption can be confirmed with updates.

Working Assumption

At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases

- Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported

  - if twoT is indicated, both of two Tx chains are switched to band B

  - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A

- Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported

  - if twoT is indicated, both of two Tx chains are switched to band C

  - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS

    - Alt.1: determined based on gNB’s configuration/indication of two prioritized bands (serving cells)e.g., new RRC parameter

      - If next transmission is 1 port transmission on the first prioritized band, another Tx chain is associated with second prioritized band.

      - If next transmission is 1 port transmission on the band other than the first prioritized band, another Tx chain is associated with first prioritized band.

    - Alt.2: based on predefined rule

    - Other alternative is not precluded

FFS for other potential cases

Proposal 7: Following potential restriction on number of supported bands for up to 2 ports transmission is applied.

- In case of switched UL with 3 bands, at least one band should support up to 2 ports UL transmission.

- In case of switched UL with 4 bands and dual UL with 3 or 4 bands, there is no restriction on the number of bands supporting up to 2 ports UL transmission.

Proposal 8: Following restriction is applied for Rel-18 UL Tx switching across 3 or 4 bands.

- The UE does not expect to perform more than one uplink switching in a reference slot based on µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, where µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active bandwidth parts of the bands in the band combination

- Additionally, when the UE is to perform uplink switching where more than two bands are involved, the UE does not expect to perform the uplink switching unless it is equal to or larger than [14] symbols based on µUL  separated from the last uplink switching, where µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, and µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active bandwidth parts of the bands in the band combination

Proposal 9: For potential ambiguity issue on the switching period location, existing semi-static configuration of switching period location (uplinkTxSwitchingPeriodLocation-r16 in ServingCellConfig) is reused, and additional semi-static configuration of priority list is also used to determine the switching period location.

- Further details on the configurations can be discussed and decided in RAN2

References

- RAN1 Chair’s Notes, RAN1#110bis-e, October 2022.

- R1-2210723	Summary#4 of discussion on multi-carrier UL Tx switching scheme	Moderator (NTT DOCOMO, INC.)

- R1-2208000	Reply LS on UL Tx switching across 3 or 4 bands	RAN4

- R1-2210819	LS on Rel-18 UL Tx switching	RAN4