3GPP TSG-RAN WG1 Meeting #111	R1-2212345

Toulouse, France, November 14th – 18th 2022

Agenda Item:	9.8.2

Source:	Ericsson

Title:	Other aspects including control signaling and procedures in NCR

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss our view on required NCR-MT capabilities considering the NCR deployment, and the need of HARQ for PDCCH. In addition to this contribution, we discuss signalling aspects for the required control information in our accompanying contribution [2].

# Discussion

## NCR-MT’s beam management capability

For Rel-18 NCR, the following agreements regarding legacy CSI measurement/report mechanisms and sounding procedure was made in the RAN1#110bis-e meetings [3].

Agreement (RAN1#110bis-e) To support CSI measurement/reporting mechanisms for NCR-MT in C-link - The necessary legacy mechanism for receiving CSI-RS is reused for NCR-MT. - The necessary legacy mechanism for reporting CSI is reused for NCR-MT. - FFS: The details of the necessary mechanisms will be further discussed and decided. - Note: this does not mean all the legacy procedures for receiving CSI-RS and reporting CSI will be supported. Agreement (RAN1#110bis-e) To support the sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported. - FFS: The details of the necessary mechanism of legacy UE sounding procedure. - Note: This does not mean all legacy UE sounding procedure will be supported.

The FFSs in the above agreements can be discussed together with the RAN1 capabilities which are needed for the basic operation of operator deployed NCR, i.e., those features without which the system cannot operate. Specifically, RAN1 should not modify existing UE functionalities that has been carefully developed and specified. As such, we think, a repeater node, similar to an IAB-node, is a stationary node deployed according to the network planning done by the operator. The planning can be expected to result in a stable control/backhaul link between the controlling gNB and the NCR. Hence, many UE features related to mobility and measurement are not needed for NCR-MTs. As a bassline, the NCR-MT can have the same mandatory features as an IAB-MT agreed in the RAN1#101-e meeting [4], where specifically basic CSI feedback (2-32) and basic SRS (2-52) [5] are included, cf. Table 1.

Agreement (RAN1#101-e) Wide-area IAB-MTs support the following Rel. 15 layer-1 mandatory UE features (as defined in TR38.822) - Without capability - 0-1, 0-3, 0-4, 1-1 (only 1 preamble for component 1, component 2, component 3 except paging), 2-1, 2-5, 2-6, 2-12, 2-16, 2-16a, 2-32 (only components 1-4 and 7), 2-50 (only components 1,2), 2-52 (only components 1, 2), 3-1 (only components 1,2,3,4,5), 4-1, 5-1 (only components 1/2/3/4/5/6/7/9/10/12), 6-1, 7-1, 8-3 - With capability signaling which shall be set to '1' - 1-3, 2-22, 4-10 - The rest of Rel-15 layer-1 UE features other than the ones as listed above are optional for wide-area IAB-MTs. - Note: Mandatory MT capabilities are independent from DU capabilities and do not imply a corresponding mandatory DU capability. - The UE feature list for local-area IAB-MTs is FFS

Table 1: Mandatory features for IAB-MT.

| ### Features | Index | Feature group | Mandated component | Without capability | With capability set to 1 |
| --- | --- | --- | --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL |  | Yes |  |
|  | 0-3 | DL modulation scheme |  | Yes |  |
|  | 0-4 | UL modulation scheme |  | Yes |  |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures | 1 preamble for component 1, component 2, component 3 except paging | Yes |  |
|  | 1-3 | SS block based RLM |  |  | Yes |
| 2. MIMO | 2-1 | Basic PDSCH reception |  | Yes |  |
|  | 2-5 | Basic downlink DMRS for scheduling type A |  | Yes |  |
|  | 2-6 | Basic downlink DMRS for scheduling type B |  | Yes |  |
|  | 2-12 | Basic PUSCH transmission |  | Yes |  |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A |  | Yes |  |
|  | 2-16a | Basic uplink DMRS for scheduling type B |  | Yes |  |
|  | 2-22 | Aperiodic beam report |  |  | Yes |
|  | 2-32 | Basic CSI feedback | component 1-4 and 7 | Yes |  |
|  | 2-50 | Basic TRS | components 1,2 | Yes |  |
|  | 2-52 | Basic SRS | components 1,2 | Yes |  |
| 3. DL control channel and procedure | 3-1 | Basic DL control channel | components 1,2,3,4,5 | Yes |  |
| 4. UL control channel and procedure | 4-1 | Basic UL control channel |  | Yes |  |
|  | 4-10 | Dynamic HARQ-ACK codebook |  |  | Yes |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation | components 1,2,3,4,5,6,7,9,10,12 | Yes |  |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction |  | Yes |  |
| 7. Channel coding | 7-1 | Channel coding |  | Yes |  |
| 8. UL TPC | 8-3 | Basic power control operation |  | Yes |  |

The deployment situation for IAB-node and NCR is similar, hence the required features for IAB-MT and NCR-MT are similar.

Use the identified and agreed IAB-MT capabilities as a starting point for repeater-MT capabilities.

## HARQ for PDCCH

The following agreement regarding ACK/NACK for the side control information was made in the RAN1#110bis-e meeting [3].

Agreement (RAN1#110bis-e) HARQ-ACK feedback for PDSCH carrying the side control information from higher layer (e.g., MAC-CE, RRC) is supported. The legacy HARQ-ACK feedback mechanism is reused. - FFS: Whether HARQ-ACK feedback for PDCCH carrying side control information is supported - Note: This does not mean all legacy HARQ-ACK feedback mechanism will be supported.

From a network perspective, the repeater deployment can be assumed to be well-planned and under operator control, with good and stable line-of-sight propagation conditions on the control and the backhaul links. Specially, we expect the availability of the control and backhaul links to be similar as the traditional wireless backhaul, i.e., at least 4-9s availability. Introducing additional complicated dynamic HARQ-ACK mechanism for a link that is assumed to be robust (e.g., an ACK message is received for almost 99.99% of time) is not justified, considering HARQ-ACK for PDCCH is not even required for legacy access links which often operate in more challenging nLoS conditions. In our opinion, the need for re-transmission of the Layer-1 side-control information should be avoided already at deployment.

It should also be noted that the ACK/NACK procedure has an inherent delay which is only applicable to the cases where longer configuration application latencies are not critical (e.g., SPS release or Rel-17 unified TCI). Even if a C-link channel is so bad that it is considered for a moment that HARQ-ACK feedback for PDCCH reception could make sense, for a repeater to support the typically low time offsets between PDCCH scheduling and PDSCH data reception, not even a single retransmission may be feasible before the beam indication needs to be applied. Considering multiple retransmissions due to improper deployment with significantly reduced availability, the HARQ scheme may increase network latency even further.

Additionally, to provide timely feedback on PDCCH reception, UL transmissions need to be scheduled accordingly. That would disqualify ACK/NACK feedback from being used in many existing networks, since for many network TDD configurations, there may not even be an UL slot configured at a suitable time (not too early, not too late) during the period for which the indication is valid. In other words, specifying HARQ-ACK for PDCCH-based dynamic indications in repeaters would substantially restrict the feasible network configurations, specially TDD UL/DL pattern configurations. Considering repeaters will be deployed in existing networks, requiring L1-HARQ functionality will have undesirable impact on UEs.

Finally, HARQ-ACK feedback for PDCCH is a completely new signalling which will require a significant specification and validation effort that is not realistic in only two meetings.

Based on the above discussion, we observe and propose the following:

HARQ is not useful for low latency scenarios such as PDCCH-based dynamic indication in repeaters.

HARQ for PDCCH-based dynamic indication in repeaters would severely restrict network configurations in terms of TDD UL/DL patterns.

Do not support HARQ feedback for PDCCH carrying side control information.

## Beam failure reporting

Although the repeater deployment can be assumed to be well-planned with stable links between the gNB and the NCR, it may still be possible that the signal propagation paths between the gNB and the NCR change due to, e.g., the movement of the objects in the area or due to interference. Also, it may be possible that, while the active C-link beam is blocked, there is another C-link beam that can be used properly. In such cases, the legacy BFR mechanism can be useful, to guarantee stable NCR operation. Thus, we believe that, in the cases where adaptive beams in C-link are supported by the NCR-MT, the legacy BFR mechanism can be supported as an optional functionality for the NCR-MT. Of course, with fixed beams there is no use for a BFR mechanism.

As an optional functionality for the NCR-MT, legacy BFR mechanism is supported, if the NCR-MT supports adaptive beams in C-link.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The deployment situation for IAB-node and NCR is similar, hence the required features for IAB-MT and NCR-MT are similar.

Observation 2	HARQ is not useful for low latency scenarios such as PDCCH-based dynamic indication in repeaters.

Observation 3	HARQ for PDCCH-based dynamic indication in repeaters would severely restrict network configurations in terms of TDD UL/DL patterns.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Use the identified and agreed IAB-MT capabilities as a starting point for repeater-MT capabilities.

Proposal 2	Do not support HARQ feedback for PDCCH carrying side control information.

As an optional functionality for the NCR-MT, legacy BFR mechanism is supported, if the NCR-MT supports adaptive beams in C-link.

# References

RP-222673, New WID on NR network-controlled repeaters, 3GPP TSG RAN #97-e, September 2022.

R1-2212344, “Side-control information and NCR behavior,” Ericsson, RAN1 #111, November 2022.

“Chairman’s Notes”, 3GPP TSG RAN1 #110bis-e, October 2022.

“RAN1 Chairman’s Notes”, 3GPP TSG RAN1 #101-e, June 2020.

TR 38.822, “User Equipment (UE) feature list”, 3GPP, V16.3.0, March 2022.