3GPP TSG RAN WG1 #111		R1-2212930

Toulouse, France, November 14th – 18th, 2022

Title:	[Draft] LS reply on RACH-less handover in NTN

Response to:	R1-2210807 (R2-2211017)

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	OPPO [to be changed to RAN1]

To:	RAN2, RAN4

Cc:

Contact Person:

#### Name:	Hao Lin

Tel. Number:

E-mail Address:	lin.hao@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN1 would like to thank RAN2 for their LS in R1-2210807 (R2-2211017) and provide response to the following question asked by RAN2.

For mobility enhancement in Rel-18 NR NTN, RAN2 has discussed RACH-less handover. RAN2 would like to check with RAN1 in which of the following listed scenarios RACH-less handover is possible. - Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB - Intra-satellite handover with different feeder links, i.e., with gateway/gNB switch - Inter-satellite handover with gateway/gNB switch - Inter-satellite handover with same gateway/gNB ACTION: RAN2 respectfully asks RAN1 to provide response to the above question.

RAN1 response:

For scenario (21)-(4), from RAN1 perspective the RACH-less handover [is / may be] possible, assuming the following notes can be satisfied, when UE UL transmission synchronization can be maintained by applying pre-compensation using the assistance information, e.g., epoch time, ephemeris, common TA, of the target cell.

Note 1: RAN1 assumes that UE fulfils the RAN4 UL synchronization requirement specified in Table 7.1C.2-1 of TS38.133 applies to the first UL transmission in the target cell.

Note 2: gNB is expected to provide valid assistance information of the target cell to UE.

Note 3: gNB is expected to ensure the UE can perform the UL transmission while respecting common TA and UE processing time.

2. Actions:

To RAN2:

RAN1 kindly respectfully asks RAN2 to take the above response into account in the future work.

To RAN4:

RAN1 respectfully asks RAN4 whether RAN1’s assumption in Note 1 is correct.

3. Date of Next TSG-RAN1 Meetings:

RAN1#112                       27 Februrary-03 March 2023	Athens

RAN1#113                       22-26 May 2023	Korea