3GPP TSG RAN WG1 #112		                                                          R1-2300633

Athens, Greece, February 27th – March 3rd, 2023

Source:	CATT,CBN

Title:	Discussion on Type1 HARQ codebook generation for fdmed-ReceptionMulticast

Agenda Item:	8.12

Document for:	Discussion and Decision

- Introduction

In this contribution, the procedure of K1 generation for multicast is discussed when the UE is provided fdmed-ReceptionMulticast parameter.

- Discussion

For improving reliability of multicast PDSCH transmission, two configurations are supported for Type-1 HARQ-ACK codebook generation. One is configured with the parameter type1-Codebook-GenerationMode, another is configured with the parameter fdmed-ReceptionMulticast.

- type1-Codebook-GenerationMode

When the parameter is indicated as mode1, K1 for Type-1 HARQ-ACK codebook generation is provided by the intersection of K1 set of unicast and K1 set of multicast. When the parameter is indicated as mode2, K1 for Type-1 HARQ-ACK codebook generation is provided by the union of K1 set of unicast and K1 set of multicast.

MulticastConfig-r17 ::= SEQUENCE { .... type1-Codebook-GenerationMode-r17 ENUMERATED { mode1, mode2} OPTIONAL -- Need M

- fdmed-ReceptionMulticast

If the UE is provided fdmed-ReceptionMulticast, UE will generate Type-1 HARQ-ACK codebook for multicast and unicast separately. In this case, K1 for Type-1 HARQ-ACK codebook of multicast is only provided by the K1 set of multicast. Moreover, type1-Codebook-GenerationMode and fdmed-ReceptionMulticast can’t be configured for UE simultaneously.

Proposal 1: It is not expected that type1-Codebook-GenerationMode and fdmed-ReceptionMulticast are configured for UE simultaneously

Proposal 2：If the fdmed-ReceptionMulticast is provided, K1 for Type-1 HARQ-ACK codebook of multicast is only provided by the K1 set of multicast.

The draft CR for TS 38.213 is as follows.

----------------------- Start of text to TS 38.213 v17.4.0 -------------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

-	If the UE is configured to monitor PDCCH for multicast DCI formats for serving cell c

-	if the UE is not provided type1-Codebook-GenerationMode = 'mode1''mode2', K1 is additionally provided by the union of dl-DataToUL-ACK from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1 or pucch-ConfigMulticast2/pucch-ConfigurationListMulticast2 and dl-DataToUL-ACK-ForDCI Format4-1

-	if the UE is not provided dl-DataToUL-ACK-ForDCI Format4-1, K1 is provided by the union of dl-DataToUL-ACK from pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 and the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}

-	if the UE is provided type1-Codebook-GenerationMode = 'mode1', the UE

-	determines a first K1,UM set as K1∩K1,M, where K1,M is a set of slot timing values for the multicast DCI formats, a second K1,U\M set as K1\K1,UM, and a third K1,M\U set as K1,M\K1,UM

    - if the UE is provided fdmed-ReceptionMulticast,  K1 for multicast is provided by the union of dl-DataToUL-ACK from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1 or pucch-ConfigMulticast2/pucch-ConfigurationListMulticast2 and dl-DataToUL-ACK-ForDCI Format4-1

      - if the UE is not provided dl-DataToUL-ACK-ForDCI Format4-1, K1 is provided by the union of dl-DataToUL-ACK from pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 and the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}

b)	on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by

-	the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c if the UE is not configured to monitor PDCCH for multicast DCI formats for serving cell c, or is not provided type1-Codebook-GenerationMode = 'mode1', or, if any, for the first K1,UM set

-	the union of row indexes of time domain resource allocation tables for DCI format 1_0 and/or DCI format 1_1 and/or DCI format 1_2 for serving cell c for the second K1,U\M set, if any

-	the union of row indexes of time domain resource allocation tables for multicast DCI formats the UE is configured to monitor PDCCH for serving cell c for the third K1,M\U set, if any

-	if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for any PDCCH monitoring occasion in any slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12  for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0

----------------------- End of text to TS 38.213 v17.4.0 -------------------------

- Conclusion

The procedure of K1 generation is discussed when the UE is provided fdmed-ReceptionMulticast. We have two proposals as following:

Proposal 1: It is not expected that type1-Codebook-GenerationMode and fdmed-ReceptionMulticast are configured for UE simultaneously

Proposal 2：If the fdmed-ReceptionMulticast is provided, K1 for Type-1 HARQ-ACK codebook of multicast is only provided by the K1 set of multicast.

The draft CR is provided in [1]

- References

R1-2300634 “Draft CR on Type1 HARQ codebook generation for fdmed-ReceptionMulticast”, CATT, RAN1#112, February 27th – March 3rd, 2023