3GPP TSG RAN WG1 #112                                                  R1-2300737

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	5

Source:	Lenovo

Title:	Discussion on PSFCH configured power with multiple resource pools

Document for:	Discussion

Introduction

RAN1 has received an LS [1] from RAN4 related to PSFCH configured power with multiple resource pools with following questions:

Question 1: RAN4 agreed to define PEMAX,c for PSFCH configured Tx Power in TS38.101-1 as sum of IEs sl-maxTransPower when multiple resource pools configured are transmitted simultaneously and would like to know if this can have any impact to RAN1. Question 2: RAN4’s understanding is that the total transmit power is shared equally between simultaneously transmitted PSFCHs regardless of whether they are from a single or multiple resource pools. RAN4 would like to confirm with RAN1 that this understanding is correct.

In this contribution, we provide related consideration from RAN 1’s perspective.

Discussion

In RAN1’s specification (TS38.213) two sections are related to the PSFCH transmission mentioned in RAN4’s LS. Section 16.2.4.2 as in Appendix is about the simultaneous PSFCH transmission/reception, in the specification it doesn’t limit the procedure for simultaneous PSFCH transmission in one resource pool or in multiple resource pools. Section 16.2.3 as in Appendix is about the power control of PSFCH, although the selection of simultaneous PSFCH transmission could be applied for both single resource pool and multiple resource pools, in the specification about power control of PSFCH the procedure is limited to one resource pool as highlighted in the Appendix.

In the RAN1’s discussion we have not discussed whether multiple simultaneous PSFCH transmissions are from single resource pool or multiple resource pools. As in the LS if the PCMAX is defined according to sum of IEs sl-maxTransPower when PSFCHs will be transmitted on multiple resource pools, the current RAN1 specification should be changed at least remove “one a resource pool”. We think multiple PSFCH transmissions on multiple resources at the same time is a valid case if FDMed resources pools are configured or slots with PSFCH from multiple resource pools are configured to be overlapped, at least for sidelink resource allocation mode 1 it allows FDMed resource pool configuration since there is a resource pool index indicator in the DCI format 3-0.

Conclusion

In this contribution we provide our views related to RAN4 questions:

Proposal 1: RAN1 informs RAN4 that RAN1 has not discussed whether simultaneous PSFCH transmissions are from single resource pool or multiple resource pools.

Proposal 2: If PEMAX,c is specified as in RAN4’s LS RAN1 may need to further discuss the simultaneous PSFCH transmissions on multiple resource pools, and CR may be needed to reflect this.

Appendix

- 16.2.4.2 Simultaneous PSFCH transmission/reception If a UE - would transmit Nsch,Tx,PSFCH PSFCHs and receive Nsch,Rx,PSFCHPSFCHs, and - transmissions of the Nsch,Tx,PSFCH PSFCHs would overlap in time with receptions of the Nsch,Rx,PSFCH PSFCHs the UE transmits or receives only a set of PSFCHs corresponding to the smallest priority field value, as determined by a first set of SCI format 1-A and a second set of SCI format 1-A [5, TS 38.212] that are respectively associated with the Nsch,Tx,PSFCH PSFCHs and the Nsch,Rx,PSFCH PSFCHs. If a UE would transmit Nsch,Tx,PSFCH PSFCHs in a PSFCH transmission occasion, the UE transmits NTx,PSFCH PSFCHs corresponding to the smallest NTx,PSFCH priority field values indicated in all SCI formats 1-A associated with the PSFCH transmission occasion. - 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH - αPSCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - if Nsch,Tx,PSFCH≤Nmax,PSFCH - if PPSFCH,one+10log10Nsch,Tx,PSFCH≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 such that NTx,PSFCH≥max1,i=1KMi where Mi is a number of PSFCHs with priority value i and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs assigned with priority values 1, 2, …, K, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions - else - the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 - if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 such that NTx,PSFCH≥max1,i=1KMi where Mi is a number of PSFCHs with priority value i and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs assigned with priority values 1, 2, …, K, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is determined for the NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1] - else PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 such that NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1].

References

- R1-2300030, LS on PSFCH configured power with multiple resource pools