3GPP TSG RAN WG1 #112	R1-2301044

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.3.2

Source: 	ETRI

Title:	Discussion on subband non-overlapping full duplex enhancements

Document for:	Discussion/Decision

Introduction

This contribution presents ETRI’s views on SBFD enhancements focusing on the following topics:

- UL/DL subband configuration for SBFD.

- Potential enhancements on DL reception and UL transmission for SBFD UEs.

- Potential enhancements on CSI feedback for SBFD.

Discussion

UL/DL subband configuration for SBFD (SBFD operation Alt.4)

Table 1. Agreements on SBFD resource configuration / indication [1], [2].

Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined. Agreement For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline. Agreement The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1. - The UL subband can be located at one side of the carrier. - The UL subband can be located at the middle part of the carrier, subject to RAN4’s study and conclusion Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible. Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance. Send an LS to RAN4 to inform the above agreement. If RAN4 has response, it will be taken into account but in the meanwhile, RAN1 work will continue based on the above. Agreement For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline. Agreement For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid. Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously

As agreed in Table 1, the time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs as the baseline (SBFD operation Alt 4) and in a given time instance at least one pair of UL and DL subbands will be explicitly and/or implicitly configured/allocated to those UEs. Since this aspect may have potential impacts as mentioned in Section 2.2 of this contribution, we would like to clarify that these multiple SBFD subbands with different D-U directions can be configured on a given time instance from a UE perspective as well as from the network perspective.

Proposal 1. Multiple SBFD subbands with different D-U directions can be configured on a given time instance from a UE perspective.

For the first step of the detailed signalling design, it was agreed that both the time location and the frequency location at least for the UL subband will be explicitly configured as a baseline. Whether the frequency location of DL or flexible subbands is explicitly indicated or implicitly determined is not yet decided. In addition, the agreed baseline SBFD scheme will follow the legacy TDD BWP operation, i.e., it will operate under a single DL/UL BWP pair with aligned center frequency. That also implies that each UL/DL subband will occupy a set of contiguous PRBs of the active BWP possibly with a guard band between the subbands subject to RAN4 discussions.

Those listed properties are very similar to the Rel-16 NR-U wideband operation, where one NR-U carrier can span multiple LBT subbands with guard bands in between to protect the per-LBT subband CCA performance from inter-subband interference. In the NR-U specification, each LBT subband is represented by a RB set, and the RB set locations are informed to UE by the RRC configuration (implicitly based on the location information of intra-cell guard bands). Also, as captured in TS 38.331 (below), the guard bands location is indicated in terms of CRBs with reference to Point A, and is UE-specifically signalled via RRC as included in IE ServingCellConfig.

ServingCellConfig ::= SEQUENCE { … intraCellGuardBandsDL-List-r16 SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16 OPTIONAL, -- Need S intraCellGuardBandsUL-List-r16 SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16 OPTIONAL, -- Need S … } IntraCellGuardBandsPerSCS-r16 ::= SEQUENCE { guardBandSCS-r16 SubcarrierSpacing, intraCellGuardBands-r16 SEQUENCE (SIZE (1..4)) OF GuardBand-r16 } GuardBand-r16 ::= SEQUENCE { startCRB-r16 INTEGER (0..274), nrofCRBs-r16 INTEGER (0..15) }

In our view, the same principles (i.e., subband configuration based on CRB grid with reference to Point A) would be the simplest and clearest way to indicate the SBFD subbands frequency location. In addition, it would be beneficial to have UE-specific RRC signalling for the SBFD subbands location considering various possibilities on the UE implementations for CLI mitigation.

For the time domain, when viewed from a broad perspective, the allocation of the SBFD symbols is a part of the TDD UL/DL slot format configuration. Thus, we think that the SBFD symbol location information can be included in or combined with the slot format information, or at least their RRC parameters can have the same IE or procedural level. The NR semi-static slot format configuration includes both cell-specific and UE-specific RRC signalling: tdd-UL-DL-ConfigurationCommon in ServingCellConfigCommon and tdd-UL-DL-ConfigurationDedicated in ServingCellConfig. If the SBFD operation is applied to connected mode UEs only, it would be more beneficial to apply the UE-specific RRC signalling for the SBFD subband time location information to achieve the commonality with the frequency location indication signalling.

Based on our observations, we propose the following:

Proposal 2. Consider UE-specific RRC signaling for time location and frequency location of the SBFD subbands as the baseline.

- FFS: Cell-specific RRC signaling for time location and frequency location of the SBFD subbands (subject to SBFD support during initial access)

More specifically, we see there are at least two different flavors for the SBFD subband configuration, depending on its interaction with the slot format configuration:

- Alt. 1: No change in the legacy slot format configuration. The UL subband is separately signalled and it overrides the configured slot format.

- Alt. 2: Extend the slot format configuration to support to make SBFD symbols.

An example configuration for Alt. 1 is depicted in Figure 1. In this approach, the configured UL subband has higher priority than the resources determined by the slot format configuration, and it overrides a part of the DL and flexible symbols as illustrated in the figure. Since the DL and flexible subbands need not be indicated, indicating a single RB set corresponding to the UL subband frequency location is sufficient. For the time domain allocation, one or more SBFD durations consisting of a set of slots and/or symbols can be indicated. The periodicity of the SBFD durations can simply be the same as the periodicity of the slot format configuration, or can be separately configured if that separation is considered beneficial.

Figure 1. An example for Alt. 1.

For Alt. 2, the extension of the legacy slot format configuration can be achieved in several directions. One simple approach is to add a new symbol type, e.g., SBFD symbol (Alt. 2-1) on top of the existing DL, UL, and flexible symbol. In this approach, the subband pattern-related information may be configured together with the SBFD symbol location. For example, SBFD symbols with ‘D’ direction may correspond to {DUD}, {DU}, and {UD} subband patterns, and SBFD symbols with ‘F’ direction may correspond to {FUF}, {FU}, and {UF} subband patterns, depending on the UL subband location as shown in Figure 2.

Figure 2. An example for Alt. 2-1.

Another approach is to configure multiple RB sets together with per-RB set slot format configuration information (Alt. 2-2), which is illustrated in Figure 3. For example, UE can be configured with three RB sets to form {XYZ} SBFD subband pattern and three slot format information each corresponding to each of the three RB sets. In this approach, symbols having different directions construct the SBFD symbols. At the expense of increased signalling overhead, this method can provide very large flexibility on DL/UL resource partitioning in use of the SBFD carrier. One clarification that may be needed in Alt. 2-2 is whether the way of determining the SBFD symbols in Alt. 2-2 falls into the “explicit” indication or implicit indication to judge whether this approach can be included in the agreed baseline scheme or not.

Figure 3. Examples for Alt. 2-2.

Since the preferred ratio of HD periods and FD periods can be varied according to various aspects, such as the number of cell-edge UEs, DL/UL traffic situations, etc., enhancements on DCI-based framework, e.g., by using SFI or rate matching resource, to enable dynamic D-U allocations across multiple RB sets can be considered for subband non-overlapping FD.

Proposal 3. RAN1 to consider the followings for SBFD subband location indication for the SBFD-capable UEs considering the following aspects:

- Interaction with slot format configuration

- Tradeoff between signaling overhead and UL/DL resource partitioning flexibility

- Explicit indication of RB set(s) for flexible subband(s)

- SFI enhancement for dynamic SBFD symbol allocation for a given time instance.

DL reception and UL transmission for SBFD UEs

Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously

Regarding the above agreements on SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, we believe option 2 should be the baseline for future study given that option 1 limits configuration flexibility without clear benefits. From our understanding, flexible symbol in TDD-UL-DL-ConfigCommon can further be configured or indicated as uplink symbol by TDD-UL-DL-ConfigDedicated or DCI format 2_0 and in such cases we don’t see a strong need to have the restriction on UL transmission outside UL subband.

Proposal 4. For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, RAN1 to consider option 2 as the baseline for SBFD aware UEs:

- Option 2:

  - UL transmissions within UL subband are allowed in the symbol

  - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same

    - FFS: SBFD aware UE behaviours

    - FFS: Whether or not signalling of guardband(s) is needed

  - FFS: Whether or not the symbol can be converted to a DL-only symbol

  - Frequency locations of DL subband(s) are known to the SBFD aware UE

  - DL receptions within DL subband(s) are allowed in the symbol

We share our views on FFS points of option 2 as follows:

- Conversion to DL-only symbol: Per the modular design principle of 5G NR, we think it would be good to pursue enabling coexistence of multiple features for various purposes and environments. For the same reason, it should be possible to co-operate SBFD and dynamic TDD (including the legacy one and the enhanced version, which is being studied in the other agenda), we think. In this regard, conversion of flexible symbol in TDD-UL-DL-ConfigCommon, which may be configured with SBFD subband configurations, to either of UL-only or DL-only symbol is an essential feature to support SBFD operation and dynamic TDD simultaneously. Otherwise, it would be clarified that SBFD operation and dynamic TDD cannot be applied in the same symbol.

- Guard band signalling: As discussed in Section 2.1, an explicit signalling of guard band provides clear SBFD operation from both gNB and UE perspectives at the cost of signalling overhead. We think either of DL subband or guard band would be necessarily configured to avoid any ambiguity given that the required guard band may not be the same according to the type of channel/signal allocated in the DL/UL subband (see Figure 7 for detailed examples). However, it needs to be clarified that the configured guard band is not applicable when the symbol is converted to a UL-only (or DL-only) symbol.

Proposal 5. RAN1 to clarify the followings:

- Explicit signalling of guardband(s) is required for SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon.

- For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, the symbol can be converted to a DL-/UL-only symbol.

- The configured guardband is not applicable for a DL-/UL-only symbol.

Table 2. Agreements on DL reception and UL transmission for SBFD UEs [1].

Agreement Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - If there are, whether/how to avoid/handle such collision cases (as second step)

As captured in Table 1, it was agreed to study UE behaviors of SBFD aware UEs in an SBFD symbol where both time and frequency locations of subbands for SBFD operation are known to those UEs. As a consequence multiple SBFD subbands with different D-U directions can be configured on a given time instance from a UE perspective (proposal 1 of this contribution), DL-UL direction change of an SBFD aware UE across SBFD symbols can be beneficial for the following use cases:

- Use case #1 (allocation of cell-specific signals / channels): Without DL-UL direction switch during the (consecutive) SBFD symbols, the network may suffer from the lack of DL or UL resources to allocate cell-specific signals and/or channels, such as SSB, type-0 PDCCH, SIBs, CSI-RS, PRACH, or SRS, etc. However, by allowing DL-UL direction switch during the (consecutive) SBFD symbols for a UE, this issue can be resolved at least in the time domain perspective while the frequency resource allocations for the cell-specific signals and/or channels still need to be handled, carefully.

- Use case #2 (Latency reduction): Since DL-UL direction switch during the (consecutive) SBFD symbols can make more flexible timing gaps between multiple DL receptions and/or UL transmissions, it could be helpful to reduce latency for UL/DL beam indication, channel measurement, or CSI feedback, during the SBFD operation periods.

- Use case #3 (co-existence with legacy UEs): For the similar reason in use case #1, the SBFD-aware UE may be able to follow legacy UE behaviour (e.g., paging procedure, etc.) in SBFD operation durations as well as in TDD operation durations.

The use cases above would be the answer to the previous agreements in Table 2.

Figure 4. DL-UL direction change of an SBFD aware UE in SBFD symbols

Proposal 6. Capture the following use cases of DL-UL direction change for an SBFD aware UE in SBFD symbols in the TR:

- Use case #1: Ease of planning on cell-specific signals / channels

- Use case #2: Latency reduction for UL/DL beam indication, channel measurement, CSI feedback, etc.

- Use case #3: Ease of co-existence with legacy UEs

Proposal 7. Study semi-static and/or dynamic DL-UL direction change of an SBFD aware UE in adjacent SBFD symbols.

For the resource allocation in SBFD symbols, RAN1 discussed high-level principles during the last meeting and the Agreement #4 in the following has been made:

Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

In our view, it is essential to first identify what the baseline operation is for the SBFD symbols. The current specification describes rules for DL reception in semi-static UL symbols and vice versa as follows:

- For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive PDCCH, PDSCH, or CSI-RS when the PDCCH, PDSCH, or CSI-RS overlaps, even partially, with the set of symbols of the slot.

- For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot.

According to the above statement, UE drops the whole DL transmission if it collides with semi-static UL symbols and vice versa. Extending this principle to frequency domain in SBFD symbols, the following operation can be considered:

- Option A: UE does not receive a DL transmission if the DL transmission overlaps, even partially, with a UL subband (and potentially corresponding guard band(s)), and UE does not transmit a UL transmission if the UL transmission overlaps, even partially, with a DL subband (and potentially guard band(s)).

We think the above option may work as a baseline operation and suggest to consider it for the resource allocation discussion. However, based on the current categorization in Agreement #4, it is not clear which option covers Option A above. In view that UE can anyway be scheduled DL (or UL) in a UL subband (or a DL subband), it may belong to Option 4, however, in our understanding dropping the whole scheduled DL or UL transmission is not the intention of Option 4 (and Option 2, 3 as well). We think this aspect needs to be clarified clearly for the future discussions. In this regard, the agreed options, i.e., from Option 1 to Option 4, can be recategorized as follows with the understanding that Option A is considered as baseline:

- Option A: UE does not receive a DL transmission if the DL transmission overlaps, even partially, with a UL subband (and potentially corresponding guard band(s)), and UE does not transmit a UL transmission if the UL transmission overlaps, even partially, with a DL subband (and potentially guard band(s)).

- Option B: If a DL transmission partially overlaps with a UL subband (and potentially corresponding guard band(s)), the UE can receive the non-overlapped DL transmission.

- Option C: If a UL transmission partially overlaps with a DL subband (and potentially corresponding guard band(s)), the UE can transmit the non-overlapped UL transmission.

Proposal 8. Consider the following options for SBFD aware UEs.

- Option A (baseline): UE does not receive a DL transmission if the DL transmission overlaps, even partially, with a UL subband (and potentially corresponding guard band(s)), and UE does not transmit a UL transmission if the UL transmission overlaps, even partially, with a DL subband (and potentially guard band(s)).

- Option B: If a DL transmission partially overlaps with a UL subband (and potentially corresponding guard band(s)), the UE can receive the non-overlapped DL transmission.

- Option C: If a UL transmission partially overlaps with a DL subband (and potentially corresponding guard band(s)), the UE can transmit the non-overlapped UL transmission.

Figure 5 provides an example of Option B (partial reception on DL transmission overlapping with UL subband). In Figure 5, a scheduled PDSCH may overlap with a UL subband (or guard band) in SBFD symbols. Based on Option B, UE may still receive the PDSCH partially on the two DL subbands, for example, by applying a rate matching around the UL subband (or guard band). Similarly, a CORESET (or a PDCCH MO) may overlap with a UL subband in SBFD symbols. Then, it may be clarified that the UE shall take the non-overlapped BD/CCE candidates only into account as the valid BD/CCE candidates for PDCCH reception in that PDCCH MO (although this clarification may not exactly fall into Option B).

Figure 5. An example of valid DL reception in SBFD symbols based on Option 2.

Table 3. Agreements on DL reception and UL transmission for SBFD UEs in [2].

Agreement Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured.

Table 3 shows the agreements on DL reception and UL transmission across SBFD and non-SBFD symbol boundary [2]. Figure 6 shows an example for UL resource allocations across SBFD/UL symbols. In some sense, the SBFD symbols and the UL symbol may have different pre/post-processing and those might cause the event to break the channel coherence for the UL signal at either/both UE or gNB, in which case the antenna port assumption does not hold. However, some implementations of high capability UEs may not allow such events.

In addition, the previous release specified DM-RS bundling of PUCCH/PUSCH, in which case a UE should maintain the power consistency/phase continuity during UL repetitions. The specification formulates a nominal/actual time domain window and describes events to break the power consistency/phase continuity. If those events occur, then an actual time domain window(s) may restart keeping the power consistency/phase continuity depending on UE capabilities. If any UL transmission with possible further repetitions spans across SBFD/UL symbols, then similar issues regarding time domain windows can be discussed.

Figure 6. Uplink power consistency / phase continuity between SBFD resources and TDD resources.

Observation 1. In case of UE with high capabilities, the channel coherence may be maintained longer than either of the allocated SBFD resource or TDD resource.

Similar with the above example, the following aspects can be considered to enable 1) PDCCH reception in SBFD symbols and 2) PDCCH reception across SBFD and non-SBFD symbols:

- The same CORESET configuration can be utilized for PDCCH monitoring on DL subbands

- Only the BD/CCE candidates fully overlapping with DL subbands are considered as vaild for PDCCH monitoring

- PDCCH repetition across SBFD symbols and non-SBFD symbols

- FFS, the exact mapping of frequencyDomainResources in DL subbands

Proposal 9. Consider the followings for PDCCH receptions in SBFD symbols.

- The same CORESET configuration can be utilized for PDCCH monitoring on DL subbands

- Only the BD/CCE candidates fully overlapping with DL subbands are considered as vaild for PDCCH monitoring

- PDCCH repetition across SBFD symbols and non-SBFD symbols

- FFS, the exact mapping of frequencyDomainResources in DL subbands

Similar with the above example, the following aspects can be considered to enable 1) PDSCH reception in SBFD symbols and 2) PDSCH reception across SBFD and non-SBFD symbols:

- The same frequency domain resource allocation [per the same PRB indexing] can be utilized for PDSCH scheduling on DL subbands

- Only the REs overlapping with DL subbands are considered as vaild for PDSCH reception

  - FFS, puncturing and/or rate-matching on the invalid REs

- FFS, whether/how to support SPS PDSCH allocation across SBFD symbols and non-SBFD symbols

Proposal 10. Consider the followings for PDSCH receptions in SBFD symbols.

- The same frequency domain resource allocation [per the same PRB indexing] can be utilized for PDSCH scheduling on DL subbands

- Only the REs overlapping with DL subbands are considered as vaild for PDSCH reception

  - FFS, puncturing and/or rate-matching on the invalid REs

- FFS, whether/how to support SPS PDSCH allocation across SBFD symbols and non-SBFD symbols

Similar with the above example, the following aspects can be considered to enable 1) PUCCH transmission in SBFD symbols and 2) PUCCH transmission across SBFD and non-SBFD symbols:

- Option 1: Separated PUCCH resource group configurations for non-SBFD symbols (e.g., covered by the 1st PUCCH resource group) and SBFD symbols (e.g., covered by the 2nd PUCCH resource group)

- Option 2: Single PUCCH resource configuration that can span across SBFD and non-SBFD symbols

- FFS, application timing of each PUCCH resource group in case of option 1

  - Slot-level and/or symbol-level PUCCH resource group switching according to the duplex mode

Proposal 11. Consider the followings for PUCCH transmissions in SBFD symbols.

- Option 1: Separated PUCCH resource group configurations for non-SBFD symbols (e.g., covered by the 1st PUCCH resource group) and SBFD symbols (e.g., covered by the 2nd PUCCH resource group)

- Option 2: Single PUCCH resource configuration that can span across SBFD and non-SBFD symbols

- FFS, application timing of each PUCCH resource group in case of option 1

  - Slot-level and/or symbol-level PUCCH resource group switching according to the duplex mode

Similar with the above example, the following aspects can be considered to enable 1) PUSCH transmission in SBFD symbols and 2) PUSCH transmission across SBFD and non-SBFD symbols:

- The same frequency domain resource allocation [per the same PRB indexing] can be utilized for PUSCH scheduling on UL subbands

- Only the REs overlapping with UL subbands are considered as vaild for PUSCH transmission

  - FFS, puncturing and/or rate-matching on the invalid REs

- FFS, whether/how to support CG PUSCH allocation across SBFD symbols and non-SBFD symbols

Proposal 12. Consider the followings for PUSCH transmissions in SBFD symbols.

- The same frequency domain resource allocation [per the same PRB indexing] can be utilized for PUSCH scheduling on UL subbands

- Only the REs overlapping with UL subbands are considered as vaild for PUSCH transmission

  - FFS, puncturing and/or rate-matching on the invalid REs

- FFS, whether/how to support CG PUSCH allocation across SBFD symbols and non-SBFD symbols

Figure 7. Guard band configurations for SSB slots and normal (non-SSB) slots.

Figure 7 depicts an example of guard band configurations for SSB slots and normal (non-SSB) slots. Given that the portion of SSB slots can be much larger than 10% per the network deployment scenario, the potential gain from SBFD operation would be limited, if the specification only supports TDM of SSB slots/symbols and SBFD slots/symbols. To enable FDM of SSB and SBFD DL/UL subband(s), the following aspects/features need to be considered in RAN1:

- Separated guard band configurations for FDMed and TDMed cases

- Dropping / puncturing / rate-matching rules for FDMed SSB and UL subbands

- Dropping / puncturing / rate-matching rules for FDMed SSB and DL subbands

Proposal 13. RAN1 to consider the following options for FDMed SSB and SBFD DL/UL subbands:

- Separated guard band configurations for FDMed and TDMed cases.

- Dropping / puncturing / rate-matching rules for FDMed SSB and UL subbands.

- Dropping / puncturing / rate-matching rules for FDMed SSB and DL subbands.

CSI feedback enhancements for subband FD

Table 4. Agreements on CSI feedback for SBFD UEs [1], [2].

Agreement Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands. Agreement Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency. Agreement Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following: - RBG for PDSCH RA type 0 - CSI reporting configuration - CSI-RS resource configuration - PRG of PDSCH

As aforementioned, flexible/dynamic resource allocations for both TDD and subband FD operations are beneficial, e.g., for co-existence of legacy (Rel-15/-16/-17) and new (Rel-18 and beyond) UEs. An accurate and proper CSI feedback would be one of the key aspects to realize such flexible/dynamic DL/UL resource allocations across multiple RB sets. Given that frequency resources for a CSI report are configured by higher-layer parameters in CSI report setting (so, there would be no explicit association between CSI report and duplex mode without CSI feedback enhancements), the frequency resources for the duplex operation mode and those for CSI feedback may or may not be identical to each other at a given time instance.

Figure 8 depicts an example of valid frequency resources for CSI derivation according to various duplex operation modes. In Figure 8, it is clear that the bandwidth configured for the corresponding CQI report does not cause any ambiguity (e.g., any configured bandwidth can be applied for both wideband/subband CSI derivations) for CSI reference resources with TDD. On the other hands, since the bandwidth configured for the corresponding CQI report can be wider than the bandwidth for subband FD, TBS derived by the bandwidth configured for the corresponding CQI report and TBS derived by the actual available DL resources for subband FD may not be aligned. These two different TBS values can be quite different and thus the derived CQI indexes could be different as well.

Therefore, to support accurate CSI feedback for various duplex operation modes, it should be clarified that the valid frequency resources for CSI feedback can be affected by the frequency resources allocated for the duplex operation mode associated with the CSI feedback.

Figure 8. An example of valid frequency resources for CSI derivation according to TDD/subband FD operation modes.

Proposal 14. RAN1 to clarify that for a CSI report, a UE shall assume the duplex mode applied for the CSI reference resource of that CSI report.

Proposal 15. RAN1 to clarify that for a CSI report with a frequency resource configuration, only the frequency resources within the DL subband are considered as the valid frequency resources for that CSI report where the corresponding CSI reference resource contains SBFD symbols.

Figure 9. CMR, IMR, CSI reference resource, and PUSCH/PUCCH for a CSI report.

Figure 9 shows CMR, IMR, CSI reference resource and PUSCH/PUCCH resources for a CSI report and their relationships. As in Figure 9, a CSI report shall be associated with 1) at least one CMR and/or IMR and 2) a CSI reference resource. It should be noted that the specification permits a UE to accumulate (or perform a weighted sum) measurements from multiple CMR/IMR occasions unless the CMR/IMR is configured with time-domain measurement restriction for the CSI report via higher-layer parameter timeRestrictionForChannelMeasurements or timeRestrictionForInterferenceMeasurements. It means that a CSI report based on the current version of specifications may be associated with different CMR/IMR occasions, which may be measured in either of SBFD symbol and non-SBFD symbol. Since it cannot be guaranteed that interferences in SBFD symbol and non-SBFD symbol will share the same pattern/tendency, the CSI accuracy in such cases also cannot be secured as well. Therefore, specification supports to align the duplex mode of CMR, IMR, and CSI reference resource for a CSI report would be beneficial to ensure accuracy of the CSI report.

Proposal 16. RAN1 to clarify that for a CSI report, a UE shall only consider the CMR and/or IMR, which shares the same duplex mode with the CSI reference resource.

- For a CSI report with CSI reference resource in SBFD symbols, UE does not take the CMR and/or IMR into account for CSI derivation, if the CMR and/or IMR is on the non-SBFD symbols

- For a CSI report with CSI reference resource in non-SBFD symbols, UE does not take the CMR and/or IMR into account for CSI derivation, if the CMR and/or IMR is on the SBFD symbols

Conclusion

In this contribution, ETRI’s views on subband non-overlapping FD were shown and the following observations and proposals were made:

Observation 1. In case of UE with high capabilities, the channel coherence may be maintained longer than either of the allocated SBFD resource or TDD resource.

Proposal 1. Multiple SBFD subbands with different D-U directions can be explicitly configured on a given time instance from a UE perspective.

Proposal 2. Consider UE-specific RRC signaling for time location and frequency location of the SBFD subbands as the baseline.

- FFS: Cell-specific RRC signaling for time location and frequency location of the SBFD subbands (subject to SBFD support during initial access)

Proposal 3. RAN1 to consider the followings for SBFD subband location indication:

- Interaction with slot format configuration

- Tradeoff between signaling overhead and UL/DL resource partitioning flexibility

- Explicit indication of RB set(s) for flexible subband(s)

- SFI enhancement for dynamic SBFD symbol allocation for a given time instance.

Proposal 4. For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, RAN1 to consider option 2 as the baseline for SBFD aware UEs:

- Option 2:

  - UL transmissions within UL subband are allowed in the symbol

  - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same

    - FFS: SBFD aware UE behaviours

    - FFS: Whether or not signalling of guardband(s) is needed

  - FFS: Whether or not the symbol can be converted to a DL-only symbol

  - Frequency locations of DL subband(s) are known to the SBFD aware UE

  - DL receptions within DL subband(s) are allowed in the symbol

Proposal 5. RAN1 to clarify the followings:

- Explicit signalling of guardband(s) is required for SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon.

- For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, the symbol can be converted to a DL-/UL-only symbol.

- The configured guardband is not applicable for a DL-/UL-only symbol.

Proposal 6. Capture the following use cases of DL-UL direction change for an SBFD aware UE in SBFD symbols in the TR:

- Use case #1: Ease of planning on cell-specific signals / channels

- Use case #2: Latency reduction for UL/DL beam indication, channel measurement, CSI feedback, etc.

- Use case #3: Ease of co-existence with legacy UEs

Proposal 7. Study semi-static and/or dynamic DL-UL direction change of an SBFD aware UE in adjacent SBFD symbols.

Proposal 8. Consider the following options for SBFD aware UEs.

- Option A (baseline): UE does not receive a DL transmission if the DL transmission overlaps, even partially, with a UL subband (and potentially corresponding guard band(s)), and UE does not transmit a UL transmission if the UL transmission overlaps, even partially, with a DL subband (and potentially guard band(s)).

- Option B: If a DL transmission partially overlaps with a UL subband (and potentially corresponding guard band(s)), the UE can receive the non-overlapped DL transmission.

- Option C: If a UL transmission partially overlaps with a DL subband (and potentially corresponding guard band(s)), the UE can transmit the non-overlapped UL transmission.

Proposal 9. Consider the followings for PDCCH receptions in SBFD symbols.

- The same CORESET configuration can be utilized for PDCCH monitoring on DL subbands

- Only the BD/CCE candidates fully overlapping with DL subbands are considered as vaild for PDCCH monitoring

- PDCCH repetition across SBFD symbols and non-SBFD symbols

- FFS, the exact mapping of frequencyDomainResources in DL subbands

Proposal 10. Consider the followings for PDSCH receptions in SBFD symbols.

- The same frequency domain resource allocation [per the same PRB indexing] can be utilized for PDSCH scheduling on DL subbands

- Only the REs overlapping with DL subbands are considered as vaild for PDSCH reception

  - FFS, puncturing and/or rate-matching on the invalid REs

- FFS, whether/how to support SPS PDSCH allocation across SBFD symbols and non-SBFD symbols

Proposal 11. Consider the followings for PUCCH transmissions in SBFD symbols.

- Option 1: Separated PUCCH resource group configurations for non-SBFD symbols (e.g., covered by the 1st PUCCH resource group) and SBFD symbols (e.g., covered by the 2nd PUCCH resource group)

- Option 2: Single PUCCH resource configuration that can span across SBFD and non-SBFD symbols

- FFS, application timing of each PUCCH resource group in case of option 1

  - Slot-level and/or symbol-level PUCCH resource group switching according to the duplex mode

Proposal 12. Consider the followings for PUSCH transmissions in SBFD symbols.

- The same frequency domain resource allocation [per the same PRB indexing] can be utilized for PUSCH scheduling on UL subbands

- Only the REs overlapping with UL subbands are considered as vaild for PUSCH transmission

  - FFS, puncturing and/or rate-matching on the invalid REs

- FFS, whether/how to support CG PUSCH allocation across SBFD symbols and non-SBFD symbols

Proposal 13. RAN1 to consider the following options for FDMed SSB and SBFD DL/UL subbands [from a UE perspective]

- Separated guard band configurations for FDMed and TDMed cases

- Dropping / puncturing / rate-matching rules for FDMed SSB and UL subbands

- Dropping / puncturing / rate-matching rules for FDMed SSB and DL subbands

Proposal 14. RAN1 to clarify that for a CSI report, a UE shall assume the duplex mode applied for the CSI reference resource of that CSI report.

Proposal 15. RAN1 to clarify that for a CSI report with a frequency resource configuration, only the frequency resources within the DL subband are considered as the valid frequency resources for that CSI report where the corresponding CSI reference resource contains SBFD symbols.

Proposal 16. RAN1 to clarify that for a CSI report, a UE shall only consider the CMR and/or IMR, which shares the same duplex mode with the CSI reference resource.

- For a CSI report with CSI reference resource in SBFD symbols, UE does not take the CMR and/or IMR into account for CSI derivation, if the CMR and/or IMR is on the non-SBFD symbols

- For a CSI report with CSI reference resource in non-SBFD symbols, UE does not take the CMR and/or IMR into account for CSI derivation, if the CMR and/or IMR is on the SBFD symbols

- References

- RAN1#110b-e Chair notes

- RAN1#111 Chair notes