3GPP TSG RAN WG1 #112			R1-2301063

Athens, February 27th – March 3rd, 2023

Agenda Item:	9.3.1

Source: 	LG Electronics

Title: 	Study on Evaluation for NR duplex evolution

Document for:	Discussion and decision

Introduction

In this contribution, we discuss the deployment scenarios and evaluation methodology for the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD. In addition, we provide our initial evaluation results on subband non-overlapping full duplex.

Deployment scenarios and evaluation methodology

In this section, we provide our views on the remaining details for deployment scenarios and evaluation methodology based on the agreements made in the RAN1#111 meeting.

Interference modelling for SBFD

In the RAN1#111 meeting, it was agreed to reuse gNB self-interference modelling with some revision for co-site inter-sector co-channel inter-subband CLI modelling.

Agreement For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling as follows. Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB - Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB. - Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale). - NDLRB is the total number of DL RBs in the DL subbands. - Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x. - αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI. - 10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+… - Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment. - Companies shall report the value of αco-site assumed in the simulations with feasibility of how these values were derived. - Send LS to RAN4 confirming the model and asking the value ranges for spatial isolation, and values of 10*log10ACLRBS and 10*log10ACSBS.

The total interference caused by two co-site inter-sector is represented as sum of interference generated from each adjacent sector TRP. The interference generated from adjacent sector reused gNB self-interference modelling, respectively.

In the RAN1#111 meeting, the possibility range of αco-site was discussed. But because of the separated opinions, it has no point of agreement. In this section, the research of the possible range of αco-site is explained.

In [2], sensitivity degradation of co-site inter sector co-channel inter-subband CLI is described. If 1 dB sensitivity degradation is the target, the total of Tx leakage and Rx path contribution should be 1 dB, i.e. -102 dBm. Then each contribution is assumed to be -105 dBm, because it is the sum contributions from all co-site sectors, so -108dBm is used in the following separate analysis. And the -108dBm contains spatial isolation, frequency isolation and other interference suppression schemes.

About spatial isolation, several companies made simulation results. In [3], the result of antenna isolation according to several distance are shown on two adjacent passive antennas using band 3 (1805 MHz – 1880 MHz) and band 39 (1880 MHz – 1920 MHz) respectively. As a result of the experiment, when antenna distance is larger than 1m, coupling loss would be larger than 65dB.

In [4], the results of spatial isolation according to the beam direction are shown on two legacy AAS (Active Antenna System) BSs separated by 2.5 m. As a result of the experiment, while the beam direction is moved from 0 to 60 degree, the isolation at worst case is ~76 dB and the isolation at best case is more than 85dB.

In [5] and [6], the results of spatial isolation according to the distance between two adjacent penal was shown at 3GHz. At the first case, which the horizontal (xy plane) edge-to-edge distance between the panels is 400 mm, and placed at the same vertical height, the result of spatial isolation was 20 dB to 60dB. At second case, which the horizontal (xy plane) edge-to-edge distance between the panels is 400 mm, and the vertical (z axis) edge-to-edge distance is 300 mm, the result of spatial isolation was 55 dB to 70dB.

Summarizing the above experimental results, with various inter distances from 400mm to 2.5m, it can be said that spatial isolation is affected by inter distance. The result shows that, by adjusting inter distance between panels, spatial isolation can be controlled.

Observation 1: By adjusting the distance between panels, spatial isolation can be controlled.

For simulation purpose, by the way, if it is needed to satisfy 1dB sensitivity, total interference of 108dBm is required. And it should be assumed the range of spatial isolation. Therefore, based on above experiments, the range of spatial isolation could be 55 to 85dB.

Proposal 1: For the evaluation purpose, it can be assumed that the range of spatial isolation is 55 to 85dB.

BS transmit power of Scenario B

In previous meetings, BS transmit power for each deployment scenario was discussed, and several agreements were made as follows:

Agreement (RAN1#110) For evaluation of SBFD and dynamic/flexible TDD, the following BS transmit power for legacy TDD are considered. These values are for the single operator case. FR1; FR2-1. Urban macro; - Option 1: [53] dBm for 100MHz - Option 2: [49] dBm for 100MHz [refer to TR 38.828 Table 5.2.1.4-1]; N.A.. Dense Urban Macro layer; - Option 1: [53] dBm for 100MHz - Option 3: [44] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1]; - Option 1: [43] dBm for 200MHz [refer to TR 38.828 Table 5.2.2.4-1]. Dense Urban Micro layer; - Option 3: [40] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1]; - Option 2: [33] dBm for 200MHz. EIRP should not exceed 68 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1]. Indoor hotspot; - Option 2: [24] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.1.1.2-1]; - Option 1: [23] dBm for 200MHz. EIRP should not exceed 58 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1].

Agreement (RAN1#110bis-e) Remove the square brackets and update the agreement made in RAN1#110 for BS transmit power for legacy TDD as below. For evaluation of SBFD and dynamic/flexible TDD, the following BS transmit power for legacy TDD are considered. These values are for the single operator case. FR1; FR2-1. Urban macro; - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz [refer to TR 38.828 Table 5.2.1.4-1]; N.A.. Dense Urban Macro layer; - Option 1: 53 dBm for 100MHz - Option 3: 44 dBm for 100MHz [refer to TR 38.802 Table A.2.1-1]; - Option 1: [43] 40 dBm for 200MHz 100MHz [refer to TR 38.828 Table 5.2.2.4-1]. Dense Urban Micro layer; - Option 3: 38 dBm for 100MHz [refer to TR 38.802 Table A.2.1-1]; - Option 1: [33] 30 dBm for 200MHz 100MHz. EIRP should not exceed 68 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1]. Indoor hotspot; - Option 2: 24 dBm for 100MHz [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.1.1.2-1]; - Option 1: [23] 23 dBm for 200MHz 100MHz. EIRP should not exceed 58 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1].

Agreement (RAN1#111) Update the agreement made in RAN1#110b for BS transmit power for legacy TDD for FR2-1 as below. For evaluation of SBFD and dynamic/flexible TDD, the following BS transmit power for legacy TDD are considered. These values are for the single operator case. FR2-1. Urban macro; N.A.. Dense Urban Macro layer; - Option 1: 40 dBm for 100MHz or 43dBm for 200MHz. Dense Urban Micro layer; - Option 1: 30 dBm for 100MHz or 33dBm for 200MHz. EIRP should not exceed 68 dBm.. Indoor hotspot; - Option 1: 23 dBm for both 100MHz and 200MHz. EIRP should not exceed 58 dBm..

The remaining issue is the assumption of the BS transmission power for deployment Scenario B.  For the deployment scenario B, the BS transmission power for layer 1 and layer 2 depending on frequency range (i.e., FR1 and FR2-1) needs to be decided. For deployment scenario B, the already agreed evaluation assumption for other deployment scenarios can be reused. In the case of FR1, the BS transmission power for Urban Macro can be reused for layer 1, and that for Indoor hotspot can be reused for layer 2. In the case of FR2, the BS transmission power for Dense Urban can be adopted for layer 1, and that for Indoor hotspot can be used for layer 2.

Proposal 2: For deployment scenario B, the already agreed evaluation assumption for other deployment scenarios can be reused.

- In the case of FR1, the BS transmission power for Urban Macro can be reused for layer 1, and that for Indoor hotspot can be reused for layer 2.

- In the case of FR2, the BS transmission power for Dense Urban can be adopted for layer 1, and that for Indoor hotspot can be used for layer 2.

Table 1. The BS transmission power for deployment Scenario B

|  | FR1 | FR2-1 |
| --- | --- | --- |
| Layer 1 | - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz [refer to TR 38.828 Table 5.2.1.4-1] | - Option 1: 40 dBm for 100MHz [refer to TR 38.828 Table 5.2.2.4-1] |
| Layer 2 | - Option 2: 24 dBm for 100MHz [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.1.1.2-1] | - Option 1: 23 dBm for both 100MHz and 200MHz. EIRP should not exceed 58 dBm. |

UE clustering for SBFD deployment Case 4

In the RAN1#111 meeting, it was agreed to apply UE clustering distribution for SBFD Deployment Case 4. Two options were identified, and it was agreed to down-select one option among the two options in RAN1#112 meeting.

But, the method of how deploy the center of UE clustering remains for further study.

Agreement (RAN1#111) UE clustering distribution is also applied for SBFD Deployment Case 4 as baseline. Down-select from the following two options in RAN1#112: - Option 1. Cluster centers for each operator are independently dropped. - Option 2. Cluster centers for operator A are dropped. The cluster centers are used for operator B. - FFS: grid shift case

Even UEs are serviced by different operators, the UE cluster in an area or building could be co-located with high possibility. In this reason, the option 2 seems more reasonable assumption for multiple operation scenario.

Proposal 3: For UE clustering distribution in SBFD deployment Case 4, select option 2 (i.e., Cluster centers for operator A are dropped. The cluster centers are used for operator B.)

Performance evaluation results

This section provides our initial evaluation results on subband non-overlapping full duplex.

In the evaluation, throughput and latency performance between legacy TDD and SBFD are compared. For performance comparison, following frame structure (i.e., Alt 1 in the agreement in RAN1#109-e [4]) for the legacy TDD and SBFD operation is assumed.

- Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

DL and UL resource for the legacy TDD and SBFD operation are illustrated in Figure 1. For the evaluation, SBFD Subband configuration#1 with {DUD} pattern and total 10 PRBs of guard band is assumed.

(a) Legacy TDD (HD only)                                       (b) SBFD (HD and SBFD)

Figure 1. DL/UL resource configuration for (a) legacy TDD and (b) SBFD

To observe the effect of inter-UE CLI, two alternatives for UE clustering distribution of Urban Macro and Dense Urban Macro layer were agreed in RAN1#110-bis-e [2].

Agreement For UE clustering distribution of Urban Macro and Dense Urban Macro layer, take Alt-2 as baseline and Alt-3 as optional. M; X; Indoor UE height (m). Alt-2; 20; 2; 1.5. Alt-3; 10; 1; 1.5.

The Alt-2 is adopted to simulation which is baseline of UE clustering distribution that has multi cluster in single Macro layer.

In addition, ASIR is assumed to reflect the effects of SI and CLI on the DL/UL performance. The ASIR implies the adjacent subband interference ratio and is defined as the ratio of the power transmitted on one subband to the total interference received by a receiver on the adjacent subband, due to both transmitter and receiver imperfections without considering channel. For the evaluation, frequency flat ASIR value as provided in Table 3 is assumed. ASIR_BS_BS and ASIR_UE_UE in the table refer the ASIR value applied to inter-site gNB-gNB co-channel inter-subband CLI and UE-UE co-channel inter-subband CLI, respectively.

For the evaluation, the same DL Tx power of gNB is assumed for legacy TDD and SBFD slot, so Tx power density per RE is increased in SBFD slot compared to legacy TDD slot. Regarding the channel model, large-scale fading is considered only.

In regard of the antenna configuration, the option-2 is applied which is the total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD.

Based on simulation assumption, same FTP arrival rate is assumed for both legacy TDD and SBFD. Other detailed evaluation assumptions are provided in Table 4 and 5 in Annex 1.

Table 2. Evaluation assumption on DL/UL packet size

|  | DL packet size | UL packet size |
| --- | --- | --- |
| Packet size 1 | 4KB | 1KB |
| Packet size 2 | 0.5MB | 0.125MB |

Table 3. Evaluation assumption on ASIR (Adjacent Subband Interference Ratio)

| ASIR_BS_BS | ASIR_UE_UE |
| --- | --- |
| 43dB | 28dB |

Under the assumptions, the evaluation results of the legacy TDD operation (i.e., HD only) and SBFD operation (i.e., HD+SBFD) are obtained.

In Annex 2, Table 6 and Figures 4, 5, 6 and 7 show the evaluation results according to small packet size in Dense Urban deployment scenario with l-layer (i.e., macro layer only). And Table 7 and Figure 8, 9, 10 and 11 show the evaluation results according to large packet size in Dense Urban deployment scenario with l-layer (i.e., macro layer only).

In Annex 3, Table 8 and Figure 12, 13, 14 and 15 show the evaluation results according to small packet size in Indoor Office deployment scenario. And Table 9 and Figures 16, 17, 18 and 19 show the evaluation results according to large packet size in Indoor Office. Each result shows Type 1/Type 2 RU, mean/5%/50%/95% packet delay of all transmission, and UE average/tail/median throughput of legacy TDD and SBFD operation.

Compared to legacy TDD operation, it is allowed that some of downlink resource can be used for uplink transmission in SBFD operation. Hence, it can be expected that the operation of SBFD causes downlink throughput performance reduction due to the lack of downlink resource. However, the tendency may not be the same in all deployment scenario.

In Figure2 and 3, downlink average throughput performance and uplink average throughput performance in the case of Dense Urban (Macro Layer Only) and Indoor Office are depicted, respectively.

|  | UE average DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 44.15; 49.43; 53.71; 49.2. TDD_Medium_RU; 25.60; 39.16; 48.17; 38.2. TDD_High_RU; 1.14; 15.51; 38.15; 16.5. SBFD_Low_RU; 43.84; 48.88; 53.33; 46.5. SBFD_Medium_RU; 22.72; 37.69; 47.16; 35.1. SBFD_High_RU; 1.14; 11.59; 35.83; 13.5. |
| --- | --- |

(a) UE average DL throughput in Dense Urban Macro layer (Mbps)

|  | UE average DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 45.61; 49.98; 54.11; 49.9. TDD_Medium_RU; 36.51; 45.45; 51.31; 44.9. TDD_High_RU; 6.35; 31.27; 45.79; 29.6. SBFD_Low_RU; 45.17; 50.19; 54.37; 50.0. SBFD_Medium_RU; 39.03; 45.89; 51.37; 45.6. SBFD_High_RU; 2.63; 28.37; 44.64; 26.8. |
| --- | --- |

(b) UE average DL throughput in Indoor Office (Mbps)

Figure 2. DL average throughput performance of legacy TDD and SBFD 
(DL: 4KB, UL: 1KB)

|  | UE average UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 3.69; 4.59; 5.61; 4.6. TDD_Medium_RU; 1.27; 2.99; 4.28; 2.9. TDD_High_RU; 0.30; 0.56; 2.83; 1.0. SBFD_Low_RU; 11.50; 12.95; 14.45; 13.0. SBFD_Medium_RU; 10.70; 12.04; 13.16; 12.0. SBFD_High_RU; 7.75; 10.49; 12.02; 10.3. |
| --- | --- |

(a) UE average UL throughput in Dense Urban Macro layer (Mbps)

|  | UE average UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 3.85; 4.70; 5.74; 4.8. TDD_Medium_RU; 2.76; 3.95; 4.98; 3.9. TDD_High_RU; 1.00; 2.48; 4.15; 2.5. SBFD_Low_RU; 11.00; 12.80; 14.50; 12.8. SBFD_Medium_RU; 10.43; 12.11; 13.52; 12.1. SBFD_High_RU; 6.89; 9.83; 12.26; 9.8. |
| --- | --- |

(b) UE average UL throughput in Indoor Office (Mbps)

Figure 3. UL average throughput performance of legacy TDD and SBFD 
(DL: 4KB, UL: 1KB)

In figure 2, DL average throughput of Indoor Office generally shows a performance increment over the Dense Urban on both legacy TDD and SBFD operation. It is because TRP-to-UE distance reduction causes pathloss reduction and it makes enhanced downlink link quality. In figure 2 (a), comparing DL mean average throughput of legacy TDD and SBFD in Dense Urban deployment scenario, legacy TDD outperforms SBFD by 8.83%. However, in figure 2 (b), comparing DL mean average throughput of legacy TDD and SBFD in Indoor Office deployment scenario, legacy TDD and SBFD show a performance difference of about -1.56%.

At least in case of small packet size, the DL mean average throughput reduction of SBFD compared to legacy TDD is large in Dense Urban. However, the DL mean average throughput reduction of SBFD compared to legacy TDD shows less or no performance degradation in Indoor Office.

Observation 2: In small packet size case, compared to Dense Urban macro layer deployment case, SBFD downlink throughput performance in Indoor Office shows less or no performance degradation than legacy TDD operation.

Because SBFD operation uses some DL resource to UL subband resource, the uplink throughput of SBFD outperforms that of legacy TDD in both deployment scenarios (i.e., Dense Urban and Indoor Office deployment). In Figure 3, in the case of medium RU in Dense Urban deployment scenario, UL mean average throughput of SBFD outperforms that of legacy TDD about 313%. In the case of medium RU in Indoor Office deployment scenario, UL mean average throughput of SBFD outperforms that of legacy TDD about 210%.

With summary, in the case of Dense Urban deployment scenario, it is observed that the uplink performance of SBFD operation outperforms that of legacy TDD, and the downlink performance of SBFD is degraded over legacy TDD because of lack of downlink resource in the case of SBFD operation.

In the case of Indoor Office deployment scenario, it is observed that the uplink of SBFD outperforms that of legacy TDD. But, for downlink case, it is observed that the performance of SBFD is degraded marginally over legacy TDD because better spectral efficiency can be achieved due to better link quality in Indoor Office deployment scenario.

Observation 3:

- In the case of Dense Urban deployment scenario, it is observed that the uplink performance of SBFD operation outperforms that of legacy TDD, and the downlink performance of SBFD is degraded over legacy TDD because of lack of downlink resource in the case of SBFD operation.

- In the case of Indoor Office deployment scenario, it is observed that the uplink of SBFD outperforms that of legacy TDD. But, for downlink case, it is observed that the performance of SBFD is degraded marginally over legacy TDD because better spectral efficiency can be achieved due to better link quality in Indoor Office deployment scenario.

Summary

In this contribution, we discussed the deployment scenarios and evaluation methodology for NR duplex evolution. In addition, some initial evaluation results on subband non-overlapping full duplex were provided. From the discussion and evaluation, we obtained following proposals and observations.

Evaluation Assumption: Interference modelling for SBFD

Observation 1: By adjusting the distance between panels, spatial isolation can be controlled.

Proposal 1: For the evaluation purpose, it can be assumed that the range of spatial isolation is 55 to 85dB.

Evaluation Assumption: BS transmit power of Scenario B

Proposal 2: For deployment scenario B, the already agreed evaluation assumption for other deployment scenarios can be reused.

- In the case of FR1, the BS transmission power for Urban Macro can be reused for layer 1, and that for Indoor hotspot can be reused for layer 2.

- In the case of FR2, the BS transmission power for Dense Urban can be adopted for layer 1, and that for Indoor hotspot can be used for layer 2.

Table 1. The BS transmission power for deployment Scenario B

|  | FR1 | FR2-1 |
| --- | --- | --- |
| Layer 1 | - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz [refer to TR 38.828 Table 5.2.1.4-1] | - Option 1: 40 dBm for 100MHz [refer to TR 38.828 Table 5.2.2.4-1] |
| Layer 2 | - Option 2: 24 dBm for 100MHz [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.1.1.2-1] | - Option 1: 23 dBm for both 100MHz and 200MHz. EIRP should not exceed 58 dBm. |

Evaluation Assumption: UE clustering for SBFD deployment Case 4

Proposal 3: For UE clustering distribution in SBFD deployment Case 4, select option 2 (i.e., Cluster centers for operator A are dropped. The cluster centers are used for operator B.)

Performance evaluation results

Observation 2: In small packet size case, compared to Dense Urban macro layer deployment case, SBFD downlink throughput performance in Indoor Office shows less or no performance degradation than legacy TDD operation.

Observation 3:

- In the case of Dense Urban deployment scenario, it is observed that the uplink performance of SBFD operation outperforms that of legacy TDD, and the downlink performance of SBFD is degraded over legacy TDD because of lack of downlink resource in the case of SBFD operation.

- In the case of Indoor Office deployment scenario, it is observed that the uplink of SBFD outperforms that of legacy TDD. But, for downlink case, it is observed that the performance of SBFD is degraded marginally over legacy TDD because better spectral efficiency can be achieved due to better link quality in Indoor Office deployment scenario.

Reference

- CMCC, “New SI: Study on evolution of NR duplex operation,” RP-213591, e-Meeting, December 6th – 17th, 2021

- R4-2218478 Further discussion on BS feasibility study for duplex evolution v1

- R4-2218648 study the feasibility of and impact on RF requirements on gNB side.

- R4-2219146 feasibility study from RF perspective

- R4-2219633 BS RF feasibility considerations

- R1-2211941 Evaluation of NR duplex evolution

- Chair’s note in RAN1#111 meeting

- Chair’s note in RAN1#110bis-e meeting

- Chair’s note in RAN1#110 meeting

- Chair’s note in RAN1#109-e meeting

- Annex 1. Evaluation Assumption

Table 4. Evaluation assumption for Dense Urban (macro layer only)

| Parameters | Evaluation assumption |
| --- | --- |
| Carrier Frequency | FR1: 4GHz (Macro layer) |
| Layout | Single layers: - Macro layer: Hex. Grid, 7 BSs, 3 sectors per BS Min. distance btw macro-to-macro: 200m |
| UE distribution | 210 UEs (10 UEs per BS) UE clustering. 80% of indoor UEs, 20% of outdoor UEs Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) Min. distance btw macro-to-UE: 35m |
| System bandwidth/ Subcarrier spacing | 4GHz: 100MHz / 30kHz (273RBs) |
| Tx power | Macro Tx power: 44dBm UE max. Tx power: 23dBm |
| BS antenna configuration | FR1: (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ |
| UE antenna configuration | FR1: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = 0.5 |
| Large-scale channel parameters | Below 6GHz: - Macro-to-UE: UMa in TR 38.901 - Macro-to-Macro: UMa in TR 38.901 (hUE =25m) - UE-to-UE: A.2.1.2 in TR36.843, penetration loss between UEs follows Table A.2.1-13 in TS38.802 |
| Traffic model | FTP traffic model 3 Downlink: 4/500 KB/packet Uplink: 1/125 KB/packet |
| DL/UL resource pattern | TDD: DDDSUDDDSU SBFD: DXXXUDXXXU UL/DL configuration in X slot S=[12D:2G:0U] DL and UL PRBs in X slot - DL RB: 208 RBs - UL RB: 55 RBs - Guard RB: 10 RBs |
| Resource pattern flexibility | Static and common DL/UL resource pattern among cells |
| ASIR for CLI | SBFD: ASIR BS-BS: 43 dB ASIR UE-UE: 28 dB |
| Residual self-interference | SBFD: Residual SI = Tx power - ASIR – SIC - ASIR: 43 dB - SIC: 80dB |
| Packet dropping timer | 60 slots (A packet is in outage if this packet failed to be successfully received by destination receiver beyond “Packet dropping timer) |
| Output | DL/UL packet delay (slot) - Packet-Latency CDF: The CDF of the packet latencies of all the packets from all the UEs. - Mean/5%/50%/95% Packet-Latency: The mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs. - Packet delay: slot index of packet transmission completion – slot index of packet generation - Minimum packet delay: 1 slot UE average/tail/median DL/UL packet throughput (Mbps) - UE average DL/UL throughput: Harmonic mean of packet size / packet delay - UE tail DL/UL throughput: 5%ile of packet size / packet delay - UE median DL/UL throughput: 50%ile of packet size / packet delay - Mean/5%/50%/95% Average-UPT: The mean/5%/50%/95% value of Average-UPTs for all users. - Mean/5%/50%/95% Tail-UPT: The mean/5%/50%/95% value of Tail-UPTs for all users. - Mean/5%/50%/95% Median-UPT: The mean/5%/50%/95% value of Median-UPTs for all users. |

Table 5. Evaluation assumption for Indoor Office

| Parameters | Evaluation assumption |
| --- | --- |
| Carrier Frequency | FR1: 4GHz |
| Layout | Single layers: Indoor floor: (12BSs per 120m x 50m) Min. distance btw macro-to-macro: 20m [TR 38.802 Table A.2.1-11] |
| UE distribution | 120 UEs (10 UEs per BS) No UE clustering, UE speed is 3km/h, UE height is 1.5m Min. distance btw BS-to-UE: 0m [TR 38.802 Table A.2.1-11] |
| System bandwidth/ Subcarrier spacing | 4GHz: 100MHz / 30kHz (273RBs) |
| Tx power | Indoor Tx power: 24dBm UE max. Tx power: 23dBm |
| BS antenna configuration | FR1: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), (dH, dV) = (0.5, 0.5)λ |
| UE antenna configuration | FR1: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = 0.5 |
| Large-scale channel parameters | Below 6GHz: - TRP -to-UE: InH-Office in TR 38.901 - TRP -to- TRP: InH-Office in TR 38.901 (hUE =3m) - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) |
| Traffic model | FTP traffic model 3 Downlink: 4/500 KB/packet Uplink: 1/125 KB/packet |
| DL/UL resource pattern | TDD: DDDSUDDDSU SBFD: DXXXUDXXXU UL/DL configuration in X slot S=[12D:2G:0U] DL and UL PRBs in X slot - DL RB: 208 RBs - UL RB: 55 RBs - Guard RB: 10 RBs |
| Resource pattern flexibility | Static and common DL/UL resource pattern among cells |
| ASIR for CLI | SBFD: ASIR BS-BS: 43 dB ASIR UE-UE: 28 dB |
| Residual self-interference | SBFD: Residual SI = Tx power - ASIR – SIC - ASIR: 43 dB - SIC: 80dB |
| Packet dropping timer | 60 slots (A packet is in outage if this packet failed to be successfully received by destination receiver beyond “Packet dropping timer) |
| Output | DL/UL packet delay (slot) - Packet-Latency CDF: The CDF of the packet latencies of all the packets from all the UEs. - Mean/5%/50%/95% Packet-Latency: The mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs. - Packet delay: slot index of packet transmission completion – slot index of packet generation - Minimum packet delay: 1 slot UE average/tail/median DL/UL packet throughput (Mbps) - UE average DL/UL throughput: Harmonic mean of packet size / packet delay - UE tail DL/UL throughput: 5%ile of packet size / packet delay - UE median DL/UL throughput: 50%ile of packet size / packet delay - Mean/5%/50%/95% Average-UPT: The mean/5%/50%/95% value of Average-UPTs for all users. - Mean/5%/50%/95% Tail-UPT: The mean/5%/50%/95% value of Tail-UPTs for all users. - Mean/5%/50%/95% Median-UPT: The mean/5%/50%/95% value of Median-UPTs for all users. |

- Annex 2. Evaluation Results of Dense Urban (macro layer only)

- Small packet size

Table 6. DL and UL Resource Utilization of HD TDD and SBFD 
in Dense Urban Macro layer (DL: 4KB, UL: 1KB)

|  | RU Type1 |  | RU Type2 |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| TDD_Low_RU | 6.7 | 2.0 | 8.6 | 10.0 |
| TDD_Medium_RU | 20.0 | 6.0 | 26.0 | 30.0 |
| TDD_High_RU | 38.9 | 10.9 | 50.4 | 54.5 |
| SBFD_Low_RU | 5.1 | 0.4 | 7.8 | 1.3 |
| SBFD_Medium_RU | 15.6 | 1.2 | 23.8 | 3.8 |
| SBFD_High_RU | 30.7 | 2.5 | 46.8 | 7.8 |

|  | UE average DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 44.15; 49.43; 53.71; 49.2. TDD_Medium_RU; 25.60; 39.16; 48.17; 38.2. TDD_High_RU; 1.14; 15.51; 38.15; 16.5. SBFD_Low_RU; 43.84; 48.88; 53.33; 46.5. SBFD_Medium_RU; 22.72; 37.69; 47.16; 35.1. SBFD_High_RU; 1.14; 11.59; 35.83; 13.5. |
| --- | --- |

(a) UE average DL throughput (Mbps)

|  | UE average UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 3.69; 4.59; 5.61; 4.6. TDD_Medium_RU; 1.27; 2.99; 4.28; 2.9. TDD_High_RU; 0.30; 0.56; 2.83; 1.0. SBFD_Low_RU; 11.50; 12.95; 14.45; 13.0. SBFD_Medium_RU; 10.70; 12.04; 13.16; 12.0. SBFD_High_RU; 7.75; 10.49; 12.02; 10.3. |
| --- | --- |

(b) UE average UL throughput (Mbps)

Figure 4. DL and UL average throughput performance of HD TDD and SBFD
 in Dense Urban Macro layer (DL: 4KB, UL: 1KB)

|  | UE median DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 64.00; 64.00; 64.00; 64.0. TDD_Medium_RU; 32.00; 64.00; 64.00; 53.4. TDD_High_RU; 1.10; 21.33; 64.00; 21.4. SBFD_Low_RU; 64.00; 64.00; 64.00; 63.9. SBFD_Medium_RU; 32.00; 64.00; 64.00; 51.9. SBFD_High_RU; 1.10; 16.00; 64.00; 18.1. |
| --- | --- |

(a) UE median DL throughput (Mbps)

|  | UE median UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 4.00; 5.33; 8.00; 5.1. TDD_Medium_RU; 1.60; 4.00; 5.33; 3.5. TDD_High_RU; 0.29; 0.59; 3.20; 1.2. SBFD_Low_RU; 16.00; 16.00; 16.00; 16.0. SBFD_Medium_RU; 16.00; 16.00; 16.00; 16.0. SBFD_High_RU; 8.00; 16.00; 16.00; 15.2. |
| --- | --- |

(b) UE median UL throughput (Mbps)

Figure 5. DL and UL median throughput performance of HD TDD and SBFD
 in Dense Urban Macro layer (DL: 4KB, UL: 1KB)

|  | UE tail DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 21.33; 32.00; 32.00; 30.0. TDD_Medium_RU; 10.67; 21.33; 32.00; 19.6. TDD_High_RU; 1.08; 5.82; 21.33; 7.4. SBFD_Low_RU; 21.33; 32.00; 32.00; 29.6. SBFD_Medium _RU; 10.67; 21.33; 32.00; 18.4. SBFD_High_RU; 1.08; 4.00; 16.00; 6.0. |
| --- | --- |

(a) UE tail DL throughput (Mbps)

|  | UE tail UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 1.60; 2.29; 3.20; 2.4. TDD_Medium_RU; 0.47; 1.23; 2.00; 1.2. TDD_High_RU; 0.27; 0.29; 1.14; 0.5. SBFD_Low_RU; 5.33; 8.00; 8.00; 7.9. SBFD_Medium _RU; 5.33; 8.00; 8.00; 7.2. SBFD_High_RU; 3.20; 5.33; 8.00; 5.3. |
| --- | --- |

(b) UE tail UL throughput (Mbps)

Figure 6. DL and UL tail throughput performance of HD TDD and SBFD
 in Dense Urban Macro layer (DL: 4KB, UL: 1KB)

|  | DL packet latency (slot). 5%; 50%; 95%; Mean. TDD_Low_RU; 0.50; 0.50; 1.00; 0.6. TDD_Medium_RU; 0.50; 0.50; 1.50; 0.8. TDD_High_RU; 0.50; 1.50; 29.00; 7.1. SBFD_Low_RU; 0.50; 0.50; 1.00; 0.6. SBFD_Medium _RU; 0.50; 0.50; 2.00; 0.8. SBFD_High_RU; 0.50; 2.00; 29.00; 8.1. |
| --- | --- |

(a) DL packet latency (slot)

|  | UL packet latency (slot). 5%; 50%; 95%; Mean. TDD_Low_RU; 0.50; 1.50; 3.50; 1.8. TDD_Medium_RU; 0.50; 2.50; 9.00; 3.2. TDD_High_RU; 1.00; 11.50; 29.00; 13.7. SBFD_Low_RU; 0.50; 0.50; 1.00; 0.6. SBFD_Medium _RU; 0.50; 0.50; 1.00; 0.7. SBFD_High_RU; 0.50; 0.50; 1.50; 0.8. |
| --- | --- |

(b) UL packet latency (slot)

Figure 7. DL and UL latency performance of HD TDD and SBFD
 in Dense Urban Macro layer (DL: 4KB, UL: 1KB)

- Large packet size

Table 7. DL and UL Resource Utilization of HD TDD and SBFD 
in Dense Urban Macro layer (DL: 0.5MB, UL: 0.125MB)

|  | RU Type1 |  | RU Type2 |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| TDD_Low_RU | 9.1 | 1.7 | 11.7 | 8.5 |
| TDD_Medium_RU | 20.4 | 4.2 | 26.5 | 20.8 |
| TDD_High_RU | 50.7 | 9.3 | 65.7 | 46.3 |
| SBFD_Low_RU | 8.4 | 0.7 | 12.8 | 2.2 |
| SBFD_Medium_RU | 18.9 | 2.0 | 28.7 | 6.1 |
| SBFD_High_RU | 39.3 | 5.8 | 59.8 | 18.0 |

|  | UE average DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 280.50; 453.75; 615.38; 452.4; 1.7; 11.7; 8.5. TDD_Medium_RU; 219.80; 320.80; 489.80; 333.8; 4.2; 26.5; 20.8. TDD_High_RU; 146.34; 171.11; 213.97; 175.9; 9.3; 65.7; 46.3. SBFD_Low_RU; 228.57; 360.36; 505.04; 346.6; 0.7; 12.8; 2.2. SBFD_Medium _RU; 179.37; 256.76; 378.28; 252.1; 2.0; 28.7; 6.1. SBFD_High_RU; 139.13; 166.67; 203.88; 159.6; 5.8; 59.8; 18.0. |
| --- | --- |

(a) UE average DL throughput (Mbps)

|  | UE average UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 36.70; 47.62; 64.52; 36.70. TDD_Medium_RU; 36.36; 46.65; 60.61; 36.36. TDD_High_RU; 35.71; 43.64; 52.98; 35.71. SBFD_Low_RU; 60.61; 90.91; 119.21; 60.61. SBFD_Medium _RU; 53.48; 76.92; 106.01; 53.48. SBFD_High_RU; 44.53; 56.56; 75.64; 44.53. |
| --- | --- |

(b) UE average UL throughput (Mbps)

Figure 8. DL and UL average throughput performance of HD TDD and SBFD
 in Dense Urban Macro layer (DL: 0.5MB, UL: 0.125MB)

|  | UE median DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 307.69; 533.33; 213.97; 514.0. TDD_Medium_RU; 230.37; 380.95; 615.38; 398.4. TDD_High_RU; 142.86; 166.67; 228.57; 175.6. SBFD_Low_RU; 245.42; 432.75; 203.88; 421.0. SBFD_Medium _RU; 188.26; 296.30; 500.00; 312.7. SBFD_High_RU; 140.35; 166.67; 219.33; 171.7. |
| --- | --- |

(a) UE median DL throughput (Mbps)

|  | UE median UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 37.04; 48.78; 66.67; 37.04. TDD_Medium_RU; 36.36; 48.26; 63.51; 36.36. TDD_High_RU; 35.71; 45.45; 58.82; 35.71. SBFD_Low_RU; 66.67; 95.24; 125.00; 66.67. SBFD_Medium _RU; 58.82; 87.12; 117.65; 58.82. SBFD_High_RU; 44.44; 62.56; 95.24; 44.44. |
| --- | --- |

(b) UE median UL throughput (Mbps)

Figure 9. DL and UL median throughput performance of HD TDD and SBFD
 in Dense Urban Macro layer (DL: 0.5MB, UL: 0.125MB)

|  | UE tail DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 156.86; 275.86; 533.33; 299.8. TDD_Medium_RU; 137.93; 173.91; 296.30; 189.6. TDD_High_RU; 135.59; 137.93; 150.94; 140.4. SBFD_Low_RU; 140.35; 210.53; 400.00; 231.2. SBFD_Medium _RU; 135.59; 150.94; 210.53; 158.6. SBFD_High_RU; 135.59; 137.93; 156.86; 140.9. |
| --- | --- |

(a) UE tail DL throughput (Mbps)

|  | UE tail UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 35.09; 45.45; 62.50; 35.09. TDD_Medium_RU; 33.90; 40.00; 57.14; 33.90. TDD_High_RU; 33.90; 35.09; 42.55; 33.90. SBFD_Low_RU; 46.04; 83.33; 117.65; 46.04. SBFD_Medium _RU; 35.71; 51.28; 95.24; 35.71. SBFD_High_RU; 33.90; 36.36; 46.02; 33.90. |
| --- | --- |

(b) UE tail UL throughput (Mbps)

Figure 10. DL and UL tail throughput performance of HD TDD and SBFD
 in Dense Urban Macro layer (DL: 0.5MB, UL: 0.125MB)

|  | DL packet latency (slot). 5%; 50%; 95%; Mean. TDD_Low_RU; 6.00; 8.00; 17.50; 9.4. TDD_Medium_RU; 6.00; 11.00; 24.50; 12.6. TDD_High_RU; 8.50; 23.00; 29.50; 21.5. SBFD_Low_RU; 7.00; 10.00; 22.00; 11.6. SBFD_Medium _RU; 7.50; 14.00; 27.00; 15.4. SBFD_High_RU; 10.50; 24.00; 29.50; 22.4. |
| --- | --- |

(a) DL packet latency (slot)

|  | UL packet latency (slot). 5%; 50%; 95%; Mean. TDD_Low_RU; 15.00; 20.50; 27.50; 20.7. TDD_Medium_RU; 15.50; 21.00; 28.00; 21.2. TDD_High_RU; 16.00; 22.50; 29.00; 22.4. SBFD_Low_RU; 8.00; 10.50; 17.50; 11.5. SBFD_Medium _RU; 8.50; 12.00; 23.00; 13.2. SBFD_High_RU; 9.50; 16.50; 28.00; 17.4. |
| --- | --- |

(b) UL packet latency (slot)

Figure 11. DL and UL latency performance of HD TDD and SBFD
 in Dense Urban Macro layer (DL: 0.5MB, UL: 0.125MB)

- Annex 3. Evaluation Results of Indoor Office

- Small packet size

Table 8. DL and UL Resource Utilization of HD TDD and SBFD 
in Indoor Office (DL: 4KB, UL: 1KB)

|  | RU Type1 |  | RU Type2 |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| TDD_Low_RU | 8.1 | 2.4 | 10.4 | 11.9 |
| TDD_Medium_RU | 18.1 | 5.4 | 23.4 | 27.0 |
| TDD_High_RU | 39.4 | 10.1 | 51.1 | 50.5 |
| SBFD_Low_RU | 6.1 | 0.5 | 9.3 | 1.5 |
| SBFD_Medium_RU | 13.7 | 1.1 | 20.8 | 3.6 |
| SBFD_High_RU | 33.3 | 3.1 | 50.7 | 9.7 |

|  | UE average DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 45.61; 49.98; 54.11; 49.9. TDD_Medium_RU; 36.51; 45.45; 51.31; 44.9. TDD_High_RU; 6.35; 31.27; 45.79; 29.6. SBFD_Low_RU; 45.17; 50.19; 54.37; 50.0. SBFD_Medium_RU; 39.03; 45.89; 51.37; 45.6. SBFD_High_RU; 2.63; 28.37; 44.64; 26.8. |
| --- | --- |

(a) UE average DL throughput (Mbps)

|  | UE average UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 3.85; 4.70; 5.74; 4.8. TDD_Medium_RU; 2.76; 3.95; 4.98; 3.9. TDD_High_RU; 1.00; 2.48; 4.15; 2.5. SBFD_Low_RU; 11.00; 12.80; 14.50; 12.8. SBFD_Medium_RU; 10.43; 12.11; 13.52; 12.1. SBFD_High_RU; 6.89; 9.83; 12.26; 9.8. |
| --- | --- |

(b) UE average UL throughput (Mbps)

Figure 12. DL and UL average throughput performance of HD TDD and SBFD
 in Indoor Office (DL: 4KB, UL: 1KB)

|  | UE median DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 64.00; 64.00; 64.00; 64.0. TDD_Medium_RU; 64.00; 64.00; 64.00; 62.6. TDD_High_RU; 7.11; 32.00; 64.00; 39.3. SBFD_Low_RU; 64.00; 64.00; 64.00; 64.0. SBFD_Medium_RU; 64.00; 64.00; 64.00; 63.8. SBFD_High_RU; 2.46; 32.00; 64.00; 35.3. |
| --- | --- |

(a) UE median DL throughput (Mbps)

|  | UE median UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 4.00; 5.33; 8.00; 5.3. TDD_Medium_RU; 3.20; 4.00; 5.33; 4.4. TDD_High_RU; 0.94; 3.20; 4.70; 3.0. SBFD_Low_RU; 16.00; 16.00; 16.00; 15.9. SBFD_Medium_RU; 16.00; 16.00; 16.00; 15.9. SBFD_High_RU; 8.00; 16.00; 16.00; 13.8. |
| --- | --- |

(b) UE median UL throughput (Mbps)

Figure 13. DL and UL median throughput performance of HD TDD and SBFD
 in Indoor Office (DL: 4KB, UL: 1KB)

|  | UE tail DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 21.33; 32.00; 32.00; 30.8. TDD_Medium_RU; 16.00; 21.33; 32.00; 25.3. TDD_High_RU; 2.56; 16.00; 21.33; 14.3. SBFD_Low_RU; 21.33; 32.00; 32.00; 30.5. SBFD_Medium _RU; 21.33; 21.33; 32.00; 25.9. SBFD_High_RU; 2.21; 12.80; 21.33; 12.7. |
| --- | --- |

(a) UE tail DL throughput (Mbps)

|  | UE tail UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 1.60; 2.67; 3.20; 2.6. TDD_Medium_RU; 1.14; 1.78; 3.20; 1.8. TDD_High_RU; 0.55; 1.00; 1.78; 1.1. SBFD_Low_RU; 5.33; 8.00; 8.00; 7.8. SBFD_Medium _RU; 5.33; 8.00; 8.00; 7.1. SBFD_High_RU; 3.20; 5.33; 8.00; 5.0. |
| --- | --- |

(b) UE tail UL throughput (Mbps)

Figure 14. DL and UL tail throughput performance of HD TDD and SBFD
 in Indoor Office (DL: 4KB, UL: 1KB)

|  | DL packet latency (slot). 5%; 50%; 95%; Mean. TDD_Low_RU; 0.50; 0.50; 1.00; 0.6. TDD_Medium_RU; 0.50; 0.50; 1.50; 0.7. TDD_High_RU; 0.50; 1.00; 7.50; 1.7. SBFD_Low_RU; 0.50; 0.50; 1.00; 0.6. SBFD_Medium _RU; 0.50; 0.50; 1.50; 0.7. SBFD_High_RU; 0.50; 1.00; 11.00; 1.9. |
| --- | --- |

(a) DL packet latency (slot)

|  | UL packet latency (slot). 5%; 50%; 95%; Mean. TDD_Low_RU; 0.50; 1.50; 3.50; 1.7. TDD_Medium_RU; 0.50; 2.00; 5.00; 2.1. TDD_High_RU; 0.50; 2.50; 11.50; 3.8. SBFD_Low_RU; 0.50; 0.50; 1.00; 0.6. SBFD_Medium _RU; 0.50; 0.50; 1.00; 0.7. SBFD_High_RU; 0.50; 0.50; 2.00; 0.8. |
| --- | --- |

(b) UL packet latency (slot)

Figure 15. DL and UL latency performance of HD TDD and SBFD
 in Indoor Office (DL: 4KB, UL: 1KB)

- Large packet size

Table 9. DL and UL Resource Utilization of HD TDD and SBFD 
in Indoor Office (DL: 0.5MB, UL: 0.125MB)

|  | RU Type1 |  | RU Type2 |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| TDD_Low_RU | 7.9 | 2.4 | 10.2 | 12.1 |
| TDD_Medium_RU | 20.0 | 6.6 | 25.9 | 32.8 |
| TDD_High_RU | 39.0 | 10.2 | 50.6 | 51.1 |
| SBFD_Low_RU | 7.6 | 1.0 | 11.6 | 3.2 |
| SBFD_Medium_RU | 18.6 | 3.4 | 28.3 | 10.6 |
| SBFD_High_RU | 33.6 | 6.4 | 51.1 | 20.0 |

|  | UE average DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 320.00; 470.59; 627.45; 472.4; 1.7; 11.7; 8.5. TDD_Medium_RU; 224.61; 324.52; 484.85; 337.6; 4.2; 26.5; 20.8. TDD_High_RU; 174.55; 239.08; 347.88; 247.8; 9.3; 65.7; 46.3. SBFD_Low_RU; 255.97; 380.95; 521.82; 382.8; 0.7; 12.8; 2.2. SBFD_Medium _RU; 190.02; 270.48; 376.84; 276.1; 2.0; 28.7; 6.1. SBFD_High_RU; 156.86; 207.79; 284.39; 212.7; 5.8; 59.8; 18.0. |
| --- | --- |

(a) UE average DL throughput (Mbps)

|  | UE average UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 45.23; 59.70; 74.07; 60.1. TDD_Medium_RU; 42.02; 52.42; 65.39; 52.9. TDD_High_RU; 40.10; 49.21; 60.27; 49.4. SBFD_Low_RU; 76.92; 107.33; 139.54; 107.3. SBFD_Medium _RU; 58.82; 78.01; 103.73; 78.9. SBFD_High_RU; 47.47; 60.22; 83.11; 62.3. |
| --- | --- |

(b) UE average UL throughput (Mbps)

Figure 16. DL and UL average throughput performance of HD TDD and SBFD
 in Indoor Office (DL: 0.5MB, UL: 0.125MB)

|  | UE median DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 347.83; 571.43; 347.88; 549.2. TDD_Medium_RU; 235.29; 379.99; 575.31; 387.9. TDD_High_RU; 173.91; 260.75; 444.44; 278.7. SBFD_Low_RU; 296.03; 470.59; 284.39; 449.4. SBFD_Medium _RU; 190.48; 307.69; 470.59; 313.4. SBFD_High_RU; 156.86; 219.22; 349.27; 233.0. |
| --- | --- |

(a) UE median DL throughput (Mbps)

|  | UE median UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 47.64; 62.50; 76.92; 63.3. TDD_Medium_RU; 42.55; 57.14; 71.43; 57.1. TDD_High_RU; 40.82; 52.63; 67.82; 53.2. SBFD_Low_RU; 83.33; 117.65; 142.86; 115.7. SBFD_Medium _RU; 64.52; 86.96; 117.65; 88.7. SBFD_High_RU; 49.45; 67.82; 97.62; 70.0. |
| --- | --- |

(b) UE median UL throughput (Mbps)

Figure 17. DL and UL median throughput performance of HD TDD and SBFD
 in Indoor Office (DL: 0.5MB, UL: 0.125MB)

|  | UE tail DL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 177.39; 320.00; 571.43; 337.4. TDD_Medium_RU; 140.35; 181.82; 333.33; 200.4. TDD_High_RU; 135.59; 148.15; 195.12; 154.6. SBFD_Low_RU; 148.15; 258.06; 470.59; 273.9. SBFD_Medium _RU; 135.59; 156.86; 250.00; 171.5. SBFD_High_RU; 135.59; 142.86; 173.91; 147.2. |
| --- | --- |

(a) UE tail DL throughput (Mbps)

|  | UE tail UL throughput (Mbps). 5%; 50%; 95%; Mean. TDD_Low_RU; 35.71; 54.05; 71.43; 53.8. TDD_Medium_RU; 33.90; 38.46; 55.79; 40.9. TDD_High_RU; 33.90; 35.71; 45.51; 37.3. SBFD_Low_RU; 54.05; 90.91; 133.33; 92.7. SBFD_Medium _RU; 35.09; 48.78; 80.00; 51.9. SBFD_High_RU; 33.90; 37.04; 54.05; 39.5. |
| --- | --- |

(b) UE tail UL throughput (Mbps)

Figure 18. DL and UL tail throughput performance of HD TDD and SBFD
 in Indoor Office (DL: 0.5MB, UL: 0.125MB)

|  | DL packet latency (slot). 5%; 50%; 95%; Mean. TDD_Low_RU; 6.00; 7.50; 16.50; 8.8. TDD_Medium_RU; 6.00; 11.00; 24.00; 12.4. TDD_High_RU; 7.23; 15.50; 27.50; 16.3. SBFD_Low_RU; 7.00; 9.50; 20.25; 10.9. SBFD_Medium _RU; 7.50; 13.50; 26.50; 14.9. SBFD_High_RU; 9.00; 18.50; 28.50; 18.7. |
| --- | --- |

(a) DL packet latency (slot)

|  | UL packet latency (slot). 5%; 50%; 95%; Mean. TDD_Low_RU; 13.00; 16.50; 25.00; 17.0. TDD_Medium_RU; 13.50; 18.00; 27.50; 19.1. TDD_High_RU; 13.50; 19.50; 28.50; 20.2. SBFD_Low_RU; 7.00; 9.00; 14.50; 9.7. SBFD_Medium _RU; 8.00; 12.00; 22.50; 13.1. SBFD_High_RU; 9.00; 15.00; 27.00; 16.3. |
| --- | --- |

(b) UL packet latency (slot)

Figure 19. DL and UL latency performance of HD TDD and SBFD
 in Indoor Office (DL: 0.5MB, UL: 0.125MB)