3GPP TSG RAN WG1#112	           R1-2301217

Athens, Greece, 27th February-03rd March 2023

Agenda item:	9.11.2

Source: 	Lenovo

Title: 	On NTN NW verified UE location

Document for:	Discussion

# Introduction

The Rel-18 NR NTN SID was agreed upon during the RAN#94-e [1] meeting and revised during the RAN#95 meeting, where one of the objectives included the study of solutions for the network verified UE location in NTN networks as indicated by the following:

Network verified UE Location: Pending on the conclusion of the RAN SI FS_NR_NTN_netw_verif_UE_loc study item, study and evaluate, if needed, solutions for network to verify UE reported location information [RAN2,RAN1,RAN3]. RAN is expected to determine by RAN#98 whether the study has identified any need for Network verified UE location specification support in Rel-18.

Further to the above, RAN completed a use case and requirement study [TR 38.882, 2] where the key recommendation of the study includes the following:

TR 38.882 Recommendation (Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR (Release 18): In this study, we have identified the need to define a network-based solution which aims at verifying the reported UE location information. The verification should be performed independently from the location information reported by UE. The UE location information for the study is considered verified if the reported UE location is consistent with the network based assessment to within 5-10 km (similar to terrestrial network macro cell size), enabling country discrimination and selection of an appropriate core network in order to support all the regulatory services (i.e. emergency call, lawful intercept, public warning, charging/billing). The solution should not impact significantly the latency of the targeted services nor infringe privacy requirements that apply to the UE location. The study in [RAN2,RAN1,RAN3], which will study and evaluate solutions for the network to verify UE reported location information, shall consider the following aspects: - The scenario of single satellite (or HAPS) in view by the UE at a time is considered with higher priority. - Multiple satellite (or HAPS) in view by the UE may be considered if time allows - Assume that the UE is attached to a network (so that its context has been set up in the network) for the purpose of positioning - Different solutions or positioning methods for NGSO, GSO or HAPS are not precluded - When considering solutions based on positioning methods, existing 3GPP defined RAT dependent positioning methods shall be considered as baseline. Other methods are not precluded. - Solutions using existing NG-RAN architecture and procedures shall be considered

Further to the above, following was concluded in RAN#98e for the normative work stage.

Network verified UE location Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. Note 1: Enhancements assume reuse of the RAT dependent positioning framework Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity) Note 4 : Multiple satellite in view by the UE may be considered if time allows Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 6 : The enhancements should take into account the mirror-image ambiguity Note 7 : Network verified UE location is an optional UE feature

This contribution provides a discussion into the different aspects for consideration in relation to the support of network verified UE location for NTN deployments.

# Multi-RTT enhancements for NW verified UE location

The positioning methods in 3GPP are devised for terrestrial networks, where the network node (e.g., gNB) is static and at least three measurements for signals transmitted (e.g., PRS) from three network entities (e.g., gNBs) are needed for computation of precise location estimates at least in the case of DL-TDoA. However, in NTN using a transparent payload architecture, a UE connects to the network via satellites that are moving along an orbital plane. Moreover, a common scenario in NTN is the case of single satellite in view, where location estimates are to be calculated using a single node. The positioning methods such as Multi-RTT may work for single satellite (although with low accuracy) by utilizing the movement of satellites, i.e., by performing measurements at multiple time instances. Performing measurements at multiple instances may add significant latency that may be another issue. Therefore, for the positioning procedures to work at least for single satellite, some signalling enhancements may be needed, while only adding an acceptable amount of latency.

Proposal 1: RAN1 to study low latency signalling enhancements to support Multi-RTT positioning with single satellite.

## Measurement configuration and reporting enhancements

In the case of Multi-RTT with single satellite, three or more measurements at different time instances may be required to estimate a location of target UE, where there should at least be some minimum gap between the measurements for Multi-RTT to work. If there is no minimum gap then the circle/rings formed at different times may overlap each other, resulting in accurate positioning estimates, whereas this measurement gap may be in the order of seconds for Multi-RTT to work. Current specifications for Multi-RTT do not define how this gap may be configured in NTN context or how the measurements may be configured for multiple time instances for single satellite, or how periodicity of a measurement configuration may mean for single satellite case.

Similarly, some reporting enhancements may be needed to relate the measurements performed by UE at different time instances with a defined measurement gap. In addition, it would be beneficial to enable the UE to report measurements at pre-defined configurable time instances. Moreover, generating multiple reports over a long period, e.g., spanning up to 60 sec, may further add to latency that is already quite high in case of single satellite. Therefore, in the context of Multi-RTT with single satellite, further study may be needed to identify how existing measurement configurations and reporting procedures may translate for the single satellite case, what additional enhancements may be needed for both downlink and uplink reference signals and how the existing procedures may be modified to reduce the latency,

Proposal 2: RAN1 to identify the measurement configuration and reporting enhancements needed for low latency Multi-RTT method with single satellite for both uplink and downlink reference signals.

## gNB Rx-Tx time difference definition for NTN

The current specification defines the gNB Rx-Tx time difference, used in Multi-RTT, as

| Definition | The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources can be used to determine the start of one subframe containing SRS. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector. |
| --- | --- |

However, this definition may be misleading in case of NTN systems with transparent payload and with single satellite, as the measurements from service link may need to be considered to calculate the gNB Rx-Tx difference, as compared to the time difference between gNB on ground transmit-receive difference (service link and feeder link propagation delay). For example, as indicated in the Figure, the gNB Rx-Tx may from t5 to t2 may be needed for multiple time instances for single satellite with transparent payload.

This may require some enhancements in the signalling or some adaptations in the definition to make it work for NTN systems with transparent payload. In addition, the current reference point needs to also be updated for Multi-RTT to work for NTN systems.

Proposal 3: Study the mechanism to adapt/modify the gNB Rx-Tx time difference definition for Multi-RTT for NTN systems with transparent payload.

## Polarization support indication for uplink and downlink reference signal for NTN positioning

In NR Rel-17, use of multiple polarization types such linear, left circular polarization (LHCP), and right hand circular polarization (RHCP) has been adopted in NTN, where it is optional for UE to support the polarization signalling in NR NTN. The polarization signalling is supported in SIB reception that indicates DL and/or UL polarization information. In addition, the polarization signalling is supported for target serving cell in handover command message and for non-serving cell in RRM measurement configuration. Currently there is no polarization indication support for reference signals used in positioning. This may be needed as a polarization mismatch can further degrade the positioning accuracy in NTN or increase the positioning latency, owing to low link budget in NTN.

Observation 1: Polarization indication signalling is not supported for reference signal used in positioning.

Observation 2: A polarization mismatch may decrease the positioning accuracy and/or increase the latency.

Proposal 4: Support an indication of polarization to be used for uplink and downlink reference signals for positioning.

The use of a polarization for uplink and downlink reference signals for positioning may strongly depend on the ability of a UE to support a polarization type. Moreover, the location server may need to know which polarizations may be supported by the network node and/or UE, in order to correctly assign a polarization type for the reference signals or to evaluate the reason for low positioning accuracy.

Proposal 5: Support a signalling mechanism to indicate the UE capability to support a polarization type for positioning.

# Mirror Ambiguity Resolution

For time based positioning methods such as Multi-RTT and DL/UL-TDOA with a single satellite, there would be multiple measurements required at different time instances to estimate the UE location. However, the geometric circle/rings formed by the measurements at three time instances would center along the satellite orbital plane, and eventually that would result in intersection of the rings/circle at two points forming a mirror image of each other. Especially, this mirror ambiguity may always exist for all those UEs that are in the beams/cells underneath the satellite orbital plane, resulting in two location estimates of target UEs.

Observation 3: The mirror ambiguity results in two positioning estimates of the target UE.

The mirror ambiguity can be resolved if it is known to the NG-RAN node or LMF that a UE is located on the left or right side of the satellite orbital plane. This can be estimated by combining measurements from other methods, e.g., using UL angle of arrival measurements along with Multi-RTT measurements. A solution may be to beamform positioning reference signal in a way that beams are always pointing either to the left or right side of the satellite orbital plane. Thus, knowing only the beam identity of the positioning reference signal may be sufficient to know that UE is located on the left or right side of the satellite orbital plane, that in return would resolve the mirror ambiguity.

Observation 4: Knowledge of positioning beam identity or uplink angle of arrival may be sufficient to resolve mirror ambiguity.

Proposal 6: Support indication of positioning beam identity or uplink angle of arrival to address mirror ambiguity in time-based positioning methods such as Multi-RTT and DL/UL-TDOA with a single satellite.

Since the location estimates are to be used for verification purposes based on a network-induced location request (NI-LR), one way forward is to let AMF decide whether there is a need to resolve the mirror ambiguity. For example, AMF may consider a UE position verified, if one or both location estimates fulfil the accuracy threshold. This can reduce the latency as no further method/procedure need to be employed.

Proposal 7: Send a LS to SA2 to clarify whether mirror ambiguity needs to be resolved by the LMF or location estimates with mirror ambiguity may be sufficient for verification using NI-LR procedures.

# Geometrical limitations of time based methods

During the study phase in RAN1, it was observed by multiple companies that UEs that are along/near the orbital plane of the satellite and are only connected through that satellite, the location estimates for those UEs would not be accurate due to geometrical limitations of time-based positioning methods to accurately calculate the location estimates. These UEs would always be rejected even though these UEs may be valid UEs. Since, the satellite is constantly moving along the orbital plane, many UEs may fall under this area of uncertainty/ambiguity. One option is to make the verification procedure optional for those UEs that are in the coverage of single satellite, or AMF may not deregister a UE, if it is indicated to AMF by LMF that the UE is located under the satellite orbital plane.

Proposal 8: Send a LS to SA1 and SA2 to clarify about the verification procedure for UEs that are located under the orbital plane of a satellite and have a single satellite in view.

# Conclusion

The following observations are summarized as follows in relation to NTN network verified UE location:

Observation 1: Polarization indication signalling is not supported for reference signal used in positioning.

Observation 2: A polarization mismatch may decrease the positioning accuracy and/or increase the latency.

Observation 3: The mirror ambiguity would result in two positioning estimates of the target UE.

Observation 4: Knowledge of positioning beam identity or uplink angle of arrival may be sufficient to resolve mirror ambiguity.

The corresponding proposals are summarized as follows:

Proposal 1: RAN1 to study low latency signalling enhancements to support Multi-RTT positioning with single satellite.

Proposal 2: RAN1 to identify the measurement configuration and reporting enhancements needed for low latency Multi-RTT method with single satellite for both uplink and downlink reference signals.

Proposal 3: Study the mechanism to adapt/modify the gNB Rx-Tx time difference definition for Multi-RTT for NTN systems with transparent payload.

Proposal 4: Support an indication of polarization to be used for uplink and downlink reference signals for positioning.

Proposal 5: Support a signalling mechanism to indicate the UE capability to support a polarization type for positioning.

Proposal 6: Support indication of positioning beam identity or uplink angle of arrival to address mirror ambiguity in time-based positioning methods such as Multi-RTT and DL/UL-TDOA with a single satellite.

Proposal 7: Send a LS to SA2 to clarify whether mirror ambiguity needs to be resolved by the LMF or location estimates with mirror ambiguity may be sufficient for verification using NI-LR procedures.

Proposal 8: Send a LS to SA1 and SA2 to clarify about the verification procedure for UEs that are located under the orbital plane of a satellite and have a single satellite in view.

# References

- RP-221819, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, RAN#96, June. 2021.

- RP-223534, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, RAN#98e, December. 2022.

- 3GPP TR 38.882, “Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR”, V18.0.0, June 2022.