3GPP TSG RAN WG1 #112	        		             R1-2301295

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.15.1

Source: 	Samsung

Title: 	Discussion on UE capability and RRC signaling for UAV beamforming

Document for:	Discussion and Decision

Background

As a possible way to manage interference of UAV aerial link, UAV side UE beamforming in FR 1 is considered. RAN1 is now studying whether further UE capability reporting for UAV beamforming may provide practical gain for terrestrial UE protection. In a previous meeting RAN1 #110b, we agreed on candidate parameters as contents of UE’s UAV beamforming capability reporting.

Agreement Study extending application of FR2-only beam management parameters e.g., spatial relation, beam correspondence, etc. to FR1 for UAV UEs FFS: Other parameters FFS: Impacts to legacy beam management for FR1 FFS: Application of beam correspondence in FDM bands Note: Identification of relevant UAV UE capabilities does not require commitment to support a specific TCI framework, and relevant parameters may change depending on the framework supported Note: Whether or not to specify above parameters should depend on the identification of the target scenarios and the potential issues faced by UAV, the identification of which existing capabilities and/or mechanisms and/or frameworks can be treated as candidate for addressing above issues, and the necessity of specifying above parameters (i.e., whether the potential issues faced by UAV can already be solved with the existing capabilities and/or mechanisms and/or frameworks) Agreement Study indication of beam characteristics, e.g., number of beams, beamwidth, beam center, radiated EIRP, etc. as UAV UE capability FFS: Feasibility/benefit of indicating orientation of beams including height dependence FFS: Necessary parameters, ranges of suitable values, and method of indication FFS: Height-dependence on relevant parameters FFS: Indication of beams as either ‘fixed’ or ‘adaptive’ Note: Whether or not to specify above parameters should depend on the identification of the target scenarios and the potential issues faced by UAV, the identification of which existing capabilities and/or mechanisms and/or frameworks can be treated as candidate for addressing above issues, and the necessity of specifying above parameters (i.e., whether the potential issues faced by UAV can already be solved with the existing capabilities and/or mechanisms and/or frameworks) Agreement Study indication of minimum beam application latency as UAV UE capability - If unifiedJointTCI-r17 is supported, suitable range of values for minBeamApplicationTime-r17 - If unifiedJointTCI-r17 is not supported, enhancements to timedurationforQCL may be considered - FFS: additional parameters, e.g., beamSwitchTiming Note: further consideration does not require commitment to support a specific TCI framework. Note: Whether or not to specify above parameters should depend on the identification of the target scenarios and the potential issues faced by UAV, the identification of which existing capabilities and/or mechanisms and/or frameworks can be treated as candidate for addressing above issues, and the necessity of specifying above parameters (i.e., whether the potential issues faced by UAV can already be solved with the existing capabilities and/or mechanisms and/or frameworks)

In this contribution, we discuss target scenario to confirm the validity of each candidate parameters. Possible benefits of each candidate parameters are discussed with proposed target scenario.

Discussions

Target Scenario

According to discussion in RAN1 #110b, target scenario is critical at confirming the necessity or benefits of each capability parameters for UAV beamforming. Considering available time units, RAN1 may need to focus on single scenario treats at least one feature for performance enhancements revealed by previous UAV study [1]. In RAN1’s perspective, ICI management or avoidance for the terrestrial UE protection should be an issue to be considered.

Proposal 1: RAN1 focuses on a single scenario for UAV performance enhancement considering terrestrial UE protection via aerial link interference management or avoidance

During RAN plenary discussion, it was noted by several companies that UAV’s beamforming may help network’s mobility management. With UAV beamforming, as an expectation, UL ICI toward terrestrial UE could be reduced and network may have flexibility to toss UAV to less crowded cell for terrestrial UE protection [3]. As an example, for a UAV flying in a crowded area, the network may allow access via a gNB which is less loaded but located at a longer distance to avoid or reduce impact on terrestrial UE communication. In this example, it is assumed that UAV avoids ICI generation toward the cell with the best link quality via proper utilization of UL beam. We think this is valid scenario also aligned with other working group’s target of enhancement.

Figure 1. Example of UE’ beam sharping for aerial communication

Figure 1 shows the example above where network needs traffic off-loading for cell A. It is also assumed cell B or cell C can be a target cell for UAV UE’s handover. UAV UE performs beam adaptation for serving cell after handover. If cell B is selected as target cell, after handover, UAV UE generates beam adjusted for the aerial link with cell B as grey colored beam above. Due to the spatial correlation between grey beam and blue beam, in LoS environment, UE may observe similar level of pathloss from cell A and cell B, after beam adjustment. It means quite high level of interference will arrive at cell A and intended protection of terrestrial UE is not achieved. As shown above, spatial multi-cell information is essential for the terrestrial UE protection, but considering the work scope defined by WID [2], that information should be available at network without introduction of new reporting format or contents. Considering these aspect together, we propose target scenario as below:

Proposal 2: RAN1 consider following case as a target scenario for Rel-18 UAV beamforming capability discussion

- Network commands handover for UAV UE to non-best cell for the terrestrial UE protection or cell traffic off-loading with following conditions

  - After handover, with UAV UE’s beam adjustment toward new serving cell, legacy serving cell observes sufficiently reduced RSRP via aerial link

    - TBD: How much reduction should be assumed/required

  - Required information to make handover decision above is extracted from legacy reporting and UE capabilities.

    - Handover decision is made without new L1 reporting

For the study in this release, we also propose to focus on simple network or cell utilization only, i.e., the same cell for DL and UL, and no multi-TRP operation. If new UE capability of UAV beamforming provides benefits only in multi-TRP scenario or with DL/UL non-symmetric cell only, we assume that new UE capability not valid.

Proposal 3: Consider single TRP and DL/UL symmetric cell only at studying the benefits of UE UAV beamforming capability

By focusing on single TRP & DL/UL symmetric cell, we assume mobility management is done based on DL measurement only. So the DL/UL beam reciprocity should be essential in target scenario. In addition, since ICI management is the critical goal of UAV beamforming, utilization of different DL/UL beam or network transparent switching of UE beam should not be supported.

Observation 2: At least in DL/UL symmetric cell, beam correspondence is required for interference aware UAV mobility management

Observation 3: Since ICI depends on UAV beam, network transparent changing of UE beam cause unexpected ICI

Proposal 4: Support UAV beamforming based mobility management for the UAV UE supporting FR1 beam correspondence only

Proposal 5: Network assumes the same UE beam applied on reported UE’s measurement and channel transmission /reception

Required UE capabilities

As a starting point, RAN1 agreed to consider UAV UE’s beam characteristics or extension of FR2-only beam management parameters. Indication of beam characteristics provides strict information on the beam shapes and directional gains UE provides. Such capability itself would not provide sufficient information to serving cell extracting interference information. But when merged together with multi-cell link quality information and geometric information, further information becomes available. Figure 2 is an example when LoS is satisfied between UAV UE and multiple of cells including serving cell. In figure 2, UE measures multi-cell link quality, e.g., L3-RSRP with UE beam adjustment. Depending on relative geometric location, same or different directional antenna is used for link quality measurement. If handover happens to a cell measured by different directional antenna, e.g. cell C, reduced amount of interference serving cell will expect, compared with the case of handover within the same directional antenna.

Observation 4: With the following conditions, reporting of UE beam characteristics may help serving cell to predict proper target cell for UAV UE off-loading

- LoS environment is provided for serving cell and target cell

- Geometric information of UAV UE and target cell is provide at used for target cell selection

Proposal 6: Low priority on UE beam characteristics reporting, if dependence on positioning is essential

As FR2 only parameters of UE beam management capabilities, following 10 parameters are defined. Extension of those parameters to FR1 is supported if it helps network to extract more information under the proposed target scenario.

| Definitions for parameters | Per | M | FDD-TDD | FR1-FR2 |
| --- | --- | --- | --- | --- |
|  |  |  | DIFF | DIFF |
| beamCorrespondenceCSI-RS-based-r16 | Band | No | TDD only | FR2 only |
| beamCorrespondenceSSB-based-r16 | Band | No | TDD only | FR2 only |
| beamCorrespondenceWithoutUL-BeamSweeping | Band | Yes | N/A | FR2 only |
| beamSwitchTiming, beamSwitchTiming-v1710 | Band | No | N/A | FR2 only |
| beamSwitchTiming-r16, beamSwitchTiming-r17 | Band | No | N/A | FR2 only |
| maxNumberRxTxBeamSwitchDL, maxNumberRxTxBeamSwitchDL-v1710 | Band | No | N/A | FR2 only |
| sfn-DefaultUL-BeamSetup-r17 | Band | No | N/A | FR2 only |
| unifiedJointTCI-BeamAlignDLRS-r17 | Band | No | N/A | FR2 only |
| uplinkBeamManagement | Band | No | N/A | FR2 only |
| beamManagementType-r16, beamManagementType-CBM-r17 | BC | Yes | TDD only | FR2 only |

As a way to inform UE’s capability of supporting beam correspondence, extension of FR2 beam correspondence parameters can be considered.

Proposal 7: Extend or modify FR2 beam correspondence parameters to FR1 as an indication of UE’s capability supporting UAV beamforming based mobility management

At measurement of DL beam based on aperiodic RS, UE may have two options, whether rely on the same UE beam used for PDCCH/PDSCH reception, or to rely on different UE beam to acquire further channel information. To support CSI measurement via different or multiple of UE beam, gNB may need to know how much latency UE needs for beam switching.

Proposal 8: Consider extension or modification of following parameters to indicated required offset between triggering and reception of aperiodic CSI-RS

- beamSwitchTiming

- beamSwitchTiming-v1710

- beamSwitchTiming-r16

- beamSwitchTiming-r17

According to RAN decision, in this study, we do not consider any enhancement on FR1 beam management. Therefore, UE beam management is not possible - though DCI can deliver SRI or TCI, QCL-type D is not configured. So we assume beam management related parameters are meaningless in FR1.

Observation 5: Following parameters indicating UE’s beam management capabilities are meaningless in FR1

- uplinkBeamManagement

- unifiedJointTCI-BeamAlignDLRS-r17

- beamManagementType-r16

- beamManagementType-CBM-r17

Since any enhancement on FR1 beam management will not be treated in this study. Following parameters indicating UE’s beam management capabilities or indicating framework of beam management are not required in target scenario.

Observation 6: Following parameters indicating UE’s beam management capabilities are not required to extend to FR1

- maxNumberRxTxBeamSwitchDL

- maxNumberRxTxBeamSwitchDL-v1710

With the same reason, default beam indication is not valid in FR 1

Observation 7: Following parameters indicating usage of default beam is not valid in FR1

- sfn-DefaultUL-BeamSetup-r17

Based on the observation above, we propose to exclude unnecessary parameters not to extend to FR1

Proposal 9: Following FR2 UE beamforming capabilities do not extend to FR1 for the purpose of supporting ICI aware UAV UE handover.

- uplinkBeamManagement

- unifiedJointTCI-BeamAlignDLRS-r17

- beamManagementType-r16

- beamManagementType-CBM-r17

- maxNumberRxTxBeamSwitchDL

- maxNumberRxTxBeamSwitchDL-v1710

- sfn-DefaultUL-BeamSetup-r17

Conclusions

In this document, we propose a single scenario for the study, and the details of scenario is proposed. Based on the proposed study, we discuss the benefits of each capability parameters agreed to consider. We propose several parameters for further discussion, and other parameters not to be considered in further study. Here is the list of observations and proposals.

Proposal 1: RAN1 focuses on a single scenario for UAV performance enhancement considering terrestrial UE protection via aerial link interference management or avoidance

Proposal 2: RAN1 consider following case as a target scenario for Rel-18 UAV beamforming capability discussion

- Network commands handover for UAV UE to non-best cell for the terrestrial UE protection or cell traffic off-loading with following conditions

  - After handover, with UAV UE’s beam adjustment toward new serving cell, legacy serving cell observes sufficiently reduced RSRP via aerial link

    - TBD: How much reduction should be assumed/required

  - Required information to make handover decision above is extracted from legacy reporting and UE capabilities.

    - Handover decision is made without new L1 reporting

Proposal 3: Consider single TRP and DL/UL symmetric cell only at studying the benefits of UE UAV beamforming capability

Observation 2: At least in DL/UL symmetric cell, beam correspondence is required for interference aware UAV mobility management

Observation 3: Since ICI depends on UAV beam, network transparent changing of UE beam cause unexpected ICI

Proposal 4: Support UAV beamforming based mobility management for the UAV UE supporting FR1 beam correspondence only

Proposal 5: Network assumes the same UE beam applied on reported UE’s measurement and channel transmission /reception

Observation 4: With the following conditions, reporting of UE beam characteristics may help serving cell to predict proper target cell for UAV UE off-loading

- LoS environment is provided for serving cell and target cell

- Geometric information of UAV UE and target cell is provide at used for target cell selection

Proposal 6: Low priority on UE beam characteristics reporting, if dependence on positioning is essential

Proposal 7: Extend or modify FR2 beam correspondence parameters to FR1 as an indication of UE’s capability supporting UAV beamforming based mobility management

Proposal 8: Consider extension or modification of following parameters to indicated required offset between triggering and reception of aperiodic CSI-RS

- beamSwitchTiming

- beamSwitchTiming-v1710

- beamSwitchTiming-r16

- beamSwitchTiming-r17

Observation 5: Following parameters indicating UE’s beam management capabilities are meaningless in FR1

- uplinkBeamManagement

- unifiedJointTCI-BeamAlignDLRS-r17

- beamManagementType-r16

- beamManagementType-CBM-r17

Observation 6: Following parameters indicating UE’s beam management capabilities are not required to extend to FR1

- maxNumberRxTxBeamSwitchDL

- maxNumberRxTxBeamSwitchDL-v1710

Observation 7: Following parameters indicating usage of default beam is not valid in FR1

- sfn-DefaultUL-BeamSetup-r17

Proposal 9: Following FR2 UE beamforming capabilities do not extend to FR1 for the purpose of supporting ICI aware UAV UE handover.

- uplinkBeamManagement

- unifiedJointTCI-BeamAlignDLRS-r17

- beamManagementType-r16

- beamManagementType-CBM-r17

- maxNumberRxTxBeamSwitchDL

- maxNumberRxTxBeamSwitchDL-v1710

- sfn-DefaultUL-BeamSetup-r17

- References

[1] TR 36.777 “Study on Enhanced LTE Support for Aerial Vehicles” December 2017

[2] RP1-222171 “Revised WID: NR support for UAV (Uncrewed Aerial Vehicles)”, September, 2022

[3] RP-213547 “Email discussion summary for 94e-19-R18-UAV”, December, 2021

[4] Chairman Notes, 3GPP RAN1#110bis-e, e-Meeting, October, 2022.

[5] R1-2209762 “Discussion on the benefits of UE beamforming capability reporting”, RAN1 #110b-e, October, 2022