3GPP TSG RAN WG1 #112                                                                      R1-2301334

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.1.4.1

Source:	Apple Inc.

Title:	Views on UL precoding indication for multi-panel simultaneous transmissions

Document for:	Discussion/Decision

# Introduction

In Rel-18, RAN1 has made couple of agreements toward specifying simultaneous uplink transmission over multi-panel (STxMP). Most importantly, in RAN1#110 it was agreed under a working assumption that in Rel-18, SDM scheme is supported for single-DCI based STxMP PUSCH transmission in mTRP based system. In RAN1#110b-e and RAN1#111, more details on SDM based STxMP was agreed like SRS resource set indication, layer indication and antenna port indication. In this contribution, we provide our views on remaining issues, for both sDCI based and mDCI based, for multi-panel simultaneous PUSCH transmissions.

# Single-DCI based STxMP

## Maximum number of layers for SDM scheme

On maximum number of layers for SDM, the following agreement is made in RAN1#111:

Agreement

For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission:

- Use the 2-bit “SRS resource set indicator” DCI field to dynamically indicate the sTRP or SDM transmission.

- FFS: how to interpret each codepoint of “SRS resource set indicator”.

- For the maximal number of layers for sTRP transmission, down-select:

  - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec

  - Option2:  Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec.

- Down-select one from the following for maximal number of layers of SDM transmission:

  - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set

  - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission.

  - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately.

- FFS:  To ensure the same size of DCI for sTRP and SDM cases, how to use/interpret the TPMI/SRI field(s) and whether to do reserved bit or zero padding.

When UE is configured with two SRS resource sets with SDM operation, UE by default assumes DCI will include bitfields for ‘SRS resource set indicator’, two independent SRI bit fields, and two independent TMPI bitfields for CB based PUSCH, all for default assumption on mTRP based SDM operation. Having said that, we think total size of SRI bit fields shall not be less than SRI bit field required for sTRP based operation. Similarly, for CB based PUSCH, total size of TMPI bit fields is not less than the required TPMI bits for sTRP transmission. This design assures not only minimal DCI overhead, but also allows UE to dynamically switch to sTRP transmission. Now, while in theory maximum number of layers associated to each SRS resource set can be different for mTRP based transmission, we don’t see a big motivation to have different maximum number of layers for sTRP based transmission (similar to R17). Based on the above discussion, we have the following proposal:

Proposal 1: For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission:

- For the maximal number of layers for sTRP transmission, support Option 1:

  - The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec

- For maximal number of layers of SDM transmission, support Alt2:

  - Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set, i.e. (L1,Mmax, L2,Mmax)

## Maximum number of layers for SFN scheme

For SFN scheme, in our view maximum number of layers per panels shall be up to 2, given that the same layer is transmitted/repeated by both panels. Thus, the only allowed maximum rank configuration for SFN, i.e. LMmax are {1,1} and {2,2}. For the association between maximum number of layers for sTRP and mTRP based with SFN, we have a similar discussion as SDM, unless the triple (LSmax, L1,Mmax, L2,Mmax) is now replace with the pair (LSmax, LMmax). We have the following proposals:

Proposal 2: For dynamic switching between SFN STxMP PUSCH and sTRP transmission, UE is indicated with a pair (LSmax, LMmax) where

- LSmax represents maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec

- LMmax: represents maximum number of layers when UE is indicated to perform SFN based mTRP

Proposal 3: For a UE with SFN STxMP indication, the only allowed combinations of LMmax are 1 or 2.

## Configuration of SRS resources for SDM scheme

On different number of SRS resources for the two configured SRS resource sets, the issue can be discussed separately for CB based and NCB based PUSCH. For CB based PUSCH, we don’t see a strong justification to have different SRS resources for the two configured sets, given that UE’s configuration for ul-FullPowerTransmission is applicable to both PUSCHs.

Proposal 4: If UE is configured with two SRS-ResourceSet with usage set to ‘codebook' and is indicated for STxMP with SDM scheme, the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets.

In theory, for NCB based PUSCH, a Rel-18 UE can be configured with two SRS resource sets and different number of SRS resources per set. We note that, support of different number of SRS resources per set together with maximal number of layers corresponding to each set, and whether such association is fixed, or it depends on sTRP vs mTRP operation, these all impact on DCI design for SRI bit fields. To keep total size of SRI bit fields is always not less than SRI bit field required for sTRP based operation, some combinations of maximum number of layers per SRS resource set for mTRP based SDM transmission, i.e. (L1,Mmax, L2,Mmax), and the maximum number of layers for sTRP based PUSCH LSmax, shall be excluded based on number of SRS resources within the 1st/ 2nd SRS resource sets, i.e. (N1,SRS , N2,SRS). More precisely:

Proposal 5: If UE is configured with two SRS-ResourceSet with usage set to ‘nonCodebook' each with more than one SRS resources, and UE is configured with SDM scheme, different number of SRS resources per set, (N1,SRS &  N2,SRS), is supported for all (LSmax, L1,Mmax, L2,Mmax) combinations except for

- LSmax, >1 && (L1,Mmax, L2,Mmax) = (1,1) && ( (N1,SRS=2,  N2,SRS=4) or (N1,SRS=4 &  N2,SRS=2))

On different number of SRS ports for the two indicated SRS resources by the two SRI fields for CB based PUSCH, similar to the number of SRS resources across the two SRS resource sets for NCB based PUSCH, the issue is directly associated to the number of layers per PUSCH. In theory, for CB based PUSCH, a Rel-18 UE can be configured with two SRS resource sets and the indicated resources by SRI may have different number of antenna ports, nrofSRS-Ports. To assure that total size of TMPI bit fields is not less than the required TPMI bits for sTRP transmission, some combinations of maximum number of layers per SRS resource set for mTRP based SDM transmission, i.e. (L1,Mmax, L2,Mmax), and the maximum number of layers for sTRP based PUSCH LSmax, shall be excluded based on some number of indicated nrofSRS-Ports across the two SRS resources. For instance, if UE is configured with codebookSubset = nonCoherent, transform precoder is disabled, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and (LSmax, L1,Mmax, L2,Mmax) = (2/3/4,1,1), and (N1,ant, N2,ant) = (4,2) or (2,4), then in this case the total number of bits for TPMI fields associated to the two SRS resource sets for mTRP based STxMP is 3 bits, while if UE is dynamically indicated to switch to sTRP based operation, size of TPMI bit field is 4 bits.

Proposal 6: If UE is configured with two SRS-ResourceSet with usage set to ‘Codebook', and UE is configured with SDM scheme, different number of SRS antenna ports across the sets is supported subject to UE capability and excluding some configurations of (LSmax, L1,Mmax, L2,Mmax) and (N1,ant, N2,ant).

## Configuration of SRS resources for SFN scheme

For SFN scheme and different number of SRS resources across the two sets, or different number of antenna ports across the two sets for CB based PUSCH, we have a similar line of discussion as SDM. That is UE assumes the total size of SRI bit fields, and TMPI bit fields for CB based PUSCH, shall not be less than the required bits for SRI (and/or TMPI for CB based) when UE is dynamically indicated to switch from SFN to sTRP based transmission. In short, we don’t see justification for different number of SRS resources across the two SRS resource sets for CB based PUSCH, while it can be supported for NCB based PUSCH. We have the following proposals:

Proposal 7: If UE is configured with two SRS-ResourceSet with usage set to ‘nonCodebook' each with more than one SRS resources, and UE is configured with SFN scheme, different number of SRS resources per set, (N1,SRS &  N2,SRS), is supported for all (LSmax, LMmax) combinations except for

- LSmax, >1 && LMmax = 1 && ( (N1,SRS=2,  N2,SRS=4) or (N1,SRS=4 &  N2,SRS=2))

Proposal 8: If UE is configured with two SRS-ResourceSet with usage set to ‘Codebook', and UE is configured with SFN scheme, different number of SRS antenna ports across the sets is supported subject to UE capability and excluding the following combination configurations of (LSmax, L1,Mmax, L2,Mmax) and (N1,ant, N2,ant).

- codebookSubset = nonCoherent && (N1,ant, N2,ant) = (4,2) and sTRP is indicated to use the 1st SRS resource set OR

- codebookSubset = nonCoherent && (N1,ant, N2,ant) = (2,4) and sTRP is indicated to use the 2nd SRS resource set

## PTRS to DMRS mapping

In current specification, DCI has 0 bits for PTRS to DMRS association if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; otherwise, DCI uses 2 bits for this indication, using three tables defined in 38.212, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively.

Proposal 9: for PTRS-DMRS association when UE is configured with SDM based STxMP:

- 0 bits is needed if maximum number of layers per panel is indicated to be 1

- 1 bit is needed if the maximum number of layers associated to each panel is indicated to be {1+2}, or {2+1}

- 2 bits is needed if the maximum number of layers per panel is indicated to be 2

In current spec, if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from (see 38.214, 6.2.3.1)

- SRI bit-field and indicated SRS resources for non-codebook based UL transmission:

  - A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRSUplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port.

- indicated TPMI(s) for codebook based UL transmission:

  - PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1.

In order to assure that for non-codebook based SDM STxMP, all SRS resources within a SRS resource set are associated to a single PTRS port which is different than that of the other SRS resource set, we have the following proposal

Proposal 10: For a UE with SDM STxMP indication, maxNrofPorts per SRS resource set is assumed to be  up to 1 (not 2), and the port-index if configured is different between the two SRS resource sets.

## DMRS port indication for layer combination {1+2}

For DMRS port indication in SDM based scheme of STxMP PUSCH transmission, one open aspect is on DMRS port association when layer combination is {1+2}. In was agreed in RAN1#111 that

Agreement

For SDM scheme single-DCI based STxMP transmission, when L1 and L2 layers are indicated/determined by two TPMI fields of CB PUSCH or two SRI fields of NCB PUSCH respectively:

- The first L1 indicated DMRS ports correspond to the L1 layers indicated by the first TPMI or SRI field

- The remaining L2 indicated DMRS ports correspond to the L2 layers indicated by the second TMPI or SRI field

- Support at least one of the following options for indication of layer combination {1+2}:

  - Option 1: new entry is added to DMRS table, e.g., {0, 2, 3}, {2, 0, 1}.

  - Option 2: use DCI field (e.g., SRS resource set indicator) to indicate that for layer combination {1+2}, the first two indicated DMRS ports correspond to the 2 layers indicated by the second TPMI or SRI field and the rest one indicated DMRS port correspond to the layer indicated by the first TPMI or SRI field.

  - Option 3: For layer combination of {1+2}, the DMRS port in the CDM group with only one port is mapped to the SRI/TPMI field indicating one layer, and the DMRS ports in the CDM group with 2 ports are mapped to the SRI/TPMI field indicating 2 layers

  - Other options are not precluded

In our view both Option 2 and 3 work, and there is no need to introduce new entries to DMRS table. We have a slight preference in option 2, as option 3 restricts DMRS ports of a PUSCH of ranks 2 to be on the same CDM group.

Proposal 11: Support Option 2 for DMRS port indication for layer combination {1+2}.

## PUSCH Mapping to REs

If UE is indicated to transmit a single TB through simultaneous PUSCH transmissions over multi-panel, current specification can be adopted to determine the TB size. For mapping of the coded symbols to the REs, different options can be discussed, For example

- Option 1: start with the first allocated symbol, the coded symbols are mapped in frequency first to all resource elements (REs) associated to the 1st PUSCH and next to REs associated to the 2nd PUSCH. Next move to the next symbol.

- Option 2: start with 1st PUSCH, coded bits are mapped in all frequency and time REs associated to 1st PUSCH (frequency next time). Next, move to 2nd PUSCH.

Proposal 12: RAN1 should study different PUSCH mapping schemes in terms of UE complexity and/or PUSCH reliability.

## UCI multiplexing procedures

If a m-TRP based PUSCH indicated for STxMP with SDM scheme overlaps in time with a PUCCH that is associated to single TRP (s-TRP), UCI multiplying procedure may follow the existing specification per PUCCH+PUSCH resources that are associated to the same TRP. In other words, UE may resolve per panel PUCCH+PUSCH collision (multiplex UCI, drop PUCCH, etc) based existing procedures.

Proposal 13: If a mTRP based PUSCH+PUSCH overlaps in time with a PUCCH that is associated to a sTRP, UCI is multiplexed only on the PUSCH resource that is associated to the same TRP/beam/panel as of PUCCH

- FFS: How the code-rate for UCI multiplexing is determined, i.e., based on REs allocated to both PUSCHs, or based on only those REs associated to the PUSCH toward the same TRP as PUCCH

# multi-DCI based STxMP

## UCI multiplexing procedures

In current specification, TDAI bit-field in uplink DCI indicates number of HARQ-ACK bits that shall be multiplexed on PUSCH. More precisely, when there is overlapping PUSCHs and PUCCH, up to one PUSCH will be selected for HARQ-ACK multiplexing, where no overlapping PUSCHs within the same carrier with the same priority index, is expected. Now in Rel-18, m-DCI based simultaneous m-TRP based PUSCH+PUSCH, when UE is scheduled with fully or partially overlapping PUSCHs in the time and frequency domain by multiple PDCCHs with two different values of coresetPoolIndex, TDAI indication should be per panel, as shown in the following example. In this case, UE will multiplex HARQ-ACKs to each PUSCH independently, unlike current spec which only one PUSCH with UL-TDAI not equal to 4 carries HARQ-ACK

Figure 1: TDAI indication for dynamic HARQ-ACK codebook and m-DCI based STxMP

Proposal 14: If UE is scheduled with fully or partially overlapping PUSCHs in the time and frequency domain by multiple PDCCHs that are associated to different ControlResourceSets having different values of coresetPoolIndex, TDAI indication is per panel.

# Conclusion

In this contribution, we provided some views on multi-panel simultaneous uplink transmissions. Based on what we discussed, the following proposals are made:

Proposal 1: For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission:

- For the maximal number of layers for sTRP transmission, support Option 1:

  - The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec

- For maximal number of layers of SDM transmission, support Alt2:

  - Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set, i.e. (L1,Mmax, L2,Mmax)

Proposal 2: For dynamic switching between SFN STxMP PUSCH and sTRP transmission, UE is indicated with a pair (LSmax, LMmax) where

- LSmax represents maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec

- LMmax: represents maximum number of layers when UE is indicated to perform SFN based mTRP

Proposal 3: For a UE with SFN STxMP indication, the only allowed combinations of LMmax are 1 or 2.

Proposal 4: If UE is configured with two SRS-ResourceSet with usage set to ‘codebook' and is indicated for STxMP with SDM scheme, the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets.

Proposal 5: If UE is configured with two SRS-ResourceSet with usage set to ‘nonCodebook' each with more than one SRS resources, and UE is configured with SDM scheme, different number of SRS resources per set, (N1,SRS &  N2,SRS), is supported for all (LSmax, L1,Mmax, L2,Mmax) combinations except for

- LSmax, >1 && (L1,Mmax, L2,Mmax) = (1,1) && ( (N1,SRS=2,  N2,SRS=4) or (N1,SRS=4 &  N2,SRS=2))

Proposal 6: If UE is configured with two SRS-ResourceSet with usage set to ‘Codebook', and UE is configured with SDM scheme, different number of SRS antenna ports across the sets is supported subject to UE capability and excluding some configurations of (LSmax, L1,Mmax, L2,Mmax) and (N1,ant, N2,ant).

Proposal 7: If UE is configured with two SRS-ResourceSet with usage set to ‘nonCodebook' each with more than one SRS resources, and UE is configured with SFN scheme, different number of SRS resources per set, (N1,SRS &  N2,SRS), is supported for all (LSmax, LMmax) combinations except for

- LSmax, >1 && LMmax = 1 && ( (N1,SRS=2,  N2,SRS=4) or (N1,SRS=4 &  N2,SRS=2))

Proposal 8: If UE is configured with two SRS-ResourceSet with usage set to ‘Codebook', and UE is configured with SFN scheme, different number of SRS antenna ports across the sets is supported subject to UE capability and excluding the following combination configurations of (LSmax, L1,Mmax, L2,Mmax) and (N1,ant, N2,ant).

- codebookSubset = nonCoherent && (N1,ant, N2,ant) = (4,2) and sTRP is indicated to use the 1st SRS resource set OR

- codebookSubset = nonCoherent && (N1,ant, N2,ant) = (2,4) and sTRP is indicated to use the 2nd SRS resource set

Proposal 9: for PTRS-DMRS association when UE is configured with SDM based STxMP:

- 0 bits is needed if maximum number of layers per panel is indicated to be 1

- 1 bit is needed if the maximum number of layers associated to each panel is indicated to be {1+2}, or {2+1}

- 2 bits is needed if the maximum number of layers per panel is indicated to be 2

Proposal 10: For a UE with SDM STxMP indication, maxNrofPorts per SRS resource set is assumed to be  up to 1 (not 2), and the port-index if configured is different between the two SRS resource sets.

Proposal 11: Support Option 2 for DMRS port indication for layer combination {1+2}.

Proposal 12: RAN1 should study different PUSCH mapping schemes in terms of UE complexity and/or PUSCH reliability.

Proposal 13: If a mTRP based PUSCH+PUSCH overlaps in time with a PUCCH that is associated to a sTRP, UCI is multiplexed only on the PUSCH resource that is associated to the same TRP/beam/panel as of PUCCH

- FFS: How the code-rate for UCI multiplexing is determined, i.e., based on REs allocated to both PUSCHs, or based on only those REs associated to the PUSCH toward the same TRP as PUCCH

Proposal 14: If UE is scheduled with fully or partially overlapping PUSCHs in the time and frequency domain by multiple PDCCHs that are associated to different ControlResourceSets having different values of coresetPoolIndex, TDAI indication is per panel.

# References

- RP-213517, “New WID: MIMO Evolution for Downlink and Uplink”, Samsung (Moderator), Dec. 2021