3GPP TSG RAN WG1 #108-e			           R1-2301618

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item: 5

Source: MediaTek Inc.

Title: Discussion on Reply LS on PDCCH Skipping

Document for: Discussion and Decision

# Introduction

RAN2 replied RAN1 LS on PDCCH skipping in R1-2300020 (R2-2213349) [1], as quoted below:

RAN2 thanks RAN1 for the LS and would like to provide following answer and agreements. Question: What is the meaning of “ignores” in the sentence “UE ignores PDCCH skipping while contention resolution timer is running.” quoted from the previous RAN2 LS to RAN1 on PDCCH skipping in RRC_CONNECTED (R2-2201960)? RAN2 answer: The sentence “UE ignores PDCCH skipping while contention resolution timer is running.” indicates that the UE shall monitor PDCCH regardless of skipping command on SpCell while contention resolution timer is running. RAN2 has agreed the attached CR to remove the cases for ignoring PDCCH skipping in stage 2 specification. RAN2 has further discussed the PDCCH skipping aspects and have agreed that the following cases are missing in the RAN1 specification: - UE shall monitor PDCCH regardless of previously received skipping command on SpCell after successful contention resolution. - UE shall monitor PDCCH regardless of skipping command on all serving cells of the corresponding Cell Group if UE detect the PDCCH skipping command after SR is sent and is pending. - UE shall monitor PDCCH regardless of previously received skipping command on all serving cells of the corresponding Cell Group after the pending SR is cancelled.

# Discussion

With RAN2 clarification and identified cases missing in RAN1 specification, RAN1 should make the corresponding corrections for aligned understanding with RAN2. Detailed CR can be discussed in Agenda Item 8.7, Maintenance on UE Power Saving Enhancements.

Proposal 1: RAN1 to discuss and decide necessary CR in Agenda Item 8.7 (Maintenance on UE Power Saving Enhancements) for corrections regarding the three missing cases identified by RAN2.

Proposal 2: RAN1 further replies RAN2 LS based on the CR outcome in Agenda Item 8.7

For converging the CR, companies can refer to [2] for further discussion and decision.

# Reference

R1-2300020 (R2-2213349), “Reply LS on PDCCH skipping”, RAN1 #112, RAN2

R1-2301619 (Draft CR), “Corrections on PDCCH Skipping”, RAN1 #112, MediaTek Inc.