3GPP TSG RAN WG1 #112	R1-2301995

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:		9.14.2

Source:	IITH

Title:	DMRS design for power domain enhancements

Document for:		Discussion and Decision

# Motivation

Several agreements have been made to enhance coverage of 5G NR via power domain enhancements. Below, we show the ones relevant to DMRS design for enhancing coverage via power domain enhancements.

Agreement

The following non-transparent solutions for MPR/PAR reduction are currently under discussion in RAN1.

- Frequency domain spectrum shaping w/ spectrum extension.

- Tone reservation w/ spectrum extension

In addition, transparent schemes, for instance but not limited to frequency domain spectrum shaping w/o spectrum extension or schemes based on clipping and filtering, are also being evaluated to serve as a benchmark to assess the benefits of non-transparent solutions. Companies are allowed to use any transparent transmission scheme of their choice.

Agreement

At least the symmetric spectrum extension option for frequency domain spectrum shaping with spectrum extension (FDSS-SE), are considered for studying MPR/PAR reduction enhancements in Rel-18.

Conclusion

It is RAN1 understanding that:

- Performance comparison based on net gain results combining transmitter and receiver performance is performed by RAN4.

- No final decision would be taken by RAN1 on which MPR/PAR reduction solution, will be specified in Rel-18, if any, since this is RAN4’s responsibility.

    - It does not preclude RAN1 specification impact

Agreement

For RAN1 link-level performance evaluation of MPR/PAR reduction solutions involving the use of spectrum shaping filter, companies are encouraged to use at least the following spectrum shaping filter configuration for calibration purpose:

- 2-tap, e.g., (1 0.28), 3-tap, e.g., (0.335 1 0.335), and (0.28 1 0.28)

- Truncated RRC (0.5, 0.1667)

There is no restriction to use other spectrum shaping filter coefficients in simulations.

Note: the above does not have spec impact.

Agreement

Study the PAPR/CM[/OBO] of DMRS with FDSS-SE, e.g., the following solutions:

  - Option 1 - Based on low PAPR Type 1 DMRS sequence:

    - 1-a:  A DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension.

    - 1-b A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. The sequence is then cyclically extended to span the PRBs in the extension.

    - 1-c A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. DMRS extension is applied similar to data to span the PRBs in the extension.

- Option 2 - Based on low PAPR type 2 DMRS sequence

    - Variances like those of Option 1 can be referred

  - Option 3 – For in-band DMRS lengths 6/12/18/24 symbols, DMRS sequence is obtained by DFT transformation of low PAPR sequence type 1. Then the sequence is extended to span the PRBs in the extension in the same way as data extension.

Note: Other solutions can be studied. Comparison with the three solutions above is encouraged. Sequence with different density between in-band and extension can be studied

In this contribution, we discuss different procedures for DMRS design when spectrum extension methods are considered for coverage extension via power domain enhancement.

# DMRS design and BLER performance

In this section, we discuss procedures to generate the DMRS sequences followed by the BLER performance with the proposed DMRS sequences with transparent and non-transparent with spectrum extension followed by the frequency domain spectrum extension.

## DMRS design for spectrum extension and shaping

In Rel-17 specification, for DFT-s-OFDM waveform, for an allocation of size M, the size of the QPSK or any other modulation data is generated for size M. The spectrum extension is performed on this data after DFT precoding to extend the symbol size to M+d, where d is the extension factor. For a DFT precoded data X(k), the spectrum extension may be performed by

Xexsk=Xk mod M;k={0, 1, 2, ……M+d-1} -> Cyclic Extension

Or

Xexsk=Xk-d2 mod M;k={0, 1, 2, ……M+d-1} -> Symmetric Extension

The PAPR for QPSK with the above-mentioned methods is presented in Fig. 1 with 25% spectrum extension and using (1+D) 2-tap filter.

Figure 1: PAPR of the QPSK data sequence with spectrum extension and shaping

Hence, to equalize the transmitted data, M+d length RS needs to be transmitted. In Rel-17 specification, either ZC or CGS are being used as RS. CGS is for lengths less than 30, and can be spectrum extended to M+d. However, for allocation size of M, ZC sequences (x(n)) are generated for a length of a prime number (Nzc) which is less than and close to M. To get a sequence of length M, x(n) is cyclically extended to M as follows,

rn=xn mod Nzc

In order to support frequency domain spectrum shaping with spectrum extension (FDSS-SE), for an extension factor of ‘d’, RS of length M+d will be needed for transmission when the allocation is of size “M”. Various method to obtain M+d length ZC-RS are discussed, and conclusions are presented in [1].

The effect of extension and shaping should be the same on both RS and data. However, since ZC is generated for length NZC<M in Rel-17, the effect of extension and shaping on data symbol is not same as the effect on the RS symbol.

Observation-1: New RS sequence design needs to be studied for QPSK or higher modulations when operated with spectrum extension and shaping.

In Rel-17, low PAPR type 2 DMRS sequence is used for demodulation of pi/2-BPSK data. The low PAPR type 2 DMRS sequences have proved to have low PAPR, and has a flexibility to generate the sequence to any desired length. For an allocation size M, the sequence can be generated to length M. Similar to data, the DFT precoded RS sequence can be spectrum extended to M+d length same as the method employed for data spectrum extension.

Note: The spectrum shaping filter is generated to at least the length of the allocation and extension factor i.e., M+d. If the length of the filter generated is more than M+d, the filter is then truncated to the length M+d. This is referred to as truncation in the rest of the contribution.

The generation of DMRS using low PAPR type 2 DMRS sequences involves the following steps:

- Generate a pi/2 BPSK sequence of length same as the allocation size M.

- Obatin DFT precoded sequence by operating the generated sequnece with M-point DFT.

- Extend the precoded symbol either cylically or symmetrically to length M+d

- Shape the extended sequence with chosen shaping filter before generating the OFDM symbol.

The PAPR performance of the low PAPR type2 DMRS sequences for different configurations is shown in Fig. 2.

Figure 2: PAPR of the RS sequences with spectrum extension and shaping

From the above figures, it is clear that using low PAPR type-2 DMRS sequences is signifcantly better than usiing the ZC sequences.

Proposal-1: Low PAPR type 2 DMRS sequences can be handled similar to data sequences for spectrum extension and shaping and therefore be used as RS sequences with appropriately chosen shaping filter.

Proposal-2: The length of the DMRS sequence generated is equal to allocation size.

## BLER performance

The BLER performance with the proposed Frequency domain spectrum extension and shaping is presented in figure 3 comparing the base line (3GPP Rel-17) with the proposed extension and shaping. It can be observed from the figure that the proposed scheme has a loss of around 1dB compared to the Rel-17. However, the loss can be compensated from the PAPR gain obtained with spectrum extension and shaping.

Figure 3: BLER performance comparing with and without spectrum extension and shaping

# 3	Conclusion

In this document, we have discussed methods to generate spectrum extended filtered symbol for data sequences to obtain low PAPR. To demodulate the data, we presented the RS sequence that can offer low PAPR with spectrum extension and shaping. Based on the discussion and results, we make the following observations and proposals:

Proposal 1:  Low PAPR type 2 DMRS sequences may be processed similar to data sequences for spectum extension and shaping and used as RS sequences with approprately chosen filter.

Proposal-2: The length of the DMRS sequence generated is equal to allocation size.

# References

- RAN1 Chair’s Notes , 3GPP TSG RAN WG1 #111, Toulouse, France, November 14th – 18th, 2022

# Annexure

Simulation assumptions

| Parameter | Value |
| --- | --- |
| Number of PRBS | 6 before extension 8 after extension and without extension |
| MCS | 2 - baseline 3 – FDD-SE |
| No. of Tx | 1 |
| No. of Rx | 2 |
| Shaping filter | 2-tap - [1 1] |
| Extension factor | 25% |
| FFT size | 4096 |
| SCS | 30 KHz |