3GPP TSG-RAN WG1 Meeting #112bis-e	R1- 2303438

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.2.3.2

Source:	NVIDIA

Title:	AI and ML for beam management

Document for:	Discussion

# 1	Introduction

The approval of the Rel-18 work package marks the start of 5G Advanced evolution. The package includes a study item on AI/ML for NR air interface, and the work item description can be found in [1].

The initial use cases focused in this study include:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

For the use cases under consideration, the study aims to finalize representative sub use cases and assess potential specification impact:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

In the previous RAN1 meetings, initial agreements were made to provide directions for further investigation. In this contribution, we discuss relevant aspects of AI/ML for beam management.

# 2	Representative sub-use cases

5G NR can operate at a wide range of frequencies, ranging from sub-6 GHz to millimeter wave frequencies. To support operation over such a wide range of carrier frequencies, NR has been designed to utilize beam-based operation, where gNB and UE may use transmit and receive beamforming for all channels and signals.

Initial transmit and receive beam finding is used in initial access and connected UEs on, for example, a non-standalone NR carrier in MR-DC. The gNB transmits multiple SSBs that are beamformed in different directions. Up to a maximum of 64 such SSBs can be transmitted in a burst of 5 ms within a period of 20 ms. The UE detects an SSB and stores the information related to the corresponding receive beam. After detecting the SSB and decoding the MIB, the UE knows which SSB it has detected.

In initial access, the UE may use the same receive beam to receive SIB1 as it used to receive the detected SSB. SIB1 conveys the association information which links the index of a SSB to a corresponding set of PRACH preambles in time and frequency. In a non-standalone NR carrier in MR-DC, the UE does not need to read SIB1 and instead may get the information from dedicated RRC signaling. The UE would perform PRACH transmission in the PRACH resource associated with its detected SSB.

The gNB can detect the PRACH using a beam direction same as the beam direction it used to transmit the SSB. When the gNB detects a PRACH from the UE, it knows which SSB the UE detected and can determine its transmit beam for communicating with the UE. The transmit and receive beams established in initial transmit and receive beam finding are used to complete random access procedure and the subsequent procedures before dedicated RRC configuration is set up.

In general, the downlink beam management procedures consist of three steps: Procedure 1 (P-1): SSB-based beam sweeping; Procedure 2 (P-2): CSI-RS based transmit-end beam refinement; Procedure 3 (P-3): CSI-RS based receive-end beam refinement. There are also uplink beam management procedures.

AI/ML based algorithms may find applications in many steps of beam management. It is worthwhile discussing which of the steps should be the focal point for evaluation. Beam prediction in time and/or spatial domain can help reduce overhead and latency, as well as improving beam selection accuracy. They are good candidates for evaluating AI/ML based algorithms for beam management.

At RAN1#109-e, it was agreed to support BM-Case 1 and BM-Case 2 for characterization and baseline performance evaluations for AI/ML-based beam management:

- BM-Case 1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case 2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

These two use cases can be the focal point for studying AI/ML based algorithms for beam management.

Proposal 1: Beam prediction in spatial domain and beam prediction in time domain should be the focal point for studying AI/ML based algorithms for beam management.

For each of these two use cases, the AI/ML model training can be at network side or at UE side. The AI/ML training process can be online, where the model being used for inference is (typically continuously) trained in (near) real-time with the arrival of new training samples. The AI/ML training process can also be offline, where the AI/ML model is trained based on collected dataset and the trained model is later used or delivered for inference. Offline training may be more feasible for the near future. But in the long run, it is vital that the AI/ML models can learn continuously to adapt to varying environments, site-specific conditions, and heterogenous configurations.

Observation 1: Offline training may be more feasible for the near future. But in the long run, it is vital that the AI/ML models can learn continuously to adapt to varying environments, site-specific conditions, and heterogenous configurations.

Proposal 2: For the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training:

- Alt.1: offline training

- Alt.2: online training

For AI/ML model inference, it is natural to study inference at network (resp. UE) side when training is performed at network (resp. UE) side. The study of other cases where training and inference reside at two different sides (e.g., training at network side and inference at UE side, or vice versa) can be postponed.

## 2.1 	Spatial-domain DL beam prediction

In BM-Case 1, spatial-domain DL beam prediction for Set A of beams is performed based on measurement results of Set B of beams, i.e., Set B is a set of beams whose measurements are taken as inputs of the AI/ML model.

There can be multiple options for Set A of beams and Set B of beams.

In one alternative, Set B of beams is a subset of Set A of beams. In another alternative, Set A and Set B are different (e.g., Set A consists of narrow beams and Set B consists of wide beams). In either case, the underlying assumption is that the measurement effort on Set B of beams is less than the measurement effort on Set A of beams, and AI/ML based algorithms based on e.g., super resolution models are applied to predict the quality of the full set of beams in Set A.

When it comes to AI/ML model input for spatial-domain DL beam prediction, there are many different possibilities. The simplest option would be L1-RSRP measurement based on Set B of beams. In our companion contribution [2], we present evaluation results for AI/ML based spatial-domain DL beam prediction, which uses L1-RSRP measurements as AI/ML input. The results show that by using L1-RSRP measurement based on Set B of beams, the AI/ML-based algorithm can achieve performance comparable to that of exhaustive beam search in Set A of beams.

Observation 2: Evaluation results show that by using L1-RSRP measurement based on Set B of beams, the AI/ML-based algorithm can achieve performance comparable to that of exhaustive beam search in Set A of beams.

Proposal 3: For BM-Case 1, at least support L1-RSRP measurement based on Set B of beams as AI/ML model input.

Additional assistance information besides L1-RSRP measurement based on Set B of beams may be used as AI/ML input for the spatial-domain DL beam prediction. Examples brought up include Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. Comprehensive evaluation results showing convincing performance gains is needed to nail down the essential assistance information needed for the spatial-domain DL beam prediction.

Proposal 4: Comprehensive evaluation results showing convincing performance gains is needed to nail down the essential assistance information needed for the spatial-domain DL beam prediction.

## 2.2 	Temporal DL beam prediction

In BM-Case 2, temporal DL beam prediction for Set A of beams is performed based on the historic measurement results of Set B of beams, i.e., Set B is a set of beams whose measurements are taken as inputs of the AI/ML model.

There can be multiple options for Set A of beams and Set B of beams.

In one alternative, Set B of beams is a subset of Set A of beams. In another alternative, Set A and Set B are different (e.g., Set A consists of narrow beams and Set B consists of wide beams). In yet another alternative, Set A and Set B are the same. In either of these cases, the underlying assumption is that the historical measurement results on Set B of beams are used as input to AI/ML based algorithms to predict future beam quality of the set of beams in Set A.

Specifically, in BM-Case 2, the measurement results of K (K>=1) latest measurement instances are used for AI/ML model input, and the AI/ML model output includes F (F>=1) predictions for F future time instances, where each prediction is for each time instance.

When it comes to AI/ML model input for temporal DL beam prediction, there are many different possibilities. The simplest option would be the historical optimal beam index. In our companion contribution [2], we present evaluation results for AI/ML based temporal DL beam prediction, which uses historical optimal beam index as AI/ML input. The results show that using historical optimal index, the AI/ML-based algorithm can satisfactorily yield optimal beam index prediction for future time instances.

Observation 3: Evaluation results show that by using historical optimal index, the AI/ML-based algorithm can satisfactorily yield optimal beam index prediction for future time instances.

Proposal 5: For BM-Case 2 (temporal DL beam prediction), at least support using historical optimal beam index based on Set B of beams as AI/ML model input.

Another option would be L1-RSRP measurement based on Set B of beams. Additional assistance information besides L1-RSRP measurement based on Set B of beams may also be used as AI/ML input for the temporal DL beam prediction. Examples brought up include Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx  beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.), increase ratio of L1-RSRP for best N beams, UE orientation information. Comprehensive evaluation results showing convincing performance gains is needed to nail down the essential assistance information needed for the temporal DL beam prediction.

Proposal 6: Comprehensive evaluation results showing convincing performance gains is needed to nail down the essential assistance information needed for the temporal DL beam prediction.

# 3	Potential specification impact

Although it is likely that most of the AI/ML algorithms for beam prediction can be up to implementation, there are several aspects that have potential specification impact.

AI/ML model training can occur at gNB or UE side. In either case, the training entity can benefit from assistance from the other entity for training data collection. Relevant areas for discussion include training data type/size, training data source determination, and assistance signalling and procedure for training data collection.

Regarding the data collection for AI/ML model training at UE side, it is important to study the potential specification impact by considering aspects including whether and how to initiate data collection, configurations (e.g., configuration related to set A and/or Set B, information on association/mapping of Set A and Set B), assistance information from network to UE, etc.

Proposal 7: For AI/ML model training for beam prediction in spatial/time domain, study potential specification impact related to training data type/size, training data source determination, and assistance signalling and procedure for training data collection.

Similar to many other functionalities in 3GPP systems, the usage of AI/ML model for a certain functionality should be under network control, if the functionality at one side cannot be made transparent to the other side. Therefore, assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, model selection, etc. should be investigated.

Proposal 8: For AI/ML based beam prediction in spatial/time domain, study potential specification impact related to assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

AI/ML models are data driving. They are trained to learn patterns from data. But the environment of a mobile communication system is dynamic and changes over time, and thus the data also keeps changing. As the data from the environment changes, the AI/ML model performance may be degraded. Therefore, it is essential to monitor the AI/ML model performance and regularly update the model to maintain satisfactory model performance. To this end, assistance signalling and procedure for model performance monitoring and model update/tuning should be investigated.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, the model monitoring can be mainly performed at the network-side. To facilitate the network-side model monitoring, it is important to study the necessity and the potential specification impacts of several aspects, including UE reporting of beam measurement(s) based on a set of beams indicated by gNB, signaling (e.g., RRC-based, L1-based).

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, there are different alternatives for model monitoring.

- UE-side model monitoring, where UE monitors the performance metric(s) and UE makes decision(s) of model selection/activation/deactivation/switching/fallback operation. Potential specification impacts include configuration/signaling from gNB to UE for performance monitoring, as well as support of indication/request/report from UE to gNB for performance monitoring.

- Network-side model monitoring, where network monitors the performance metric(s) and network makes decision(s) of model selection/activation/deactivation/switching/ fallback operation. Potential specification impacts include configuration/signaling from gNB to UE for measurement and/or reporting, UE reporting to network (e.g., for the calculation of performance metric), and indication from network for UE to perform life cycle management operations.

- Hybrid model monitoring, where UE monitors the performance metric(s) and network makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation.

Different performance metrics can be considered for AI/ML model monitoring for BM-Case1 and BM-Case2. Examples include beam prediction accuracy related KPIs (e.g., Top-K/1 beam prediction accuracy), link quality related KPIs (e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER), performance metric based on input/output data distribution of AI/ML, L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP, etc.

The potential specification impact of utilizing the aforementioned alternatives and the performance metrics for model monitoring requires investigation.

Proposal 9: For AI/ML based beam prediction in spatial/time domain, study potential specification impact related to assistance signalling and procedure for model performance monitoring and model update/tuning.

When it comes to AI/ML model inference input, report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) may be needed. In general, the type of model input, and model input acquisition and pre-processing may have potential specification impact. Similarly, when it comes to AI/ML model inference output, outputs generated by an AI/ML model may need to be delivered from gNB to UE or from UE to gNB. The post-processing of AI/ML model inference output may have potential specification impact as well.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, it is beneficial to study potential specification impact of L1 reporting enhancements for AI/ML model inference, such as UE reporting of the measurement results of more than 4 beams in one reporting instance.

For BM-Case1 with a UE-side AI/ML model, the information of AI/ML model inference such as (1) the beam(s) that is based on the output of AI/ML model inference and (2) predicted L1-RSRP corresponding to the beam(s) should be reported to network. Besides, confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams) can also be reported from UE to network. The potential specification impact of L1 signaling to support the reporting requires investigation.

For BM-Case2 with a UE-side AI/ML model, the information of AI/ML model inference such as (1) the beam(s) of N future time instance(s) that is based on the output of AI/ML model inference, (2) predicted L1-RSRP corresponding to the beam(s), and (3) information about the timestamp corresponding the reported beam(s) should be reported to network. Besides, confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams) can also be reported from UE to network. The potential specification impact of L1 signaling to support the reporting requires investigation.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable, can be beneficial. Also, beam indication from network for UE reception is needed.

Proposal 10: For AI/ML based beam prediction in spatial/time domain, study potential specification impact related to report/feedback of model input for inference, type of model input, and model input acquisition and pre-processing.

Proposal 11: For AI/ML based beam prediction in spatial/time domain, study potential specification impact related to report/feedback of model inference output and post-processing.

Different UEs may have different capabilities when it comes to the support of AI/ML algorithms for beam prediction in spatial/time domain. Therefore, UE capability for AI/ML based beam prediction including model training, model inference and model monitoring needs to be investigated and defined.

Proposal 12: For AI/ML based beam prediction in spatial/time domain, study potential specification impact related to UE capability for AI/ML based beam prediction including model training, model inference and model monitoring.

# Conclusion

In the previous sections, we discuss general aspects of AI/ML framework for NR air interface and make the following observations:

Observation 1: Offline training may be more feasible for the near future. But in the long run, it is vital that the AI/ML models can learn continuously to adapt to varying environments, site-specific conditions, and heterogenous configurations.

Observation 2: Evaluation results show that by using L1-RSRP measurement based on Set B of beams, the AI/ML-based algorithm can achieve performance comparable to that of exhaustive beam search in Set A of beams.

Observation 3: Evaluation results show that by using historical optimal index, the AI/ML-based algorithm can satisfactorily yield optimal beam index prediction for future time instances.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Beam prediction in spatial domain and beam prediction in time domain should be the focal point for studying AI/ML based algorithms for beam management.

Proposal 2: For the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training:

- Alt.1: offline training

- Alt.2: online training

Proposal 3: For BM-Case 1, at least support L1-RSRP measurement based on Set B of beams as AI/ML model input.

Proposal 4: Comprehensive evaluation results showing convincing performance gains is needed to nail down the essential assistance information needed for the spatial-domain DL beam prediction.

Proposal 5: For BM-Case 2 (temporal DL beam prediction), at least support using historical optimal beam index based on Set B of beams as AI/ML model input.

Proposal 6: Comprehensive evaluation results showing convincing performance gains is needed to nail down the essential assistance information needed for the temporal DL beam prediction.

Proposal 7: For AI/ML model training for beam prediction in spatial/time domain, study potential specification impact related to training data type/size, training data source determination, and assistance signalling and procedure for training data collection.

Proposal 8: For AI/ML based beam prediction in spatial/time domain, study potential specification impact related to assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

Proposal 9: For AI/ML based beam prediction in spatial/time domain, study potential specification impact related to assistance signalling and procedure for model performance monitoring and model update/tuning.

Proposal 10: For AI/ML based beam prediction in spatial/time domain, study potential specification impact related to report/feedback of model input for inference, type of model input, and model input acquisition and pre-processing.

Proposal 11: For AI/ML based beam prediction in spatial/time domain, study potential specification impact related to report/feedback of model inference output and post-processing.

Proposal 12: For AI/ML based beam prediction in spatial/time domain, study potential specification impact related to UE capability for AI/ML based beam prediction including model training, model inference and model monitoring.

# Reference

RP-221348, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP TSG RAN #96, June 2022.

R1-2303438, “Evaluation of AI and ML for beam management,” RAN1#112bis-e, NVIDIA.