3GPP TSG RAN WG1 #112bis-e									 	         R1- 2303669

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.2.3.2

Source:	NEC

Title:               	Discussion on AI/ML for beam management

Document for:	Discussion and Decision

- Introduction

Rel-18 study item on AI/ML for NR air interface has been agreed in RAN#94. Specifically, beam management was agreed as an initial use case. Among the objectives, the scope of this SI mainly includes finalizing representative sub use cases in the agreed use case and assessing potential specification impact for the agreed sub use cases.

In RAN1#109 [1], BM-Case1 (spatial-domain beam prediction) and BM-Case2 (time-domain beam prediction) have been agreed as representative sub use cases. In this contribution, we provided our views on the details of BM-Case1 and BM-Case2, and potential specification impacts.

- Discussion on details of representative sub use cases

In this section, some suggestions on the input and output of BM-Case1 and BM-Case2 are provided.

  - Input of BM-Case1 and BM-Case2

For AI/ML input of BM-Case1 and BM-Case2, the following conclusions were reached in RAN1#109 [1] and RAN1#112 [5].

Conclusion Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input: - Alt.1: Only L1-RSRP measurement based on Set B - Alt.2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companions in the discussion: Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: CIR based on Set B - Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose. Conclusion Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance): - Alt.1: Only L1-RSRP measurement based on Set B - Alt 2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam pointing angles beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose. Conclusion Regarding the explicit assistance information from UE to network for NW-side AI/ML model, RAN1 has no consensus to support the following information - UE location - UE moving direction - UE Rx beam shape/direction Conclusion Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information - NW-side beam shape information - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc. - Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion - e.g., some information following the same principle of Rel-17 positioning agreement

Alt.2: L1-RSRP measurement based on Set B and assistance information

From the evaluation results provided some companies, if Set B is variable, the performance (e.g., beam prediction accuracy) of model inference using only L1-RSRP measurements based on Set B as AI input will deteriorate. In this case, it is necessary to introduce additional assistance information as AI input, such as angle related information (e.g., Tx/Rx beam angle information, UE direction/orientation information) and positioning related information (e.g., UE position). Furthermore, compared to DL Tx/Rx beam ID, i.e., Alt.3, angle related information can provide better generalization for the AI/ML model.

Proposal 1: Support angle related information (e.g., beam angle information, UE direction/orientation information) and positioning related information (e.g., UE position) as assistance information.

In previous meetings, some companies have raised concerns about proprietary/privacy of some assistance information (e.g., Tx/Rx beam shape/angle/direction, UE location, UE moving direction). For this reason, it was concluded at the last meeting that such assistance information cannot be explicitly disclosed to the other side. From our point of view, considering the irreplaceable importance of the assistance information while avoiding the proprietary/privacy, we should study implicitly providing the assistance information from one side to the other side. For instance, the angle related information can be converted or mapped into another space (e.g., L1-RSRP measurement) based on the trigonometric function and then disclosed in another form.

Proposal 2: For avoiding the proprietary/privacy of the angle related information, study implicitly providing the assistance information (e.g., angle related information) from one side to the other side.

  - Output of BM-Case1 and BM-Case2

For AI/ML output of BM-Case1 and BM-Case2, the following agreement was reached in RAN1#110 [2].

Agreement Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output: - Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure) - E.g., N predicted beams can be the top-N predicted beams - Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - FFS: details of Beam angle(s) - FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s)) - Note1: It is up to companies to provide other alternative(s) - Note2: Beam ID is only used for discussion purpose - Note3: All the outputs are “nominal” and only for discussion purpose - Note4: Values of N is up to each company. - Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side. - Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output

How to select the N DL Tx and/or Rx beams

According to the contributions about evaluations provided some companies in previous meetings, the AI/ML approaches for BM-Case1/2 can divided into the following 2 types:

- Classification: use Set B to predict the best beam ID in Set A.

  - In output, each beam in Set A refers to a “category” and corresponds to a probability (i.e., the probability being the best beam).

- Regression: use Set B to predict the qualities (e.g., L1-RSRP) to all beams in Set A.

  - In output, each beam in Set A corresponds to a predicted L1-RSRP.

In general, the AI/ML model can output the best beam in Set A, e.g., the beam having the maximum probability for type of classification, or the beam having the maximum predicted L1-RSRP for type of regression.

However, in some cases, more than one best beam in Set A may exist. For example, multiple beams in Set A have high and very close beam qualities in reality. In this case, for type of classification, it is difficult for the AI/ML model to distinguish these beams (i.e., categories) due to they have similar probabilities. For type of regression, these beams may correspond similar predicted L1-RSRP. Furthermore, the above cases may also occur when some unexpected errors of AI/ML model inference happen, e.g., generalization performance of the AI/ML model is impacted.

In any case, the actual best beam is almost one of these predicted beams having the similar beam qualities. Therefore, in order to obtain the actual best beam, these predicted beams should be used as outputs of the AI/ML model, in other words, they should be regarded as candidates of the actual best beam. Ulteriorly, these beams in Set A can be selected according to some pre-defined rules. Specifically, for type of classification, the pre-defined rule can be: a sum probability of being the best beam higher than a threshold. For type of regression, the pre-defined rule can be: L1-RSRP higher than a threshold.

Proposal 3: Support selecting Top-N1 DL Tx and/or Rx beams according to some pre-defined rules, e.g., a sum probability of being the best beam higher than a threshold, L1-RSRP higher than a threshold.

- Discussion on potential spec impacts for BM-Case1/2

  - Data collection

For data collection for NW side AI/ML model trained at NW side, the following proposals were reached in RAN1#112 [5].

Proposal 3.2.1: Regarding the training data collection mechanism for NW-side AI/ML model training trained at NW side, study the following options as a starting point for the contents of collected data - Opt.1: UE reports sends M1 L1-RSRPs (corresponding to M1 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to a beam set (e.g., Set A, Set A+B, Set B), where M1 can be larger than 4 - FFS: the range of M1 - Opt.2: UE reports sends M2 L1-RSRPs (corresponding to M2 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to a beam set (e.g., Set B), reports sends M3 L1-RSRPs (corresponding to M3 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to another beam set (e.g., Set A), where M2 (M2>0) and M3 can be larger than 4 - FFS: the range of M2, M3 - Opt.3: UE r reports sends M4 L1-RSRPs (corresponding to M4 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to a beam set (e.g., Set B), reports sends M5 beams (beam pair) indicators based on the measurement corresponding to another beam set (e.g., Set A), where M4 can be larger than 4 - FFS: the range of M4, M5 - Other option(s) is not precluded - Note1: From UE perspective, the measurement and reporting related to one beam set may be separate from/transparent to the operations related to another beam set - Note2: Data collection for model training may be implemented by gNB in a transparent way - Note3: Potential down-selection/prioritization will be discussed later - Note4: Overhead, UE complexity and power consumption should be considered for the above options Proposal 3.2.2: Regarding the training data collection mechanism for NW-side AI/ML model training trained at NW side, study necessity and beam-management-specific potential specification impact (if necessary) from the following additional aspects - Signaling type Mechanism of the reporting, e.g., RRC signaling, L1 signaling, user plane, control plane - Information of timestamp corresponding to the reported data samples, e.g., timestamps, [UE speed], SNR, etc. - Signaling and/or condition(s) to trigger/stop data logging (including buffering) and/or reporting - Signaling and/or condition(s) for trigger/stop reporting - Quantization of the measurement results (e.g., L1-RSRP) - Reporting overhead reduction - Note: non-3GPP based solution is a separate issue.

As mentioned in Section 2.2, the AI/ML model for BM-Case1/2 can be designed and trained based on regression method or classification method. For different methods, the required data and (or) labels may be different. For example, for an AI/ML model designed on regression method, it is enough for model training that UE sends the L1-RSRPs of the beams in Set B and the L1-RSRPs of the beams in Set A. For an AI/ML model designed on classification method, it is enough for model training that UE sends the L1-RSRPs of the beams in Set B and the indication of the best beam in Set A. Therefore, considering the diversity of model training methods, we should study all proposed options.

Furthermore, especially for Set B is a subset of Set A or Set B is the same as Set A, the beam information in output (or label) may overlap with the beam information in input (or data). For instance, the best beam in Set A may be one of the beams in Set B. Due to the periodic nature of data collection, repeated reporting of beam information may result in unnecessary overhead of UL resources. Therefore, for all proposed options, we should study unnecessary reporting overhead reduction.

Proposal 4: Regarding the training data collection mechanism for NW-side AI/ML model training trained at NW side, study unnecessary reporting overhead reduction.

  - Model inference

For model inference for BM-Case2, the following agreement was reached in RAN1#111 [4].

Agreement For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on the following L1 reporting enhancement for AI/ML model inference - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered

In order to ensure the real-time performance of beam, especially in some scenarios of high speed movement (e.g., HST, freeway), gNB can trigger a P/SP beam report, and UE needs to perform beam measurement and reporting frequently. Based on BM-Case2, it becomes a reality to use the historical beams to predict the future beam(s), which will greatly save overhead of beam measurement and reporting. Specifically, the beams measured in multiple historical time instances are be used to predict the beam(s) in one or more future time instance(s). And the time interval (or time stamp) between the historical or future time instances is generally is the same. It means that, for a triggered P/SP beam report, UE only needs to receive the beams transmitted in partial time instances and measure the beams. And for the future time instance(s) corresponding to the predicted beam(s), UE does not need to receive the beam measurement RS(s) or (and) perform beam measurement and reporting. For this purpose, gNB can release or deactivate the P/SP beam report. However, due to beam tracking may be a long-term, continuous and periodic behavior, gNB needs to configure/release or activate/deactivate the same beam report frequently. Obviously, it will lead to huge and unnecessary overhead of signaling (e.g., RRC, MAC CE). Therefore, for reduction of unnecessary signaling overhead, we should study discontinuous P/SP beam report.

Proposal 5: Regarding model inference for BM-Case2 with NW side AI/ML model, study discontinuous P/SP beam report.

  - Model monitoring

For model monitoring for UE-side AI/ML model, three monitoring methods including UE-side, NW-side and hybrid performance monitoring have been agreed in RAN1#110-bis [3].

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

For hybrid performance monitoring, the following proposal was reached in RAN1#112 [5].

Proposal 5.3.3: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding Alt.3 (Hybrid model monitoring), study the following aspects as a starting point including the study of necessity: - Signaling from gNB to UE for performance monitoring (e.g., dedicated RS configuration for measurement) - The contents of UE reporting and the UE reporting mechanism to NW - Other aspect(s) is not precluded - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered

For Alt.3 Hybrid performance monitoring, it has been agreed that UE is responsible for monitoring (i.e., calculating) the performance metric(s). From our point of view, we should study what information needs to be reported to NW. Naturally, UE can directly report the performance metric(s), such as beam prediction accuracy related KPIs, link quality related KPIs, performance metric related to data distribution and L1-RSRP difference. But we need to study additional mechanism of reporting the performance metric(s). Alternatively, UE can report some information about non-performance metric, which is determined based on the performance metric(s). For instance, top-1/K or top-K/1 beam prediction accuracy can be used as a performance metric of the AI/ML model for BM-Case1/2. Actually, the value of K reflects the K predicted beam(s) that are needed for finding the real best beam (i.e., top-1 genie-aided beam). In our opinion, if the value of K is reported, not only can NW make reasonable LCM related decisions (e.g., model selection, model activation/deactivation, model switching, fallback), but more importantly, it can know how many predicted beams are needed for finding the real best beam to allocate or reserve for the predicted beams to be reported. As another example, for BM-Case2, within a given period of time, only performance metric(s) (e.g., beam prediction accuracy) corresponding to partial future time instance(s) can satisfy some predefined conditions about performance. In this case, the number of future time instances satisfying such conditions needs also to be reported to NW. Otherwise, unnecessary reporting overhead may be caused. Therefore, we should study the information (or contents) of UE reporting and corresponding reporting mechanism.

Proposal 6: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding Alt.3 (Hybrid model monitoring), study the following information of UE reporting and corresponding reporting mechanism.

- Performance metric.

- Non-performance metric, which is determined based on the monitored performance metric(s).

- Conclusion

In this contribution, we provided our views on the details of BM-Case1 and BM-Case2, and potential specification impacts. We have the following proposals:

Proposal 1: Support angle related information (e.g., beam angle information, UE direction/orientation information) and positioning related information (e.g., UE position) as assistance information.

Proposal 2: For avoiding the proprietary/privacy of the angle related information, study implicitly providing the assistance information (e.g., angle related information) from one side to the other side.

Proposal 3: Support selecting Top-N1 DL Tx and/or Rx beams according to some pre-defined rules, e.g., a sum probability of being the best beam higher than a threshold, L1-RSRP higher than a threshold.

Proposal 4: Regarding the training data collection mechanism for NW-side AI/ML model training trained at NW side, study unnecessary reporting overhead reduction.

Proposal 5: Regarding model inference for BM-Case2 with NW side AI/ML model, study discontinuous P/SP beam report.

Proposal 6: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding Alt.3 (Hybrid model monitoring), study the following information of UE reporting and corresponding reporting mechanism.

- Performance metric.

- Non-performance metric, which is determined based on the monitored performance metric(s).

- References

- Chair's notes RAN1#109

- Chair's notes RAN1_110

- Chair's notes RAN1_110-bis

- Chair's notes RAN1_111

- Chair's notes RAN1_112