3GPP TSG RAN WG1 #113	R1-2304464

Incheon, Korea, May 22nd – May 26th, 2023

Source: 	vivo

Title:	Further discussion on two TAs for multi-DCI-based multi-TRP operation

Agenda Item:	9.1.1.2

Document for:	Discussion and Decision

Introduction

Great progress has been achieved in the last few meetings. In this contribution, we discuss the remaining issues

Impact on random access procedure

Transmission power determination of PRACH

Agreement For multi-DCI based inter-cell multi-TRP and intra-cell multi-TRP operation with two TAGs configured in a CC, for a CFRA based PDCCH order from one TRP triggering PRACH towards another TRP, study whether and, if needed, how to determine the transmit power of the triggered PRACH preamble. Working Assumption For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by TRPX triggers RACH procedure towards either TRPX or TRPY. - FFS: details of PRACH power control

For intra-cell multi-DCI based Multi-TRP operation, UE cannot distinguish whether the PDCCH order sent by TRPX triggers RACH procedure towards TRPX or TRPY until the RAR is received. So, PL-RS for determining PRACH power should be decoupled with the PDCCH order. For inter-cell multi-DCI based Multi-TRP operation, PRACH resource is configured per additional PCI, PCI should be informed to UE together with the SSB index. Even UE has knowledge that the PRACH triggered by PDCCH order is towards to the same TRP or the other TRP, PL-RS can be determined based on the indicated SSB for simplicity. To make sure that the indicated SSB is the known SSB for UE, gNB should ensure that the SSB is the SSB reported by UE through beam reporting.

For both intra-cell and inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, PL-RS for PRACH can be the indicated SSB, and gNB ensure that the SSB is known SSB at UE.

Association between absolute TA and TAG in RACH procedure

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, down-select one of the following alternatives: - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1, 2), then the TA obtained via the RACH procedure corresponds to the nth TRP. Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2: - One of two TAG IDs configured in the SpCell can be indicated. Agreement For intercell multi-DCI based Multi-TRP operation with two TA enhancement, support indication of which PRACH configuration to be used in the RACH procedure in the PDCCH order. - FFS: Whether additionalPCI or a generic identifier is indicated in PDCCH order - FFS: The detail of the indication in PDCCH order in terms of whether to support PRACH triggered for inactive additionalPCI. Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support at least RAR-based solution where RAR is only received from a TRP that is associated with Type 1 CSS - RAR based - FFS: RAR-less solution reusing the solution agreed in Rel-18 Mobility Enh

As for the UE-initiated RACH procedure, UE can transmit RACH preamble towards any of the TRPs. However, the target TAG should be indicated for the absolute TA carried in RAR or absolute TAC MAC CE received in both gNB-initiated and UE-initiated RACH since two TAGs are configured for a cell.

Association between TAG and the abosolute TA carried in absolute TAC or RAR is required for both PDCCH triggered-RACH and UE-initiated RACH.

For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, the following alternatives are agreed for down selection:

- Alt 1: indicate TAG ID as part of TA command in RAR

- Alt 2: indicate TAG ID as part of PDCCH order

- Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1, 2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

Alt 3 provides new framework of SSB/RACH/preamble which will lead to more spec efforts. Since we have Coresets groups with different coresetpoolindex to identify TRP already, introducing two SSB/RACH/preamble group seems redundant along with the current design.

Alt 1 is a  concise solution with little spec impact. As shown in Figure 1, at least one reserved bit can be found in absolute TAC MAC CE and MAC RAR. The reserved bit can be used to indicate TAG ID. By reinterpreting the reserved bit in absolute TAC and RAR (e.g. ‘0’ indicates the TAG with lower ID, ‘1’ indicates the TAG with higher ID), association between the absolute TAC and RAR and the TAG can be achieved for all  RACH procedures.  However in Alt 2, RAR or absolute TAC MAC CE reiceved in UE initiated RACH procedure, UE cannot acquire the TAG ID for UE-initiated RACH without PDCCH order.

Abosolute TAC and MAC RAR

Based on the above analysis, we propose to support direct indication of TAG in RAR or absolute TAC is preferred by using one reserved bit (e.g. ‘0’ indicates the TAG with lower ID, ‘1’ indicates the TAG with higher ID).

Send LS to RAN2 and ask them to support direct indication of TAG in RAR or absolute TAC (e.g. by using one reserved bit, ‘0’ indicates the TAG with lower ID, ‘1’ indicates the TAG with higher ID).

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, RACH towards cell with additional configured PCIs different from the PCI of the serving cell can be triggered for tracking UL timing ahead of scheduling. Mechanism to determine which PRACH configuration (i.e., RACH configuration corresponding to serving cell PCI or an additional PCI) to be used in the RACH procedure triggered by PDCCH order should be further determined. Maximum 7 additional PCI can be configured for measurement. So, 3 bits identifier should be introduced for PDCCH order to indicate PRACH configuration.

Support PRACH triggering for inactive additional PCI.

Support explicitly indicating PRACH configuration by PDCCH order (e.g. 3 bits indicates PCI).

In Rel-18 mobility enhancement, UE-based TA measurements is supported. UE receives one relative TAC associated with one TRP, and TAC associated with the other TRP can be determined by the received TAC together with DL receiving timing difference.  The TAC received by UE is determined based on the reference TRP for example TRP 1, then UE calculates the TA for the other TRP by utilizing the timing offset between DL receiving timing from two TRPs. Support UE-based TA maintaining for 2 TA enhancement will reduce TA maintaining delay, and signaling overhead also be reduced. So, we propose to also support UE-based TA maintaining agreed in Rel-18 Mobility Enh for 2 TA enhancement.

Support UE-based TA maintaining agreed in Rel-18 Mobility Enh for 2 TA enhancement.

Further Details for two TAs

UE behavior in out-of-sync state

TAG configured for Pcell is regarded as PTAG which governs synchronization of all UL transmissions. For two TA enhancement, two TAGs can be configured for Pcell. One of the TAGs can be defined as PTAG. TAG indicated in initial access procedure can be defined as PTAG for simplicity.

TAG associates to initial access can be defined as PTAG.

In current spec [3], UL transmission should be released when the associated TA is out-of-sync that is the associated timeAlignmentTimers expires. When UL synchronization of PTAG is lost, all UL transmission should be released. For two TA procedure, the UE behavior for one of the TAs being out-of-sync should be further discussed. When UL sync with one TRP is lost, potential transmission can proceed towards the other TRP which maintains UL sync. For UE capable of deriving TAC from one TRP and DL timing difference, UL transmission of the TAG with expired timeAlignmentTimers can keep on.

Support per TAG release or keep UL transmission depending on whether the  timeAlignmentTimers expiring or not for the two TAGs defined for the serving cell.

Support keep on the UL transmission towards the TRP associated with the TAG with expired timeAlignmentTimers if UE capable of deriving TA using DL timing difference.

Relation between channels/signals and the TAs

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state for FR1/FR2 and Rel-15/16 spatial relation in FR2 - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel-15/16 spatial relation framework is used for FR1 - PUCCH - DG/CG Type 1/Type 2 PUSCH - AP/SP/P SRS Ran1 #110bis AI 9.1.1.1 Agreement: On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Ran1 #111 AI 9.1.1.1 Agreement On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value.

On unified TCI framework extension for M-DCI based MTRP, UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value. In addition, the existing TCI field in a DCI associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value. In other words, activated TCI states are grouped by Coresetpoolindex. UL channels are also grouped by Coresetpoolindex for the purpose of applying TCI state. Regarding 2 TAGs for TRP-specific TA purpose, associating one TAG for the TCI states grouped in one group is necessary. Otherwise, one TRP will maintain 2 UL timing which is not the purpose of this item.

Revert the working assumption on UE may report that it supports the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs.

Associating TAG to CORESETPoolIndex is a common design for PUCCH, DG/CG Type 1/Type 2 PUSCH, and AP/SP/P SRS when Rel-15/16 spatial relation framework is used, or FR1. For a PUCCH resource scheduled by a PDCCH with the CORESET associated with a coresetPoolIndex value, the indicated joint/UL TCI state corresponding to the same coresetPoolIndex value can be applied to the PUCCH resource. This works well for HARQ-ACK feedback where the PUCCH resource is determined based on the last DCI of the scheduling TRP. Thus, at least for PUCCH resources for HARQ-ACK feedback, configuring an association between the PUCCH resource and a coresetPoolIndex value is unnecessary. There are two fold benefits, reduced spec work on PUCCH configuration and PUCCH resources saving because a PUCCH resource/group can be dynamically assigned with different joint/UL TCI states corresponding to different TRPs by the scheduling DCI. For AP-SRS, coresetPoolIndex should be RRC-configured per SRS resource set. Regarding AP-SRS, triggering SRS resource set associated with two TRPs simultaneously using one DCI from a TRP is beneficial considering DCI overhead.

By the way, it should be noted that the association between SRS resource set with usage set to CB or NCB and coresetpoolindex is achieved by default rule in StxMP discussed in AI 9.1.4.1. It states that the first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1.  To keep consistency, same approach can be adopted in Option 2.

For PUCCH, DG/CG Type 1/Type 2 PUSCH, and AP/SP/P SRS when Rel-15/16 spatial relation framework is used, support associating TAG to CORESETPoolIndex:

for dynamically scheduled/activated PUSCH and PUCCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission

for Type 1 CG, P/SP-SRS, and P/SP-PUCCH, coresetPoolIndex is RRC-configured

for AP-SRS coresetPoolIndex is RRC-configured per SRS resource set.

- Note:  Association between SRS resource sets for CB or NCB transmission configured for STxMP and coresetPoolindex value defined in AI 9.1.4.1 can be reused.

DL reference timing definition

Agreement For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG - baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length - as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length - FFS: the maximum Rx timing difference (could be up to RAN4) - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected Ran4 LS reply: For a UE capable of supporting Receive Time Difference (RTD) > CP, MRTD/MTTD value for FR1 is 33/34.6 µs and MRTD/MTTD value for FR2 is 8/8.5 µs. For a UE not capable of supporting RTD>CP, MTTD is within (CP + M1 µs) for FR1 and MTTD is within (CP + M2 µs) for FR2. Where M1 and M2 are FFS in RAN4.

Two DL reference timings are supported to adjust UL timing for each TAG respectively regardless of whether maximum Rx timing difference exceeds CP length or not. Maximum Rx timing difference for UE supporting Rx timing difference exceeding CP length has been provided in RAN4 LS reply [4]. MRTD value for FR1 is 33 µs and MRTD value for FR2 is 8 µs. Since no additional RAN1 specification enhancement specific for this case is expected, the only issue left to be discussed is how to derive such two-reference timing. DL RS/channels transmissions are associated with two TRPs through coresetpoolindex. So, UE can derive two-reference timing by measuring RSs associated with different coresetpoolindex. Spec impact of introducing two-reference timing has impact on RAN4 spec TS 38.133. So, a LS is needed to inform the progress in RAN1.

UE derives two reference timing by measuring DL RSs associated with coresetpoolindex 0 and coresetpoolindex 1 respectively.

Send LS to RAN4 to inform agreement on two-reference timing.

Transmission interruptions with two TAs

Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs. - Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region Ran4 LS: For a UE capable of supporting Receive Time Difference (RTD) > CP, MRTD/MTTD value for FR1 is 33/34.6 µs and MRTD/MTTD value for FR2 is 8/8.5 µs. For a UE not capable of supporting RTD>CP, MTTD is within (CP + M1 µs) for FR1 and MTTD is within (CP + M2 µs) for FR2. Where M1 and M2 are FFS in RAN4. Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, down-select at least one of the following in RAN1#112bis-e: - Alt 1: Introducing a time gap X between two UL transmissions associated with two different TA values - E.g., X symbols in the slot(s) corresponding to the two UL transmission remain unused - FFS: How X is determined - Alt 2: Reduce the overlapping duration of one of the two UL transmissions - Alt 3: Scheduling restriction is applied such that the UE does not expect the two UL transmissions to overlap - Other alternatives are not precluded TBD: how to capture the down-selected alternative(s) in the specifications in case specification impact is deemed needed.

As shown in Figure 2, two TA loops maintained for two TACs may cause UL transmission overlapping due to two separate timing adjustments of two TAGs. How to handle the overlapped channels is left to be further discussed.

Two TA process loops for two TACs

To address this issue, following three options were discussed in last meeting:

Option 1: Introduce scheduling restriction in overlapping part.

Option 2: Introduce dropping rules for overlapping parts.

Option 3: Leave it for UE or NW implementation to handle this issue.

Scheduling gap is required for Option 1. Since it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs. The scheduling gap should be properly defined to avoid overlapping.

Dropping rule for UL transmission associating with a TAG is introduced in Option2. If the UL transmission is frequently scheduled from different TRPs which is typical for mTRP scenario, invalid scheduling will disable mTRP UL transmission, for frequently occurring overlap causes frequent UL transmission being dropped. Priority rule defined in current spec is more useful to achieve dynamic dropping UL transmission towards one TRP.

As for Option 3, neither UE implementation nor NW implementation can work well to deal with the overlapping UL transmission. If UE implementation determines the dropped UL transmission, data with higher priority may be delayed unfortunately. Length of overlapping part is unknown at NW, so it’s hard for NW to avoid the overlapping UL transmission.

Based on the above discussion, scheduling restriction or dropping rules need to be defined to handle the overlapping issue.

Scheduling restriction or dropping rules need to be defined to handle the overlapping issue.

Overlapping part derives from MTTD, Table 1 provides maximum number of overlapped OFDM symbols due to MTTD based on RAN4 LS assuming DL timing exceed CP. Number of overlapped OFDM symbols varies for different SCS.

Maximum number of overlapped OFDM symbols based on Ran4 LS assuming DL timing exceed CP

| Μ | FR1 | FR2 | Maximum number of overlapped OFDM symbols |
| --- | --- | --- | --- |
| 0 | 34.6 |  | 1 |
| 1 | 34.6 |  | 1 |
| 2 | 34.6 | 8.5 | 2 for FR1, 1 for FR2 |
| 3 |  | 8.5 | 1 |

Number of overlapped OFDM symbols varies for different SCS.

In addition to the UL transmission timing difference which will cause UL transmission interruption, UE capabilities need to be defined for the switch time between two TA loops to guarantee UL transmission towards both TRPs. Due to the RF switch or baseband processing delay when switching TA loop, transmission from the TRP may be interrupted if UL transmission is scheduled exactly during the switch time. So, UE capability of TA switch time between two TA loops can be reported to avoid such transmission interruption.

Scheduling restriction or dropping rules should be defined considering MTTD and UE capabilities on the switching time between two TA loops.

Conclusions

In this contribution, we have the following observation and proposals:

- Timing offset p between non-synchronized TRPs can be obtained through two RACH   procedures.

UL timing for the second TRP which is adjusted by the DL timing difference is aligned well.

Number of overlapped OFDM symbols varies for different SCS.

- For both intra-cell and inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, PL-RS for PRACH can be the indicated SSB, and gNB ensure that the SSB is known SSB at UE.

Association between TAG and the abosolute TA carried in absolute TAC or RAR is required for both PDCCH triggered-RACH and UE-initiated RACH.

Send LS to RAN2 and ask them to support direct indication of TAG in RAR or absolute TAC (e.g. by using one reserved bit, ‘0’ indicates the TAG with lower ID, ‘1’ indicates the TAG with higher ID).

Support PRACH triggering for inactive additional PCI.

Support explicitly indicating PRACH configuration by PDCCH order (e.g. 3 bits indicates PCI).

Support UE-based TA maintaining agreed in Rel-18 Mobility Enh for 2 TA enhancement.

TAG associates to initial access can be defined as PTAG.

Support per TAG release or keep UL transmission depending on whether the timeAlignmentTimers expiring or not for the two TAGs defined for the serving cell.

Support keep on the UL transmission towards the TRP associated with the TAG with expired timeAlignmentTimers if UE capable of deriving TA using DL timing difference.

Revert the working assumption on UE may report that it supports the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs.

For PUCCH, DG/CG Type 1/Type 2 PUSCH, and AP/SP/P SRS when Rel-15/16 spatial relation framework is used, support associating TAG to CORESETPoolIndex:

for dynamically scheduled/activated PUSCH and PUCCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission

for Type 1 CG, P/SP-SRS, and P/SP-PUCCH, coresetPoolIndex is RRC-configured

for AP-SRS coresetPoolIndex is RRC-configured per SRS resource set.

- Note:  Association between SRS resource sets for CB or NCB transmission configured for STxMP and coresetPoolindex value defined in AI 9.1.4.1 can be reused.

UE derives two reference timing by measuring DL RSs associated with coresetpoolindex 0 and coresetpoolindex 1 respectively.

Send LS to RAN4 to inform agreement on two-reference timing.

Scheduling restriction or dropping rules need to be defined to handle the overlapping issue.

Scheduling restriction or dropping rules should be defined considering MTTD and UE capabilities on the switching time between two TA loops.

References

Chair's Notes RAN1#112-bis-e.

3GPP TS 38.133,V 17.1.0,2021-06.

3GPP TS 38.321,V 17.1.0,2021-06.

R4-2217279, Reply LS on MTTD for multi-DCI multi-TRP with two TAs.