3GPP TSG RAN WG1 #114	R1-2306448

Toulouse, France, August 21st - August 25th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Co-channel Coexistence for LTE Sidelink and NR Sidelink

Agenda item:	9.4.2

Document for:	Discussion and Decision

# 1	Introduction

The following justification is given as background for the objective to support LTE SL and NR SL co-channel coexistence in V2X deployments in ITS spectrum:

Another aspect to consider is the V2X deployment scenario where both LTE V2X and NR V2X devices are to coexist in the same frequency channel. For the two different types of devices to coexist while using a common carrier frequency, it is important that there is mechanism to efficiently utilize resource allocation by the two technologies without negatively impacting the operation of each technology. This requirement was also mentioned as part of the input from 5G Automotive Association to the Rel-18 RAN Workshop.

The main motivation to consider co-channel coexistence between LTE SL and NR SL, is associated to the expectation that most (if not all) of the ITS spectrum will be allocated by regulators to LTE SL, leaving limited (or no) dedicated spectrum for NR SL. The motivation to prioritize LTE SL in the ITS spectrum, is related to the need to enable the basic safety V2X use cases (such as the ones described in TS 22.885) in a relatively short term in as many vehicles as possible to minimize the occurrence of traffic related accidents and fatalities. As new vehicles (that support both LTE SL and NR SL, and further in the future potentially NR SL only) are introduced into the market then at some point in time there will be enough market penetration to enable the use of advanced V2X use cases (such as the ones described in TS 22.886). However, for these advanced V2X use cases to be feasible it is required that enough spectrum is made available for NR SL both in non-ITS bands as well as the ITS band While the former case is being tackled by the introduction in NR SL of features such as carrier aggregation, operation in unlicensed band and beam management at FR2, the latter is to be enabled via LTE SL and NR SL co-channel coexistence.

In RAN Meeting #99, the WID on NR sidelink evolution was revised in RP-230077 [1] such that the objective on co-channel coexistence of NR sidelink and LTE sidelink operation in common spectrum is now:

4. Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. o For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer  No change to the R16/17 resource allocation procedure in PHY due to this restriction  The existing SL slot structure from Rel-16 is unchanged  The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe o For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain  Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. o Mode 2 operation only

Post RAN1#113 the objective of sidelink co-channel coexistence was marked as complete, which means that the objective now enters maintenance. Despite being marked complete, there are still several issues that needs to be addressed to ensure that sidelink co-channel coexistence and the agreed features, will work properly.

# 2	Discussion

## 2.1	30 kHz SCS

### 2.1.1	Issue 1: For 30kHz SCS, there is a risk that PHY will only return candidate resources in the second slot.

The guidance from RAN#99 states the following PHY and MAC behavior when supporting 30kHz SCS:

… - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. o For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer  No change to the R16/17 resource allocation procedure in PHY due to this restriction  The existing SL slot structure from Rel-16 is unchanged …

This behavior will likely result in cases where no suitable candidate resources are provided to MAC. The issue arising particular when the load increases, it may occur that PHY will only identify candidate resources in the second slot, if PHY considers candidate resources in the first and second slot as independent resources. This means that PHY will not increase its RSRP threshold due to insufficient resources in the first slot only, and it is not aware that it has returned no candidate resources in a first slot.

Observation 1: There is a risk that set SA will contain only candidate single-slot resources from “second slot” and MAC will not be able to identify candidate resources for the case of 30kHz SCS NR SL with dynamic co-channel coexisting with LTE SL.

This issue, can be resolved in at least three ways, but requires changes to the resource selection procedure;

a.	PHY is configured to always provide only candidate resources from the first slot.

b.	PHY is configured to target 20% resources available in first slots (and increase RSRP thresholds based on the number of first slot candidate resources only), but provides candidates from first and second slot to MAC.

c.	PHY is configured to provide a multi-slot candidate resource set for first and second slot

d.	MAC layer instructs PHY on whether it wants candidate resources for either;

	The first slot only

	The second slot only, if first and second slot has been requested for different TBs

Approach a. is a simple solution that would require a change in the PHY resource selection procedure such that the initial set of candidate resources is defined to be only in the first slot, which would also mean that the procedure of increasing RSRP thresholds if there are not sufficient number of candidate resource remaining continues to work. The drawback of this approach a. is that it requires MAC to autonomously extend the candidate resource from the first slot into the second slot. This requires MAC to know when it can extend to the second slot and when it cannot.

Approach b. changes the evaluation in Step 7 and the check of X ∙ Mtotal to be evaluated for the candidate resources available in the first slot when NR use 30kHz SCS and dynamic co-channel coexistence. While this approach also seems simple, and the number of candidates in the first slot might be similar (from MAC perspective) compared to approach a, it leaves other questions to be addressed by RAN1, such as whether this check should also be applied in Step 5A, and whether there should be two X ∙ Mtotal conditions, one for the first slot and one fir the second slot.

Approach c. is a solution where PHY provides a multi-slot candidate resource set which contains pairs of first and second slots. This removes the responsibility of determining when extensions to the second slot can be done, but it has a larger specification impact to PHY.

Approach d. is a more flexible solution where MAC can decide whether it needs candidate resources for the first only, or for both. This solution is simple to implement in PHY and simple to handle in MAC where the only thing it need to do is to select a pair of candidate resources (one from the first and another from the second set) and check that the pair is in consecutive slots. This approach is also useful when MAC is seeking to fit a transmission in a second slot where another transmission is already scheduled in the first slot.

The minimum RAN1 impact seems to come from Approach a or d, where a restriction in the resource (re-)selection procedure is introduced. From RAN2 point-of-view approach a or b seems to be the least impacting, options. However, if this approach is agreed, the term from the guideline stating that “… can select the subsequent overlapping NR SL slot in MAC layer” will need to clarified how that works when there are no candidate resource to select in the second slot from PHY. This would also imply that MAC will be responsible for creating a resource allocation for a transmission in the second slot which will require a mapping rule for frequency resources from the first to the second slot as well as a method for MAC to identify when it is allowed to do such extension and when it is not (for example when an extension exceeds the NR SL RP or when the slots are not overlapping the LTE SL RP.

Proposal 1: Solve the issue that PHY might return candidate resources only in the second slot when operating in 30kHz SCS.

### 2.1.2	Issue 2: Rules for MAC selection of resources in the second slot when using 30kHz SCS

For now, we have no agreement on how MAC can select resources in the second slot. A rule is needed to make sure that predictability is in place by other UEs, and to make sure that MAC will not cause another AGC issue which can occur due to the channel gain difference on different sub-channels and hence the interference power from NR to LTE can change if NR is allowed to select an arbitrary combination of candidate resources in the first and second slot.

Proposal 2: For 30kHz SCS, MAC may only select resources in the second slot that occupies the same resources in frequency as the selected candidate resource from the first slot.

Issue 9: Whether PSFCH should be allowed to be transmitted in the first and/or second slot.

During the discussions in RAN1#112bis-e it was raised whether PSFCH should be constrained to be only in first or second NR slot when 30kHz SCS is used. The argumentation for placing PSFCH in only the second NR slot is that, in the case that the NR SL Rx UE transmits PSFCH on top of an LTE SL transmission (despite of NR SL Tx UE doing exclusion to prevent this case), the NR PSFCH would only interfere with the last LTE PSSCH/PSCCH symbol. If PSFCH would occur in the first slot it would interfere with two LTE PSSCH/PSCCH symbols. By interfere, it can be either by transmitting directly on top of the resources used by the LTE SL transmission, but it can also be by influencing the LTE receiver ADC with an energy difference that it was not taking into account in its AGC setting. The drawback of limiting PSFCH in specification is that a constraint of PSFCH periodicity is needed (to only allow 2 or 4 slot periodicities) and then a mechanism for ensuring that the modulo 2 NR logical slots are following the logical subchannels such that this constraint is maintained. With that in mind, if a mechanism for alignment between NR logical slots and LTE logical subframes is introduced, placement of PSFCH in the second slots can be ensured by configuration, e.g. by setting the NR resource pool bitmap such that bits are grouped into pairs of adjacent slot and bits in a pair are set to the same value. The effort for introducing this feature for the sole purpose of PSFCH interference reduction to LTE does not seem to outweigh the specification effort.

Proposal 3: For NR with SCS=30 kHz, no need to introduce a constraint in specification that PSFCH can only be in NR slots whose starting (or ending) symbol is overlapped with the starting (or ending) symbol of an LTE SL subframe.

## 2.2	Issue 3: Whether or not to boost ThLTE.

In the CR drafting process post RAN1#113, one aspect that was not completely clear was the aspect of whether or not to boost ThLTE (if set) in Step 7 in 38.214 Section 8.1.4. It is our view that despite no explicit agreement this makes good sense to do, as it treats both NR and LTE fairly, and is essential for NR to identify candidate resources when the load starts to increase. Therefore we propose to explitly agree to boost ThLTE when the NR RSRP threshold list is also boosted.

Proposal 4: LTE RSRP threshold list ThLTE is also boosted when the NR RSRP threshold list is boosted in Step 7 in 38.214 Section 8.1.4.

Text proposal:

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 … The following steps are used: … 6b) This step is executed only if the procedure in clause 8.1.4C is triggered. 7) If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Thpi,pj [ and ThLTEpi,pj, if set, ] is increased by 3 dB for each priority value pi,pj and the procedure continues with step 4. 7a) If sidelink DRX active time of RX UE is provided by the higher layer and there is no candidate single-slot resource remained within the sidelink DRX active time in the set SA, the UE based on its implementation additionally selects and includes at least one candidate single-slot resources within the sidelink DRX active time in the set SA.

## 2.3	Preemption and re-evaluation triggered by LTE

Issue 4: Preemption triggered by LTE.

Higher layers can provide PHY with a set of resources that are to be checked for the need to do re-evaluation or pre-emption. These resources are currently only checked against whether they are included in SA, and if not re-evaluation will be triggered (which also captures re-evaluation due to reasons caused by LTE), but pre-emption checking does not currently consider the LTE specific causes. We do not see why an LTE reservation should not be allowed to cause pre-emption of NR resources as an LTE reservation is anyway used to check for re-evaluation and exclusion behavior, so the step 6LTE should also be considered in the pre-emption procedure. Additionally, an NR candidate resource can also be excluded when it maps to a PSFCH slot that overlaps with an LTE reservation, which means that if pre-emption is not triggered for this resource, the Rx UE will transmit PSFCH on top of LTE, on a resource that would otherwise had been excluded by the TX UE, which is not fair behavior towards LTE compared to the intended behavior with step 5LTE3.

Proposal 5: Enable an LTE reservation to trigger pre-emption of an NR resource.

Text proposal:

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 … If a resource ri from the set r0,r1,r2,… is not a member of SA, then the UE shall report re-evaluation of the resource ri to higher layers. If a resource ri' from the set r0',r1',r2',… meets the conditions below then the UE shall report pre-emption of the resource ri' to higher layers. - ri' is not a member of SA, and - ri' meets the conditions for exclusion in step 6, with ThprioRX,prioTX set to the final threshold after executing steps 1)-7), i.e. including all necessary increments for reaching X⋅Mtotal, or for exclusion in step 6LTE, with ThLTEprioRX,prioTX set to the final threshold after executing steps 1)-7), i.e. including all necessary increments for reaching X⋅Mtotal, or for exclusion in step 5LTE3, and - the associated priority prioRX, satisfies one of the following conditions: - sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX - sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX …

Issue 5: Re-evaluation triggering on second slot candidate resources provided by MAC.

When MAC is responsible to select resources in the second slot, and if PHY are enforcing a restriction to only consider first slot single-slot resources for the candidate resources set, then if MAC asks PHY to check a resources in the second slot, re-evaluation would always be triggered for the second-slot resources. A simple solution to this issue is that MAC will not provide candidate resources from second slots to PHY for re-evaluation.

Proposal 6: For 30 kHz SCS, if PHY is restricted to provide candidate resources only in the first slot, then MAC should not provide resources in the second slot for re-evaluation.

Issue 6: Pre-emption checking of first and second slot candidate resources provided by MAC.

The issue of re-evaluation triggering also applies for pre-emption, but with the additional layer that priority is also taken into account. One solution could be to only do pre-emption checking on the candidate resource in the first slot, and then leave it to MAC to derive the pre-emption result for the second slot candidate. This solution would, however, only work when the first slot candidate has the highest or equal priority of the first and second slot candidates, which is an unreasonable constraint as it conflicts with the transmit power constraint on the first and second slot transmissions. For example, broadcast traffic is often both low priority and high power.

Another solution is that MAC may only provide a candidate from the first slot, but then associate it with the highest priority of the first and second slot candidate. This allows MAC flexibility of high/low priority in the first or second slot, while also allowing a proper pre-emption check by PHY. This is a simple solution that allows MAC to get a proper pre-emption result that can be applied to both the first and second slot candidates.

Proposal 7: For 30 kHz SCS, when MAC provides a resource for pre-emption check, MAC only provides first slot candidates to PHY, and associates the highest priority of the first and second slot candidate to the resource provided to PHY.

## 2.4	TDM-based semi-static resource pool partitioning for 30 kHz SCS

Issue 7: RP bitmap definition for 30kHz SCS in case of TDM based co-channel coexistence.

In the RAN1#110 meeting the following conclusion was made:

Conclusion For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs. o Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning. o Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe o FFS: potential enhancements for synchronization can be further investigated

If TDM based co-existence with 30 kHz SCS is used together with LTE, NR RP bitmap would typically be as long as LTE RP bitmap in terms of time. For example if LTE RP bitmap length of 20 subframes is used, NR RP bitmap length could be 40 slots. The starting point of NR RP bitmap should be aligned with the starting point of LTE RP bitmap. In order to guarantee that RP bitmaps are continuously aligned, the determination of NR reserved slots should be modified so that the locations of reserved subframes in LTE and reserved slots in NR are aligned. This can be achieved if pairs of reserved slots in NR are allocated to overlap reserved subframe in LTE.

Proposal 8: Agree on the TP to 38.214 Section 8 provided below to align reserved slot/subframe locations between NR and LTE.

# 8 Physical sidelink shared channel related procedures A UE can be configured by higher layers with one or more sidelink resource pools. A sidelink resource pool can be for transmission of PSSCH, as described in Clause 8.1, or for reception of PSSCH, as described in Clause 8.3 and can be associated with either sidelink resource allocation mode 1 or sidelink resource allocation mode 2. In the frequency domain, a sidelink resource pool consists of sl-NumSubchannel contiguous sub-channels. A sub-channel consists of sl-SubchannelSize contiguous PRBs, where sl-NumSubchannel and sl-SubchannelSize are higher layer parameters. The set of slots that may belong to a sidelink resource pool is denoted by (t0SL,t1SL,⋯,tTmax-1SL) where - 0≤tiSL<10240×2μ, 0≤i<Tmax, - the slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0, - the set includes all the slots except the following slots, - NS-SSB slots in which S-SS/PSBCH block (S-SSB) is configured, - NnonSL slots in each of which at least one of Y-th, (Y+1)-th, …, (Y+X-1)-th OFDM symbols are not semi-statically configured as UL as per the higher layer parameter tdd-UL-DL-ConfigurationCommon of the serving cell if provided or sl-TDD-Configuration if provided or sl-TDD-Config of the received PSBCH if provided, where Y and X are set by the higher layer parameters sl-StartSymbol and sl-LengthSymbols, respectively. - The reserved slots which are determined by the following steps. In case of SL with 30 kHz SCS coexisting in the same carrier with LTE SL, reserved slots are allocated in pairs overlapping with LTE reserved subframes defined in section 14.1.5 of 36.213. Otherwise, 1) the remaining slots excluding NS-SSB slots and NnonSL slots from the set of all the slots are denoted by (l0,l1,⋯,l(10240×2μ-NS-SSB-NnonSL-1)) arranged in increasing order of slot index. 2) a slot lr (0≤r<10240×2μ-NS-SSB-NnonSL) belongs to the reserved slots if r=m∙(10240×2μ-NS-SSB-NnonSL)Nreserved, here m=0,1,⋯,Nreserved-1 and Nreserved=10240×2μ-NS-SSB-NnonSL mod Lbitmap where Lbitmap denotes the length of bitmap configured by higher layers. - The slots in the set are arranged in increasing order of slot index. The UE determines the set of slots assigned to a sidelink resource pool as follows:

Issue 8: Synchronization in case of TDM based co-channel coexistence

If LTE SLSS/PSBCH and NR S-SSB are used in the carrier configured for coexistence of LTE and NR SL then configuring non-overlapping RPs is challenging because in dedicated LTE SL carrier the supported RP bitmap lengths are 16, 20 and 100 subframes and if SLSS/PSBCH are configured then RP bitmap lengths cannot be aligned with synchronization period length (160ms). Because of this it is not possible to configure RP bitmaps so that PDSCH transmissions in LTE SLSS/PSBCH subframes and NR S-SSB slots are avoided. To guarantee that RP bitmap starting points are always aligned, NR S-SSBs need to be configured to overlap with LTE SLSS/PSBCH transmissions. According to the in-device coexistence framework, device type A UE would then transmit only higher priority synchronization signal which would typically be LTE SLSS/PSBCH. This can result in that synchronization in coexistence carrier is based on just LTE SLSS/PSBCH transmissions and receptions. This should be clarified in the specifications.

Proposal 9: Agree on the TP to 38.213 Section 16.1 provided below to support synchronization based on LTE SLSS/PSBCH

## 16.1 Synchronization procedures A UE receives the following SL synchronization signals in order to perform synchronization procedures based on S-SS/PSBCH blocks: SL primary synchronization signals (S-PSS) and SL secondary synchronization signals (S-SSS) [4, TS 38.211]. For co-channel coexistence, a UE may use LTE SLSS/PSBCH for DFN and slot boundary determination if S-SS/PSBCH blocks are not detected.

## 2.5	Conflict of PSFCH RX with own LTE SL TX

Issue 10: Resource exclusion of a candidate resource mapping to a PSFCH occasion that is overlapping an LTE PSSCH from the same device LTE module.

During the discussions in RAN1#112bis-e the issue on whether the NR SL Tx UE should exclude a candidate resource if it maps to a PSFCH resource where the Type-A module own LTE SL module has selected a resource to transmit, with the consequence that the IDC framework would step-in and can force the NR SL Tx UE to not be able to receive the PSFCH transmission. The PSFCH transmission is only useful if it is both transmitted and received, and since the NR SL Tx UE is already agreed to take into account PSFCH overlapping with LTE SL reservations from other LTE SL modules, and also take into account resource selections from its own LTE module overlapping a candidate resource, it is straight forward that the NR SL Tx module should also exclude candidate resources that are mapping to a PSFCH resource that is overlapping a resource selected to be used by its own LTE module.

Proposal 10: The NR SL Tx module excludes candidate resources associated to a PSFCH resource that overlaps an LTE SL resource selected to be used by its own LTE module; and agree on TP to 38.214 Section 8.1.4 provided below.

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 … The following steps are used: … 5LTE4) In case of dynamic co-channel coexistence of LTE sidelink and NR sidelink: The UE shall exclude any candidate single-slot resource Rx,y from the set SA if all the following conditions are met: - the resource pool is configured with PSFCH resources; - the UE has a selected sidelink grant for LTE V2X according to [19, TS 36.321] . - the selected sidelink grant for LTE V2X determines the set of LTE resource blocks and LTE subframes which overlaps in time with PSFCH slots associated with Rx,y+j×PrsvpTX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1 , where the PSFCH association is according to [6, TS 38.213]. PrsvpTX' and Q are determined as in condition c) of step 6LTE. 5a) If the number of candidate single-slot resources Rx,y remaining in the set SA is smaller than X⋅Mtotal, the set SA is initialized to the set of all the candidate single-slot resources as in step 4. …

## 2.6	Other

Issue 11: RP selection with dynamic co-channel coexistence only by devices supporting dynamic co-channel coexistence

The current resource pool selection specification in TS 38.321 only distinquishes between RP with and without PSFCH configured, and a LC with HARQ feedback enabled will then result in the selection of an RP with PSFCH configured. This objective on sidelink co-channel coexistence introduces the new capability of dynamic co-channel coexistence, which should be a mandatory capability when using the RP where the framework is configured. It is therefore clear that the RP selection procedure should also make sure that it is only devices that supports dynamic co-channel coexistence that can select an RP where dynamic co-channel coexistence is configured.

Proposal 11: The resource pool with dynamic co-channel coexistence may only be used by SL UEs which support dynamic resource pool sharing.

There are several of the issues highlighted in this tdoc that have RAN2 impact, when RAN1 has agreed on a solution. This includes, ensuring sufficient candidate resources in the first slot, rule for selecting second slot candidates, re-evaluation triggering, pre-emption triggering and RP selection. It is therefore important that RAN1 informs RAN2 on the solutions that have RAN2 impact.

Proposal 12: Send an LS to RAN2 with RAN1 decisions on issues with RAN2 impact.

# 3	Conclusion

In this contribution, we discussed physical channel design for SL-U and make the following observations and proposals:

Observation 1: There is a risk that set SA will contain only candidate single-slot resources from “second slot” and MAC will not be able to identify candidate resources for the case of 30kHz SCS NR SL with dynamic co-channel coexisting with LTE SL.

Proposal 1: Solve the issue that PHY might return candidate resources only in the second slot when operating in 30kHz SCS.

Proposal 2: For 30kHz SCS, MAC may only select resources in the second slot that occupies the same resources in frequency as the selected candidate resource from the first slot.

Proposal 3: For NR with SCS=30 kHz, no need to introduce a constraint in specification that PSFCH can only be in NR slots whose starting (or ending) symbol is overlapped with the starting (or ending) symbol of an LTE SL subframe.

Proposal 4: LTE RSRP threshold list ThLTE is also boosted when the NR RSRP threshold list is boosted in Step 7 in 38.214 Section 8.1.4.

Proposal 5: Enable an LTE reservation to trigger pre-emption of an NR resource.

Proposal 6: For 30 kHz SCS, if PHY is restricted to provide candidate resources only in the first slot, then MAC should not provide resources in the second slot for re-evaluation.

Proposal 7: For 30 kHz SCS, when MAC provides a resource for pre-emption check, MAC only provides first slot candidates to PHY, and associates the highest priority of the first and second slot candidate to the resource provided to PHY.

Proposal 8: Agree on the TP to 38.214 Section 8 provided below to align reserved slot/subframe locations between NR and LTE.

Proposal 9: Agree on the TP to 38.213 Section 16.1 provided below to support synchronization based on LTE SLSS/PSBCH

Proposal 10: The NR SL Tx module excludes candidate resources associated to a PSFCH resource that overlaps an LTE SL resource selected to be used by its own LTE module; and agree on TP to 38.214 Section 8.1.4 provided below.

Proposal 11: The resource pool with dynamic co-channel coexistence may only be used by SL UEs which support dynamic resource pool sharing.

Proposal 12: Send an LS to RAN2 with RAN1 decisions on issues with RAN2 impact.

# References

[1]	RP-230077, WID on NR sidelink evolution, 3GPP TSG RAN#99.