3GPP TSG RAN WG1 #114			R1-230nnnn

Toulouse, France, August 21st – August 25th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 9.16.16

Agenda Item:		9.16.16

Document for: 	Endorsement

- 9.16.13	Others

Including UE features for endorsed TEI proposals.

Agreement

- Introduce following FGs for HARQ multiplexing for PDSCH scheduling after UL grant on PUSCH

| 55. TEI18 | 55-4a | Multiplexing Type-1 HARQ-ACK codebook for PDSCH scheduling after UL grant on PUSCH | 1. UE multiplexes Type-1 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where ACK/NACK is generated for the HARQ-ACK codebook including HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission. FFS dependency between 55-4d/55-4e | 4-1, 4-11, one of {5-17, 11-5, 11-6} | Yes | N/A | UE does not support to multiplex Type-1 HARQ-ACK codebook on non-initial a PUSCH repetition when the Type-1 codebook includes HARQ-ACK information for PDSCH scheduling after a UL grant. | Per Band | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. TEI18 | 55-4b | Multiplexing Type-2 HARQ-ACK codebook for PDSCH scheduling after UL grant on PUSCH | 1. UE multiplexes Type-2 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where the HARQ-ACK codebook includes HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission. FFS dependency between 55-4d/55-4e | 4-1, 4-10, one of {5-17, 11-5, 11-6} | Yes | N/A | UE does not support to multiplex Type-2 HARQ-ACK codebook on non-initial a PUSCH repetition when the Type-1 codebook includes HARQ-ACK information for PDSCH scheduling after a UL grant. | Per Band | N/A | N/A | N/A |  | Optional with capability signaling |
| 55. TEI18 | 55-4c | Multiplexing Type-3 HARQ-ACK codebook for PDSCH scheduling after UL grant on PUSCH | 1. UE multiplexes Type-3 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where the HARQ-ACK codebook includes HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission FFS dependency between 55-4d/55-4e | 4-1, 10-16, one of {5-17, 11-5, 11-6} | Yes | N/A | UE does not support to multiplex Type-3 HARQ-ACK codebook on non-initial a PUSCH repetition when the Type-1 codebook includes HARQ-ACK information for PDSCH scheduling after a UL grant. | Per Band | N/A | N/A | N/A |  | Optional with capability signaling |
| 55. TEI18 | 55-4d | Determining a different PUCCH resource to transmit HARQ-ACK for PDSCH scheduled after UL grant | 1. Support determining a different PUCCH resource in a slot from the PUCCH resource indicated by the last DCI format before a UL grant in the slot, to include HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling a PUSCH transmission with repetitions and the HARQ-ACK information are multiplexed on a repetition of the PUSCH transmission other than a first repetition in the same slot. | 4-1, 4-10 | Yes | N/A | UE does not support to determine a different PUCCH resource to transmit HARQ-ACK for PDSCH scheduled after UL grant. | Per Band | N/A | N/A | N/A |  | Optional with capability signaling |
| 55. TEI18 | 55-4e | Determining different codebook size to transmit HARQ-ACK for PDSCH scheduled after UL grant | 1. Support determining different codebook size in a PUCCH slot from the size determined based on HARQ-ACK information associated with PDSCH reception(s) scheduled before a UL grant, to include HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling a PUSCH transmission with repetitions and the HARQ-ACK information are multiplexed on a repetition of the PUSCH transmission other than a first repetition in the same slot. | 4-1, 4-10 | Yes | N/A | UE does not support to determine different codebook size to transmit HARQ-ACK for PDSCH scheduled after UL grant. | Per Band | N/A | N/A | N/A |  | Optional with capability signaling |

Agreement

- Introduce FG 55-5 for pathloss RS updates for Type 1 CG-PUSCH

| 55. TEI18 | 55-5 | Enable MAC CE based pathloss RS updates for Type 1 CG-PUSCH | Support configuration of enablePL-RS-UpdateForType1CG-PUSCH-SRS-r18 | 5-19, 16-1e | Yes | N/A | MAC CE based pathloss RS updates for Type 1 CG-PUSCH is not supported | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Introduce FG 55-6 for (2, 2) span-based PDCCH monitoring with additional restriction(s)

| 55. TEI18 | 55-6 | (2, 2) span-based PDCCH monitoring with additional restriction(s) | Support of (2, 2) span-based PDCCH monitoring as per FG11-2 with the following additional restriction(s) - There is at least one OFDM symbol gap between two PDCCH monitoring occasions |  | Yes | N/A |  | Per FS | N/A | N/A |  | This capability is signalled for SCS 15 kHz and 30 kHz This capability is reported for processing capability #1 and for processing capability #2 respectively | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2306528	UE features for endorsed Rel-18 TEI on HARQ multiplexing	Huawei, HiSilicon, Ericsson, China Unicom

R1-2306598	On UE features for TEI18	Nokia, Nokia Shanghai Bell

R1-2306994	Discussion on UE feature for Rel-18 TEI	ZTE

R1-2307367	UE features for endorsed Rel-18 TEI on pathloss RS for Type 1 CG-PUSCH	xiaomi