3GPP TSG RAN WG1 #114			R1-230nnnn

Toulouse, France, August 21st – August 25th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 9.16.7

Agenda Item:		9.16.7

Document for: 	Endorsement

- 9.16.7	UE features for NR sidelink evolution

Agreement

- Introduce following FGs

| 47. NR_SL_enh2 | 47-k1 | SL channel access for dynamic channel access mode | UE supports 1. SL Type 1 channel access and contention window size adjustment 2. SL Type 2A channel access 3. SL Type 2B channel access 4. SL Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol in 15kHz SCS if the UE supports 15 kHz SCS 7. CP extension up to 2 symbols in 30kHz SCS [8. CP extension up to 2 symbols if the UE supports 60kHz SCS] [9. Switching Type 1 to Type 2 channel access when UE is able to use a shared COT] FFS whether to merge with 47-k3 | At least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | No | UE does not support channel access for NR sidelink operation in unlicensed spectrum. | [Per band] | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47- k2 | SL multi-channel access for dynamic channel access mode | 1. UE supports multi-channel access procedures for PSSCH transmission in multiple RB sets 2. UE supports Type A and Type B multi-channel access procedures for concurrent PSFCH transmissions in multiple RB sets [3. UE supports Type A and Type B multi-channel access procedures for concurrent S-SSB transmissions in multiple RB sets] FFS whether to introduce FG(s) for transmission on multiple RB sets | 47-k1, 15-11, and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | [No] | UE does not support multi-channel access in dynamic channel access mode for NR sidelink operation in unlicensed spectrum. | [Per band / Per UE or Per FS (for 20MHz BW only UE)] | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with or without capability signalling] |

Agreement

- Introduce following FGs

| 47. NR_SL_enh2 | 47- k3 | Receiving UE to UE COT sharing information | 1. UE supports monitoring SCI to read COT sharing information FFS whether to merge with 47-k1 | 47-k1, 15-1, and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | No | [No] | UE does not support UE-to-UE COT sharing for NR sidelink operation in unlicensed spectrum. | [Per band] | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-k4 | Transmitting UE to UE COT sharing information | 1. UE supports using ue-toUE-COT-SharingED-Threshold for Type 1 channel access for UE to UE COT sharing 2. UE supports indicating in SCI COT sharing information | 47-k1 and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | No | [No] | UE does not support UE-to-UE COT sharing for NR sidelink operation in unlicensed spectrum. | [Per band] | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |

Agreement

- Introduce following FGs

| 47. NR_SL_enh2 | 47-k6 | Type1 LBT blocking Option 1 | UE supports 1. [may] avoid selection of N consecutive resource(s) before a reserved resource with high L1 SL priority 2. [may] avoid selection of M consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource | 47-k1 and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | No | UE does not support Type1 LBT blocking Option 1 | [Per UE] | n/a | n/a |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-k7 | Type1 LBT blocking Option 2 | UE supports 1. If transmission in slot(s) before a reserved resource is able to share its initiated COT to the reservation [with high L1 SL priority], UE [may] prioritize / select resource(s) in the slot(s) for transmission | 47-k1 and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | No | UE does not support Type1 LBT blocking Option 2 | [Per UE] | n/a | n/a |  |  | Optional with capability signalling |

Agreement

- Introduce following FG

| 47. NR_SL_enh2 | 47-m1 | Interlace RB-based SL transmission/reception | 1. UE supports interlace RB-based SL transmissions for the physical layer channels that it is capable of transmit 2 UE supports interlace RB-based SL receptions for the physical layer channels that it is capable of receive FFS whether to separate this FG for Tx/Rx and for PSCCH/PSSCH/PSFCH | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | Yes | No | UE does not support Interlace RB-based PSCCH/PSSCH/PSFCH transmission/reception | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum, where PSD and/or OCB requirements are defined by regulation.] | [Optional with capability signalling] [For UE supports NR sidelink in unlicensed spectrum, where PSD and/or OCB requirements are defined by regulation, UE must indicate this FG is supported.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Introduce following FGs

| 47. NR_SL_enh2 | 47-m3 | Transmitting PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports transmitting PSCCH/PSSCH from 2nd starting symbol in a slot in addition to the first starting symbol | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | No | No | UE transmits PSCCH/PSSCH only from 1st starting symbol in a slot | Per band | N/A | N/A |  |  | [Optional without capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m4 | Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot in addition to the first starting symbol [2. UE can monitor up to [A] PSCCHs in a slot] | [15-1 except Component 5] | No | No | UE receives PSCCH/PSSCH transmitted only from 1st starting symbol in a slot | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] | [Optional without capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |

Agreement

- Introduce following FG

| 47. NR_SL_enh2 | 47-m5 | Multiple PSFCH occasions per PSCCH/PSSCH [with shared spectrum channel access] | 1. UE supports [N] PSFCH occasion(s) per PSCCH/PSSCH [with shared spectrum channel access] FFS whether to separate this FG for Tx and Rx capabilities FFS whether N is same for Tx and Rx | 15-11 | Yes | No | UE supports only one PSFCH occasion per PSCCH/PSSCH transmission | Per band | N/A | N/A |  | Candidate values for N are {1,2,3,4} | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Introduce following FG

| 47. NR_SL_enh2 | 47-m6 | Transmitting SSB repetitions within one RB set | 1. UE supports transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set FFS whether/how to introduce Rx capability | 15-4, 47-k1 | No | No | UE does not support transmission of legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] | [Optional without capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Introduce following FG

| 47. NR_SL_enh2 | 47-m8 | Transmitting S-SSB on additional S-SSB occasion(s) | 1. UE supports transmitting S-SSB on additional S-SSB occasion(s) FFS whether/how to introduce Rx capability | 15-4, 47-k1 | No | No | UE does not support transmitting S-SSB on additional S-SSB occasion(s) | Per band | N/A | N/A |  |  | [Optional without capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 47-s1 is updated as follows

| 47. NR_SL_enh2 | 47-s1 | Transmission using dynamic resource pool sharing | 1) Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing using LTE sidelink resource reservation information in NR mode2 resource (re)selection 2) UE supports NR PSCCH/PSSCH/[PSFCH] TXs in 15kHz and/or 30kHz SCSs. Candidate value sets: {[15KHz, 30kHz, both]} FFS Rx capability including supported SCS | [15-3, 15-6, 15-11] | [Yes] | No |  | [Per band] | N/A | N/A |  | [This is the basic FG for LTE/NR sidelink co-channel coexistence.] [UE supports EUTRA mode 4 SL in the same band] | [Optional with capability signalling] [For UE supports NR sidelink co-channel coexist with EUTRA sidelink, UE must support this FG.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2306525	UE features for NR sidelink evolution	Huawei, HiSilicon

R1-2306589	On UE features for NR sidelink evolution	Nokia, Nokia Shanghai Bell

R1-2306674	Discussion on UE features for NR sidelink evolution	Spreadtrum Communications

R1-2306782	Discussion on UE features for Rel-18 sidelink	vivo

R1-2307072	Discussion on UE features for NR sidelink evolution	CATT, GOHIGH

R1-2307314	Discussion on UE features for Rel-18 NR Sidelink Evolution	Apple

R1-2307370	Discussion on UE features for NR sidelink evolution	xiaomi

R1-2307502	Discussion on UE features for NR SL evolution	NTT DOCOMO, INC.

R1-2307577	UE features list for Rel-18 NR sidelink evolution WI	OPPO, Huawei, HiSilicon, LG Electronics

R1-2307604	Discussion on UE features for NR sidelink evolution	ZTE, Sanechips

R1-2307715	UE features for NR sidelink evolution	Samsung

R1-2307965	UE Features for Sidelink Evolution	Qualcomm Incorporated

R1-2308078	Discussion on UE feature for Rel-18 SL evolution	MediaTek Inc.