3GPP TSG RAN WG1 #114			R1-230nnnn

Toulouse, France, August 21st – August 25th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 9.16.8

Agenda Item:		9.16.8

Document for: 	Endorsement

- 9.16.8	UE features for eRedCap

Agreement

- Reporting type of FG 48-1/48-2 is “per UE” without FDD/TDD differentiation and without FR1/FR2 differentiation

  - The column of “Need of FR1/FR2 differentiation” is updated to “FR1 only”

Agreement

- Component 11 in FG 48-1 is confirmed as “DL/UL peak data rate target of 10 Mbps corresponding to vLayers·Qm·f = 3.2”

- Add following in the component column of FG 48-2

  - Component 11 in FG 48-1 does not apply and DL/UL peak data rate target of 10 Mbps corresponding to vLayers·Qm·f = 0.75 when vLayers = 1 and vLayers·Qm·f = 0.8 when vLayers = 2

Agreement

- Component 13 in FG 48-1 is revised as follows

  - Relaxed processing timeline of 1/0.5 ms for 15/30 kHz SCS when the RAR PDSCH and MsgB PDSCH (if supported) is larger than 25/12 PRBs for 15/30 kHz SCS

Agreement

- Add Component 14 in FG 48-1 as “Network-configurable additional separate early indication in Msg1, for Rel-18 eRedCap UEs”

- Add a note in FG 48-1 as “It is up to RAN2 whether/how to capture the capabilities for additional separate early indication of Rel-18 eRedCap UE in Msg 3 and Msg A PUSCH.”

R1-2306436	Discussion on UE features for R18 RedCap	FUTUREWEI

R1-2306530	UE features for eRedCap	Huawei, HiSilicon

R1-2306590	On UE features for eRedCap	Nokia, Nokia Shanghai Bell

R1-2306675	Discussion on UE features for eRedCap	Spreadtrum Communications

R1-2306684	UE features for eRedCap	Ericsson

R1-2306783	Discussion on UE features for R18 eRedCap	vivo

R1-2307003	UE Feature for Rel-18 RedCap UE	NEC

R1-2307073	Discussion on UE features for R18 RedCap	CATT

R1-2307146	Discussion on UE features for eRedcap	ZTE, Sanechips

R1-2307221	Discussion on UE features for R18 eRedCap	CMCC

R1-2307315	On UE features for eRedcap	Apple

R1-2307371	Discussion on UE features for eRedCap	xiaomi

R1-2307503	Discussion on UE features for eRedCap	NTT DOCOMO, INC.

R1-2307561	Rel-18 eRedCap UE features	OPPO

R1-2307716	UE features for eRedCap	Samsung

R1-2307966	UE featuers for eRedCap	Qualcomm Incorporated

R1-2308041	UE features for eRedCap	MediaTek Inc.