- 3GPP TSG RAN WG1 #115                                                                                  R1-2311204

Chicago, USA, November 13th - November 17th, 2023

Source:	ZTE

Title:	Discussion on the evaluation methodology for the self-evaluation

Agenda Item:	8.15.1

Document for:    Discussion

- Introduction

In RAN1#114bis meeting, following agreements have been achieved for ITU submission [1]. In this contribution, link budget for eMBB-s, HRC-s, mMTC-s and NB-IoT NTN are discussed.

Agreement Proposal 1.1 in section 6 of R1-2310471 is endorsed with TBS value 1736 for NR NTN Rural eMBB-s table. For the link budget template, the target elevation angle is 30 degrees, and the link budget template includes the link margin (in dB) at that elevation angle. Include the receiver interference in the link budget template for NTN, and reuse the following average INR results from TR 38.821: - 4.4 dB for FRF3 DL, - 7.6 dB for FRF3 UL The UE speed is updated as 3 km/h in the template for results collection for connection density. The maximum delay for reliability should be reported with at least consideration on the impact of assumed layout, number of repetitions, usage of HARQ, and feeder link delay. At least for eMBB-s spectral efficiency evaluation, a value of 0dB for scintillation loss can be optionally used (in addition to the already agreed 2.2dB) and results can be separately captured in TR 37.911.

- Discussion on the link budget

According to the agreements on link budget calculation, the required SNR is obtained via LLS using the corresponding simulation configurations. The link budget for different scenarios are provided below.

Table 1 Link budget for NR NTN Rural eMBB-s

| PDSCH |  |  |  | PUSCH |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TBS | BW | #reps | Target BLER | TBS | BW | #reps | Target BLER |
| 1736 | 9.36MHz | 1 | 10% | 184 | 180kHz | 20 | 10% |
| Required SNR = -5.7dB Coverage margin at elevation angle 30° = 3.9dB |  |  |  | Required SNR = -8.2dB Coverage margin at elevation angle 30° = 8.3dB |  |  |  |

Table 2 Link budget for NR NTN Rural HRC-s

| PDSCH |  |  |  | PUSCH |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TBS | BW | #reps | Target BLER | TBS | BW | #reps | Target BLER |
| 256 | 1.44MHz | 4 | 0.1% | 256 | 180kHz | 16 | 0.1% |
| Required SNR = -5.4dB Coverage margin at elevation angle 30° = 3.6dB |  |  |  | Required SNR = -0.8dB Coverage margin at elevation angle 30° = 0.8dB |  |  |  |

Table 3 Link budget for NR NTN Rural mMTC-s

| PDSCH |  |  |  | PUSCH |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TBS | BW | #reps | Target BLER | TBS | BW | #reps | Target BLER |
| 184 | 1.08MHz | 1 | 10% | 256 | 180kHz | 20 | 10% |
| Required SNR = -5.1dB Coverage margin at elevation angle 30° = 3.3dB |  |  |  | Required SNR = -3.8dB Coverage margin at elevation angle 30° = 3.9dB |  |  |  |

Table 4 Link budget for NB-IoT NTN

| NPDSCH |  |  |  | NPUSCH fmt1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TBS | BW | #reps | Target BLER | TBS | BW | #reps | Target BLER |
| 256 | 180kHz | 4 reps, 10 subframes each (total 40ms) | 10% | 256 | 15kHz | 8 reps, 4 RUs (8 subframes per RU) | 10% |
| Required SNR = -10.6dB Coverage margin at elevation angle 30° = 8.8dB |  |  |  | Required SNR = -5.7dB Coverage margin at elevation angle 30° = 16.6dB |  |  |  |

It is observed that the coverage margin at minimum elevation angle is larger than 0 in all scenarios, which means the link budget is satisfactory.

Observation 1: The link budget can be fulfilled for eMBB-s, HRC-s, mMTC-s and NB-IoT NTN.

Proposal 1: Capture the link budget results in Table 1~Table 4 into TR 37.911.

- Conclusions

In this contribution, the link budget results are reported and evaluation methodology is discussed with following observations and proposals.

Observation 1: The link budget can be fulfilled for eMBB-s, HRC-s, mMTC-s and NB-IoT NTN.

Proposal 1: Capture the link budget results in Table 1~Table 4 into TR 37.911.

References

- RAN1 Chair's notes, RAN1#114bis.