3GPP TSG RAN WG1 #115		                                                                                                                              	             R1-2311399

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:     8.16.8

Source:	Xiaomi

Title:	Discussion on UE features for Coverage Enhancements

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our high level views on Rel-18 UE features for coverage enhancements.

PRACH coverage enhancements

For FGs for PRACH coverage enhancements, two main issues are discussed in this section.

Separate FGs for CBRA and CFRA: During the last RAN2 meeting, an agreement was reached regarding the UE capability details for CBRA and CFRA. Therefore, there is no need to further discuss whether/how to separate this FG for CBRA and CFRA in RAN1.

UE capability details RAN2 assumes that a separate UE capability for CFRA with MSG1 repetition is not needed

FG type: From UE implementation perspective, it is more reasonable to support per band reporting considering that the UE may not support FG 54-1 in some bands e.g. unlicensed bands. Besides, for NR coverage enhancement in Rel-17, all the FGs are adopted as per band reporting, so the UE supporting the FG(s) of Rel-18 coverage enhancement can also follow the legacy operation.

Based on above discussions, our recommend UE feature list for PRACH coverage enhancements is shown in proposal 1.

Based on the discussions above, we propose:

Proposal 1: For PRACH coverage enhancement, revise FG 54-1 as follows.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_cov_enh2 | 54-1 | PRACH coverage enhancements | Support of multiple PRACH transmissions with the same Tx spatial filter. Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx spatial filter. |  | Yes |  | UE doesn’t support multiple PRACH transmissions with the same Tx spatial filter. | Per Band | N/A | N/A | N/A |  | Optional with capability signalling. |

Dynamic switching between DFT-s-OFDM and CP-OFDM

According to the proposed FGs which is the outcome of RAN1 #114bis for DWS [1][2], the FFS needs to be discussed further.

- For 54-3,

  - Currently, RAN1 hasn’t made any agreements on DCI 0_3 yet, and it is suggested to discuss this feature in the MC AI;

  - UE reports whether different waveforms are supported for certain carrier combinations for intra-band CA and inter-band CA, so the report granularity is suggested as per FSPC.

Based on the discussions above, we propose:

Proposal 2: Adopt the following for DWS operation UE features

- For 54-3,

  - Currently, RAN1 hasn’t made any agreements on DCI 0_3 yet, and it is suggested to discuss this feature in the MC AI;

  - UE reports whether different waveforms are supported for certain carrier combinations for intra-band CA and inter-band CA, so the report granularity is suggested as per FSPC.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_cov_enh2 | 54-3 | Dynamic waveform switching | Support of dynamic waveform switching for DCI format 0_1/0_2 |  | Yes |  | Dynamic waveform switching is not supported | per FSPC | N/A | N/A | N/A | If UE supporting this FG supports FG 11-1, the UE supports FG 54-3 with DCI format 0_2 | Optional with capability signaling. |
| 54. NR_cov_enh2 | 54-3a | PHR enhancement for dynamic waveform switching | Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH | 54-3 | Yes | N/A | PHR enhancement for dynamic waveform switching is not supported. |  | N/A | N/A | N/A |  | Optional with capability signaling. |

Conclusions

This contribution presents our views on Rel-18 UE features for coverage enhancements, which can be found in above for detailed comments and suggestions. Further discussion and consolidation in RAN1 is required.

References

[1]   R1-2310635 RAN1 UE features list for Rel-17 NR after RAN1 #114, RAN1#114bis, October 2023

[2]   Chairman Notes, RAN1#114bis, October 2023