3GPP TSG RAN WG1 #115	R1-2311874

# Chicago, USA, November 13th – November 17th, 2023

Agenda Item:		8.16.8

Source:				Samsung

Title:					UE features for coverage enhancements

Document for:	Discussion

# Introduction

This contribution discusses UE features for coverage enhancements [1] based on the agreements achieved in RAN1#114bis [2].

# Discussion

## 2.1 PRACH coverage enhancements

FG 54-1

Separate FGs for CBRA and CFRA seems not needed.

The type is not decided yet. Our preference is per UE. Differentiation between FR1 and FR2 is also acceptable.

Proposal 1: A single FG for the support of multiple PRACH transmissions for CBRA and CFRA is introduced.

Proposal 2: FG 54-1 is per UE.

## 2.2 Power domain enhancements

Enhancements for UE power high limit for CA and DC

Reporting of ΔPPowerClass when configured duty cycle is exceeded was agreed in RAN4. A new UE capability needs to be introduced to support reporting of ΔPPowerClass. RAN1 may agree to introduce it at this time and further details would need to be added later by RAN4/RAN2. Alternatively, the discussion of UE capabilities for the support of ΔPPowerClass reporting can happen in RAN4/RAN2 since the support of ΔPPowerClass reporting impacts RAN4/RAN2 specifications.

Observation 1: A UE capability for the support of ΔPPowerClass reporting needs to be introduced. Since ΔPPowerClass reporting has impact on RAN4/RAN2 specifications, the UE capability can be discussed in RAN4/RAN2.

## 2.3 Dynamic switching between DFT-S-OFDM and CP-OFDM

A single FG for the support of dynamic waveform switching for DCI formats 0_1 and 0_2 is sufficient.

A single FG for the support of dynamic waveform switching for single carrier case and multi-carrier case is sufficient.

Proposal 3: A single FG for the support of dynamic waveform switching for DCI format 0_1 and 0_2 is introduced.

Proposal 4: A single FG for the support of dynamic waveform switching for single carrier and multiple carriers.

Proposal 5: FGs 54-3/3a are per UE.

# Conclusion

The proposals and observation made in this contribution are summarized as below:

Proposal 1: A single FG for the support of multiple PRACH transmissions for CBRA and CFRA is introduced.

Proposal 2: FG 54-1 is per UE.

Observation 1: A UE capability for the support of ΔPPowerClass reporting needs to be introduced. Since ΔPPowerClass reporting has impact on RAN4/RAN2 specifications, the UE capability can be discussed in RAN4/RAN2.

Proposal 3: A single FG for the support of dynamic waveform switching for DCI format 0_1 and 0_2 is introduced.

Proposal 4: A single FG for the support of dynamic waveform switching for single carrier and multiple carriers.

Proposal 5: FGs 54-3/3a are per UE.

# References

- RP-221858, Revised WID on Further NR coverage enhancements, China Telecom, RAN#96.

- R1-2310627, Session Notes for 9.16.8, Ad-Hoc Chair (NTT DOCOMO), RAN1#114bis.