3GPP TSG RAN WG1 #115			R1- 23xxxxx

Chicago, USA, November 13th – November 17th, 2023

Agenda item:	7.1

Source: 	Moderator (Apple)

Title: 	FL summary #1 of slot offset indication for A-CSI without data

Document for:	Discussion/Decision

# Introduction

In R1-2311670, UE implementation complexities related to slot offset determination for A-CSI without data is discussed. In summary, based on current specification:

- From R15, for A-CSI without data (unlike for A-CSI with data or a for PUSCH without A-CSI) the slot offset K2 is not directly determined from the TDRA table.

- The procedure needs UE to be configured per uplink CC with up to 48 slot offset lists, called reportSlotOffsetList in Rel-15, where number of K2 entries in each list is the same as number of rows in TDRA table.

- No repetition is applied to an A-CSI PUSCH without data.

- This needs a lot of memory, which shall be accessed as quick as possible, and yet is scaled by 4 going to R16, if UE indicates capability to support repetition Type-A (or B),

The document is provided to the discuss how to resolve this implementation issue with minimized specification impact.

# Problem Statement

## UE procedure to determine K2 for A-CSI without data

In Rel-16, dynamic indication of repetition factor was introduced for slot based and sub-slot based PUSCH repetitions, called repetition Type-A and Type-B respectively. Such dynamic indication is through additional column, numberOfRepetitions in TDRA table; once “Time domain resource assignment” bit-field indicates a TDRA row, UE is also indicated about repetition factor. More precisely, for a UE capable of repetition Type-A and/or Type-B, as given by FG 11-6 and FG 11-5, if/once UE is configured with TDRA table pusch-TimeDomainAllocationListDCI-0-1 for DCI 0_1, or pusch-TimeDomainAllocationListDCI-0-2 for DCI 0-2, numberOfRepetitions is present in TDRA table. It should be noted that to provide the flexibility at the scheduler, number of entries in the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 for DCI 0_1, or pusch-TimeDomainAllocationListDCI-0-2 for DCI 0-2, can be up to maxNrofUL-Allocations-r16 which is 64 (see 38.331 for details).

When UE is scheduled to transmit a transport block with or without A-CSI, the indexed TDRA row defines the slot offset K2, the start and length indicator SLIV, etc, but for an A-CSI without data the slot offset K2 is determined differently. More precisely, from 38.214, 6.1.2.1 we have:

#### 6.1.2.1 Resource allocation in time domain …. When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a 'CSI request' field on a DCI, the 'Time domain resource assignment' field value m of the DCI provides a row index m + 1 to the allocated table as defined in Clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where are the corresponding list entries of the higher layer parameter - reportSlotOffsetListDCI-0-2, if PUSCH is scheduled by DCI format 0_2 and reportSlotOffsetListDCI-0-2 is configured; - reportSlotOffsetListDCI-0-1, if PUSCH is scheduled by DCI format 0_1 and reportSlotOffsetListDCI-0-1 is configured; - reportSlotOffsetList, otherwise; in CSI-ReportConfig for the triggered CSI Reporting Settings and is the (m+1)th entry of . ….

Fig.1: UE procedure to determine K2 for A-CSI without data

Fig.1 summarizes current specification based on which UE determines K2 when UE is scheduled to transmit A-CSI without data. As it is shown in this figure, ‘CSI request’ bit-field in DCI will trigger one state out of the configured trigger states. Each trigger state consists of up to maxNrofReportConfigPerAperiodicTrigger (=16) CSI report configs where each CSI report config is associated with a report slot offset list (so NRep in 38.214, 6.1.2.1 is up to 16). From 38.331, slot offset list must have the same number of entries as the configured TDRA table. Finally, as shown in 38.214 6.1.2.1, K2 offset value is determined by taking the maximum over the (m+1)th entry of slot offset lists in the triggered state, where m is provided by he 'Time domain resource assignment' bit-field in DCI.

## Proposed Solution

We should note that within the procedure in Sec. 2, CSI report configs in each trigger state are selected from maxNrofCSI-ReportConfigurations (=48) CSI report configs per UL CC. So, although per trigger state there are up to 16 report slot offset lists that will go through maximization, but UE must save in memory up to 48 slot offset lists per UL CC, where number of K2 values in each list and number of rows in the configured TDRA table must be the same. In other words, per UL CC UE has to budget 48x64 K2 numbers. For example, if UE supports 5 UL CCs, then it must budget a costly memory of size 30.72 KB for quick access to K2 numbers. This memory number is 4 times more than required memory in R15, where slot offset list cannot have more than 16 rows.

To reduce memory issue, in general there could be two options. One option is to report reduced capabilities (like to limit maxNumberAperiodicCSI-PerBWP-ForCSI-Report or to reduce number of DL/UL CCs, etc) so the actual number of CSI-Report configurations is much less than the nominal maxNrofCSI-ReportConfigurations (=48). This option is not desired given that it has direct impact on system performance. The second option is to limit number of entries in report slot offset list. It is worth noting that unlike TDRA table there is no strong justification to increase number of entries in report slot offset list from 16 in R15 to 64 in R16, given that no repetition is applied to A-CSI on PUSCH without data. An alternative solution to the second option which has no NBC impact is that UE may still be configured with report slot offset lists which have same number of entries as the configured TDRA table, but NW only indicates up to 16 entries for A-CSI without data. That is for A-CSI without data, the 'Time domain resource assignment' field value m of the DCI cannot be 16 or more. This is the desired solution for both UE and NW given that it has no restriction on NW configurations, and NW does not have to deal with different UE implementations.

# Discussion

## 1st round

Based on this discussion in previous sections, the following two proposals are given.

Proposal 1: When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a 'CSI request' field on a DCI, UE does not expect that 'Time domain resource assignment' field value m of the DCI is 16 or larger.

Please provide your feedback regarding the above Proposal 1.

| Company | Comments |
| --- | --- |
| Apple | We support the proposal, given that it has no restriction on NW’s configuration (UE can still be configured with up to 64 K2 entries in each report slot offset list), and it has no impact on existing UEs. Besides NW does not have to deal with different UE implementations in the field. |
| QC | We support this proposal. We have a question: since which release this proposal (and a CR later) applies to? On this, we prefer to take the proposal/CR from Rel-17. But we are open to hear others’ views and discuss more. |
| MTK | Fine to support the proposal. |
| Samsung | Based on the offline discussion, we can accept this proposal. It can be beneficial to save memory for the UE. |
| ZTE | We are fine with this proposal. |
| vivo | We are fine with above proposal, with the understanding that there is no new UE capability introduced for this. |

Proposal 2: Agree with the following TP, to capture the above discussion.

#### 6.1.2.1 Resource allocation in time domain <Unchanged parts are omitted> When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a ‘CSI request’ field on a DCI, the ‘Time domain resource assignment’ field value m of the DCI provides a row index m + 1 to the allocated table as defined in Clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where are the corresponding list entries of the higher layer parameter - reportSlotOffsetListDCI-0-2, if PUSCH is scheduled by DCI format 0_2 and reportSlotOffsetListDCI-0-2 is configured; - reportSlotOffsetListDCI-0-1, if PUSCH is scheduled by DCI format 0_1 and reportSlotOffsetListDCI-0-1 is configured; - reportSlotOffsetList, otherwise; in CSI-ReportConfig for the triggered CSI Reporting Settings and is the (m+1)th entry of , where the UE does not expect that (m+1) is larger than 16. ….

| Company | Comments |
| --- | --- |
| Apple | We prefer the specification change from the earliest release, i.e., R16. |
| QC | We support this proposal. Regarding taking this TP for which release, we prefer to take the TP from Rel-17. But we are open to hear others’ views and discuss more. |
| MTK | Fine with the above TP. |
| Samsung | We also prefer to take this TP in Rel-17. |
| ZTE | We are fine with this TP. |
| vivo | We are fine with the spec change and open for Rel-16 and/or Rel-17. |

### Summary of 1st round

Moderator would like to thank all companies contributed to offline discussions and/or left comments to the FL summary. Based on the received comments, Proposal 1 and Proposal 2 are agreeable. After further offline discussions, except one company, companies are fine to adopt the proposed TP from Rel-16. For online discussion, the following is proposed:

FL’s Proposal: Agree on Proposal 1 and Proposal 2 (proposed TP).

- The proposed TP is adopted in TS38.214, from Rel-16.

Proposal 1: When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a 'CSI request' field on a DCI, UE does not expect that 'Time domain resource assignment' field value m of the DCI is 16 or larger.

Proposal 2: Agree with the following TP, to capture the above discussion.

#### 6.1.2.1 Resource allocation in time domain <Unchanged parts are omitted> When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a ‘CSI request’ field on a DCI, the ‘Time domain resource assignment’ field value m of the DCI provides a row index m + 1 to the allocated table as defined in Clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where are the corresponding list entries of the higher layer parameter - reportSlotOffsetListDCI-0-2, if PUSCH is scheduled by DCI format 0_2 and reportSlotOffsetListDCI-0-2 is configured; - reportSlotOffsetListDCI-0-1, if PUSCH is scheduled by DCI format 0_1 and reportSlotOffsetListDCI-0-1 is configured; - reportSlotOffsetList, otherwise; in CSI-ReportConfig for the triggered CSI Reporting Settings and is the (m+1)th entry of , where the UE does not expect that (m+1) is larger than 16. ….