3GPP TSG RAN WG1 #115	        		   			R1-2312712

# Chicago, USA, November 13th – 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0568 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of multiplexing in a PUSCH with repetitions HARQ-ACK associated with DL assignments received after an UL grant for the PUSCH [HARQ-ACK MUX on PUSCH] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-11-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of multiplexing in a PUSCH with repetitions HARQ-ACK associated with DL assignments received after UL grant for the PUSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce support for multiplexing in a PUSCH with repetitions HARQ-ACK associated with DL assignments received after an UL grant for the PUSCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for multiplexing in a PUSCH with repetitions HARQ-ACK associated with DL assignments received after UL grant for the PUSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9, 9.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged parts are omitted ***

# 9	UE procedure for reporting control information

## *** Unchanged parts are omitted ***

A UE does not expect to multiplex in a PUSCH transmission in one slot with SCS configuration μ1 UCI of same type that the UE would transmit in PUCCHs in different slots with SCS configuration μ2 if μ1<μ2.

A UE does not expect to multiplex in a PUSCH transmission or in a PUCCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs of a same priority index.

A UE does not expect a PUCCH resource that results from multiplexing overlapped PUCCH resources, if applicable, to overlap with more than one PUSCHs if each of the more than one PUSCHs includes aperiodic CSI reports.

A UE does not expect to detect a first DCI format having associated HARQ-ACK information without scheduling a PDSCH reception, and indicating a resource for a PUCCH transmission with a HARQ-ACK codebook that would include the HARQ-ACK information in a slot if the UE

-	detects a second DCI format in a PDCCH monitoring occasion, that starts before a PDCCH monitoring occasion for the first DCI format, that schedules a PUSCH transmission in the slot, and

-	multiplexes the HARQ-ACK codebook in the PUSCH transmission in the slot.

If a UE

-	is not provided any of enable-Type1-HARQ-ACK-mux-forDLassignmentafterULgrant, or enable-Type2-HARQ-ACK-mux-forDLassignmentafterULgrant, or enable-Type3-HARQ-ACK-mux-forDLassignmentafterULgrant, or

-	is provided uci-MuxWithDiffPrio, or

-	transmits a PUSCH without repetitions or transmits a first repetition of a PUSCH transmission

the UE does not expect to detect a first DCI format scheduling a PDSCH reception and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information that would be included in a HARQ-ACK codebook in a slot if the UE

-	previously detects a second DCI format scheduling the PUSCH transmission in the slot, and

-	multiplexes the HARQ-ACK codebook in the PUSCH transmission in the slot.

If a UE

-	is provided enable-Type1-HARQ-ACK-mux-forDLassignmentafterULgrant, or enable-Type2-HARQ-ACK-mux-forDLassignmentafterULgrant, or enable-Type3-HARQ-ACK-mux-forDLassignmentafterULgrant, and

-	is not provided uci-MuxWithDiffPrio, and

-	transmits a repetition of a PUSCH transmission other than a first repetition,

the UE includes, in a HARQ-ACK codebook, HARQ-ACK information associated with a PDSCH reception scheduled by a first DCI format indicating a resource for a PUCCH transmission in a slot, when

-	the UE detects a second DCI format, in a PDCCH monitoring occasion that starts before the PDCCH monitoring occasion for the first DCI format, scheduling a PUSCH transmission in the slot, and

-	the UE multiplexes the HARQ-ACK codebook in the PUSCH transmission in the slot, and

-	the timeline conditions in clause 9.2.3 for PUCCH resource determination and the timeline conditions of Tproc,1mux and Tproc,2mux for multiplexing the HARQ-ACK information in the PUSCH, as described in clause 9.2.5, are satisfied, and

-	the UE does not determine a different PUCCH resource in time domain for the PUCCH transmission with the HARQ-ACK information in the slot if the UE is not provided enable-different-PUCCHresource, and

-	the UE does not determine a different size for the HARQ-ACK codebook after including the HARQ-ACK information if the UE is not provided enable-different-CBsize.

If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH and the timing conditions for overlapping PUCCHs and PUSCHs in clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH.

When a UE transmits multiple PUSCHs on respective serving cells in a slot with reference to slots for PUCCH transmissions and the multiple PUSCHs overlap with a PUCCH carrying UCI in the slot, the UE selects all the PUSCHs overlapping with the PUCCH as the candidate PUSCHs for UCI multiplexing within the slot.

## *** Unchanged parts are omitted ***

#### 9.1.2.2	Type-1 HARQ-ACK codebook in physical uplink shared channel

If a UE is not provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast or multicast HARQ-ACK information, the UE does not multiplex the unicast or multicast HARQ-ACK information in the PUSCH transmission, respectively.

If a UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast and/or multicast HARQ-ACK information, and would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then

-	if the UE has not received any PDSCH or SPS PDSCH release or TCI state update that the UE multiplexes corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or the TCI state update, or on the value of dl-DataToUL-ACK or dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_1 or DCI format 1_3, or on the value of dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-DCI-1-2-r17 if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_2 and the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast HARQ-ACK information, or on the value of dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 4_2 and the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for multicast HARQ-ACK information, in any of the Mc occasions for candidate PDSCH receptions by a DCI format or SPS PDSCH on any serving cell c, as described in clause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission

-	else the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH, unless the UE receives only a SPS PDSCH release, or only SPS PDSCH receptions, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast HARQ-ACK information, or is scheduled by DCI format 4_1 with a counter DAI field value of 1 if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for multicast HARQ-ACK information, on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception as described in clause 9.1.2.

A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to

-	PDSCH reception scheduled or activated by a DCI format that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format scheduling the PUSCH transmission, if the corresponding conditions in Clause 9 for multiplexing associated HARQ-ACK information in a HARQ-ACK codebook in the PUSCH are not valid

-	SPS PDSCH release or TCI state update that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format scheduling the PUSCH transmission.

A UE does not expect to detect a DCI format switching a DL BWP within N2 symbols prior to a first symbol of a PUSCH transmission where the UE multiplexes HARQ-ACK information, where N2 is defined in [6, TS 38.214].

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format that includes a DAI field, and

-	is not provided fdmed-ReceptionMulticast and is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for both unicast and multicast HARQ-ACK information, or

-	is provided pdsch-HARQ-ACK-Codebook = 'semi-static' only for one of unicast and multicast HARQ-ACK information

the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1 when a value of the DAI field is VT-DAIUL=1 except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when VT-DAIUL=0 unless the UE receives

-	only a SPS PDSCH release, or

-	only unicast SPS PDSCH(s) associated with transport blocks having enabled HARQ-ACK information report, or only a TCI state update, or only a PDSCH that is scheduled by a DCI format 1_0 having enabled associated HARQ-ACK information report if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast HARQ-ACK information,

-	multicast SPS PDSCH(s) with transport blocks having enabled associated HARQ-ACK information report or scheduled by a DCI format 4_1 having enabled associated HARQ-ACK information report if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for multicast HARQ-ACK information,

with a counter DAI field value of 1 on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the TCI state update or only for the PDSCH reception as described in clause 9.1.2.

VT-DAIUL=0 if the PUSCH is scheduled by a DCI format that includes a DAI field and the DAI field is set to '0'; otherwise, VT-DAIUL=1.

## *** Unchanged parts are omitted ***