3GPP TSG RAN WG1 #116bis		  R1-2402500

Changsha, China, April 15th– 20th, 2024

Agenda Item:	9.8.2

Source:	Lenovo

Title:	Discussion of FR3 channel model

Document for:	Discussion

# Introduction

One of the areas of the new SID on channel model approved in RAN #102e [1] is to study the near-field effect and the spatial non-stationarity of the wireless channel in the 7-24 GHz range:

Lessons learned from 5G/5G Advanced also provide insights for new considerations for specification development. Large MIMO antenna array deployments being envisioned for mid-band would test the limits of the existing channel models. Such considerations include near-field effects of the channel, and spatial non-stationary effects of the channel - the modelling of ray cluster blockages and/or channel parameter correlation effect on a subset of the antenna elements of a large antenna array. Additional considerations may also include the number/power of paths, cluster structure, material/building penetration loss models, and spatial consistency between a UE and different non-co-located TRPs, for example. As part of this study, assessment of these limitations and potential updates to better reflect new use cases in the 7 to 24 GHz band are proposed.

In this contribution we present our view on the near-field effect and the spatial non-stationarity of the channel and the impact on the modeling methodology.

# Discussion

## The near-field and far-field area

Figure 1. Electric field of an electric dipole oscillator.

Traditional channel modeling method assumes planar wave field from the TX antenna array to the RX antenna array, directly from the TX to the RX (LOS path) and/or indirectly through reflection/scattering on other objects (NLOS path). A planar EM wave is charactered by its uniform strength of the electric field and linear phase with respect to the receiver position. Strictly speaking for dipole radiation the planar wave is only an approximation where the distance from the TX to the RX is sufficiently large. This assumption does not hold when the antenna array becomes very large and the aperture size cannot be ignored compared to the distance between the TX and the RX. Figure 1 shows the electric field of an electric dipole oscillator. At the microscopic level, we can start by considering every antenna element as an oscillating electric dipole,

pt=p0eiωt

whose radiating electric field (E(r)) is a function of the dipole moment (p0) and the relative position from the TX antenna element (r)

Er=14πε0k2p0⊥ei(ωt-k⋅r)r=14πε0k2p0⊥ei(ωt-kr)r

Where k=2πλr  is the propagation wave vector, and p0⊥=p0-r(r⋅p0) is the projection of the dipole in the direction of the propagation r. Of particular interest to us is the last term ei(k⋅r-ωt)r, which represents a spherical wave radiating from the dipole oscillator at the origin. The strength of the electric field is inversely proportional to the distance from the oscillator r, and its phase is related to the distance r. Of particular interest to the 3GPP channel model is the case of uniform linear or planar array (ULA or UPA) where the antenna elements are evenly spaced. Under the far-field approximation where the electric field can be treated as a planar wave, the signals on all the antenna elements have the same amplitude (uniform power) and linear phase w.r.t. their positions/indices (linear phase). The uniform power assumption requires the distance ri,j between different pairs of (TX antenna j, RX antenna i) the same for different i,j, and the linear phase assumption requires the phase term wave vector k identical to the (TX antenna, RX antenna) pair. These are approximations that only hold within a certain error margin. As we are particularly interested in the phase linearity and power uniformity, we need to look at the phase and amplitude in close details. Regarding the phase, Rayleigh distance DR is defined as the minimal distance between an antenna array and a single receiver where the phase error introduced by the planar wave assumption is no larger than π8:

DR=2D2λ

D is the aperture of the antenna array which equals to the largest length of in any direction. For 1D linear array (ULA), D is the length of the linear array. For 2D rectangular array (UPA) with the size of the array Dx and Dy in the X and Y direction, D is the length of the diagonal of the array D=Dx2+Dy2.  We usually consider the large antenna array located at the base station, and the size of the array at the UE side insignificant compared with the base station array. If the dimension of the UE also needs to be considered, D can be taken as the sum of the apertures at the base station (DBS) and at the UE (DUE), D=DBS+DUE.

Figure 2 shows the Rayleigh distance for 7GHz and 24GHz as a function of the array size. For array size of 5 meters, the Rayleigh distance reaches 1100m and 4000m respectively. This exceeds the expected cell size for these frequencies and implies that the near-field effect needs to be considered for all the UEs in the cell.

Figure 2. Rayleigh distance for 7 GHz and 24 GHz.

However, the definition of Rayleigh distance assumes the receiver to be at the boresight of the antenna array. When the receiver is in a direction not aligned with the boresight, the Rayleigh distance (defined as the minimal distance with the phase error no larger than π8) also varies with the direction. In other words, the Rayleigh distance is angular dependent. As an example, Figure 3 shows the angular dependent Rayleigh distance of a ULA array with 257 antennas separated by half wavelength operating at 28 GHz. The aperture D of the antenna array is 1.37m [2].

It can be seen that the Rayleigh distance (blue dashed line) decreases with the AOD of the UE w.r.t. to the center of the array, and reaches 0 when the receiver is parallel to the antenna array. The angular dependent Rayleigh distance forms an area of an egg shape with its long axes (and the obtuse end) pointing away from the antenna array. The exact shape of this area has to be computed numerically. We can call the area within the Rayleigh distance non-linear phase area. Outside of the area, we can assume the phase is linear (with maximal phase error of  π8) with respect to the antenna placement; inside this area, non-linear phase has to be considered. Another factor needs consideration is whether the signal power is uniform or not across the TX-RX antenna pairs. Figure 3 shows 95% uniform power distance (red line) delineating the region where the ratio of signal power between the minimal and the maximal antenna is 95% or more. This uniform power distance also varies with the AOD of the UE w.r.t. to the center of the array. The uniform power distance or area also has to be computed numerically. We can call the area enclosed by the red line non-uniform power area. Outside this region we can assume the signal power between any antenna pairs is the same (with error of 5% or less).

Figure 3. Angular dependent Rayleigh distance, uniform power distance, and their approximations for a 1 dimensional ULA array.

The angular dependent Rayleigh distance and angular dependent uniform power distance together divide the area in front of the antenna array into four separate regions based on phase linearity and power uniformity. These four areas are marked out in Figure 3. It can be observed that the area with non-uniform power, including both green and the purple areas, are relatively small compared with the area with the yellow or grey area. The gray area (with both uniform power and linear phase) is the area where the EM wave propagates very much like a planar wave and is the traditional far-field area. In Figure 3 the maximal nonuniform power distance to the antenna array plane is 15D, while the maximal Rayleigh distance is 250D. Uneven power needs to be considered only when a UE is within 15D (20.55m) from the base station. We can further bound the nonuniform power area within a box (the green rectangle), and any UE outside of the box can be safely assumed to have uniform power to all the antennas. The 3GPP channel model defines a minimal distance between the gNB and UE and avoids dropping UE within the minimal distance. The minimal distance is comparable to the size of this box. We suggest to either define the minimal distance as the maximal non-uniform power distance (15D in Figure 3), or define this box an a non-drop zone in addition to the minimal distance. This way all the UEs are dropped beyond the uniform power distance or outside of this box, and we can safely assume the signal power on all the antenna elements are the same.

Proposal 1: Define the maximal uniform power distance as the minimal distance between a UE to a base station, or define a rectangular non-drop zone from the antenna array based on the uniform power distance. No UEs are dropped in this area. This makes the signal power uniform on all the antennas for simulated UEs.

The angular dependent Rayleigh distance can only be calculated numerically. This makes it difficult to define or to use in the channel model. The non-linear phase area enclosed by the blue curve in Figure 3 forms an egg shape point away from the antenna array, with the furthest reach in the boresight. We can bound this area with a circle (black circle in Figure 3) with diameter DR tangent to the center of the antenna array (the origin) and pointing away towards the boresight. For UE outside of this circle, we can ignore the nonlinear phase and assume planar EM wave. This is the far-field area we are familiar with, and the fast fading channel model in TR38.901 readily applies. Nonlinear phase of EM wave or spherical wave only needs to be considered for UEs within this circle. Because the spherical wave model is always more accurate than the planar wave model for UE at any location, we did not lose any modeling accuracy by enlarging the area where the spherical wave model is applied.

Proposal 2: Approximate the non-linear phase area as a circle (or sphere in 3D) with the Rayleigh distance DR (2D2λ) as its diameter pointing away from the center of the antenna array. The area outside this circle (or sphere) can be treated as planar wave. Area inside this circle (or sphere) needs to be treated as spherical wave.

## The near-field channel model in the spatial domain

Figure 4. Channel between a TX array and a RX array.

We have defined the near-field area where the spherical model applies. UEs outside of this area are in the far-field and the traditional channel model based on the planar wave assumption can be applied. For UEs inside this area (but outside the box of nonuniform power), the spherical wave model can be applied with nonlinear phase but uniform power. In this section we will present the spatial channel model for near-field UEs. Without loss of generality we consider the DL channel from the BS antenna array to a UE. So we refer the BS antenna array as the transmitter and the UE antenna array as the receiver. The channel can be decomposed as a LOS component and K NLOS components, where the LOS part reaches from the transmitter to the receiver directly, and each NLOS component is reflected or scattered through a scatter i.

H=HLOS+k=1KHkNLOS

For the LOS channel HLOS, the spherical wave model with non-linear phase but uniform power can be applied. For a pair of (RX antenna i, TX antenna j) located at (ri,tj), the channel coefficient between them is given by

hi,jLOS=β(fc,ro-to)e-ikri-tj

where β(f,ro-to) is a free space propagation coefficient dependent on the carrier frequency fc and the relative position (ro-to) from the center of the TX array (to) to the center of the RX array (ro). The position of each antenna is determined from its relative position in the antenna array and the orientation of the array. Because the only UEs modeled are outside of the nonuniform power area, we can apply a same parameter β(f,ro-to) to all TX-RX antenna pairs or equivalently to the whole channel matrix. For the NLOS channel, the signal transmitted from the TX may reach the RX through reflection or scattering, each path form a NLOS path. A NLOS channel is a composite channel where the channel from one part is cascaded to the channel to the next part. For a two part NLOS channel where the signal reaches the receiver through only one reflection/scattering, such as NLOS path m Figure 4, the channel

HmNLOS=βmeiϕmHRX,mLOSHm,TXLOS

is the product of two parts, Hm,TXLOS (the LOS channel from the TX antenna array to scatter m) and HRX,mLOS (the LOS channel from scatter m to the RX antenna array), and a reflection coefficient 0<βm≤1 and its phase eiϕm. If the size of a scatter is small enough, it can be considered as point reflector, and only the position of the scatter m  (sm) needs to be modelled. When the dimension of a scatter (such as path n in Figure 4) is comparable to the wavelength or larger, its size cannot be neglected. It is a continuous dielectric or metallic reflector that can be treated with the Green’s function. Integration with the Green’s function is cumbersome to say the least and is not friendly to our model. We can approximate this continuous reflector with a set of regularly placed discrete reflecting elements on a plane, like a RIS (reconfigurable intelligent surface) with the same reflection coefficient (βneiϕn) on all its elements.  For example, for a reflection surface n with size xn,yn in its X and Y direction, we can approximate it as a virtual planar array of 2xnλ,2ynλ reflection elements in the X and Y direction. The NLOS channel

HnNLOS=βneiϕnHRX,nLOSHn,TXLOS

is the product of its two segments, the first part Hn,TXLOS is the LOS channel TX antenna array to virtual reflector array at location sn, the second part Hn,TXLOS is the LOS channel from the virtual reflector array at location sn, and the reflection coefficient βneiϕn. In addition to its location and its size, the orientation  of reflector n is also needed to compute HnNLOS. Whether the LOS channel HRX,nLOS needs to be computed using the spherical or planar wave assumption is determined on whether the scatter n (sn) is located within the Rayleigh distance circle (black circle in Figure 3) from the TX antenna array or not. If the size of the UE antenna array is non-negligible compared with the distance to the scatter, the scatter may be within the Rayleigh distance circle to the UE antenna array. In this case the channel HRX,nLOS needs to be treated by the spherical wave model.

Proposal 3: Model the reflection on the scatters on NLOS paths explicitly. When the size of a scatter is comparable to the wavelength or larger, model it as a set of virtual discrete reflecting element spaced at half wavelength.

Proposal 4: Treat each NLOS path as a product of cascading channel segments, where each segment is a LOS channel in the far-field or the near-field depending on the location of the scatter relative to the antenna arrays.

The NLOS channel is determined by the position of the scatters, their reflection coefficients, and the dimension and orientation of each scatter if the its size larger than the wavelength. Given these parameters, the calculation of the NLOS channel is deterministic. As part of the channel model, we need to study the distribution of these scatters, including the number of the scatters and their positions, and their reflection coefficients, and the size and orientation of each of them in the statistical sense, so these parameters can be drawn from a sample space realistically representing the 7-24 GHz channel in typical deployment scenarios.

Proposal 5: Study the statistical distribution of the scatters for NLOS channel, including the number of the scatters, their positions, reflection coefficients, and their sizes and orientations for different scenarios.

## The near-field channel in the time domain

In the current 38.901 channel model each NLOS path is featured by a pair of (AOD, AOA) and a set of delays represented as a cluster. Because in the far-field the channel model is affected only by the (AOD, AOA) instead of the exact location of the scatters, multiple delays represent multiple reflectors in the same direction. But we have observed that for the near-field channel the exact position, its size and orientation all matter. This makes it hard to cluster multiple scatters together like in the current 38.901 model. Different scatters need to be modeled separately as they are likely to occupy different positions, have different sizes and orientations. In other words, each scatter only have a single delay and not a cluster of delay taps. If necessary, multiple scatters can be placed in close proximity to generate the effect of cluster of delays, but each scatter should be modeled separately.

Proposal 6: Model each NLOS path as a single delay tap. Multiple delay taps (delay cluster) can be modeled by placing multiple NLOS paths in close proximity.

## The Doppler effect in the near-field

When a UE is in the near-field, the Doppler effect needs to be considered for different TX-RX antenna pairs instead of applying to the entire channel matrix as a whole. For every pair of TX-RX antenna, the Doppler frequency can be calculated based on the velocity of the UE (vUE) and the relative position between the TX and RX antenna (ri-tj). The Doppler shift for the antenna pair (RX antenna i, TX antenna j) is given by

fDoppleri,j=vUE∙(ri-tj)cri-tjfc

The Doppler shift fDoppleri,j is different for different antenna pairs i,j. We can call this the near-field Doppler effect. By expanding the Doppler frequency in terms of the UE position, we can tell it has the similar form to the non-linear phase effect. Therefore the region where the near-field Doppler effect applies is similar to the region for the non-linear phase area. We have proposed to bound the near-linear phase are with a circle (or sphere) in Proposal 2. Given the similarity between the nonlinear-phase and the near-field Doppler effect, we can choose to model the near-field Doppler effect in the same area for nonlinear-phase (the circle or sphere). Outside of this area, the Doppler shift is the same for all TX-RX antenna pairs,

fDoppler=vUE∙(ro-to)cro-tofc

a frequency dependent Doppler term ei2πfDopplert can be applied to the entire channel matrix.

Proposal 7: Apply antenna-pair-wise Doppler shift only for UEs in the approximate nonlinear phase area. Outside of this area the common Doppler shift applies to all the antennas.

## Partial blockage and spatial non-stationarity in the near-field channel

Another issue making the near-field channel different from the far-field is the nonstationary blocking effect introduced when a small object is in front of the large antenna array. An object may block only some of the antenna elements but not the rest. For a UE moving through the space, it experiences this shadow fading differently in its different antennas. In the LOS+NLOS channel model we proposed, both the LOS path and each segment of a NLOS part are in fact a deterministic LOS channel once properties (the position, reflection coefficient, size and orientation) of all the objects of interest (the UE and the scatters) are given. To model the partial blockage effect,  we can drop some blockers with limited size in the LOS or the NLOS path. Given the size and the position of a blocker, the set of TX-RX antenna pairs that it blocks can be determined from geometry. Partial blockage only needs to be modelled for UEs in the approximate nonlinear-phase area (the circle or sphere). As a statistical channel model, we need to study the distribution of such blockers, in terms of their positions relative to the BS large antenna array and their sizes.

Proposal 8: Study the statistical distribution of blockers in terms of their positions relative to the BS large antenna array and their sizes.

Proposal 9: Partial blockage is modelled only for UEs in the approximate non-linear phase area.

# Conclusion

As a summary, we have the following proposals on the modeling of the near-field effect for the 7-24 GHz channel.

Proposal 1: Define the maximal uniform power distance as the minimal distance between a UE to a base station, or define a rectangular non-drop zone from the antenna array based on the uniform power distance. No UEs are dropped in this area. This makes the signal power uniform on all the antennas for simulated UEs.

Proposal 2: Approximate the non-linear phase area as a circle (or sphere in 3D) with the Rayleigh distance DR (2D2λ) as its diameter pointing away from the center of the antenna array. The area outside this circle (or sphere) can be treated as planar wave. Area inside this circle (or sphere) needs to be treated as spherical wave.

Proposal 3: Model the reflection on the scatters on NLOS paths explicitly. When the size of a scatter is comparable to the wavelength or larger, model it as a set of virtual discrete reflecting element spaced at half wavelength.

Proposal 4: Treat each NLOS path as a product of cascading channel segments, where each segment is a LOS channel in the far-field or the near-field depending on the location of the scatter relative to the antenna arrays.

Proposal 5: Study the statistical distribution of the scatters for NLOS channel, including the number of the scatters, their positions, reflection coefficients, and their sizes and orientations for different scenarios.

Proposal 6: Model each NLOS path as a single delay tap. Multiple delay taps (delay cluster) can be modeled by placing multiple NLOS paths in close proximity.

Proposal 7: Apply antenna-pair-wise Doppler shift only for UEs in the nonlinear phase area. Outside of this area the common Doppler shift applies to all the antennas.

Proposal 8: Study the statistical distribution of blockers in terms of their positions relative to the BS large antenna array and their sizes.

Proposal 9: Partial blockage is modelled only for UEs in the approximate non-linear phase area.

# References

- RP-234018, “New SID: Study on channel modeling enhancements for 7-24GHz for NR”, Nokia, 3GPP RAN #102e, Dec 2023.