3GPP TSG RAN WG1 #118                                                                                             R1-2406189

Maastricht, NL, August 19th – 23rd, 2024

Source:	vivo

Title:	Discussion on CW waveform and interference handling at AIoT UL receiver

Agenda Item:	9.4.2.4

Document for:	Discussion and Decision

- Introduction

In RAN #117 meeting, the following agreements have been reached for the study on the design of carrier wave [1].

Agreement For the study of characteristics of CW waveforms, the following table is adopted as a template for capturing observations. Note 1: Further row(s) can be added, if other CW waveform characteristic is agreed. CW waveform characteristics; Observations and/or comparisons of single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform for backscattering; … (if any). D2R reception performance. Spectrum utilization of backscattered signal corresponding to the CW waveforms. CW interference suppression at D2R receiver. Relative complexity of CW generation. Note: For two unmodulated single-tones waveform, the two tones are transmitted from the same CW node.. Observation For D2R reception performance, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides [X Y] dB frequency diversity gain in a fading channel, at least depending on the gap between the two tones and the channel’s coherence bandwidth. - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. Observation For CW interference suppression at D2R receiver, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Requires additional complexity if RF interference cancellation is used at least with CW waveform reconstruction. - Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. Agreement For multiple unmodulated single-tone transmitted by one CW node, other number of tones (i.e. >2) is deprioritized. - Note: other number of tones (i.e. >2) is studied only when obvious gains are provided. Observation For relative complexity of CW generation - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Leads to higher PAPR of the generated CW, which impacts the implementation of the power amplifier in the CW node. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

Based on the progress of the last meeting, we focus on the CW waveform comparison, CW transmission, consideration on the control of CW node and provide some observations for the interference suppression for different interference types.

- Consideration on multiple single-tone CW waveform

  - Comparison between multiple single-tone and single tone CW

In previous meeting, the comparison between single tone CW and multiple single tone CW was discussed, and observations are made for interference suppression complexity and CW generation complexity. In this section, we will discuss the comparison between single tone CW and multiple single tone CW from perspectives including D2R reception performance, spectrum utilization.

The waveform of carrier wave is agnostic to AIoT devices, they will perform the same impedance switching to generate backscattered signal on the carrier waveform regardless the CW is single tone or multiple single tone. Hence, single tone or multiple single-tone does not have any impact on harmonized design on AIoT device. Besides, the backscattered transmission does not require additional power consumption and complexity at IoT device side.

Observation 1: The CW waveform is agnostic to AIoT devices

- There is no additional power consumption and complexity at AIoT device side by adopting multiple single-tone CW compared to single tone CW.

For two single-tones CW transmission from different nodes proposed in previous meetings, it is also agnostic to AIoT devices that whether the two single -tones CW are coming from the same or different node. A single tone CW is transmitted in each CW node, and each CW node is not aware of whether CW at a different frequency tone is transmitted. Hence, two single tones CW from different nodes is a deployment choice, and no additional spec impact is expected if CW is transmitted from different nodes.

Observation 2: Two single tones CW from different nodes is a deployment choice, and no additional spec impact is expected.

Proposal 1: Deprioritize the cases two tones are transmitted from the different CW nodes for two unmodulated single-tones waveform.

- D2R reception performance

Single tone transmission may suffer from frequency selective fading in NLOS channel. To overcome the frequency selectivity, multiple single-tone CW can be used for frequency diversity. To check the D2R reception performance by different CW waveform, we conduct link level simulation, assuming TDL-A channel with 30 ns delay spread, and the CW gap are chosen from {180kHz, 1.08MHz,2.16MHz,5MHz}. More detail simulation parameters assumption can be found in Appendix A. As shown in Figure 1, when the BLER target is 1%, 9dB additional performance gain can be achieved compared with single tone CW, under the assumption of two-tone gap is 5MHz.

Figure 1 Performance comparison of backscattered signal by using single tone CW and Two tone CW

Observation 3: Around 9 dB performance gain for backscattered link can be achieved by using two-tone CW compared to the usage of single tone CW when the target BLER is 1%.

- Spectrum utilization of backscattered signal corresponding to the CW waveforms

To obtain the frequency diversity by multiple single-tone CW, larger system bandwidth should be allocated for AIoT transmission. For a minimum AIoT transmission with 180kHz system bandwidth, it is difficult to adopt multiple single tone. While if deployed with around 1MHz system bandwidth, there is enough application space for multiple single-tone CW, for example, two single-tone with 5kbps date rate with 1.08MHz separate between the two tones, then frequency diversity can be achieved in NLOS channel condition, such as TDL-A NLOS with 30 ns delay spread, according to Figure 1. Since AIoT transmission may hardly co-exist with legacy NR transmission in the same band at indoor scenario according to our evaluation [2], hence, ~1MHz BW or even larger BW for AIoT transmission can be expected. In this sense, the system bandwidth for AIoT would be sufficient to adopt multiple single tone as CW waveform. Besides, D2R based on single tone CW can achieve higher spectrum efficiency, while D2R reception based on two tone CW achieves better performance at expense of lower spectrum efficiency.

Observation 4: ~1MHz BW or even larger BW for maximum AIoT BW can be expected, which facilitate reader to adopt multiple single-tone CW.

Observation 5: D2R based on two tone CW achieves better performance at expense of lower spectrum efficiency.

Based on the discussion above, the comparison of the single tone CW and multiple single-tone CW is provided in the following Table 1.

Table 1 Comparison between single tone CW and multiple single tone CW

| CW waveform characteristics | single-tone vs two single-tones |
| --- | --- |
| D2R reception performance | Two unmodulated single-tones waveform provides ~[X=1 Y=8] dB frequency diversity gain in a fading channel. |
| Spectrum utilization of backscattered signal | D2R based on Single tone CW achieves higher spectrum efficiency. D2R based on Two tone CW achieves better performance at expense of Lower spectrum efficiency. |
| Note: For two unmodulated single-tones waveform, the two tones are transmitted from the same CW node. |  |

Proposal 2: Adopt Table 1 in R1-2406189 for the comparison between single tone CW and multiple single-tone CW.

  - Gap between tones

For single tone CW, the spectrum of backscatter signal is generated at both sides of CW frequency. The occupied frequency resource for D2R transmission is determined by the backscatter link frequency (where BLF =1/(2*chip length), aka., frequency shift) and transmission  BW of the D2R backscatter signal. The spectrum of the backscatter signal is composed of main sidelobes and several harmonic components, main sidelobe should be kept to detect the backscatter signal.

For two single-tone CW, overlapping between spectrum of D2R signal and CW frequency should be avoided to facilitate interference suppression on CW frequency. Hence, the gap between two frequency tones should be properly set to cover two main sidelobes. As shown in Figure 2, the gap should cover at least 2*frequency shift and BW of main sidelobe.

Figure 2 Frequency gap between two tones for two tone CW

Besides, the gap between frequency tones for multiple tone CW should be properly set to achieve diversity gain, i.e., BW of 2*BLF+ BW of main sidelobe should be sufficient to achieve frequency diversity. For small frequency shift with several tens of kHz and BW of main sidelobe with a few tens of kHz, the minimum gap of 2*frequency shift + BW of main sidelobe may not sufficient to achieve frequency diversity. Larger gap between the two frequency tones can be set subject to system bandwidth for AIoT deployments.

Proposal 3: The gap between two frequency tones should be properly set to achieve at least following:

- Good interference suppression, the gap should be able to cover two main sidelobes, i.e., at least 2*frequency shift plus BW of main sidelobe.

- Good frequency diversity gain subject to the total BW allocated for AIoT.

  - Frequency hopping of single tone CW

In previous meeting, frequency hopping for single tone CW is also proposed to achieve frequency diversity for D2R transmission. Different from multiple single tone CW which are transmitted simultaneously, single tone CW is transmitted at a time and the single tone CW can hop among different frequency locations in different time. Consequently, frequency hopping of the D2R transmission can be achieved by the CW frequency hopping, and interference suppression is less complicated compared with multiple single tone CW.

However, to achieve frequency diversity on D2R transmission based on CW frequency hopping, some preconditions on D2R transmission should be fulfilled. Typically, replicas of the same information should be transmitted on different frequencies to achieve the frequency diversity, replica/redundancy of the same information is generated either through simply repetition or channel coding. For example, frequency hopping can be performed on different PUSCH repetitions. For single PUSCH transmission w/o repetition, intra-slot frequency hopping can be used to achieve the frequency hopping, and for this case, the replica/redundancy of the D2R transmission is generated by channel coding with interleaving.

For D2R transmission w/o channel coding, the frequency diversity gain can only be achieved through TB level repetition, as shown in Figure 3 (a). For D2R transmission w/ channel coding, most likely the coding scheme is convolutional code with limited number of shift registers, if supported, the memory depth of the coding scheme is limited. Besides, interleaving of the convolutional code may not be feasible due to restricted by power consumption and complexity. For the case TB with convolutional code and w/o interleaving, as shown in Figure 3 (b), if the encoded bits in first hop suffer from burst errors, these bits cannot be recovered by bits transmitted in another frequency hop which have no correlation with the bits suffered from burst error. In this case, the frequency diversity gain cannot be achieved. Hence, TB level repetition of the PDRCH should also be used together with the CW frequency hopping to achieve frequency diversity. And the simulation result for TB repetition and FEC are shown in Figure 4 (a) and Figure 4 (b). The simulation assumptions are as follows in Table 2.

Table 2 The simulation assumption of the single tone hopping

| Parameter | value |
| --- | --- |
| Channel structure | data length: 96bits preamble: 22 bits (refer to RFID) CRC: 16 bits |
| Channel | TDL-A |
| Delay Spread | 30ns |
| BLF | 40kHz |
| Coding | Manchester |
| CW gap | 5MHz |
| Receiver | Non-Coherent |
| repetition | Block-level repetition |
| FEC | Tail-biting Convolutional Code with 1/3 coding rate |

Figure 3 Frequency hopping schemes for D2R transmission with CW frequency hopping

(a) Simulation with TB repetition 	                        (b) Simulation with FEC

Figure 4 performance of CW frequency hopping for D2R w/ FEC and repetition

Observation 6: For D2R transmission w/o channel coding, the frequency diversity gain can only be achieved through TB level repetition.

Observation 7: For D2R transmission w/ convolutional code and w/o interleaving, the frequency diversity gain can only be achieved through TB level repetition.

- Spectrum of CW transmission

In RAN1#106, following agreements are made on frequency spectrum of CW wave.

Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. - Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum - Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum - Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. - Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum - Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Besides, in the previous meetings, companies have discussed the CW transmission cases by considering at least the following aspects.

- Capability/complexity at D2R receiver side

- CW interference type at D2R receiver side (e.g., self-interference or CLI)

For Capability/complexity at D2R receiver side, following aspects should be considered, which may require additional capability compared with legacy gNB/UE.

- Whether gNB/UE needs to support full-duplex capability for inside CW transmission

- Whether gNB need to support D2R reception on DL spectrum or whether UE need to support D2R reception on UL spectrum.

- Interference type at D2R receiver side

The CW can be transmitted in the same node as the D2R receiver or in a different node from the D2R receiver. When the CW is transmitted inside the topology and CW is transmitted in the same node as the D2R receiver, the receiver sensitivity depends on the RFIC capability, if CW source and the receiver for backscatter signal is the same node, which may degrade the receiver sensitivity if self-interference cancellation is not well performed.

If CW is transmitted outside topology or if CW source, or the receiver for backscatter signal are different nodes, self-interference is converted to cross link interference, which are largely degraded compared with self-interference, resulting simpler interference suppression capability or better detection performance.

For interference types other than CW interference, e.g., NR transmission aggressor and AIoT victim or AIoT aggressor, AIoT victim, belongs to co-existence study. These co-existence issues may or may not lead to tough problems for the CW transmission cases. Since RAN4 will perform co-existence evaluation on these interferences, observations obtained through qualitative analysis in RAN1 would be premature. Further comparison and down-selection of the CW transmission cases can be considered after observations from RAN4 co-existence evaluation are ready.

Proposal 4: For interference types other than CW interference, e.g., NR transmission aggressor and AIoT victim or AIoT aggressor AIoT victim, which belongs to co-existence study, can be left to RAN4 co-existence evaluation.

- Further comparison and down-selection of the CW transmission cases can be considered after observations from RAN4 co-existence evaluation are ready.

Based on the discussion above, we summarize the relative pros (highlight in green) and cons (highlight in yellow) for each CW transmission case in terms of both capability/complexity and interference type at D2R receiver side in Table 3.

Table 3 Summary of Pros and cons for cases for CW transmission

| Cases |  | Capability or complexity at D2R receiver side | Interference type at D2R receiver side |
| --- | --- | --- | --- |
| Case 1-1: T1/inside/DL | Same node for CW Tx and D2R Rx | - gNB needs to support full-duplex capability in DL - gNB need to support D2R reception in DL spectrum | CW Self-interference in FDD DL spectrum |
|  | Different node for CW Tx and D2R Rx | - NO needs for gNB to support full-duplex capability in DL - gNB need to support D2R reception in DL spectrum | CW Cross link interference in FDD DL spectrum |
| Case 1-2: T1/inside/UL | Same node for CW Tx and D2R Rx | - gNB needs to support full-duplex capability in DL - gNB needs to transmit CW in UL spectrum - gNB naturally support D2R reception in UL spectrum | CW Self-interference in FDD UL spectrum |
|  | Different node for CW Tx and D2R Rx | - NO needs for gNB to support full-duplex capability in DL - gNB naturally support D2R reception in UL spectrum | CW Cross link interference in FDD UL spectrum |
| Case 1-4: T1/outside/UL | - | - NO needs for gNB to support full-duplex capability in DL - gNB naturally support D2R reception in UL spectrum | CW Cross link interference in FDD UL spectrum from CW source outside topology |
| Case 2-2: T2/inside/UL | Same node for CW Tx and D2R Rx | - UE needs to support and operate in full-duplex mode - UE need to support D2R reception in UL spectrum. | CW Self-interference in FDD UL spectrum |
|  | Different node for CW Tx and D2R Rx | - NO needs for UE to support and operate in full-duplex mode - UE need to support D2R reception in UL spectrum. | CW Cross link interference in FDD UL spectrum |
| Case 2-3: T2/outside/DL | - | - NO needs for UE to support and operate in full-duplex mode - UE naturally support D2R reception in DL spectrum | CW Cross link interference in FDD DL spectrum from CW source outside topology |
| Case 2-4: T2/outside/UL | - | - NO needs for UE to support and operate in full-duplex mode - UE need to support D2R reception in UL spectrum. | CW Cross link interference in FDD UL spectrum from CW source outside topology |
| Note: table shows the relative pros (highlight in green) and cons (highlight in yellow) for each CW transmission case in terms of both capability/complexity and interference. |  |  |  |

Proposal 5: Adopt Table 3 of R1-2406189 for comparison of different CW transmission cases.

- Consideration on the CW control for the CW node

In RAN #103 meeting, the following clarification for the work scope of CW study in Rel-19 ambient IoT has been agreed.

- Revised SID in RP-240826 - Following clarifications on the work scope of Rel-19 ambient IoT have been agreed - Refer to RP-240854 for additional details Proposal 3v2 - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc. - No SID revision is necessary.

And in the last meeting, some discussion has been revolved around the topic of CW control for the CW node. Accordingly, we provide some analyses on the control for the CW nodes. In general, timing, transmission Power, frequency resources, spectrum, frequency hopping patterns of CW etc. parameters can be considered to be adjusted through the control for the CW nodes. For example, for the timing of CW transmission, the basic things need to be controlled for CW node are when CW is transmitted or not transmitted. Besides, the CW transmission duration also needs to be managed to provide enough time for AIoT device to do backscattering once it receives a certain R2D command. In this sense, how to control the CW node and if there is any specification impact caused by the CW control for the CW node for different topologies needs to be carefully studied. For topology 1 with gNB as the CW node, it can be up to gNB implementation to control and adjust CW characteristics, so no spec impact can be seen. And for topology 1 with the CW node outside, maybe gNB can achieve the CW node via backhaul link, thus it can also up to gNB implementation to control CW node. Besides, for topology 2 and CW is transmitted by UE inside topology, the control for the CW transmission is up to UE internal implementation based on the time/frequency resources for A-IoT operation allocated by gNB. For topology 2 with outside CW node, it seems gNB implementation to do CW control will also be the way.

Observation 8: For the case that the CW node is outside topologies (both for topology 1 and 2), the control of the CW node can be achieved by gNB implementation.

Additionally, for topology 1 with the CW node outside topology, we think the case that UE plays the role of CW node without R2D transmission or D2R reception, should not be considered. Since, UE would be scheduled to perform CW transmission, while agnostic to the traffic it is involved in and in addition interrupts UE’s current communication, the use case should be justified first. That is to say, UE can be a potential CW node only when it makes communication with AIoT device.

Proposal 6: Not support UE to be the CW node if the UE does not participate in AIoT D2R/R2D transmission.

- Interference suppression for different interference types at D2R receiver

As for interference suppression, scenario A1/B and A2 can be discussed respectively. For A1/B scenario, the receiver and transmitter and separated, which means spatial domain isolation for the CW interference can be obtained through this deployment. The amount of spatial isolation can be composed of two parts, including the pathloss isolation and the transmitter and receiver antenna beam nulling [3].

- Pathloss: For the D1T1 scenario, the pathloss isolation can be reached 61 dB, which assumes the distance between CW emitter and receiver are 20 meters for InH-DH-NLOS path loss model. Meanwhile, for the D2T2 scenario, the pathloss isolation is 65 dB when the CW emitter and receiver distance is 20 meters and the pathloss model is InH-Office-NLOS.

- Beam nulling: At the same time, the transmitter or the receiver can achieve about 10 dB antenna radiation beam nulling for D1T1 scenario [3], which means EIRP of the CW to D2R receiver direction is 10dB lower than CW to tag direction, and same amount of CW leakage reduction can be obtained at D2R receiver if the receiver is also base station. While 0 dB is assumed for UE as D2R receiver.

The total interference suppression can be calculated according to the analysis, which are shown in the following Table 4.

Considering sufficient spatial domain isolation is obtained, the impact to D2R performance is limited, the additional analog domain and digital domain interference can be avoided and not reported in following table.

Table 4 CW interference suppression capability at D2R receiver for A1/B scenario

| Scenario | Part1 spatial domain | Part2 analog domain | Part3 Digital Domian | Total capability | Note(sing) |
| --- | --- | --- | --- | --- | --- |
| D1T1 | 20dB(Note1) + 61 dB(Note3) | No | No | 149(Note4) | Single tone |
| D2T2 | 10 dB(Note2) +65dB(Note3) | No | No | 151(Note4) | Single tone |
| Note 1: Assuming 20dB for beam isolation between CW Tx and D2R Rx for T1, 10 dB for Tx and 10 dB for Rx [3]. Note 2: 10dB can be obtained from beam isolation, or obtained from analog filter suppression at the receiver side for T2 . Note 3: The value 65dB and 61dB represent pathloss between the CW emitter and D2R receiver with 20 meters distance, for InH-NLOS and InF-DH-NLOS respectively; Note 4: Detailed calculation refer to , which considers IM3. |  |  |  |  |  |

For A2 scenario, the interference suppression capability consists of two components, one is the antenna isolation capability, which can be reached 30dB isolation capability for D1T1[4] and 15 dB for D2T2[5]. And the other is RF-IC suppression capability, which reconstruct the CW interference and cancels the CW before LNA, according to [6][7], the RF-IC suppression capability between 30dBb~ 60dB, we assume 50dB in the evaluation. The total interference suppression capability is shown in the following Table 5.

Table 5 CW interference suppression capability at D2R receiver for A2 scenario

| Scenario | Part1 spatial domain | Part2 analog domain | Part3 Digital Domain | Total capability | Note(sing) |
| --- | --- | --- | --- | --- | --- |
| D1T1 | 30(Note1) | 50(Note3) | No | 146(Note4) | Single tone |
| D2T2 | 15(Note2) | 50(Note3) | No | 121(Note4) | Single tone |
| Note 1: 30dB for Tx/Rx antenna isolation at BS Note 2: 15dB for Tx/Rx antenna isolation at UE Note 3: 50dB is CW cancellation obtained from RF-IC Note 4: Detailed calculation refer to , which considers IM3. |  |  |  |  |  |

Note that digital domain suppression is not counted in above tables. Since if there is no blocking or non-linear distortion, the single tone CW interference is still single tone and converted to DC in baseband, which has no impact to detection performance as long as the ratio between desired signal and residual interference is within ADC dynamic range. If blocking happens, it is not clear how to model the distortion and how to overcome it in digital baseband, hence the digital domain interference is not reported.

Observation 9: The digital domain interference suppression is not modelled due to

- If there is no blocking or non-linear distortion, the single tone CW interference is still single tone and converted to DC in baseband, which has no impact to detection performance as long as the ratio between desired signal and residual interference is within ADC dynamic range.

- If blocking happens, it is not clear how to model the non-linear distortion and how to overcome it in digital baseband.

The detailed calculation of total interference suppression capability considering spatial domain and analog domain suppression is provided as follows.

The calculation for receiver sensitivity loss in AI 9.4.1.1 is summarized as follows - Remaining CW interference 2K1=CW transmit power 1E1+CW Tx antenna gain 1E2-CW cancellation [2K] - Receiver sensitivity loss [2K2] =lin2dBdB2linNoise Power [2F]dB2linNoise Power [2F]+dB2linRemaining CW interference [2K1] - Receiver sensitivity 2L= Required SNR 2G+ Noise Power 2F- Receiver sensitivity loss [2K2]

However, according to the [7] and the discussion above, the IM3 in receiver is the factor which leads to sensitivity. Hence, the parameter ‘Remaining CW interference’ in Receiver sensitivity loss [2K2] calculation, is replaced by Rx IM3 in our calculation, as follows.

- Remaining CW interference 2K1=CW transmit power 1E1+CW Tx antenna gain 1E2-CW cancellation - where CW cancellation = Part1 spatial domain suppression + Part2 analog domain suppression - RxIM3=3*Remaining CW interference 2K1-2*IIP3 - IM3 modelling is widely used in 3GPP[8] - Receiver sensitivity loss [2K2] =lin2dBdB2linNoise Power [2F]dB2linNoise Power [2F]+dB2linRx IM3 => Total CW cancellation Value[2K] = CW transmit power 1E1+CW Tx antenna gain 1E2 - Rx IM3

The detailed total CW interference cancellation capability values, which are used for D2R coverage evaluation in AI 9.4.1.1, are provided in Appendix B.

- Conclusion

In this contribution, we provide our views on CW waveform and interference handling at AIoT UL receiver. The observations and proposals are summarized as follows.

Observation 1: The CW waveform is agnostic to AIoT devices

- There is no additional power consumption and complexity at AIoT device side by adopting multiple single-tone CW compared to single tone CW.

Observation 2: Two single tones CW from different nodes is a deployment choice, and no additional spec impact is expected.

Observation 3: Around 9 dB performance gain for backscattered link can be achieved by using two-tone CW compared to the usage of single tone CW when the target BLER is 1%.

Observation 4: ~1MHz BW or even larger BW for maximum AIoT BW can be expected, which facilitate reader to adopt multiple single-tone CW.

Observation 5: D2R based on two tone CW achieves better performance at expense of lower spectrum efficiency.

Observation 6: For D2R transmission w/o channel coding, the frequency diversity gain can only be achieved through TB level repetition.

Observation 7: For D2R transmission w/ convolutional code and w/o interleaving, the frequency diversity gain can only be achieved through TB level repetition.

Observation 8: For the case that the CW node is outside topologies (both for topology 1 and 2), the control of the CW node can be achieved by gNB implementation.

Observation 9: The digital domain interference suppression is not modelled due to

- If there is no blocking or non-linear distortion, the single tone CW interference is still single tone and converted to DC in baseband, which has no impact to detection performance as long as the ratio between desired signal and residual interference is within ADC dynamic range.

- If blocking happens, it is not clear how to model the non-linear distortion and how to overcome it in digital baseband.

Proposal 1: Deprioritize the cases two tones are transmitted from the different CW nodes for two unmodulated single-tones waveform.

Proposal 2: Adopt Table 1 in R1-2406189 for the comparison between single tone CW and multiple single-tone CW.

Proposal 3: The gap between two frequency tones should be properly set to achieve at least following:

- Good interference suppression, the gap should be able to cover two main sidelobes, i.e., at least 2*frequency shift plus BW of main sidelobe.

- Good frequency diversity gain subject to the total BW allocated for AIoT.

Proposal 4: For interference types other than CW interference, e.g., NR transmission aggressor and AIoT victim or AIoT aggressor AIoT victim, which belongs to co-existence study, can be left to RAN4 co-existence evaluation.

- Further comparison and down-selection of the CW transmission cases can be considered after observations from RAN4 co-existence evaluation are ready.

Proposal 5: Adopt Table 3 of R1-2406189 for comparison of different CW transmission cases.

Proposal 6: Not support UE to be the CW node if the UE does not participate in AIoT D2R/R2D transmission.

# References

- 3GPP RAN1#117 meeting, Chairman’s note.

- R1-2406184, Evaluation methodologies assumptions and results for Ambient IoT, vivo.

- R4-2313983, TP to TR 38.858 on SBFD Implementation feasibility for FR1 BS, Samsung.

- TS 38.104, Base Station (BS) radio transmission and reception.

- Guo J , Fan J , Sun L ,et al. A Four-Antenna System With High Isolation for Mobile Phones. IEEE Antennas and Wireless Propagation Letters, 2013.

- Shen C, Xiong H, Wang X,et al. A Fast Self-Jamming Cancellation Architecture and Algorithm for Passive RFID Sensor System[J].IEEE Communications Letters, 2021, PP(99):1-1.DOI:10.1109/LCOMM.2021.3066177.

- Ali Ghahremani, V. D. Rezaei and M. Sharif Bakhtiar, “A UHF-RFID Transceiver with a Blocker-Canceller Feedback and 30 dBm Output Power”, IEEE Transactions on circuits and systems, November 2013.

- R4-2304433, Analysis on Local area BS feasibility for duplex evolution, CATT.

# Appendix A: Evaluation assumptions for multiple single tone CW

The following Table provides the LLS assumption for multiple single tone CW.

Table 错误!未指定顺序。 The LLS assumption for multi-tone carrier wave

| Parameter | Value |
| --- | --- |
| CW type | multi-tone signal (unmodulated sinusoid in two frequencies) |
| data size | 96 bits |
| CRC | 16 bits |
| Modulation | OOK |
| Channel type | TDL-A NLOS |
| Delay spread | 30 ns |
| BLF (backscatter link frequency) | 40 kHz |
| CW Gap | [5MHz, 2.16MHz, 1.08MHz, 180kHz] |
| Coding method | Miller-8 |

# Appendix B: CW cancellation capability Calculation

Table 6 The model of receiver sensitivity loss and CW total cancellation capability

| system configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0A | Scenarios | D1T1-A2 InF-DH-NLOS | D1T1-B and D1T1-A1 InF-DH-NLOS | D2T2-A2 InH-LOS | D2T2-B and D2T2-A1 InH-LOS |
| 0A1 | CW case |  |  |  |  |
| 0B | Device type | Device type1 | Device type1 | Device type1 | Device type1 |
| 0C | Center frequency(GHz) | 0.9 | 0.9 | 0.9 | 0.9 |
| Transmitter |  |  |  |  |  |
| 1E | Total Tx power (dBm) |  |  |  |  |
| 1E'1 | CW Tx power(dBm) | 33.00 | 33.00 | 23.00 | 23.00 |
| 1E'2 | CW Tx antenna gain(dBi) | 2.00 | 2.00 | 0.00 | 0.00 |
| 1F | Transmission bandwidth used for the evaluated channel(Hz) | 160000 | 160000 | 160000 | 160000 |
| 1G | Tx antenna gain(dBi) | 0 | 0 | 0 | 0 |
| 1N (cable, connector loss) |  | 3 | 3 | 1 | 1 |
| Receiver |  |  |  |  |  |
| 2B | bandwidth used for the evaluated channel(Hz) | 160000 | 160000 | 160000 | 160000 |
| 2C | Receiver antenna gain(dBi) | 2 | 2 | 0 | 0 |
| 2D | Receiver Nosie Figure(dB) | 5 | 5 | 7 | 7 |
| 2E | Thermal Noise(dBm/Hz) | -174 | -174 | -174 | -174 |
| 2F | Noise Power(dBm) | -116.96 | -116.96 | -114.96 | -114.96 |
| 2H | FFS: Ambient IoT on-object antenna penalty | 0 | 0 | 0 | 0 |
| spatial isolation capability for outside topology, this item is antenna beam nulling |  | 30 | 20 | 15 | 10 |
| 2X cable, connector loss |  | 3.00 | 3.00 | 1 | 1 |
| Rx RF-IC suppression capability for outside topology, this item is pathloss suppression interference |  | 50 | 61 | 50 | 65 |
| Total interference cancellation capability | CW cancellation (dB) | 80 | 81 | 65 | 75 |
| 2K1' | Remaining CW interference(dB) | -49.00 | -50.00 | -44.00 | -54.00 |
| 2K3 | Rx IIP3 capability(dBm) | -16.00 | -16.00 | -16.00 | -16.00 |
| 2K4 | Rx IM3 contribution(dBm) = 2K1'-2*(2k3-2K1') | -115.00 | -118.00 | -100.00 | -130.00 |
| 2K5 | INR(interference to noise ratio) = 2K4-2F | 1.96 | -1.04 | 14.96 | -15.04 |
| 2K2 | Receiver sensitivity loss(dB) = 10*log10(1+2K5) | 4.10 | 2.52 | 15.10 | 0.13 |
| 2K1 | equivalent the item 2K4 | -115.00 | -118.00 | -100.00 | -130.00 |
| 2K | equivalent cw cancellation capability =1E'1+1E'2+2C-C22-C13-C28 | 146.00 | 149.00 | 121.00 | 151.00 |