3GPP TSG RAN WG1 #118		R1-2406232

Maastricht, NL, August 19th – 23rd, 2024

Source:	OPPO

Title:	Discussion on IoT-NTN uplink capacity/throughput enhancement

Agenda Item:	9.11.4

Document for:	Discussion and Decision

Introduction

IoT NTN is being deployed to support massive IoT devices, while the system capacity faces enormous challenge due to the large beam footprint size from hundreds to thousands of kilometers in NTN scenario. Regarding this matter, the WID proposes to study and specify, if beneficial, the application of orthogonal cover code (OCC) to NPUSCH format 1 an NPRACH to improve the uplink capacity [1].

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2] - Multi-tone support for 15 kHz SCS should also be considered Note: Impact of impairment shall be taken into account

In this contribution, we provide our view and analysis on the enhancements to support UE multiplexing via OCC for NPUSCH format 1. The OCC structures and potential spec impact of the candidate OCC schemes for NPUSCH format 1 are firstly discussed. Then, we discuss the candidate DMRS scheme for single-tone NPUSCH format 1 and present our preliminary evaluation results.

Uplink capacity enhancement for IoT-NTN

OCC schemes for NPUSCH format 1

Regarding multiplexing UEs via OCC for NPUSCH format 1, according to the WID, both single-tone and multi-tone NPUSCH transmission for 3.75kHz and 15kHz SCS should be taken into account. Considering OCC can be applied in time or frequency domain, RAN1 agreed to further study the following OCC schemes for single-tone and multi-tone NPUSCH format 1 in RAN1#116 meeting:

Agreement For single-tone NPUSCH format 1 transmissions with both 3.75kHz and 15kHz SCS, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level - Repetition-level - RV-level For multi-tone NPUSCH format 1 transmissions, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level (including Nslot level) - Repetition-levels - RV-level - Intra-symbol pre-DFT spreading OCC

For single-tone NPUSCH format 1, 1 RU consists of 16 NPUSCH slots. The long time span of an OCC sequence for repetition-level and RV-level OCC causes serious performance degradation, so RAN1 agreed to focus on slot-level and symbol-level OCC with shorter time span in the last meeting:

Agreement For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

For multi-tone NPUSCH format 1, 1 RU consists of 8/4/2 slots depending on the tone number. Also, 1 TB is mapped to NRU RUs, where NRU≥1. To avoid the time span of an OCC sequence too long, the similar conclusion can also be made for multi-tone NPUSCH transmission. That is, for multi-tone NPUSCH format 1, repetition-level OCC and symbol-level OCC are not pursued.

Proposal 1: For multi-tone NPUSCH format 1, repetition-level OCC and RV-level OCC are not pursued.

Then, we discuss the OCC spreading design on the following candidate OCC schemes:

- Slot-level OCC

According to the current spec, for NPUSCH repetition, partial repetitions (i.e., MidenticalNPUSCH of MrepNPUSCH) are firstly performed with slot-level repetition, i.e., across Nslots slots, and then the rest of the repetitions are performed across NPUSCH. The details in TS36.211 are specified as follows:

10.1.3.6 Mapping to physical resources Each NPUSCH codeword can be mapped to one or more than one resource units, NRU, as given by clause 16.5.1.2 of TS 36.213 [4], each of which shall be transmitted MrepNPUSCH times. The block of complex-valued symbols z(0),...,z(Msymbap−1) shall be multiplied with the amplitude scaling factor βNPUSCH in order to conform to the transmit power PNPUSCH specified in [4], and mapped in sequence starting with z(0) to subcarriers assigned for transmission of NPUSCH. The mapping to resource elements k,l corresponding to the subcarriers assigned for transmission and not used for transmission of reference signals, shall be in increasing order of first the index k, then the index l, starting with the first slot in the assigned resource unit. After mapping to Nslots slots, the Nslots slots shall be repeated MidenticalNPUSCH−1 additional times, before continuing the mapping of z(⋅) to the following slot, where MidenticalNPUSCH=minMrepNPUSCH/2,4NscRU>11NscRU=1 Nslots=1Δf=3.75 kHz2Δf=15 kHz

In our understanding, slot-level OCC can be supported based on the current slot-level repetition procedure with foreseeable spec impact. Take OCC length NSF=2 for example, NSF=2 UEs can be multiplexed on every NSF=2 slot with separate OCC sequence, and the slot-level OCC structure is depicted in Fig 1.

Fig 1. Slot-level OCC with OCC length NSF=2

To accommodate the above slot-level OCC structure, the formula of MidenticalNPUSCH and Nslots should be modified. For example, Nslots=1 for Δf=3.75 kHz and Δf=15 kHz, and MidenticalNPUSCH is determined based on OCC length NSF. Then, the slot-level OCC can be supported for single-tone and multi-tone NPUSCH transmission.

Proposal 2: Slot-level OCC can be supported based on the current slot-level repetition procedure with small spec impact.

- Symbol-level OCC

For symbol-level OCC, the  NSF adjacent OFDM symbols assigned for NPUSCH transmission form an OCC group. Then, OCC spreading is performed across the NSF OFDM symbols within an OCC group. Fig 2 illustrates the symbol-level OCC structure with OCC length NSF=2, where NSF=2 UEs can be multiplexed on every NSF=2 OFDM symbols with separate OCC sequence.

Fig 2. Symbol-level OCC with OCC length = 2

The symbol-level OCC may present better detection performance due to the shorter time span than slot-level OCC, but the spec impact is not negligible. Firstly, in the legacy physical resource mapping procedure, the complexed-valued symbols are continuously mapped to the OFDM symbols within a slot, which will be changed for symbol-level OCC scheme. In addition, according to the current slot structure of NPUSCH format 1, 1 slot only consists of 6 OFDM symbols for data transmission. In this case, if the OCC length NSF=4 is supported, the slot structure of NPUSCH format 1 needs redesign to accommodate the symbol-level OCC group. Therefore, when it comes to symbol-level OCC scheme, at least the spec impact on physical resource mapping procedure and slot structure should be taken into account.

Proposal 3: For symbol-level OCC, at least the spec impact on physical resource mapping procedure and slot structure should be taken into account.

- Intra-symbol pre-DFT spreading OCC

For multi-tone NPUSCH transmission, intra-symbol pre-DFT spreading OCC scheme can be additionally considered. For the application of OCC in frequency domain, an OCC group is composed of NSF OCC blocks of subcarriers, where an OCC block consists of NscRU subcarriers. Then, block-wise spreading is performed across the NSF OCC blocks, and NSF UEs can be multiplexed on the NSF OCC blocks within an OCC group. The intra-symbol pre-DFT spreading OCC structure with OCC length NSF=2 is illustrated in Fig 3. For 3-tones NPUSCH transmission, when OCC length NSF=2 or NSF=4 is configured, the actually allocated subcarriers will be 6 or 12, which is equivalent to 6-tones and 12-tones NPUSCH transmission. Apparently, the physical resource mapping is different from the legacy procedure.

Fig 3. Intra-symbol pre-DFT spreading OCC with OCC length = 2

Observation 1: For intra-symbol pre-DFT spreading OCC, the physical resource mapping is different from the legacy procedure.

Given that intra-symbol pre-DFT spreading OCC only applies to multi-tone NPUSCH transmission, RAN1 still need to consider other OCC schemes for single-tone NPUSCH transmission, which increases unnecessary workload. Moreover, OCC spreading in time domain, whether for slot-level OCC or symbol-level OCC, applies to both single-tone and multi-tone NPUSCH transmission. Thus, we prefer to prioritize the time domain OCC scheme to apply different types of NPUSCH transmission.

Proposal 4: For multi-tone NPUSCH format 1, RAN1 prioritizes time-domain OCC schemes, including slot-level OCC and symbol-level OCC, over frequency-domain OCC schemes.

DMRS for NPUSCH format 1 with OCC

The legacy DMRS on NPUSCH format 1 does not support UE multiplexing. Regarding this issue, RAN1 has discussed the following two DMRS schemes for further study in RAN1#116b meeting:

Agreement For single-tone DMRS when OCC is applied to NPUSCH format 1, RAN1 considers at least the following for further study: - TDM of DMRS. The time domain locations of DMRS for different UEs are different. No OCC is applied for the DMRS of different UEs. - FFS: Detailed mapping - CDM of DMRS. The time domain locations of DMRS for different UEs are the same. Different OCCs are applied for the DMRS of different UEs. - FFS: Detailed mapping - Other schemes are not precluded, including combinations of the above

According to the current DMRS pattern of single-tone NPUSCH format 1, only one DMRS symbol is transmitted within one NPUSCH slot. When NSF UEs are multiplexed via OCC for NPUSCH format 1, at least NSF DMRS symbols on NSF NPUSCH slots are needed to ensure the orthogonality of channel estimation between the OCC-multiplexed UEs, whether for TDMed DMRS or CDMed DMRS.

As depicted in Fig 4, when OCC length NSF=2, for CDM of DMRS, the same DMRS symbols but different OCC sequences are used for the NSF=2 multiplexed UEs, where the potential gain from DMRS combination may be obtained. For TDM of DMRS, the different DMRS symbols are used for the NSF=2 multiplexed UEs, where the orthogonality of channel estimation between the NSF=2 multiplexed UEs can be well guaranteed. In addition, the detailed mapping for TDM of DMRS needs further discussion. For example, for TDM of DMRS pattern 1, there is one DMRS symbol per NSF slots used for channel estimation, and for TDM of DMRS pattern 2, there is 2 DMRS symbols per 2∗NSF slots used for channel estimation. The DMRS overhead for TDM of DMRS pattern 1 and pattern 2 are the same, but the specific DMRS mapping is different.

Fig 4. Illustration of CDM DMRS and TDM DMRS with OCC length = 2

Proposal 5: RAN1 further discuss the detailed mapping for TDM of DMRS, e.g., TDM of DMRS pattern 1 (one DMRS symbol per NSF slots), and TDM of DMRS pattern 2 (2 DMRS symbols per 2∗NSF slots).

Performance evaluation of candidate OCC schemes

The preliminary evaluation results of candidate OCC schemes (including slot-level OCC and symbol-level OCC) with CDM of DMRS are provided in Fig 5-6, and the detailed simulation parameters are listed in Table 1 in the Appendix. Note that the total REs used for TB size determination and NPUSCH transmission for different OCC schemes are the same as the NPUSCH transmission without OCC.

When 10% BLER is taken as the performance metric, Fig 5 shows that, for single-tone NPUSCH transmission, slot-level OCC-2 and symbol-level OCC-2 presents 0.3dB and 0.2dB performance degradation on the required SNR respectively. However, from the NPUSCH capacity perspective, the aggregated throughput increases linearly with the OCC-multiplexed UE number. For multi-tone NPUSCH transmission, slot-level OCC-2 and symbol-level OCC-2 presents 0.4dB and 0.3dB performance degradation on the required SNR respectively, and the NPUSCH capacity increases linearly with the OCC-multiplexed UE number. In addition, we note that the single-tone NPUSCH shows worse BLER performance than multi-tone NPUSCH due to less DMRS symbols in frequency domain.

Observation 2: For single-tone NPUSCH transmission with CDM of DMRS:

- Slot-level OCC-2 and symbol-level OCC-2 presents 0.3dB and 0.2dB performance degradation on the required SNR respectively.

- The aggregated throughput increases linearly with the OCC-multiplexed UE number.

Observation 3: For multi-tone NPUSCH transmission with CDM of DMRS:

- Slot-level OCC-2 and symbol-level OCC-2 presents 0.4dB and 0.3dB performance degradation on the required SNR respectively.

- The aggregated throughput increases linearly with the OCC-multiplexed UE number.

Fig 5. Performance evaluation of non-OCC, slot-level OCC, and symbol-level OCC for single-tone NPUSCH

Fig 6. Performance evaluation of non-OCC, slot-level OCC, and symbol-level OCC for multi-tone NPUSCH

Conclusion

In this contribution, we present our view and analysis on enhancements to support UE multiplexing with OCC for NPUSCH format 1 and DMRS. The following observations and proposals are made:

Proposal 1: For multi-tone NPUSCH format 1, repetition-level OCC and RV-level OCC are not pursued.

Proposal 2: Slot-level OCC can be supported based on the current slot-level repetition procedure with small spec impact.

Proposal 3: For symbol-level OCC, at least the spec impact on physical resource mapping procedure and slot structure should be taken into account.

Proposal 4: For multi-tone NPUSCH format 1, RAN1 prioritizes time-domain OCC schemes, including slot-level OCC and symbol-level OCC, over frequency-domain OCC schemes.

Proposal 5: RAN1 further discuss the detailed mapping for TDM of DMRS, e.g., TDM of DMRS pattern 1 (one DMRS symbol per NSF slots), and TDM of DMRS pattern 2 (2 DMRS symbols per 2∗NSF slots).

Observation 1: For intra-symbol pre-DFT spreading OCC, the physical resource mapping is different from the legacy procedure.

Observation 2: For single-tone NPUSCH transmission with CDM of DMRS:

- Slot-level OCC-2 and symbol-level OCC-2 presents 0.3dB and 0.2dB performance degradation on the required SNR respectively.

- The aggregated throughput increases linearly with the OCC-multiplexed UE number.

Observation 3: For multi-tone NPUSCH transmission with CDM of DMRS:

- Slot-level OCC-2 and symbol-level OCC-2 presents 0.4dB and 0.3dB performance degradation on the required SNR respectively.

- The aggregated throughput increases linearly with the OCC-multiplexed UE number.

Reference

[1]     RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc. (Rapporteur), RAN#104, June 17-20, 2024

[2]       R1-2405494, “FL Summary #2 for IoT-NTN”, Sony, RAN1#117 meeting

Appendix

Simulation assumptions

Table 1. LLS parameters for NPUSCH format 1

|  | Parameter | value |  |
| --- | --- | --- | --- |
| scenario | orbit | GEO | LEO600 |
|  | Elevation angle | 12.5 degree | 30degree |
| Channel and impairments | carrier frequency | 2GHz |  |
|  | Channel model | NTN-TDL-C The channels from different UE are independent. |  |
|  | Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. |  |
|  | Timing error | Uniform random selection from [-29Ts, +29Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO. |  |
| transmitter | SCS | 15KHz | 15kHz |
|  | Number of tones | Single tone | 12 tones |
|  | Waveform | DFT-s-OFDM |  |
|  | Frequency hopping | w/o frequency hopping |  |
|  | MIMO scheme | SISO |  |
|  | DMRS configuration | For baseline evaluations: OS#3 per slot for 15kHz For OCC evaluations: CDMed DMRS | For baseline evaluations: OS#3 per slot for 15kHz For OCC evaluations: CDMed DMRS |
|  | Number of resource unit (NRU) | 1 | 1 |
|  | Modulation order (Qm) | QPSK | QPSK |
|  | TBS (ITBS) | ITBS = 4 (56bits) | ITBS = 6 (88bits) |
|  | Number of repetitions (Nrep) | 8 |  |
|  | OCC length | 2, 4 |  |
|  | OCC sequence | OCC-2: [+1 +1] [+1 -1] OCC-4: [+1 +1 +1 +1] [+1 -j -1 +j] [+1 -1 +1 -1] [+1 +j -1 -j] |  |
|  | Number of UE | 2, 4 |  |
|  | Velocity of UE | 3km/h |  |
| receiver | Receiver algorithm | MMSE |  |
|  | Channel estimation | Real channel estimation |  |

Agreements in RAN1#117 meeting

Agreement

For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

Agreement

Inter-repetition OCC for NPRACH is not studied further in RAN1.

Agreement

- For the time-domain DMRS pattern (including blanked DMRS, if any):

  - For 15kHz single-tone, RAN1 strives to reuse the Rel-17 DMRS pattern

  - For 3.75kHz single-tone

    - RAN1 studies

      - Rel-17 DMRS pattern

      - A new DMRS pattern

  - The DMRS overhead (including blanked DMRS, if any) for OCC is the same as for Rel-17

Agreement

The Rel-17 guard period locations and length for NB-IoT 3.75kHz UL slot are preserved when OCC is applied to NPUSCH format 1.