3GPP TSG RAN WG1 #118		                                                      R1-2406331

Maastricht, NL, August 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on higher layer parameters for DCI format 2_3

Agenda Item:	7

Document for:	Discussion and Decision

- Introduction

This contribution discusses the higher layer parameters for DCI format 2_3.

- Discussion

DCI format 2_3 is used for the transmission of a group of TPC commands for SRS transmissions by one or more UEs.

As specified in TS 38.212 and TS 38.213, DCI format 2_3 is applicable for UL carrier(s) of serving cells where a UE is not configured with PUSCH/PUCCH transmission or for UL carrier(s) of a serving cell where SRS power control is not tied with PUSCH power control (i.e., srs-PowerControlAdjustmentStates indicates a separate power control adjustment state between SRS transmissions and PUSCH transmissions). For DCI format 2_3, RRC IE SRS-CarrierSwitching is essential as the higher layer parameter srs-TPC-PDCCH-Group in SRS-CarrierSwitching is used for indicating whether the type of DCI format 2_3 is typeA or typeB. If srs-TPC-PDCCH-Group = typeA is configured, a DCI format 2_3 field includes a TPC command for each serving cell from the set of serving cells and can also include a SRS request for SRS transmission on the set of serving cells. If srs-TPC-PDCCH-Group = typeB is configured, DCI format 2_3 field includes a TPC command for a serving cell index and can also include a SRS request for SRS transmission on the serving cell.

In current TS 38.331, the general description of IE SRS-CarrierSwitching and the descriptions on the fields in the IE are as follows:

#### – SRS-CarrierSwitching The IE SRS-CarrierSwitching is used to configure for SRS carrier switching when PUSCH is not configured and independent SRS power control from that of PUSCH. SRS-CarrierSwitching information element -- ASN1START -- TAG-SRS-CARRIERSWITCHING-START SRS-CarrierSwitching ::= SEQUENCE { srs-SwitchFromServCellIndex INTEGER (0..31) OPTIONAL, -- Need M srs-SwitchFromCarrier ENUMERATED {sUL, nUL}, srs-TPC-PDCCH-Group CHOICE { typeA SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config, typeB SRS-TPC-PDCCH-Config } OPTIONAL, -- Need M monitoringCells SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL, -- Need M ... } SRS-TPC-PDCCH-Config ::= SEQUENCE { srs-CC-SetIndexlist SEQUENCE (SIZE(1..4)) OF SRS-CC-SetIndex OPTIONAL -- Need M } SRS-CC-SetIndex ::= SEQUENCE { cc-SetIndex INTEGER (0..3) OPTIONAL, -- Need M cc-IndexInOneCC-Set INTEGER (0..7) OPTIONAL -- Need M } -- TAG-SRS-CARRIERSWITCHING-STOP -- ASN1STOP SRS-CC-SetIndex field descriptions. cc-IndexInOneCC-Set Indicates the CC index in one CC set for Type A (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network always includes this field when the srs-TPC-PDCCH-Group is set to typeA.. cc-SetIndex Indicates the CC set index for Type A associated (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network always includes this field when the srs-TPC-PDCCH-Group is set to typeA. The network does not configure this field to 3 in this release of specification.. SRS-CarrierSwitching field descriptions. monitoringCells A set of serving cells for monitoring PDCCH conveying SRS DCI format with CRC scrambled by TPC-SRS-RNTI (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.3).. srs-SwitchFromServCellIndex Indicates the serving cell whose UL transmission may be interrupted during SRS transmission on a PUSCH-less SCell. During SRS transmission on a PUSCH-less SCell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH in the same CG to allow the PUSCH-less SCell to transmit SRS. (see TS 38.214 [19], clause 6.2.1.3).. srs-TPC-PDCCH-Group Network configures the UE with either typeA-SRS-TPC-PDCCH-Group or typeB-SRS-TPC-PDCCH-Group, if any.. typeA Type A trigger configuration for SRS transmission on a PUSCH-less SCell (see TS 38.213 [13], clause 11.4). In this release, the network configures at most one entry (the first entry) of typeA, and the first entry corresponds to the serving cell in which the SRS-CarrierSwitching field is configured. SRS carrier switching to SUL carrier is not supported in this version of the specification.. typeB Type B trigger configuration for SRS transmission on a PUSCH-less SCell (see TS 38.213 [13], clause 11.4).. SRS-TPC-PDCCH-Config field descriptions. srs-CC-SetIndexlist A list of pairs of [cc-SetIndex; cc-IndexInOneCC-Set] (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network does not configure this field for typeB..

It can be seen that according to current TS 38.331, The IE SRS-CarrierSwitching is configured only when PUSCH is not configured and the SRS power control is not tied with PUSCH power control, and typeA and typeB are only used to trigger configuration for SRS transmission on a PUSCH-less SCell. These restrictions cause DCI format 2_3 cannot be used in the UL carrier(s) where PUSCH is configured and the SRS power control is not tied with PUSCH power control. In order to allow DCI format 2_3 to be used in the UL carrier(s) where PUSCH is configured and SRS power control is not tied with PUSCH power control, modification on the descriptions for SRS-CarrierSwitching, typeA and type B in TS 38.331 is needed. As DCI format 2_3 was introduced in Rel-15, the modification can be adopted by the specifications from Rel-15.

Observation 1: To allowing DCI format 2_3 to be used for the transmission of a group of TPC commands for SRS transmissions in the UL carrier(s) in a serving cell where PUSCH is configured and the SRS power control is not tied with PUSCH power control, modifications on the descriptions for SRS-CarrierSwitching, typeA and type B in TS 38.331 are needed.

Proposal 1: Send a LS to RAN2 to ask for adding the missing case “for SRS transmission on a serving cell with independent SRS power control from that of PUSCH” into the field description of “typeA/typeB” and clarifying that the IE SRS-CarrierSwitching is used to configure for SRS carrier switching when PUSCH is not configured or when independent SRS power control from that of PUSCH is applied.

- Conclusions

In this contribution, we provided our views on higher layer parameters for DCI format 2_3, with the following observation and proposal provided:

Observation 1: To allowing DCI format 2_3 to be used for the transmission of a group of TPC commands for SRS transmissions in the UL carrier(s) in a serving cell where PUSCH is configured and the SRS power control is not tied with PUSCH power control, modifications on the descriptions for SRS-CarrierSwitching, typeA and type B in TS 38.331 are needed.

Proposal 1: Send a LS to RAN2 to ask for adding the missing case “for SRS transmission on a serving cell with independent SRS power control from that of PUSCH” into the field description of “typeA/typeB” and clarifying that the IE SRS-CarrierSwitching is used to configure for SRS carrier switching when PUSCH is not configured or when independent SRS power control from that of PUSCH is applied.