3GPP TSG-RAN WG1 Meeting #118                                     R1-2406398

Maastricht, The Netherlands, 19th – 23rd August, 2024

Source: 	ZTE Corporation, Sanechips

Agenda item:	5

Title:	Discussion on requirements for Ambient IoT

Document for:	Discussion/Decision

- Introduction

The question to RAN1 extracted from SA2 LS regarding the requirements for Ambient IoT is as below.

SA2 have discussed different aspects with respect to non-volatile memory: - whether or not an Ambient IoT device can incorporate a non-volatile memory in the device design i.e., include a non-volatile memory in the Bill-of-Materials (BoM). - whether an Ambient IoT device will be able to update its non-volatile memory based on its energy status and energy storage capabilities at some point after receiving a trigger from the Reader. SA2 has not reached any common understanding. While there is no consensus in SA2 whether it is in RAN1 scope to determine devices' level of capability to support non-volatile memory; SA2 would like to ask the following question to RAN1: Question 5: SA2 would like to ask RAN1 if RAN1 can provide feedback on the non-volatile memory aspects listed above.

In this contribution, we provide our views on Ambient IoT device’s memory.

- Discussion

The device architecture has been discussed and agreed in RAN1#116 and 116bis meeting. According to the agreed architectures in Appendix, it can be seen that all of three device types include non-volatile memory.

- Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

Observation 1: Ambient IoT device can incorporate a non-volatile memory in the Bill-of-Materials.

Non-Volatile Memory (NVM), which is essential for storing identification data on tags, is one of the contributor to the cost of Ambient IoT. One feasible solution is the integration of One-Time Programmable (OTP) memory. However, due to the need for multi-write capability in numerous applications, the development of cost-effective and energy-efficient NVM has garnered considerable attention. The current state-of-the-art technology allows for the implementation of a 256-bit re-writeable NVM array using a single-poly C-Flash bitcell, which boasts a low static power consumption of 3.8μW [4]. Additionally, an ultra-low power 192-bit memory with a register array, fabricated using a 0.18μm standard CMOS process, operates with a supply voltage of 1.2 volts [5]. The memory's current draw is 1.8μA during read operations and 3.6μA during write operations, with respective data rates of 1.3Mb/s and 0.8Kb/s[5].

Therefore, the Ambient IoT device is able to update its non-volatile memory per requirements.

Observation 2: Ambient IoT device is able to update its non-volatile memory per requirements.

- Conclusion

Based on the discussion above, we have the following observation.

Observation 1: Ambient IoT device can incorporate a non-volatile memory in the Bill-of-Materials.

Observation 2: Ambient IoT device is able to update its non-volatile memory per requirements.

- References

- R1-2405796. LS on Clarification of requirements for Ambient IoT.SA2.

- RAN1#116 meeting, chairman’s notes

- RAN1#116bis meeting, chairman’s notes

- Dagan H, Teman A, Fish A, et al. A low-cost low-power non-volatile memory for RFID applications. 2012 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2012: 1827-1830.

- Feng P, Li Y, Wu N. An ultra low power non-volatile memory in standard CMOS process for passive RFID tags. 2009 IEEE Custom Integrated Circuits Conference. IEEE, 2009: 713-716.

- Appendix

RAN1#116:

Agreement Study at least the following blocks for device 1 architecture. - Antenna could be either shared or separate for RF energy harvester and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester and receiver related blocks). - RF energy harvester can include rectifier performing RF signal (AC) to DC conversion. - Energy storage (e.g., capacitor) stores harvested energy from RF energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Reception related blocks - RF BPF for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - RF Envelope Detector converts RF signal to baseband. - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator. - Depending on implementation, it may not exist. Presence of BB LPF is assumed for the study. - Comparator determines high/low of input signal. - Transmission related blocks - Backscatter modulator switches impedance to modulate backscattered signal with tx signal from BB logics. Waveform/Modulation type is FFS.

Agreement Study at least following blocks for device 2a architecture w/ RF-ED receiver. - Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks). - Energy harvester. - Energy storage (e.g., capacitor) stores harvested energy from energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Reflection amplifier can amplify reflected backscattered signal. - FFS study applicability of amplification of rx signal, power consumption. - At least one of R2D/CW2D and D2R could be amplified by either reflection amplifier or LNA. - Reception related blocks - RF BPF filter for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - FFS: LNA for improving signal strength and sensitivity of receiver. - At least one of R2D/CW2D and D2R could be amplified by either reflection amplifier or LNA. - RF envelope detector (RF-ED) detects envelope from RF signal. - BB amplifier amplifies BB signal to improve signal strength. - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator/ADC. - Depending on implementation, it may not exist. - Comparator or N-bit ADC - Transmission related blocks - Backscatter modulator switches impedance to modulate backscattered signal with tx signal from BB logics. - FFS: Large Frequency shifter (e.g., tens of MHz) for shifting backscattered signal from one frequency (e.g., FDD-DL frequency) to another frequency (e.g., FDD-UL frequency).

RAN1#116bis:

Agreement Study device 2b architecture w/ RF-ED receiver with following blocks. - Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks). - Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc - Energy storage (e.g., capacitor) stores harvested energy from energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Reception related blocks - RF BPF filter for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - FFS: LNA for improving signal strength and sensitivity of receiver, if present - RF envelope detector (RF-ED) detects envelope from RF signal. - BB amplifier amplifies BB signal to improve signal strength. - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator/ADC. - Depending on implementation, it may not exist. - Comparator or N-bit ADC - Transmission related blocks - Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic. - Digital to Analog Converter (DAC) converts digital signal to analog signal. - Low pass filter for filtering out undesired signal - Mixer performs up converting baseband signal to RF range. - Local oscillator (LO) for carrier frequency generation - FFS: PLL/FLL - FFS: Power amplifier (PA) amplifies tx signal, if present - Details on transmitter related blocks depends on tx waveform/modulation. Agreement Study device 2b architecture with ZIF receiver with following blocks. - Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks). - Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc - Energy storage (e.g., capacitor) stores harvested energy from energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, detector, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Local oscillator (LO) for generating carrier frequency for Tx and Rx - FFS: PLL/FLL - FFS: one LO or separate LOs for Tx and Rx - Reception related blocks - RF BPF filter for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - FFS: LNA for improving signal strength and sensitivity of receiver. - Mixer down converts RF signal to BB stage. - Depending on implementation, there could be one or two mixers for Rx and Tx - BB amplifier amplifies BB signal - BB LPF can filter out undesired frequency components to improve input signal quality to comparator/ADC. - Depending on implementation, it may not exist. - Comparator or N-bit ADC - Transmission related blocks - Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic. - Digital to Analog Converter (DAC) converts digital signal to analog signal. - Low pass filter for filtering out undesired signal - Mixer performs up converting baseband signal to RF range. - FFS: Power amplifier (PA) amplifies tx signal, if present - Details on transmitter related blocks depends on e.g., waveform/modulation, etc Agreement Study device 2b architecture with IF-ED receiver with following blocks. - Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks). - Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc - Energy storage (e.g., capacitor) stores harvested energy from energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Local oscillator (LO) for generating carrier frequency for Tx, or for generating carrier frequency offset by the IF for Rx - FFS: PLL/FLL - FFS: one LO or separate LOs for Tx and Rx - Reception related blocks - RF BPF filter for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - FFS: LNA for improving signal strength and sensitivity of receiver, if present - Mixer down converts RF signal to IF stage. - Depending on implementation, there could be one or two mixers for Rx and Tx - IF amplifier amplifies IF signal - IF filter for filtering out unwanted RF and LO signals - IF envelope detector (IF-ED) detects envelope from IF signal. - BB amplifier - Depending on implementation, one or both of IF amplifier and BB amplifier may exist. - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator/ADC. - Depending on implementation, it may not exist. - Comparator or N-bit ADC - Note: image rejection is required - Transmission related blocks - Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic. - Digital to Analog Converter (DAC) converts digital signal to analog signal. - Low pass filter for filtering out undesired signal - Mixer performs up converting baseband signal to RF range. - FFS: Power amplifier (PA) amplifies tx signal, if present - Details on transmitter related blocks depends on e.g., waveform/modulation, etc