3GPP TSG-RAN WG1 Meeting #118	R1- 2407167

Maastricht, Netherlands, August 19th – August 24th, 2024

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on LS on synchronization source change at the transmitting UE in SL positioning

Document for:	Discussion

# Introduction

In the RAN4 LS reply “LS on synchronization source change at the transmitting anchor UE in SL positioning”[1] RAN4 asks the following to RAN1:

LS from R1-2405788 - Overall Description: During the discussion on RRM core requirements for SL positioning, RAN4 reached the following agreement. - Agreement - It is a RAN4 understanding that UE performing measurements may not be aware on the synchronization source change at the Tx UE. - For synchronization reference source change occurring at Tx side, measurement accuracy requirements do not apply and no specific UE behaviour is defined. - Note: the agreement can be revisited if a RAN1/2 solution is introduced to inform the UE performing measurements on the synchronization source change at the Tx UE. RAN4 would like to check whether RAN1 and RAN2 have introduced or are working on any solutions to inform a UE performing an SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement. 2. Actions: To RAN1, RAN2: ACTION: RAN4 kindly requests RAN1 and RAN2 to clarify whether they have introduced or are working on any solutions to inform an SL UE performing an SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement.

# discussion

## Change of sync source for SL PRS transmitting UE

The current specification only provide sync source from a SL PRS transmitting UE as part of the assistance data attached to the SL PRS. There is no way to update the sync source without updating the assistance data for SL PRS as a whole.  Additionally, sync source information is optional. The case of sync source change is just limited to the case where sync source is provided in assistance data.  By default, the UE should assume that the assistance data is valid unless it is notified of other wise, hence without a mechanism, the measuring UE should be able to assume that the AD provided from the SL PRS transmitting UE is consistent, i.e. no sync source change has happened.

Confirm RAN4 understanding that to not impact a timing measurement, the transmitting UE's sync source should not change while another UE is performing a measurement based on its transmission, otherwise the measurement needs to be restarted.

The case where the SL PRS Tx UE changes sync source can be handled either by specific time stamped messages between the tx and measuring UE informing of the new source and when it is applicable. However, at this stage in the specification, we see such solution as an optimization. A change in sync source can instead be handled by an update of the assistance data.

A UE changing its sync source for SL PRS transmission can update the measuring UE by sending an updated assistance data.

RAN1 should not plan further enhancements for notification of a change in sync ref source from a tx UE to another measuring UE.

# References

R1-2405788 LS on synchronization source change at the transmitting anchor UE in SL positioning