3GPP TSG RAN WG1 #118																							R1-2407228

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	9.3.2

Source: 	Moderator (CMCC)

Title: 	Summary#1 on SBFD random access operation

Document for:	Discussion/decision

# Introduction

The New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved in RAN plenary #102 meeting [1], and the updated WID was approved in RAN plenary#104 meeting [2].

In this contribution, we summarized the related issues and proposals based on the contributions submitted in RAN1#118 under the agenda item 9.3.2 [3]–[39].

The following sections are structured as follows. We categorize the key issues raised by contributions into three sections and some sections may cover more than one sub-issue. For each issue/sub-issue, the related submitted proposals, the summary and initial proposals/questions suggested by moderator are provided in sub-sections. For each identified proposal/question, one table is provided.

# Issue#1: Common issues for IDLE/INACTIVE/CONNECTED mode

## Issue#1-1: General RACH configuration framework

    - Submitted proposal

#### Unified design for RRC_CONNECTED and RRC_IDLE/INACTIVE

| Company | Proposals |
| --- | --- |
| Spreadtrum | - Extent all the previous agreements of RACH procedure specified for RRC CONNECTED to RRC IDLE state. |
| Tejas | Proposal 7: SSB -RO mapping rule applied to SBFD UEs in RRC_CONNECTED shall be applicable to SBFD UEs in RRC_IDLE/ RRC_INACTIVE state as well. Proposal 8: SBFD RACH related configurations shall be sent in SIB to perform RACH operation for SBFD capable UEs in RRC_IDLE/ RRC_INACTIVE state. |
| CMCC | Proposal 1: Extend the following agreements for UEs in RRC_CONNECTED mode to UEs in RRC_IDLE/INACTIVE mode. |
| LG Electronics | Proposal 3: RAN1 to support the RACH configuration relevant signal via SIB1 to enable PRACH transmission in SBFD symbols for both a SBFD aware UE in RRC_IDLE/INACTIVE mode and a SBFD aware UE in RRC_CONNECTED mode. Proposal 4: The agreement further make sure it is applicable to both RRC CONNECTED mode and RRC_INACTIVE/IDLE mode. |
| TCL | Proposal 9: For RACH configuration to SBFD-aware UEs in RRC idle/inactive state, consider the following options: - Option 1: use one single RACH configuration, and only based on the existing parameters of the single RACH configuration. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration. |
| Ericsson | Proposal 1 As a baseline, the same agreements as has been agreed for RRC_CONNECTED that are adopted for RRC_IDLE/INACTIVE. Individual agreements can be further discussed upon identified needs by companies. |
| vivo | Proposal 18: Strive for unified solution for RA in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode. |
| Kookmin University | Proposal 9: For UEs in IDLE/INACTIVE mode, both RACH configuration Option 1 with Alt 1-1 and Option 2 are supported by SIB. Proposal 10: For UEs in IDLE/INACTIVE mode, the validity of the ROs (i.e., whether the ROs across SBFD and non-SBFD symbols in the same/different slot(s) is invalid or not) is configured by SIB. |
| CATT | Proposal 23: Common design can be used for random access in SBFD symbols by UEs in both in RRC_CONNECTED mode and in RRC_IDLE/INACTIVE mode. |
| Samsung | Observation 3: Existing RAN1 agreements for support of RACH configuration options 1 and 2 and associated RO validation, SSB-RO mapping rules, etc. by SBFD-aware UEs in RRC_CONNECTED mode need to be re-confirmed for RRC_IDLE/INACTIVE mode. |
| Nokia, Nokia Shanghai Bell | Proposal 1. RAN1 to strive for a unified solution for RRC_IDLE and RRC_CONNECTED modes, which would help reducing redundant discussions for future meetings and having less specification impacts. |
| ETRI | Proposal 1: Support a unified design between connected mode RA and non-connected mode RA for SBFD UEs. Proposal 3: Additional feature combination preamble set can be introduced for ROs at least on non-SD symbols. |
| NTT DOCOMO | Proposal 1: For random access in SBFD symbols, unified design for UEs in RRC connected mode and RRC idle/in-active mode is pursued. - Extend the previous agreements for UE in RRC connected mode to UE in RRC idle/in-active mode. |
| Qualcomm | Proposal 20: The design of SBFD random access operation for RRC Idle/Inactive should leverage the same RACH design of for RRC-Connected UE with the following consideration: - SIB indication of the time/freq. locations of the SBFD - For PRACH configuration option 2, SIB indication of the additional PRACH configuration for SBDD random access, - For PRACH configuration option 1, SIB indication whether the configured ROs in SBFD-symbols are valid or not. |

#### Terminology (legacy-ROs and additional-ROs)

| Company | Proposals |
| --- | --- |
| Spreadtrum | - For RAN1 discussion purpose, the following terminologies are used. - Valid legacy-ROs: The valid ROs that are valid for non-SBFD aware UEs. - Valid additional-ROs: The valid ROs that are not valid legacy-ROs. |
| CMCC | Proposal 4: For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the configured ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the configured ROs across SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the configured ROs configured by the additional RACH configuration. |
| Sharp | Proposal 3: Clarify that ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon are recognized as legacy ROs. |

#### Enabling of two RACH configuration options

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 15: Support to allow gNB to enable/disable random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode. |
| ZTE, Sanechips | Proposal 1: Explicit indication of the options enabled by the network should be supported. Proposal 2: The UE behavior on whether or how to initiate SBFD RACH procedure should be defined in the following cases, - Case 1: The option enabled by the network is not supported by a UE. - Case 2: The UE supports two options, and both options are enabled by the network. Proposal 10: The priority relationship between different RACH configurations should be defined, and RO from RACH configuration with a lower priority is determined as invalid RO when collision occurs among ROs from different RACH configurations. − Legacy RACH configuration can be defined with a higher priority than the additional RACH configuration. |
| Tejas | Proposal 2: If only Option 1 with no additional parameters does not create any new RO in SBFD symbols, then UE supporting Option 1 needs to support Option 2 as well. |
| Ericsson | Proposal 12 Network enabling of SBFD RACH is indicated in RRC for the single RACH configuration. |
| NEC | Proposal 3: - Consider the following two approaches by which a UE determines whether ROs configured by legacy RACH configuration are valid during SBFD symbols. - Option-1: Define a parameter in PRACH configuration for Alt-1 which indicates to the UE whether ROs in SBFD symbols are considered valid. - Option-2: UE always assumes that ROs configured by legacy RACH configuration are valid during SBFD symbols if UL subband configuration is provided to the UE and UE is not configured with additional RACH configuration for SBFD symbols based on Alt-2. |
| NTT DOCOMO | Proposal 2: For RACH configuration option 1, enabling random access operation in SBFD symbols is indicated by gNB based on the legacy RACH configuration. |
| Qualcomm | Proposal 20: The design of SBFD random access operation for RRC Idle/Inactive should leverage the same RACH design of for RRC-Connected UE with the following consideration: - SIB indication of the time/freq. locations of the SBFD - For PRACH configuration option 2, SIB indication of the additional PRACH configuration for SBDD random access, - For PRACH configuration option 1, SIB indication whether the configured ROs in SBFD-symbols are valid or not. |

### Summary

#### Unified design for RRC_CONNECTED and RRC_IDLE/INACTIVE

Many companies propose to adopt a unified design for UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode and extend the agreements made in previous meetings to RRC_IDLE/INACTIVE mode, moderator suggests the Initial proposal 1-1-1.

#### Terminology (legacy-ROs and additional-ROs)

The terminologies of legacy-ROs and additional-ROs were discussed in RAN1#116bis and RAN1#117, which have been used in previous agreements as below.

Agreement (RAN1#116-bis) For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives: - Alt 2-3: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. - FFS: The case where the additional-ROs partially overlap with non-SBFD symbols - Alt 2-4: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs. For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs. Agreement (RAN1#116-bis) For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), - For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed. - For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used

In this meeting, [Spreadtrum, CMCC, Sharp] propose to further clarify these terms. Many companies use similar terminologies (e.g., ‘legacy-ROs’, ‘additional-ROs’) for both RACH configuration Option 1 and 2. It is straightforward to understand ‘legacy-ROs’ for both options and it is also straightforward to understand ‘additional-ROs’ for RACH configuration Option 2, but companies may have different understandings when they use ‘additional-ROs’ for RACH configuration Option 1.  Therefore, to facilitate RAN1 discussion, it is better to align companies’ understandings on the terminology ‘additional-ROs’ for both RACH configuration Option 1 and 2. Moderator suggests Initial proposal 1-1-2

#### Enabling of two RACH configuration options

In RAN1#117 meeting, we confirmed the following working assumption about supporting both RACH configuration option 1 with Alt 1-1 and RACH configuration option 2, and also agreed that ‘Enabling both options at the same time for a UE is not supported’.

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

In this meeting, some companies discussed how to indicate SBFD aware-UEs which RACH configuration option is enabled and whether both two options can be enabled from network side. The enabling methods are summarized as bellow:

- Enable/disable RACH configuration Option 1: [Huawei, HiSilicon, ZTE, Sanechips, Ericsson, NEC, NTT DOCOMO] propose to introduce an explicit indication for this purpose

- Enable/disable RACH configuration Option 2: This can be served by the existence of the additional RACH configuration

Moderator suggests Initial proposal 1-1-3 and initial proposal 1-1-4.

### 1st Round Proposals

#### Initial proposal 1-1-1:

Extend the following previous agreements for UEs in RRC_CONNECTED mode to UEs in RRC_IDLE/INACTIVE mode.

Agreement@RAN1#116 For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH) Agreement@RAN1#116 For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols. - FFS PRACH repetition in SBFD symbols. - FFS PRACH repetition across SBFD symbols and non-SBFDs symbols. Agreement@RAN1#116 For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects: - Msg2[/Msg4 PDSCH] reception in DL subband(s) - Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping - Msg3 repetition - Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control - FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE Note: Strive to make progress in accordance to the discussion in AI 9.3.1. Agreement@RAN1#117 Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options. Agreement@RAN1#116bis For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, consider the following alternatives to derive the time and frequency resources of the configured ROs in SBFD symbols. - Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon). - FFS the details - FFS: Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s). Agreement@RAN1#116bis For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, no separate prach-ConfigurationIndex to be configured in this option. Agreement@RAN1#116bis For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, use existing random access configurations tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2 in TS38.211). Agreement@RAN1#116bis For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), - For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed. - For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used Agreement@RAN1#116bis For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, - no enhancements for the RO validation rule for the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any). - FFS: the ROs in non-SBFD symbols that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - FFS: It’s up to network configuration to ensure the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, which are valid for non-SBFD aware UEs based on legacy RO validation rule, are also valid for SBFD aware UEs (i.e., the configured ROs in SBFD symbols, if configured as flexible by tdd-UL-DL-ConfigurationCommon, are within the UL usable PRBs) - the RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon is valid if at least: - Time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB - FFS: Other condition. Note: For the case that all the SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, there is no restriction that all the configured ROs in SBFD symbols should be within the UL usable PRBs. Agreement@RAN1#117 For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, consider whether additional condition(s) on top of what was agreed in RAN1#116bis for RO validation are necessary. For example, - Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB. - Condition#3: A valid RO does not precede a SSB in the PRACH slot. Agreement@RAN1#116bis For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives: - Alt 2-3: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. - FFS: The case where the additional-ROs partially overlap with non-SBFD symbols - Alt 2-4: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs. For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs. Agreement@RAN1#117 Update the following agreement made in RAN1#116-bis meeting: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, adopt the following: - Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - For FR1, consider from the following alternatives: - Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. - Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211)

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | Support |
| Spreadtrum | support |
| ZTE | Support. Alternatively, we can first focus on what additional enhancements are needed to idle/inactive mode. |
| New H3C | OK |
| China Telecom | Support |
| Tejas | Support the previous agreements |
| Xiaomi | Support. |
| InterDigital | We support the proposal for the purpose of unified design for RRC_CONNECTED and RRC_IDLE/INACTIVE. |
| Samsung | Support |
| Sharp | We prefer removing the square brackets in the following agreement. It is clear that msg4 PDSCH and msg4 HARQ-ACK PUCCH are in the scope of the study for RRC_IDLE mode. Agreement@RAN1#116 For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects: - Msg2[/Msg4 PDSCH] reception in DL subband(s) - Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping - Msg3 repetition - Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control - FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE Note: Strive to make progress in accordance to the discussion in AI 9.3.1. |
| LGE | Support |
| QC | Support. In the earlier agreements, we can either - delete ‘in RRC CONNECTED state’, or - generalize by adding RRC idle/inactive state: ‘in RRC CONNECTED state and RRC Idle/Inactive state’ |
| WILUS | Support |
| DCOCOMO | Support. |
| Ericsson | Support |
| ETRI | Support |
| Sony | Support |
| Nokia | Support. |
| NEC | Support |
| OPPO | Support |
| ITRI | Support |

#### Initial proposal 1-1-2:

For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following:

- For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon.

- For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | Support |
| Spreadtrum | support |
| ZTE | ‘Additional RO’s has been used for many proposals. It seems no ambiguity identified. In this sense, this proposal may not be needed. If this is pursued, for RACH configuration Option 1, it can be simplified as ‘additional-ROs include the ROs in SBFD symbols’. |
| New H3C | OK |
| China Telecom | Support. We think introducing the concept of “additional-ROs” will facilitate the discussion, or else, it’ll cause repeated description in different proposals. |
| Tejas | Support the definition of “Additional-RO” |
| Xiaomi | Support. |
| InterDigital | Support the proposal. |
| Samsung | Support |
| Sharp | Agree with FL. |
| LGE | Agree for the discussion purpose. |
| QC | Support. It makes sense to agree on common terminology for the additional ROs in option1 and option2. Also, it is good to have common understanding for legacy-ROs in both options as well. |
| WILUS | Support |
| Ericsson | Do not support. The term additional ROs is easily confused with the additional RACH configuration (Option 2), and that should be avoided. We think it is better to adopt the term SBFD ROs for ROs configured by SBFD PRACH, either Option 1 or Option 2. Since the two options are never coinciding, this term is not subject to misinterpretation. We can support the definition of this term per the above but we would also support defining the SBFD ROs as ROs validated by SBFD-aware UEs but not legacy UEs. |
| ETRI | Support the proposal. |
| Fujitsu | Support |
| Sony | Support |
| Panasonic | Support. |
| Nokia | Support. |
| NEC | For clarification, does this suggest that additional ROs in option 2 will not refer to ROs that are scheduled in SBFD slots. Also, the definition also includes the ROs which are configured SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, but we have already discussed that some of these ROs may be valid for legacy UEs and hence the subsequent enhancements should not be applicable to such ROs. |
| OPPO | Support |
| ITRI | Support |

#### Initial proposal 1-1-3:

Network explicitly indicates whether RACH configuration Option 1 for SBFD random access operation is enabled or not from network side.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | We agree that indication from gNB to SBFD aware UEs on whether Option 1 is enabled or not is necessary. However, we think it is premature to agree on an explicit indication. For example, separate power control for PRACH transmissions in SBFD and non-SBFD symbols is discussed. If additional RRC parameters need to be introduced for separate power control, these parameters can also serve the purpose to indicate whether Option 1 is enabled or not. |
| Spreadtrum | Too early to discuss an extra indication. Actually, the way to configure RACH resource set for RA in SBFD is more related. For example, an RACH resource set configured by a feature combination and set SBFD as true, no network explicitly indication is needed. From our side, this issue matters a general structure of how to configure RACH resource in SBFD, using RACH resource set or not. |
| ZTE | Support. Option 1 is based on the existing parameters of the single RACH configuration. An explicit RRC signalling enabling Option 1 is required to notice UEs, otherwise it may cause ambiguity among gNB and UE. Regarding CATT’s comment, it expects that the separate power control parameters (even if introduced) would be optional parameters. So, an explicit indication of enabling Option 1 is still necessary. |
| New H3C | OK |
| China Telecom | We don’t support this proposal at current stage. If additional RACH configuration for SBFD is not configured, but SBFD symbols are configured, we think Option 1 is then enabled in an implicit manner. In this case, explicit indication seems redundant. |
| Tejas | Support the proposal. But an explicit indication is required, as in the earlier meetings it was discussed that UE can support either option1 or option 2. |
| Xiaomi | We are a little bit confused on the potential result with suck kind of configuration. As discussed in the previous meeting, option 2 may be up to UE capability if not treated as default. If option 1 is not enabled, what is the UE assumption on RACH configuration? Does this mean UE cannot use RO on SBFD symbols? If so, random access cannot enjoy the advantage of SBFD operation. |
| InterDigital | In our opinion, as the first step NW should explicitly indicate whether RACH transmission on SBFD symbols is enabled/allowed or not. The option that is used can be further indicated implicitly, for example, if two sets of RACH configurations are provided, it implicitly indicates Option 2 and otherwise it will be Option 1. |
| Samsung | Support in principle. |
| Sharp | Agree with FL. We think it’s natural consequence from the previous agreement. |
| LGE | If RACH configuration Option 1 is transparent without extra signaling overhead, with any form of indication/configuration by gNB, SBFD aware UE needs to know whether RACH configuration Option 1 is enabled to figure out the additional-ROs configured by the legacy RACH configuration. However, considering we discuss on separate PRACH power controls, it is better to defer as of now. |
| QC | Support. SBFD-aware can’t differentiate whether the PRACHConfigCommon is interpretated as legacy PRACH with only legacy-ROs in UL/FL symbols or as PRACH configuration option-1 with additional ROs in SBFD symbols. gNB should explicitly indicate, e.g. by SIB1, such that SBFD-aware UE would know whether the additional ROs are valid or not. |
| WILUS | We agree with indication from network side whether or not to enabling either Option 1 or Option 2. And it can be implicitly indicated depending on having the configuration e.g. an additional RACH configuration. |
| DOCOMO | Support. |
| Ericsson | Support. |
| ETRI | We agree in principle, but we have similar understanding with CATT and Spreadtrum that some other means may be provided to serve this purpose. |
| Fujitsu | Support |
| Sony | Support. |
| Nokia | Fine with the proposal in case no additional parameters is added to the RACH configuration Option 1. |
| OPPO | From our understanding, under the premise of configuration of SBFD subbands, then If gNB configures additional RACH configuration, then option2 is enabled by network, otherwise, option 1 is enabled implicitly. |
| ITRI | Support. An explicit signal to indicate whether RACH configuration option 1 is operate can avoid the ambiguous of RO used in SBFD resource. |

#### Initial proposal 1-1-4:

Further discuss whether network can enable both RACH configuration Option 1 and Option 2 for SBFD random access operation from network side.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | We are fine to discuss. But from our perspective, we do not see the need to enable both options from network side. |
| Spreadtrum | RACH configuration is in SIB1 which is per cell configuration. If Option 1 and 2 are enabled from network side, it also enabled from UE side, which is against the previous agreement, enabling both options at the same time for a UE is not supported. So we do not support this proposal. |
| ZTE | Support that NW can enable both Option 1 and Option 2. According to the agreement, a UE is not required to support both of Option 1 and Option 2. Then, there may be three types of SBFD UEs in the network (supporting Option 1 only, or Option 2 only or both options), and gNB may not know which type a UE is. In such situation, NW may enable both options to accommodate different UEs. For a certain UE, only one option is enabled. |
| New H3C | OK to discuss |
| Tejas | Support for further discussion |

| xiaomi | As there are two options on RACH configuration and it has been agreed that enable two options for a UE simultaneously is not supported, the basic assumption is different UE has different capability, i.e., - UE only support option 1 - UE only support option 2 A UE supports both option 1 and option 2 is not the case, otherwise it makes no sense for ‘enabling both options at the same time for a UE is not supported’. Accordingly, NW has to enable both option 1 and option 2 from NW perspective. On the other hand, UE can determine the valid configuration according to its capability. |
| --- | --- |
| InterDigital | We are fine to discuss. |
| Samsung | Ok to discuss. |
| Sharp | OK. |
| LGE | It’s up to network’s decision whether to enable both RACH configurations Option 1 and Option 2 to accommodate different type of SBFD aware UEs enabling either Option 1 or Option 2. |
| QC | Not needed neither motivated. Suggest to depriotrized this discussion. |
| WILUS | Fine to discuss |
| DOCOMO | We prefer to not enable both from NW side. |
| Ericsson | Do not support. We don’t see the use case for this functionality and RAN1 should not spend time on it. |
| ETRI | We are fine with the proposal. We initially think that it is an implementation choice to have both options. |
| Sony | This should be up to network implementation but from UE perspective, it can only be configured with one option. |
| Panasonic | We are OK to discuss it further. Our view is both of option 1 and option 2 would not be needed simultaneously because we don't see the need to allocate many PRACH resources in SBFD symbol. |
| Nokia | It’s unclear to us what is the impact to UE from this proposal and why do we need to discuss this from network (NW) side. It’s worth noting that enabling both Options at the same time might increase the complexity on the NW side since the NW is required to deal with two different additional ROs configurations at the same time: additional ROs configured with the legacy RACH configuration (Option 1) and the additional ROs configured with the additional RACH configuration (Option 2). Besides, for UE side, it should be clear based on the previous agreement: “Enabling both options at the same time for a UE is not supported.” |
| NEC | Support this discussion and support enabling both options for network |
| OPPO | Do not see the need to enabling both options simultaneously. |
| ITRI | Fine with the proposal. |

## Issue#1-2: Details of RACH configuration Option 1

    - Submitted proposal

#### RACH resource

| Company | Proposals |
| --- | --- |
| MediaTek | - For RACH configuration Option 1 with Alt 1-1, support the following. - Do not reinterpret msg1-FrequencyStart in rach-ConfigCommon for random access operation in SBFD symbols. - ROs (including ROs obtained because of msg1-FDM) overlapping (partially or fully) with resources outside UL-SB are considered as invalid. - ROs in SBFD symbols configured as DL by tdd-UL-DL-ConfigurationCommon is valid if at least the time and frequency resource of the ROs are fully within UL usable PRBs of the SBFD symbols. - Reuse existing SSB-to-RO mapping rules. |
| New H3C | Proposal 6: Frequency start offset for SBFD which is relative to the original PRACH frequency starting point is supported for determining frequency start of PRACH Occasion in SBFD. Proposal 5: Unified configuration on PRACH by RRC signalling can be supported. |
| Huawei, HiSilicon | Proposal 1: For RACH configuration Option 1 with Alt 1-1, the following new rules are introduced - msg1-FrequencyStart does not apply for PRACH resource for SBFD aware UEs in SBFD symbols and PRACH resource in SBFD symbols starts from the starting point of UL usable PRBs - The number of FDMed ROs in one time instance N is the largest number belonging to {1, 2, 4, 8} that is not greater than the maximum number of ROs that can be carried in UL usable PRBs and the number of configured FDMed ROs for non-SBFD aware UEs to ensure that the PRACH resources are confined within the UL usable PRBs |
| Spreadtrum | - Option 1 (Alt 1-1) is one single RACH configuration and only based on the existing parameters of the single RACH configuration, without any interpretations. |
| ZTE, Sanechips | Proposal 3: For RACH configuration Option 1 with Alt 1-1, the frequency domain reference point of 'msg1-FrequencyStart' can be re-interpreted as the lowest PRB of the UL usable PRBs for RO in SBFD symbols - If needed, the configured value of ‘msg1-FrequencyStart’ for SBFD symbols can be set by a fix value, e.g., 0. |
| CMCC | Proposal 5: For RACH configuration Option 1 with Alt 1-1, support network configuring whether to re-interpret the parameter msg1-FrequencyStart in rach-ConfigCommon as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs for SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. |
| LG Electronics | Proposal 7: For Option 1, consider to discuss if the mechanism to manage the proper number of valid ROs should be supported. |
| China Telecom | Proposal 5: For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, further study following alternatives: - Alt-1: Support to reinterpret msg1-FrequencyStart in rach-ConfigCommon as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - FFS: details. - Alt-2: Not support to reinterpret msg1-FrequencyStart in rach-ConfigCommon. |
| TCL | Proposal 1: For RACH configuration option 1, consider the following alternatives for re-interpretation of Msg1-FrequencyStart: - Alt 1: Configure msg1-FrequencyStart based on the starting RB of the initial/active UL BWP. The ROs which lies within the UL subband usable PRBs in SBFD symbols are valid, while the ROs outside are invalid. - Alt 2: Configure an RB offset between the UL subbands usable PRB and the initial/active UL BWP. The SBFD aware UEs can identify the starting location of the lowest RO in SBFD symbols, based on msg1-FrequencyStart + RB offset. Proposal 2: In RACH configuration option 1, for a single PRACH configuration index to indicate the time domain parameters of ROs in both SBFD and non-SBFD symbols, consider the following alternatives: - Alt1: Add a new column in existing tables [6.3.3.2-2 for FR1, 6.3.3.2-3 for FR2, TS 38.211] which contains the information of sub-frames/slots for RACH in SBFD symbols. This can allow a single PRACH configuration index to indicate the time domain parameters for RACH in both SBFD and non-SBFD symbols. - Alt2: Explicitly configure the sub-frames/slots for RACH in SBFD symbols. For other time domain parameters the SBFD UE can derive it from the single PRACH configuration Index of the non-SBFD symbols. |
| Ericsson | Proposal 2 For ROs within SBFD symbols, msg1-FrequencyStart indicates the offset of the 1st RO in relation to the start of the UL subband. The start of the nth RO is determined by the legacy start of the nth RO modulus of the UL subband less the RO bandwidth such that all ROs are always comprised within the UL subband. |
| vivo | Proposal 1: RACH configuration Option 1 with Alt 1-2 should be deprioritized in Rel-19. Proposal 2: For Option 1 with Alt 1-1 for RACH configuration, reinterpretation of msg1-FrequencyStart in rach-ConfigCommon is not considered. |
| Kookmin University | Proposal 3: Support reinterpret msg1-FrequencyStart in rach-ConfigCommon for SBFD symbols as follows: - The indicated offset value is reinterpreted with scaling, such that Δsbfd=Δ·K where K=NPRBULsub/NPRBUL, NPRBULsub is the number of UL usable PRBs and NPRBUL is the number of PRBs for active UL BWP. - The re-interpreted offset value Δsbfd is applied with respective to the lowest PRB of UL subband. - The ROs located outside of UL usable PRBs are invalid. |
| OPPO | Proposal 1: For option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), support using different reference points to interpret msg1-FrequencyStart for SBFD-DL symbols (SBFD symbols configured as downlink) and UL/flexible symbols. Proposal 2: For option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), the frequency offset of the lowest RO with respective to the lowest PRB of UL usable PRB is msg1-FrequencyStart*scaling factor, where the scaling factor=UL usable PRB sizeUL BWP size |
| Xiaomi | Proposal 2: msg1-FrequencyStart is directly used to determine frequency location of RO on SBFD symbols for RACH configuration Option 1 with Alt 1-1. |
| CATT | Proposal 1: For RACH configuration Option 1 with Alt 1-1, if reinterpretation of msg1-FrequencyStart is supported, it is not applicable to all the cases. - Explicit or implicit scheme needs to be defined for SBFD aware UEs to determine whether to reinterpret msg1-FrequencyStart. Proposal 2: For RACH configuration Option 1 with Alt 1-1, if reinterpretation of msg1-FrequencyStart is supported and UE determines to reinterpret msg1-FrequencyStart, frequency offset of lowest RO with respective to the lowest UL usable PRB is mod (msg1-FrequencyStart, bandwidth of UL usable PRBs) in SBFD symbols configured as DL by tdd-UL-DL-ConfigurationCommon. |
| Sony | Proposal 1: Support the reinterpretation of msg1-FrequencyStart for PRACH Configuration Option 1, such that the ROs are contained within the UL subband using a MOD function, and if an RO is wrapped around to the lower part of the UL subband that collides with another RO, the RO that would be wrapped around is dropped. Proposal 2: Scale the number of FDM RO so that it fits within the UL subband. |
| Fujitsu | Proposal 1: New configurations to allow random access on SBFD symbols should be supported. - By default, random access on SBFD symbols is not allowed. Proposal 3: For Option 1, reinterpret msg1-FrequencyStart for ROs within DL SBFD symbols and for ROs across DL SBFD symbols and flexible SBFD symbols. - For these ROs, the offset indicated by msg1FrequencyStart refers to the offset between the lowest RO in frequency domain and the lowest PRB of the UL usable PRBs. |
| Panasonic | Proposal 1: For PRACH configuration Option 1, not support to reinterpret msg1-FrequencyStart in rach-ConfigCommon. |
| Hyundai | Proposal #3: RAN1 needs to consider allowing UE to readjust configured the number of repeated preamble sequences and it can transmit as many as possible when configured preamble format may not be used properly due to the limited time duration. |
| Samsung | Proposal 5: For RACH configuration option 1 with Alt 1-1, do not support to reinterpret msg1-FrequencyStart in rach-ConfigCommon. |
| Lenovo | Proposal 1: For option 1 with Alt 1-1, there is no needed to reinterpret msg1-FrequencyStart in rach-ConfigCommon. - At least for ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon |
| Transsion Holdings | Proposal 1: The frequency resources of the configured ROs in SBFD symbols should be derived from the existing parameters of the single RACH configuration with the reinterpretation of msg1-FrequencyStart. |
| ETRI | Proposal 11: Regarding Option 1, keep the same number of FDMd ROs in SBFD symbols and in non-SBFD symbols. Proposal 12: Regarding Option 1, discuss whether to support either/both localized FDMd ROs and/or distributed FDMd ROs in SBFD symbols and in non-SBFD symbols. |
| InterDigital | Proposal 1. Support Alt 1-2 for introducing extra parameters in configuring ROs in SBFD symbols based on Option 1 (using one single RACH configuration). |
| Google | Proposal 1: For Option 1, Alt 1-1, support two different interpretation for msg1-FrequencyStart in rach-ConfigCommon depending if the symbols are SBFD or non-SBFD. • PRB 0 of the UL BWP as the frequency start reference for the non-SBFD symbols and first full PRB at the start of the UL subband for the SBFD symbols. • A modulo operation can be used for SBFD symbols to keep the ROs within the UL usable PRBs. |
| KT | Proposal 1. Consider the RACH configuration with newly introduced parameters for RACH configuration Option 1. |
| NTT DOCOMO |  |
| Qualcomm | Proposal 2: For SBFD random access operation using option 1, support the following: - The start-RBs of the ROs in SBFD-DL symbols can be interpreted with respect to the first usable PRB in the uplink subband. - FFS: Separate power control parameters for PRACH transmission in SBFD symbols. |
| WILUS | Proposal 1: For RACH configuration Option 1, it is suggested to consider implicitly different RO resource setting from the legacy UE’s RO resource by reinterpreting at least msg1-FrequencyStart to configure the RO within usable PRBs in the UL subband. |

#### RO validation

| Company | Proposals |
| --- | --- |
| MediaTek | - For RACH configuration Option 1 with Alt 1-1, support the following. - Do not reinterpret msg1-FrequencyStart in rach-ConfigCommon for random access operation in SBFD symbols. - ROs (including ROs obtained because of msg1-FDM) overlapping (partially or fully) with resources outside UL-SB are considered as invalid. - ROs in SBFD symbols configured as DL by tdd-UL-DL-ConfigurationCommon is valid if at least the time and frequency resource of the ROs are fully within UL usable PRBs of the SBFD symbols. - Reuse existing SSB-to-RO mapping rules. |
| New H3C | Proposal 12: For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, - no enhancements for the RO validation rule for the ROs in non-SBFD symbols and the ROs in SBFD symbols configured indicated as flexible by tdd-UL-DL-ConfigurationCommon (if any). - FFS: the ROs in non-SBFD symbols that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - FFS: It’s up to network configuration to ensure the ROs in SBFD symbols configured indicated as flexible by tdd-UL-DL-ConfigurationCommon, which are valid for non-SBFD aware UEs based on legacy RO validation rule, are also valid for SBFD aware UEs (i.e., the configured ROs in SBFD symbols, if configured indicated as flexible by tdd-UL-DL-ConfigurationCommon, are within the UL usable PRBs). - the RO in SBFD symbols configured indicated as downlink by tdd-UL-DL-ConfigurationCommon is valid if at least: - Time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB. - FFS: Other condition. Note: For the case that all the SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, there is no restriction that all the configured ROs in SBFD symbols should be within the UL usable PRBs. |
| Huawei, HiSilicon | Proposal 8: For RACH configuration Option 1 with Alt 1-1, the valid ROs of the PRACH resources for SBFD aware UEs in SBFD symbols occupying flexible symbols should be satisfied with following condition - Condition#1: A valid RO starts at least X symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least X symbols after the SSB. - Condition#3: A valid RO does not precede a SSB in the PRACH slot. where the value of X is the same as that in PRACH resources for non-SBFD aware UEs, illustrated in Table 8.1-2 in 38213. |
| Spreadtrum | - For RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, support the following additional condition(s) on top of what was agreed in RAN1#116bis for RO validation: - Condition#1: A valid RO starts at least Ngap symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least Ngap symbols after the SSB. - Condition#3: A valid RO does not precede a SSB in the PRACH slot. Proposal 5. For RACH configuration Option 1 with Alt 1-1, it is not expected ROs are valid for non-SBFD aware UE and the ROs are in non-SBFD symbols and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - Adopt the following changes to the previous agreements of ROs for RACH configuration Option 1 with Alt 1-1. - RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon ->RO in SBFD symbols configured as downlink w/o flexible by tdd-UL-DL-ConfigurationCommon - For RACH configuration Option 1 Alt 1-1, the valid additional ROs are valid ROs in SBFD symbols configured as downlink w/o flexible by tdd-UL-DL-ConfigurationCommon. |
| ZTE, Sanechips | Proposal 4: For RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, consider the additional conditions for RO validation. - Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB. |
| Tejas | Proposal 1: If an RO duration starts from SBFD symbols and ends in flexible symbols of TDD-UL-DLConfig Common (non-SBFD symbols), then these ROs should be considered as valid in Option 1 as well. Proposal 4: For SBFD, valid RO can start at least Ngap symbols after the SSB block. Ngap value can be same as proposed in 8.1 of TS38.213 [6]. Proposal 5: If a SBFD UE is aware of the SBFD frame structure and RO is inside the Uplink subband then these are RO should be considered as valid. No additional time gap is required. |
| CMCC | Proposal 6: Regarding RO validation for RACH configuration Option 1 with Alt 1-1, an RO across SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon is treated the same as an RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. Proposal 7: For RACH configuration Option 1 with Alt 1-1, support the following condition for RO validation of additional-ROs. Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. |
| LG Electronics | Proposal 7: It’s RAN1’s common understanding that “Time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB” in the above agreement means “Time and frequency resource of the RO are fully within UL usable PRBs, and time resource of the RO is not overlapped with SSB” |
| China Telecom | Proposal 8: For PRACH configuration Option 1 with Alt 1-1, and for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, support the following RO validation rules on top of what was agreed in RAN1#116bis for RO validation: • A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. • A valid RO starts at least X>0 (FFS the value) symbols after the SSB. |
| Ericsson | Proposal 3 Adopt the same SSB-to-RO Ngap margins as are valid for legacy operation. Observation 1 There is no need to consider the general Rx-to-Tx gap in TS 38.211, § 4.3, as a criterium for RO validation. |
| vivo | Proposal 5: The following RO validation rule can be considered for RACH configuration Option 1: A RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon is valid if: - Time and frequency resource of the RO are fully within UL usable PRBs, and - The RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol, and - The RO starts at least X>0 (FFS the value) symbols after the SSB, and - The RO does not precede a SSB in the PRACH slot. |
| Kookmin University | Proposal 1: For Condition#1 of RO validation for Option 1 with Alt 1-1, X value is explicitly configured by higher layer. If X value is not configured, the default value of X is Ngap. Proposal 2: For Condition#2 of RO validation for Option 1, X is equal to Ngap. |
| OPPO | Proposal 3: For RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, additional RO validation conditions should include at least Condition #1 (A valid RO starts at least X symbols after a last downlink non-SBFD symbol). |
| Xiaomi | Proposal 3:For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, - The RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon is valid if at least: - Frequency resource of the RO is fully within UL usable PRBs, and - Not overlapped with SSB (Ngap can be 0 for all preamble SCS), and - Starts at least X>=0 symbols after a last downlink symbol, and - Value of X depends on required transition time - Not across SBFD symbols and non-SBFD symbols within a slot or across slots |
| CATT | Proposal 6: For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, a RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon is valid if at least: - Frequency resources of the RO are fully within UL usable PRBs and not overlap with a SS/PBCH block. - Starts at least Ngap symbols after a last non-SBFD downlink symbol. |
| Sony | Proposal 3: No further validation rules apart from requiring the RO to be fully contained in the UL subband and does not overlap with the SSB. |
| Fujitsu | Proposal 2: Regarding RO validation and SSB-RO mapping for Option 1, an RO across SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon is treated in the same way as an RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. |
| NEC | Proposal 2: - For SBFD-aware UEs, and for RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, support at least following additional condition on top of what was agreed in RAN1#116bis for RO validation. - A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol |
| Samsung | Proposal 6: For RACH configuration option 1 with Alt 1-1, defer discussion on additional RO validation rules with respect to SSB reception until further progress in AI 9.3.1 |
| Nokia, Nokia Shanghai Bell | Proposal 7 RAN1 to discuss on whether/how to apply the RO mask index, when configured, on the legacy and new ROs, for both PRACH configuration Option 1 and Option 2. |
| Transsion Holdings | Proposal 3: For RACH configuration Option 1, a valid RO can only be on the SBFD symbols or non-SBFD symbols. |
| ETRI | Proposal 6: The validity test introduces frequency resource of an RO in SD symbols. Proposal 8: Strive to a unified framework for RO validation and SSB-RO mapping rule for both options, for connected UEs. Proposal 14: Additional RO validation rule if introduced are applicable for RRC non-connected mode. Proposal 15: Determine whether Condition #1 is used for RO validation or for RO dropping. Proposal 16: Condition #2 is supported with Ngap. |
| InterDigital | Proposal 3. For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, support additional condition(s) for RO validation for FDM-ed ROs in SBFD symbols. |
| Google | Proposal 2: For Option 1, Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, the following two condition(s) are supported • Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. • Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB. |
| Sharp | Proposal 4: ROs overlapping with SSB in the time domain is invalid for Option 1 with Alt 1-1. |
| Apple | Proposal 1: For Option 1 (i.e., use one single RACH configuration with possible enhancement), the following RO validation conditions are applied to the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon - A valid RO starts at least 2 symbols after a last downlink non-SBFD symbol and at least 2 symbols after the last SSB symbol |
| NTT DOCOMO | Proposal 3: For RACH configuration option 1, for ROs in SBFD DL symbols, following additional conditions are included for RO validation. - Condition#1: A valid RO starts at least X>=0 symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least X>0 symbols after the SSB. - Condition#3: A valid RO does not precede a SSB in the PRACH slot. - FFS value of X in condition #1 and condition #2. |
| Qualcomm | Proposal 7: Support only condition #2 to have at least X gap symbols before the start of a valid RO. - Relax the Ng symbols between last SSB and an RO such that a configured RO in SBFD-DL symbols that is proceeding SSB symbols is considered valid if there is enough timeline to switch from Rx to Tx. |
| CEWiT | Proposal 2: The ROs, which are valid for legacy UEs, are valid for SBFD aware UE irrespective of its alignment with SBFD subbands. |
| WILUS | Proposal 8: Regardless of which RACH configuration option is applied, Condition#1/#2 of the following conditions should be further considered to be necessary for RO validation as in legacy NR system. And Condition#3 can be relaxed and Condition#4 should be also discussed for RO validation. ‐ Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. ‐ Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB. ‐ FFS: Condition#4: A valid RO starts at least X>=0 (FFS the value) symbols after a last SBFD symbol with DL reception for a given UE. |

#### SSB-RO mapping

| Company | Proposals |
| --- | --- |
| MediaTek | - For RACH configuration Option 1 with Alt 1-1, support the following. - Do not reinterpret msg1-FrequencyStart in rach-ConfigCommon for random access operation in SBFD symbols. - ROs (including ROs obtained because of msg1-FDM) overlapping (partially or fully) with resources outside UL-SB are considered as invalid. - ROs in SBFD symbols configured as DL by tdd-UL-DL-ConfigurationCommon is valid if at least the time and frequency resource of the ROs are fully within UL usable PRBs of the SBFD symbols. - Reuse existing SSB-to-RO mapping rules. |
| New H3C | Proposal 2: Mapping SSB to RO in SBFD symbol and SSB to RO in non-SBFD symbol separately should be supported. Proposal 3: If supporting unified mapping SSB to RO in SBFD symbol and non-SBFD symbol, dedicated preamble allocation for SBFD-aware UE should be considered. Proposal 4: Regardless of unified mapping SSB to RO in SBFD symbol and non-SBFD symbol or separated mapping SSB to RO in SBFD symbol and non-SBFD symbol, if RO isn’t mapped with SSB, the RO is invalid and can’t be used for transmitting preamble. |
| Huawei, HiSilicon | Proposal 4: For RACH configuration Option 1 with Alt 1-1, a descending order of SSB index is used for the SSB-RO mapping for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. |
| Tejas | Proposal 3: SSB-RO mapping rule for SBFD UE should be allowed to cover ROs used by non SBFD aware UEs as well. |
| CMCC | Proposal 8: For RACH configuration Option 1 with Alt 1-1, the legacy SSB-RO mapping rule is reused for additional-ROs. |
| LG Electronics | Proposal 12: For Option 1, reuse legacy mapping rule applied to each legacy ROs and additional ROs where separate SSB-RO mapping is agreed to be used, respectively. - For additional ROs for SBFD aware UE, further enhancement on legacy SSB-RO mapping rule such as allowing subset of SSBs or different start SSB index for additional ROs could be discussed. |
| TCL | Proposal 4: Consider the following alternative options for SSB to ROs mapping: - Option 1: Separate SSB to ROs mapping of ROs configured in SBFD symbols and ROs configured in non-SBFD symbols. - Option 2: Joint ROs configured in SBFD symbols with ROs in non-SBFD symbols based on their indexes and map to SSBs. |
| Korea Testing Laboratory | Proposal 4: For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, the legacy SSB-RO mapping rule is reused for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. |
| OPPO | Proposal 4: For option 1 with Alt 1-1, legacy SSB-RO mapping is applied for ROs configured in SBFD symbols configured as downlink. |
| CATT | Proposal9: For RACH configuration Option 1, legacy SSB-to-RO mapping rule is reused for the additional valid ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. |
| Sony | Proposal 4: For PRACH configuration Option 1 with Alt 1-1 (i.e., use one single PRACH configuration, and only based on the existing parameters of the single RACH configuration), the SBFD UE performs two SSB-RO associations using legacy (Rel-18) SSB-RO association method, on ROs validated using different RO validation rules, i.e.: - 1st SSB-RO association is performed on ROs validated using legacy (Rel-18) RO validation rules - 2nd SSB-RO association is performed on ROs validated using new (Rel-19) RO validation rules, where a RO is valid if it is fully contained with usable UL PRB and does not overlap with SSB |
| Panasonic | Proposal 2: For PRACH configuration Option 1, use the legacy SSB-RO mapping rule for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. Further optimization is not needed. |
| Nokia, Nokia Shanghai Bell | Proposal 2. For RACH configuration Option 1, RAN1 to confirm whether new SSB-to-RO mapping rule is introduced or not by down-selecting one of the following alternatives: - Alt. 1: Legacy SSBs to ROs mapping rules are used separately on the SBFD ROs. - Alt. 2: Legacy SSBs to ROs mapping rules are used to continue the mapping for SBFD-ROs starting from the SSB index/indices associated with the latest legacy RO (i.e., previous legacy RO with highest frequency index). - Alt. 3: For each frequency index, applying the same SSB index/indices of the latest legacy RO for the subsequent SBFD-RO(s). |
| ETRI | Proposal 13: Regarding Option 1, SSB-RO mapping is performed at non-SBFD symbols in the legacy association period firstly, and then additionally is done at SBFD symbols. . |
| InterDigital | Proposal 5. Support SSB-RO mapping in ROs in SBFD symbols based on descending order of SSB indexes. |
| Sharp | Proposal 2: Independent SSB-to-RO mapping for the additional ROs is supported for Option 1 with Alt 1-1. |
| NTT DOCOMO | Proposal 4: For RACH configuration option 1, for SSB-to-RO mapping for valid ROs in SBFD DL symbols, the legacy SSB-to-RO mapping rule (i.e., preamble domain first, then frequency domain, then time domain) is applied. |
| Qualcomm | Proposal 9: For Option-1, reuse the legacy SSB-RO mapping rule for mapping SSBs to the valid ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. - FFS: the start index of the first SSB and the subsets of SSBs. Proposal 10: For both option-1 and option-2 of PRACH configuration, RAN1 to study the determination of the association period and association period pattern for the SBFD-ROs. |
| CEWiT | Proposal 7: SBFD aware UE treating the ROs valid for legacy UEs and ROs created due to SBFD separately and independently performing SSB-RO mapping for the two sets is supported |
| WILUS | Proposal 3: For RACH configuration Option 1, we propose to support legacy SSB to RO mapping rule for validated ROs within UL subband for Rel-19 SBFD aware UEs considering BW size difference between RO’s BW size of UL subband and legacy RO’s BW size for non-SBFD aware UEs. Proposal 4: For RACH configuration Option 1, we propose to have separate SSB index to RO mapping for new validated ROs in SBFD symbols for SBFD aware UEs, regardless of being configured as downlink or flexible by tdd-UL-DL-ConfigurationCommon. |

#### PRACH power control

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 14: For PRACH transmission of SBFD-aware UEs, consider separate power control for different FDMed ROs of PRACH in SBFD symbols and PRACH in non-SBFD symbols, where separate parameter sets for different FDMed ROs in SBFD symbols are introduced, including target PRACH receive power and maximum transmission power. |
| Spreadtrum | - For RACH configuration Option 1 Alt 1-1, for PRACH transmission of SBFD aware UEs in RRC CONNECTED state, support an additional set of PRACH power control parameters configuration, including P0, alpha and target power - Legacy power control parameters applies to legacy ROs, and additional power control parameters applies to additional ROs. |
| ZTE, Sanechips | Proposal 12: Support separate PRACH power control parameters configuration in SBFD symbols and non-SBFD symbols. − The separate configuration of PRACH power control parameters can be an absolute value of each power control parameter, or can be derived based on a relative value, e.g., a power offset. − The above mechanism should applicable to both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-4. |
| Tejas | Proposal 9: RACH parameters on Preamble Received Target Power, Maximum Preamble Transmission and Power Ramping Step can have different configuration values in SBFD symbols from that of non SBFD RACH configurations. Proposal 10: Separate PRACH power control parameters are required for RACH operation in RRC_IDLE/ RRC_INACTIVE mode and shall be sent in SIB. |
| CMCC | Proposal 9: For RACH configuration Option 1 with Alt 1-1, introduce a parameter for the power offset between the transmission powers in SBFD symbols and non-SBFD symbols. |
| LG Electronics | Proposal 17: For Option 1, support configuring single PRACH power control parameter for RO in non-SBFD symbol and reinterpreting the PRACH power control parameter for RO in SBFD symbol. |
| China Telecom | Proposal 10: Support separate configuration of PPRACH,target,f,c for SBFD operation. |
| Korea Testing Laboratory | Proposal 5. For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, do not prefer separate PRACH power control configuration in SBFD symbols and non-SBFD symbols. |
| Ericsson | Proposal 4 Support separate power control for SBFD ROs for both single and dual RACH configurations. |
| Kookmin University | Proposal 5: For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, support separate PRACH power control for SBFD symbols and non-SBFD symbols. For this purpose, the higher layer parameters related to the power control (e.g., preambleReceivedTargetPower) are reinterpreted for PRACH transmission in SBFD symbols. |
| OPPO | Proposal 11: For PRACH power control, support separate configuration of preambleReceivedTargetPower for SBFD symbol and non-SBFD symbol for both RACH configuration option 1 and option 2, FFS other parameters, e.g., powerRampingStep, preambleTransMax, etc. |
| Xiaomi | Proposal 4: Same power control parameters are applied to SBFD symbols and non-SBFD symbols for RACH configuration Option 1 with Alt 1-1. |
| CATT | Proposal 15: Support to configure separate power control parameters including preambleReceivedTargetPower, powerRampingStep and powerRampingStepHighPriority for SBFD symbols and non-SBFD symbols. |
| NEC | Proposal 9: • Power control for PRACH enhancements on SBFD symbols should be considered. |
| Panasonic | Proposal 3: For PRACH configuration Option 1, not support separate PRACH power control. |
| Nokia, Nokia Shanghai Bell | Observation 2 It is recommended to avoid any changes to the specification of Option1. Observation 3 Option 2 already covers the issue of having separate PRACH power control parameters for the legacy UL ROs and the additional SBFD ROs. Introducing new power control parameters to Option 1 is not justified. |
| Lenovo | Proposal 8: Separate power control parameters are supported for PRACH transmission in SBFD symbols and non-SBFD symbols. |
| Transsion Holdings | Proposal 5: For PRACH power control, support separate configuration of preambleReceivedTargetPower for SBFD symbol and non-SBFD symbols. |
| ETRI | Proposal 4: Introduce distinct parameter sets for ROs on SBFD symbols at least including power control parameters. |
| InterDigital | Proposal 6. Support configuring different PRACH power control parameters for SBFD and non-SBFD ROs, also for FDM-ed SBFD ROs that are closer or farther from DL subband edges, in consideration of potential CLI caused by PRACH transmission. |
| Sharp | Proposal 5: Power control related parameters like power ramping steps and power ramping counters should be separately configured for additional ROs in Option 1 with Alt 1-1. |
| Apple | Proposal 2: For RACH configuration Option 1 (i.e., use one single RACH configuration with possible enhancement), separate PRACH power control parameters can be configured for transmission in SBFD symbols. |
| KT | Proposal 3. Support separate PRACH power control parameters in SBFD symbols and non-SBFD symbols. Proposal 4. Discuss how to enhance the PRACH power control for PRACH transmission in SBFD symbols for PRACH configuration Option 1. |
| NTT DOCOMO | Proposal 5: For RACH configuration option 1, support configuration of separate target preamble power for SBFD in the legacy RACH configuration. |
| Qualcomm | Proposal 2: For SBFD random access operation using option 1, support the following: - The start-RBs of the ROs in SBFD-DL symbols can be interpreted with respect to the first usable PRB in the uplink subband. - FFS: Separate power control parameters for PRACH transmission in SBFD symbols. |

### Summary

#### RACH resource

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

One FFS about RACH configuration Option 1 with Alt 1-1 is whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon. In this meeting, companies’ views on this issue are summarized as below.

- Not support to reinterpret msg1-FrequencyStart in rach-ConfigCommon

  - MediaTek, New H3C, Spreadtrum, vivo, Xiaomi, Panasonic, Samsung, Lenovo

  - Reason: less complexity, the RO resource configuration flexibility can be achieved by RACH configuration Option 2

- Support to reinterpret msg1-FrequencyStart in rach-ConfigCommon

  - Huawei, HiSilicon, ZTE, Sanechips, TCL, Ericsson, Kookmin University, OPPO, sony, Fujitsu, Transsion Holdings, Google, Qualcomm, WILUS

- Support gNB configure whether to reinterpret msg1-FrequencyStart in rach-ConfigCommon

  - CMCC, CATT

Regarding how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, most companies propose that the parameter msg1-FrequencyStart in rach-ConfigCommon can be reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. However, as some companies point out, there are some cases the reinterpretation of msg1-FrequencyStart may not be workable, e.g., the msg1-FrequencyStart is larger than UL usable PRB size.

  - [CATT, Ericsson, Sony, Google] propose to add the mod operation to the value of frequency offset in SBFD symbols, but the detailed solutions among companies are different.

  - [Kookmin University, OPPO] propose to scale the value of frequency offset in SBFD symbols according to the ratio of size of UL BWP and UL usable PRBs, e.g., scaling factor = UL usable PRB size/UL BWP size

  - [Huawei, ZTE] proposes that PRACH resource for SBFD aware UEs starts from the starting point of UL subband and has N FDM-ROs, where N is the largest number belonging to {1,2,4,8} that is not greater than the maximum number of ROs that can be carried in UL subband and the number of FDM-ROs for non-SBFD aware UEs to ensure that the PRACH resources are confined within the UL subband.

  - [Sony] propose to scale the number of FDM ROs in SBFD symbols so that it fits within the UL subband..

Considering the situation, Moderator suggests Initial proposal 1-2-1 as a compromise solution.

#### RO validation

Regarding RO validation for RACH configuration Option 1 with Alt 1-1, [Spreadtrum, Fujitsu, CMCC] mention one missing case is that an RO is configured across SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon and SBFD symbols configured as flexible by configured by tdd-UL-DL-ConfigurationCommon, and propose to treat it the same as the RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. Moderator suggests Initial proposal 1-2-2.

In RAN1#117, we made the following agreement about additional RO validation rule:

Agreement For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, consider whether additional condition(s) on top of what was agreed in RAN1#116bis for RO validation are necessary. For example, - Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB. - Condition#3: A valid RO does not precede a SSB in the PRACH slot.

Companies’ views in this meeting are summarized as the following:

- Condition#1:

  - Supported by: Huawei, HiSilicon, Spreadtrum, ZTE, Sanechips, CMCC, China Telecom, vivo, Kookmin University, Xiaomi, CATT, NEC, ETRI, Google, Apple, NTT DOCOMO, WILUS

- Condition#2:

  - Supported by: Huawei, HiSilicon, Spreadtrum, ZTE, Sanechips, Tejas, China Telecom, Ericsson, vivo, Kookmin University, Xiaomi, ETRI, Google, Apple, NTT DOCOMO, Qualcomm (with relaxed Ng value), [Samsung](Defer the discussion until further progress in AI 9.3.1), WILUS

- Condition#3:

  - Supported by: Huawei, HiSilicon, Spreadtrum, NTT DOCOMO

Most companies support condition#1 and condition#2, some companies [Huawei, HiSilicon, Spreadtrum, Tejas, Kookmin University, CATT, ETRI, Apple, WILUS] propose to set the value of X as the Ngap value in the current specification. Regarding the condtion#3, most companies think it is unnecessary because there is only DL-to-UL and no UL-to-DL in one slot in the legacy TDD frame structure, while this is no longer true for SBFD operation. Thus, moderator suggests Initial proposal 1-2-3.

#### SSB-RO mapping

For RACH configuration Option 1 with Alt 1-1, it has been agreed that separate SSB-RO mapping is used for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon from legacy-ROs.

Agreement For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), - For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed. - For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used

One remaining issue is the details of the SSB-RO mapping rule for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. Companies’ views are summarized as below:

- Reuse legacy SSB-RO mapping rule:  MediaTek, CMCC, Korea Testing Laboratory, OPPO, CATT, Sony, Panasonic, Nokia, NSB, NTT DOCOMO, WILUS

- SSB-RO mapping with descending order of SSB indexes: Huawei, HiSilicon, InterDigital

- Mapping on subset of SSBs or new start index of the first SSB: LG Electronics, Qualcomm

- Legacy SSBs to ROs mapping rule is used to continue the mapping for SBFD-ROs starting from the SSB index/indices associated with the latest legacy RO (i.e., previous legacy RO with highest frequency index): Nokia, NSB

- For each frequency index, applying the same SSB index/indices of the latest legacy RO for the subsequent additional RO(s): Nokia, NSB

Moderator suggests initial proposal 1-2-4 based on majority view.

#### PRACH power control

Whether to support separate power control for PRACH transmission in SBFD symbols and non-SBFD symbols were discussed in the previous meetings. Companies’ views on the power control for RACH configuration option 1 with Alt 1-1 are summarized as below.

- Not support separate PRACH power control for additional-ROs for RACH configuration option1 with Alt 1-1:

  - LGE, Xiaomi, Panasonic, Nokia, Nokia Shanghai Bell

- Support separate PRACH power control for additional-ROs for RACH configuration option1 with Alt 1-1:

  - Spreadtrum, ZTE, Sanechips, CMCC, Korea Testing Laboratory, Ericsson, Kookmin University, OPPO, Sharp Apple, KT, NTT DOCOMO, Qualcomm (FFS)

    - Option 1: configure absolute value(s) of one or several existing power control parameters for additional-ROs, e.g., preambleReceivedTargetPower, powerRampingStep and preambleTransMax

    - Option 2: configure a power offset for additional-ROs compared to legacy-ROs

- Some companies propose to support separate PRACH power control without differentiation between RACH configuration Option 1 and Option 2

  - [China Telecom?] [CATT?], [Lenovo?], [Transsion Holdings?], [InterDigital?], [ETRI?], [Tejas?]

Consider majority companies support separate power control, moderator suggests initial proposal 1-2-5.

### 1st Round Proposals

#### Initial proposal 1-2-1:

For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, support network configuring to use Alt 1 or Alt 2 (Alt 2 to be down-selected from Alt 2-1 and Alt 2-2).

- Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation.

- Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- Alt 2-2: Use a fixed value (e.g., 0) for the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | We want to add one more Alt for further discussion. - Alt 2-3: Frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs). |
| Spreadtrum | Before we have a clear view on alt 2, it is too early to agree with a network configuration. From our side, alt 2 has the following disadvantages. - Firstly, reinterpretation is necessary only under some cases, it is not common for all configuration, e.g. when the RO configured by legacy msg1-FrequencyStart is within or partially within UL subband. There is no need to do reinterpretation. - Second some cases the reinterpretation of msg1-FrequencyStart may not be workable, e.g., the msg1-FrequencyStart is larger than usable PRB size. Mod operation was also suggested for this case. But these mod operations are only define additional and complex rules to have some RO in UL usable PRB. Frankly speaking, all the interpretations are unnecessary and unfriendly for gNB RACH resource configuration and UE determine the RO position. - last, Option 2 has same ability as reinterpretation. If the RO configured by legacy msg1-FrequencyStart is outside of UL subband, Option 2 can be used. Otherwise Option 1 and Option 2 can be used. It is not essential to support reinterpretation for Option 1. |
| ZTE | Down-selecting one option is sufficient, and it’s no need to define two different UE behaviors. We prefer Alt 2-1, and are also ok with Alt 2-2 which might be a good midground since it can address the concerns that the ROs may be out of the UL subband if a large value is configured for msg1-FrequencyStart. |
| New H3C | OK in general |
| China Telcom | We share the similar view with CATT to add Alt 2-3. We also think down-select one single option is enough. |
| Tejas | Support the proposal. We support Alt 1, i.e no reinterpretation of msg1-FrequencyStart |
| Xiaomi | Firstly we want to clarify the main bullet. Our understanding is that we will boil down to single solution in the end, not support both alt 1 and one of options from alt 2 series. Secondly, we support alt 1. The spirit of Alt 1-1 is to fully reuse existing parameters of the single PRACH configuration to determine time/frequency resources of RO. If reinterpretation of same parameters is introduced in order to determine the RO in SBFD symbols, it will complicate not only UE implementation but also specification. It is concluded in RAN1#117 that network ensures the ROs are within the UL usable PRBs if legacy ROs are configured in SBFD symbols configured as flexible. Therefore, reinterpretation is not needed for legacy ROs on flexible symbols. On the other hand, UE may need to sustain a different understanding for RO configuration on DL symbols containing SBFD subband, if reinterpretation is allowed. The reason is that it opens the door that gNB doesn’t need to guarantee a legacy RO is fully contained by a UL subband on DL symbol as SBFD aware UE can reinterpret some of parameters and derive a valid RO accordingly. As a result, UE has to implement different mechanisms for determining msg1-FrequencyStart across different symbol type, which complicates UE implementation. Furthermore, we do see duplication between reinterpretation on partial parameters with option 1 and separate configuration with option 2, as both of them provide different T/F resources for RO located on non-SBFD symbol and SBFD symbol respectively. |
| InterDigital | The re-interpretation in Alt. 2 could result in inconsistencies in frequency resources used for RACH transmission if different SBFD symbols have different UL subbands or guardbands. The Option 1 with Alt 1-1 should only be used if RACH configuration for both legacy and additional ROs are within UL subbands of SBFD symbols, where no re-interpretation is required. Otherwise, if the RACH configuration for legacy and additional ROs fall outside of UL subbands in SBFD symbols, the additional ROs are invalid and Option 1 with Alt 1-2 should be used, that is explicit indication of msg1-FrequencyStart in rach-ConfigCommon for SBFD symbols. |
| Samsung | We do not support the FL proposal. In our view, Alt.1 is sufficient. |
| Sharp | We have removed our name from the support for Alt.1 with change remark because we did not submit any proposals related to this. We are open to discuss pros/cons of the alternatives above. |
| LGE | Support only Alt 2-1. Purpose of the RACH configuration Option 1 is to enable SBFD random access without extra signaling overhead at the price of less configuration flexibility. Given the agreement and conclusion, since gNB ensures the ROs are within the UL usable PRBs if legacy ROs are configured in SBFD symbol configured as flexible by tdd-UL-DL-ConfigurationCommon, there is no reason for gNB to configure such non-workable use cases (e.g., configure ROs outside UL usable PRB with msg1-FrequencyStart to be reinterpreted. |
| QC | Alt 2-2 is not clear. Does it mean there is another parameter to explicitly indicate the freq. offset w.r.t the first usable PRB in the UL-SB? If so, how the startRB is interpretated? In general, we think that RAN1 should make a down selection between Alt 1 and Alt 2 and avoid extra RRC overhead to indicate which alternative. This is needed, as RAN1 agreed there is no extra parameters. |
| WILUS | We think that the re-interpretation of msg1-freq.start is necessary to configure the ROs within the UL usable PRBs. The details can be discussed including Alt 2-1, Alt-2-2 and Alt2-3 proposed by CATT. |
| DOCOMO | We don’t see the necessity of re-interpretation. Alt 1 is enough. As the ROs in SBFD flexible symbols are based on legacy interpretation, enhanced re-interpretation for additional-ROs is NOT necessary. Moreover, if flexibility on RO frequency resource for additional-ROs is desired, RACH configuration option 2 can be applied. |
| Ericsson | Do not support. We do not see how these can be the options given that a clear majority wants to start with the lowest PRB of UL usable PRBs and a larger number of companies wants an additional modification compared to the number of companies wanting no modification at all. We propose to first agree whether a reinterpretation should be done or not, and second, if applicable, agree on possible additions or refinements to the reinterpretation. |
| ETRI | We support Alt 2, and between sub-alts, we prefer Alt 2-1. |
| Fujitsu | Support and we prefer Alt 2-1. |
| Sony | Alt 2-1 would still lead to ROs being outside of the UL useable PRBs as shown by several companies. We need a mechanism to contain the ROs within UL useable PRBs and also to avoid any collisions of ROs if wrapped around (e.g. via a MOD function) is used. Alt 2-2 is unclear. |
| Panasonic | Support the proposal. Our preference is Alt 1. |
| Nokia | We prefer to down-select between Alt. 1 and Alt. 2, instead of making this configurable. Our preference is to go for Alt. 1 directly. If other solution other than Alt. 1 is needed, then that should be done in Option 2. Indeed, Option 2, with a dedicated msg1-FrequencyStart parameter for the additional ROs, is a more straightforward and better approach than the reinterpretation of the msg1-FrequencyStart parameter configured by the legacy RACH configuration for Option 1. Besides, Option 2, with simple enhancement, can cover this issue without any additional complexity or overhead compared to Option 1. Option 2 enhancement: For Option 2, in case the SBFD aware UEs did not receive a dedicated parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration. |
| NEC | This proposal in principal is fine but the exact wording may need to be changed depending on wording for additional ROs |
| OPPO | We would like to add one more alternative for further discussion: Alt 2-4: Frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to ⌊msg1-FrequencyStart*scaling factor⌋, where the scaling factor=UL usable PRB size/UL BWP size |
| ITRI | Support and we prefer Alt. 1. |

#### Initial proposal 1-2-2:

Regarding RO validation for RACH configuration Option 1 with Alt 1-1, an RO across SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon is treated the same as an RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | Support |
| Spreadtrum | Support the proposal. In addition, we also identify a RO in RACH configuration option 1, which is also across SBFD symbols and non-SBFD symbols, and they are valid for non-SBFD aware UE. Then, one issue is whether this RO is valid or not for SBFD aware UE, or clarify this RO is not expected by gNB configuration. Considering a RO across SBFD and non-SBFD symbols facing lots of issues, such as frequency resources, UL transmit power, UL spatial domain filter, transition period/gap and phase coherency requirements. So we suggest this RO is avoided by gNB configurations. |
| ZTE | Support. Regarding the case mentioned by Spreadtrum, our understanding is such RO is a legacy RO, and we have agreed that the legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. |
| China Telecom | Support. Regarding Spreadtrum’s comment, we share the similar view with ZTE that this is a legacy RO, thus is valid for SBFD aware UE. |
| Tejas | Support the proposal |
| Xiaomi | Support. Regarding the case raised by Spreadtrum, we think it is valid. From our understanding, the principle of proposal 1-2-2 can be directly applied to this case. We are open to discuss. |
| InterDigital | Support the proposal. |
| Samsung | Ok in principle. But our understanding is that “an RO starting in legacy DL and (possibly) ending in legacy F is treated same as an RO in legacy DL”. The wording of the FL proposal should reflect that. |
| Sharp | Agree with FL. |
| LGE | We support proposal 1-2-2. As for a RO across SBFD symbols and non-SBFD symbol, during the discussion last meeting, my observation was majority view was it is not configured by gNB for RACH configuration Option 1. The intention and benefit of the Spreadtrum’s case need to be clarified. |
| QC | Support. This is in line with FL proposal 1-1-2 as such RO is not valid for legacy UE as resource overlap with DL symbol. |
| WILUS | Support the proposal |
| DOCOMO | Fine with the proposal. |
| Ericsson | Support, with the understanding that at least one symbol must be configured DL. We are not sure this agreement is needed since both DL and F symbols are configured as SBFD symbols. |
| ETRI | Support the proposal. |
| Fujitsu | Support. Regarding the problem proposed by Spreadtrum, whether RO across SBFD and non-SBFD symbols is treated as same as legacy-RO or not depends on whether the RO is valid or not for legacy UEs. |
| Sony | Support |
| Panasonic | Support |
| Nokia | We are open for discussion. |
| NEC | Support |
| OPPO | Support |
| ITRI | Support. |

#### Initial proposal 1-2-3:

For RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, support the following conditions on top of what was agreed in RAN1#116bis for RO validation.

- Condition#1: A valid RO starts at least Ngap symbols after a last downlink non-SBFD symbol.

- Condition#2: A valid RO starts at least Ngap symbols after the SSB.

- Note: The Ngap here is the same as the Ngap in the current spec.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | We support Condition#1. For Condition#2, we may need to wait a bit for the discussions in AI 9.3.1, where it is under discussion whether SSB symbols fall back to full DL symbols. In that case, SSB symbols are DL non-SBFD symbols. If it is agreed that SSB symbols do not fall back to full DL symbols, we are not quite clear whether Condition#2 is needed and would like to understand the reasons from proponents. |
| Spreadtrum | Support Condition#1 and 2. We would like to clarify more about condition #3, which we also support. It intends to avoid too much DL-UL switching points in a slot in the legacy. This reason also valid for SBFD. It was agreed SSB symbols cannot have UL in 9.3.1. There was also a related proposal to set the whole SSB slot as DL slot, which can have less switching points. |
| ZTE | Support |
| New H3C | OK |
| China Telecom | Support. |
| Tejas | Support the proposal |
| Xiaomi | Support condition#1. Condition#2 may be not needed as SBFD gNB supports simultaneous reception and transmission. |
| InterDigital | Support the proposal. |
| Samsung | Do not support |
| Sharp | Support. |
| LGE | Generally fine with the initial proposal and FL’s intention. However, depending on the agreement on whether the slot containing SSB symbols is considered as a full DL slot or SBFD symbols when SSB collides with UL transmission in 9.3.1, values of Ngap could be impacted (potentially different Ngap symbols under two Conditions). So better to defer the decision at least for Condition#2 in 9.3.2. |
| QC | Condition#2 is only sufficient. Condition #1 should be avoided. The UE may not be scheduled in the last DL-symbols before the RO. Hence, this will always restrict UE/gNB from utilization the RO. In addition, if there is not enough timeline for Rx-Tx UE switching between DL reception and valid-RO, it should be considered as a collision scenario as per agreement in AI 9.3.1. |
| WILUS | Support the proposal. And it should also be further discussed that a valid RO starts at least X>=0 (FFS the value) symbols after a last SBFD symbol with DL reception for a given UE. |
| DOCOMO | Support. |
| Ericsson | We do not think condition 1 is strictly necessary considering a flexible symbol can be expected to be located in-between the DL and SBFD symbols within the normal Rx-to-Tx transition restrictions. What is the difference to this compared to a gap between DL and F symbols in Rel-15? |
| ETRI | Support |
| Sony | Do not support. |
| Nokia | We are generally fine with the proposal. However, can we clarify the value of Ngap? Is that similar to legacy? |
| OPPO | OK |
| ITRI | Support. |

#### Initial proposal 1-2-4:

For RACH configuration Option 1 with Alt 1-1, the legacy SSB-RO mapping rule is reused for additional-ROs.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | Support |
| Spreadtrum | Support |
| ZTE | Support |
| New H3C | OK |
| China Telecom | Support |
| Tejas | Support in general. But does the RO counting considered as separate? |
| Xiaomi | Support. |
| InterDigital | Clarification is needed, including the question form Tejas. We think it is needed FFS on whether the SSB-RO mapping for additional ROs is separate from the SSB-RO mapping for the legacy-ROs. NW should be able to configure UEs to reuse OR use another (indicated) SSB-RO mapping in SBFD symbols based on NW’s traffic, load, etc. |
| Samsung | Support |
| Sharp | Support |
| LGE | Support |
| QC | Support, however, we would like to add FFS on these two aspects for further discussion. FFS: The set SSB for SSB-to-additional RO mappings and start SSB index. The motivation is explained in our tdoc: - The SSB mapping to SBFD-ROs can start from certain SSB index to further reduce the latency for SBFD-aware UE. This is clarified by the example in figure below where SSB mapping in SBFD-ROs start from SSB#2. This will create a shorter SSB-RO mapping cycle across both legacy and SBFD ROs Not all SSBs should be associated/mapped to SBFD-ROs. For example, due to interference at gNB, some SSBs beam directions may suffer from clutter and inter-gNB CLI and can’t be used for PRACH reception. Also, the additional SBFD-ROs may be used to improve PRACH coverage/capacity in certain directions, e.g. UE cluster and hence there is no need to map all SSBs directions. |
| WILUS | Support |
| DOCOMO | Support. |
| Ericsson | Support, provided this assumes a separate SSB-to-RO mapping compared to legacy. |
| ETRI | Support. |
| Fujitsu | Support |
| Sony | Support |
| Panasonic | Support |
| Nokia | Support |
| NEC | Support |
| OPPO | Support |
| ITRI | Support |

#### Initial proposal 1-2-5:

For RACH configuration Option 1 with Alt 1-1, network can configure separate PRACH power control parameter(s) for additional-ROs from legacy-ROs. Down-select from the following alternatives:

- Alt 1: configure absolute value(s) of one or several existing power control parameters for additional-ROs.

  - FFS which parameters, e.g., preambleReceivedTargetPower, powerRampingStep and preambleTransMax

- Alt 2: configure a power offset for additional-ROs compared to legacy-ROs.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | Support. We prefer Alt 1. |
| ZTE | Support |
| Tejas | Support. We prefer Alt 1. Alt 2 does not offer the flexibility that Alt 1 offers. |
| Xiaomi | The spirit of option 1 is to fully reused the same RACH configuration as legacy UE. If separate parameter is needed, option 2 can be used. |
| InterDigital | Support the proposal. |
| Samsung | Support |
| Sharp | Support |
| LGE | If the separate power control parameters are majority view, we can live with it. |
| QC | We are supportive of having separate PC parameters for option. However, we want to clarify first whether this is okay given the earlier agreement that single PRACH configuration (option -1) is based on the existing parameters of the single RACH configuration). |
| WILUS | Support |
| DOCOMO | Fine with the proposal. In our understanding, Alt 2 with power offset is equal to separate configuration of preambleReceivedTargetPower. |
| Ericsson | Support. Prefer Alt. 1 since this is similar to what we already have for legacy and hence implies the least implementation efforts. |
| ETRI | We support Alt 1. |
| Fujitsu | Support |
| Sony | Support |
| Panasonic | We are not supportive the proposal because RACH configuration Option 1 with Alt 1-1 was agreed as "only based on the existing parameters of the single RACH configuration". As Qualcomm commented, whether the separate PRACH power control parameter is in line with the earlier agreement would need to be clarified. |
| Nokia | We do not support the proposal. Adding a separate PRACH power control in Option 1 will pull the discussion on adding some other parameters in the future, hence going back to discussing Option 1 Alt 1-2, which was already discarded. Therefore, it is highly recommended that no new parameters be added to Option 1 to simplify the discussion in the future. Besides, Option 2, with the enhancement previously mentioned by Nokia in initial proposal 1-2-1 , can already cover the issue of having separate PRACH power control without any additional overhead compared to Option 1 with the additional PRACH power control parameters. Therefore, further specification efforts for Option 1 are not justified. |
| NEC | Support |
| OPPO | Support |
| ITRI | Support |

## Issue#1-3: Details of RACH configuration Option 2

### Submitted proposal

#### RACH resource

| Company | Proposals |
| --- | --- |
| MediaTek | - For RACH configuration Option 2, support the following. - Do not provide all parameters in rach-ConfigCommon in the additional RACH configuration. - If a parameter is not provided in the additional RACH configuration, UE uses the corresponding parameter in the legacy RACH configuration - FFS which parameters are included in the additional RACH configuration - Reuse the existing SSB-RO mapping rules - SSB-RO mapping can be performed separately for valid ROs on SBFD symbols and valid ROs on SBFD symbols. |
| New H3C | Proposal 7: Separated configuration on PRACH by RRC signalling can be supported. Proposal 8: Frequency start parameter for SBFD is supported for determining frequency start of PRACH Occasion in SBFD. |
| Huawei, HiSilicon | Proposal 2: For RACH configuration Option 2, all parameters in RACH-ConfigCommon are included in the additional RACH configuration, and more candidate values are included for the following parameters - Add {3,5,6,7} into the candidate values of msg1-FDM - Add {n1, n2} into the candidate values of preambleTransMax Introduce the following parameters - Multiple parameters preambleReceivedTargetPower to set target power for FDM-ROs - Multiple parameters preamblemaxOutputPower to limit maximum power for FDM-ROs |
| Spreadtrum | - Parameters in additional RACH configuration in Option 2 have three types: - Type1: parameters should be included, such as rach-ConfigGeneric - Type2: parameters cannot be included, and the same parameters in legacy RACH configuration always applied. E.g rsrp-ThresholdSSB/rsrp-ThresholdSSB-SUL, msg1-SubcarrierSpacing, msg3-transformPrecoder - Type3: parameters may or may not be included. If included, they are applied, if not, the same parameters are reused. |
| ZTE, Sanechips | Proposal 9: Parameters in rach-ConfigCommon except for the following can be reused for the additional RACH configuration. − preambleTransMax, ra-ResponseWindow, groupBconfigured, ra-ContentionResolutionTimer and rsrp-ThresholdSSB-SUL |
| LG Electronics | Proposal 5: With two separate RACH configurations, same or different preamble formats by each PRACH configuration index can be configured by gNB. (i.e., one for legacy RACH configuration, one for additional RACH configuration) |
| Ericsson | Proposal 11 Support the following additional parameters for the additional RACH configuration (in addition to power control that is discussed elsewhere): a. prach-ConfigurationIndex b. msg1-SubcarrierSpacing c. msg1-FrequencyStart d. msg1-FDM e. ssb-perRACH-OccasionAndCB-PreamblesPerSSB f. totalNumberOfRA-Preambles g. prach-RootSequenceIndex |
| Kookmin University | Proposal 7: For additional RACH configuration in Option 2, at least the following parameters are excluded from rach-ConfigCommon: - preambleTransMax, ra-ResponseWindow, ra-ContentionResolutionTimer, rsrp-ThresholdSSB, msg1-SubcarrierSpacing, msg3-transformPrecoder - msg1-FrequencyStart (if re-interpretation is agreed) |
| Xiaomi | Proposal 7: With option 2, either same or different preamble format can be configured via separate PRACH configuration. |
| CATT | Proposal 3: For RACH configuration Option 2, at least the following parameters should be included in additional RACH configuration. - Frequency resource configuration - Time resource configuration and preamble format - PRACH format related parameters - SSB-to-RO mapping parameters - Power control parameters - Preamble partitioning parameters |
| Sony | Proposal 5: For PRACH configuration Option 2 (i.e., Use two separate PRACH configurations, including one legacy RACH configuration and one additional PRACH configuration), all the parameters currently in rach-ConfigCommon are included in the additional PRACH configuration. |
| Fujitsu | Proposal 4: The legacy additional RACH configuration framework introduced for feature combination can be reused for Option 2 additional RACH configuration. |
| Hyundai | Proposal #3: - RAN1 needs to consider allowing UE to readjust configured the number of repeated preamble sequences and it can transmit as many as possible when configured preamble format may not be used properly due to the limited time duration. |
| Samsung | Proposal 8: For RACH configuration option 2, all parameters currently in rach-ConfigCommon should be included in the additional RACH configuration. |
| Nokia, Nokia Shanghai Bell | Proposal 4 For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration parameters. Only the legacy RACH configuration will be configured with this parameter. Proposal 5 For Option 2, in case the SBFD aware UEs did not receive a dedicated parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration. |
| Lenovo | Proposal 3: For option 2(i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), all the parameters currently in rach-ConfigCommon should be kept. |
| NTT DOCOMO | Proposal 6: For RACH configuration option 2, following parameters are included in the additional RACH configuration: - Parameters for RO time domain resources: prach-ConfigurationIndex - Parameters for RO frequency domain resources: msg1-FDM, msg1-FrequencyStart - Parameters for SSB-to-RO mapping: ssb-perRACH-OccasionAndCB-PreamblesPerSSB, totalNumberOfRA-Preambles |
| Qualcomm | Proposal 3: For SBFD random access operation using option 2, support the following PRACH parameters for the additional PRACH configuration: - Time resource configuration - Freq. resource configuration - Power control parameters - SSB-RO mapping parameters - SSB parameters - PRACH subcarrier spacing. |
| CEWiT | Proposal 5: Additional RACH configuration providing additional set of parameters that act upon legacy RACH configuration to generate additional ROs is supported. Proposal 6: Introducing additional parameter in additional RACH configuration, that act upon the conventional frequency domain RACH configuration, to confine ROs within UL-SB in SBFD symbol is supported. |
| WILUS | Proposal 2: For RACH configuration Option 2, it may be advisable to include parameters such as msg1-FrequencyStart, msg1-FDM, ssb-perRACH-OccasionAndCB-PreamblesPerSSB, which are deemed to require reinterpretation in Option 1, and prach-ConfigurationIndex for setting RO resources on the time domain in an additional RACH configuration. |

#### RO validation

| Company | Proposals |
| --- | --- |
| New H3C | Proposal 9: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, the following alternatives 2-3 is supported: • Alt 2-3: o The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. o FFS: The case where the additional-ROs partially overlap with non-SBFD symbols Proposal 10: If partial or all frequency resources of RO falls outside of UL BWP frequency resource or UL SBFD frequency resource, the RO is considered as invalid. Proposal 11: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, • for the additional-ROs in SBFD symbols configured by additional RACH configuration, they are valid if at least: o time and frequency resource of the RO are fully within UL usable PRBs o FFS: Other condition. |
| Huawei, HiSilicon | Proposal 6: For RACH configuration Option 2, the additional-ROs configured by additional RACH configuration in non-SBFD symbols and SBFD symbols configured as flexible symbols by tdd-UL-DL-ConfigurationCommon should be considered as invalid for SBFD aware UEs. Proposal 7: For RACH configuration Option 2, the preamble format configured by additional RACH configuration determines the validity of additional-ROs across SBFD symbols and non-SBFD symbols. - If a short preamble format is configured, the additional-ROs across non-SBFD symbols and SBFD symbols is considered as invalid for SBFD-aware UEs. - If a long preamble format is configured, the additional-ROs across non-SBFD symbols and SBFD symbols is considered as valid for SBFD-aware UEs. |
| Spreadtrum | - For RACH configuration Option 2, the additional-ROs are valid if time and frequency resource of the RO are fully within UL usable PRBs in SBFD symbols, and the Condition#1/2/3 are satisfied. - Condition#1: A valid RO starts at least Ngap symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least Ngap symbols after the SSB. - Condition#3: A valid RO does not precede a SSB in the PRACH slot. - Alt 2-3 is adopted. The additional-ROs in non-SBFD symbols are invalid for SBFD-aware UEs. - For a configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration, and up to one SBFD/non-SBFD transition. |
| ZTE, Sanechips | Proposal 6: The additional ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs - For collision handling between additional ROs and legacy ROs in non-SBFD symbols, reuse the same rules defined for collision between additional ROs and legacy ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For power control of the additional ROs configured by additional RACH configuration, reuse the same mechanism defined for Option 1. |
| Tejas | Proposal 11: A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be considered as valid for RRC_IDLE/RRC_INACTIVE state too. S |
| CMCC | Proposal 10: For RACH configuration Option 2, both Alt 2-3 and Alt 2-4 are supported, and network configures which alternative should be used. Proposal 11: For RACH configuration Option 2, the following conditions are supported for RO validation of additional-ROs. - A valid RO should not overlap with SSB - A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. |
| LG Electronics | Proposal 9: RAN1 to support Option 2 Alt 2.4 since it provides more configuration flexibility and less specification impact. Proposal 10: For Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD aware UEs, - For the additional ROs configured by additional RACH configuration, they are valid if at least, - time and frequency resource of the RO are fully within UL usable PRBs in SBFD symbol, and not overlapped with SSB - For the additional RO in non-SBFD symbol configured by additional RACH configuration, the legacy RO validation rules are followed for SBFD aware UEs. - Other conditions in Option 1 are followed for SBFD aware UEs. Proposal 11: For dynamic or semi-static DL vs. valid RO, - Reuse the existing collision handling principles for HD-FDD RedCap UE, i.e., leave to UE implementation. |
| China Telecom | Proposal 6: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), support Alt 2-4 (o The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs). Proposal 9: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, - for the additional-ROs in SBFD symbols configured by additional RACH configuration, they are valid if at least: - time and frequency resource of the RO are fully within UL usable PRBs - FFS: Other condition. |
| Ericsson | Proposal 8 For the additional RACH configuration, support further RO validation restrictions, e.g., by an RO puncturing bitmap. Proposal 9 Additional ROs entirely located in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs (Alt. 2-3). Proposal 10 Unless Alt. 2-3 is agreed, support configuration of RO locations into: a. SBFD symbols only (default configuration) b. SBFD and UL/F symbols (per previous agreement) c. UL/F symbols only (new configuration) |
| vivo | Proposal 6: For SBFD aware UEs and for RACH configuration Option 2, additional ROs configured by additional RACH configuration and only in non-SBFD symbols are invalid. Proposal 7: The following RO validation rule can be considered for RACH configuration Option 2: A RO configured by additional RACH configuration in SBFD symbols configured as downlink/flexible by tdd-UL-DL-ConfigurationCommon is valid if: - Time and frequency resource of the RO are fully within UL usable PRBs, and - The RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol, and - The RO starts at least X>0 (FFS the value) symbols after the SSB, and - The RO does not precede a SSB in the PRACH slot. |
| OPPO | Proposal 5: For RACH configuration Option 2, Alt 2-3 is adopted, i.e., the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. Proposal 6: For RACH configuration Option 2, the additional-ROs in SBFD symbols configured by additional RACH configuration are valid if at least: - time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB - FFS other conditions. |
| Xiaomi | Proposal 5:For RACH configuration Option 2 to support random access operation for SBFD-aware UEs in RRC CONNECTED state, Alt 2-3 is supported: - Alt 2-3: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD aware UEs. Proposal 6:For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, - For the additional-ROs in SBFD symbols configured by additional RACH configuration, they are valid if at least: - Frequency resource of the RO is fully within UL usable PRBs, and - Not overlapped with SSB (Ngap can be 0 for all preamble SCS), and - Starts at least X>=0 symbols after a last downlink symbol, and - Value of X depends on required transition time |
| CATT | Proposal 7: For RACH configuration Option 2, the additional ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. Proposal 8: For RACH configuration Option 2, a RO in SBFD symbols is valid if: - Starts at least 𝑁gap symbols after a last non-SBFD downlink symbol. - Not overlap with a SS/PBCH block. - FFS frequency resource of the RO are fully within UL usable PRBs |
| Sony | Proposal 6: For PRACH configuration Option 2 (i.e., Use two separate PRACH configurations, including one legacy PRACH configuration and one additional PRACH configuration) an RO is valid if all of the following conditions are met: - Time and frequency resource of the RO are fully within UL usable PRBs - The RO does not overlapped with SSB - Alt 2-4: The additional-ROs in non-SBFD symbols configured by additional PRACH configuration can be valid for SBFD-aware UEs. |
| Fujitsu | Proposal 5: For Option 2, an RO for SBFD-aware UEs overlapping with valid ROs configured by legacy RACH configuration is invalid. Proposal 6: Clarify the benefits of validating ROs in non-SBFD symbols using the additional RACH configuration. |
| NEC | Proposal 4: - For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), support Alt 2-3. Proposal 5: • For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), an RO configured by an additional RACH configuration is considered invalid if it overlaps in the time domain with a valid RO configured by legacy RACH configuration. |
| Panasonic | Proposal 4: For PRACH configuration Option 2, support Alt 2-3 for the additional-ROs in non-SBFD symbols configured by additional RACH configuration. |
| Samsung | Proposal 7: For RACH configuration option 2, support Alt 2-3 |
| Nokia, Nokia Shanghai Bell | Proposal 7 RAN1 to discuss on whether/how to apply the RO mask index, when configured, on the legacy and new ROs, for both PRACH configuration Option 1 and Option 2. Proposal 10 For RACH configuration Option 2, RAN1 considers additional-ROs in non-SBFD symbols configured by additional RACH configuration as valid ROs for SBFD-aware UEs. |
| Lenovo | Proposal 4: For option 2(i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. (Alt 2-3) Proposal 5: For option 2(i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), - RO validation rule for the additional configuration is same as the one for option 1 with Alt 1-1. - FFS: how to handle the collision between legacy valid ROs and additional valid ROs. |
| Transsion Holdings | Proposal 2：The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. |
| ETRI | Proposal 6: The validity test introduces frequency resource of an RO in SD symbols. Proposal 7: The RO that have two symbol types are valid for RRC non-connected modes as well as RRC connected mode. Proposal 8: Strive to a unified framework for RO validation and SSB-RO mapping rule for both options, for connected UEs. Proposal 9: A PRACH preamble transmission can be across SBFD symbols and non-SBFD symbols, which is applicable for RRC connected and RRC non-connected modes. |
| InterDigital | Proposal 4. For RACH config Option 2 (based on two separate RACH configs), support Alt 2-4 to allow SBFD UEs to use additional-ROs that overlap with non-SBFD symbols. |
| Google | Proposal 3: If Option 2, using two RACH configurations, is supported then support: • Alt 2-3: o The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. |
| Sharp | Proposal 6: Support Alt 2-4. The additional ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs. |
| Apple | Proposal 4: For RACH configuration Option 2, the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. Proposal 7: For RACH configuration Option 2, for additional ROs configured in SBFD symbols by additional RACH configuration, the valid RO should be within SBFD UL symbols, fully within the SBFD UL subband, and not overlapped with the SSB. |
| KT | Proposal 2. For RACH configuration Option 2, the additional-ROs in non-SBFD symbols configured by additional RACH configuration should be invalid for SBFD-aware UEs. |
| NTT DOCOMO | Proposal 9: The RO validation rule for ROs in SBFD DL symbols designed for RACH configuration option 1 is reused for RO validation for ROs in SBFD symbols configured by additional RACH configuration for RACH configuration Option 2 (i.e., additional RACH configuration). Proposal 10: For RACH configuration Option 2 (i.e., additional RACH configuration), the ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. |
| Qualcomm | Proposal 4: Support Alt 2-3 for PRACH configuration option 2 where the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. Proposal 5: For RACH configuration Option 2, SBFD-aware UE first determine valid legacy ROs in non-SBFD symbols (UL and FL symbols) then UE determines valid ROs by the additional PRACH configuration in SBFD-symbols. - SBFD-RO in FL symbols that overlap with legacy-ROs are considered invalid. Proposal 8: RAN1 to discuss timeline of SBFD-aware UE for switching SBFD to non-SBFD mode when transmission a preamble in a valid RO starting in SBFD symbols and ending in non-SBFD symbols. |
| CEWiT | Proposal 1: The ROs, configured by additional RACH configuration in option 2, outside UL-SB or overlapping with frequency resources outside UL-SB are invalid for SBFD aware UE. Proposal 2: The ROs, which are valid for legacy UEs, are valid for SBFD aware UE irrespective of its alignment with SBFD subbands. Proposal 3: ROs, configured by additional RACH configuration, in non-SBFD symbol or partially overlapping with non-SBFD symbol is invalid (Alt 2-3). Proposal 4: For an RO across SBFD symbols and non-SBFD symbols, the UE uses frequency resources in the RO overlapping with UL-SB for transmission of RACH in both SBFD and non-SBFD segment. |
| WILUS | Proposal 6: For RACH configuration Option 2, we propose to set the RACH occasion in non-SBFD symbols as valid according to both the legacy RACH configuration and the additional RACH configuration by validity check of the PRACH slot as the legacy manner. (i.e., Alt 2-4) ‐ However, it should be further configured that the SSB-RO mapping in valid RO for legacy UEs and in valid RO across SBFD and non-SBFD symbols for SBFD-aware UEs should be set identically in the same symbols. Proposal 7: For RACH configuration Option 2, it is suggested that the configured ROs in SBFD symbols configured by additional RACH configuration are valid if at least time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB as the same manner with RACH configuration Option 1. ‐ FFS whether it is permissible for the time and frequency resources of the RO to be fully within the UL usable PRBs, even if they overlap with the SSB in the time domain. Proposal 8: Regardless of which RACH configuration option is applied, Condition#1/#2 of the following conditions should be further considered to be necessary for RO validation as in legacy NR system. And Condition#3 can be relaxed and Condition#4 should be also discussed for RO validation. ‐ Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. ‐ Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB. ‐ FFS: Condition#4: A valid RO starts at least X>=0 (FFS the value) symbols after a last SBFD symbol with DL reception for a given UE. |

#### SSB-RO mapping

| Company | Proposals |
| --- | --- |
| MediaTek | - For RACH configuration Option 2, support the following. - Do not provide all parameters in rach-ConfigCommon in the additional RACH configuration. - If a parameter is not provided in the additional RACH configuration, UE uses the corresponding parameter in the legacy RACH configuration - FFS which parameters are included in the additional RACH configuration - Reuse the existing SSB-RO mapping rules - SSB-RO mapping can be performed separately for valid ROs on SBFD symbols and valid ROs on SBFD symbols. |
| New H3C | Proposal 2: Mapping SSB to RO in SBFD symbol and SSB to RO in non-SBFD symbol separately should be supported. Proposal 3: If supporting unified mapping SSB to RO in SBFD symbol and non-SBFD symbol, dedicated preamble allocation for SBFD-aware UE should be considered. Proposal 4: Regardless of unified mapping SSB to RO in SBFD symbol and non-SBFD symbol or separated mapping SSB to RO in SBFD symbol and non-SBFD symbol, if RO isn’t mapped with SSB, the RO is invalid and can’t be used for transmitting preamble. |
| Huawei, HiSilicon | Proposal 3: For SBFD-aware UEs and for RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), • For the legacy-ROs configured by legacy RACH configuration, the legacy SSB-RO mapping rules are followed • For the additional ROs configured by additional RACH configuration, separate SSB-to-RO mapping will be used Proposal 5: For RACH configuration Option 2, a descending order of SSB index is used for the SSB-RO mapping for the additional ROs configured by additional RACH configuration. |
| Spreadtrum | - For RACH configuration Option 2, the legacy SSB-RO mapping rule can be used for the additional-ROs, and they have separate SSB-RO mapping with legacy ROs. |
| ZTE, Sanechips | Proposal 8: For RACH configuration Option 2, − Support separate SSB-RO mapping between the additional ROs configured by additional RACH configuration and the ROs configured by legacy RACH configuration. − Support joint SSB-RO mapping for additional ROs in SBFD symbols and additional ROs in non-SBFD symbols. |
| CMCC | Proposal 12: For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration. - For Alt 2-4, the SSB-RO mapping is jointly performed for the additional-ROs in SBFD symbols and non-SBFD symbols. |
| LG Electronics | Proposal 13: For Option 2, ensuring that the legacy ROs in non-SBFD symbols configured by legacy RACH configuration and the additional ROs in non-SBFD symbols configured by additional RACH configuration does not overlap, we support separate legacy SSB-RO mapping rule for additional RACH configuration and legacy RACH configuration respectively. - For additional ROs configured by additional RACH configuration for SBFD aware UE, further enhancement on legacy SSB-RO mapping rule such as allowing subset of SSBs or different start SSB index for additional ROs could be discussed. |
| TCL | Proposal 4: Consider the following alternative options for SSB to ROs mapping: - Option 1: Separate SSB to ROs mapping of ROs configured in SBFD symbols and ROs configured in non-SBFD symbols. - Option 2: Joint ROs configured in SBFD symbols with ROs in non-SBFD symbols based on their indexes and map to SSBs. |
| Korea Testing Laboratory | Proposal 2. For SBFD-aware UEs in RRC CONNECTED state and RACH configuration Option 2, supports applying legacy SSB-RO mapping rules separately to additional-ROs and legacy-ROs. |
| vivo | Proposal 8: For Option 2 for RACH configuration, support separate SSB-to-RO mapping for ROs configured by additional RACH configuration. |
| Kookmin University | Proposal 8: For SBFD-aware UEs in RRC CONNECTED state and RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration. |
| OPPO | Proposal 8: For RACH configuration Option 2, support separate SSB-RO mapping for ROs configured by legacy RACH configuration and ROs configured by additional RACH configuration. |
| CATT | Proposal 10: For RACH configuration Option 2, separate SSB-to-RO mapping is performed for the additional ROs configured by the additional RACH configuration and legacy SSB-to-RO mapping rule is reused. |
| Sony | Proposal 7: For PRACH configuration Option 2 (i.e., Use two separate PRACH configurations, including one legacy PRACH configuration and one additional PRACH configuration), the SBFD UE performs two legacy SSB-RO associations, one on each PRACH configuration, using different RO validation rule, i.e.: - 1st SSB-RO association performed on legacy PRACH configuration using legacy RO validation rule - 2nd SSB-RO association performed on the additional PRACH configuration using a new RO validation rule, where an RO is valid if: - The RO is contained in UL OFDM symbols - The RO is contained in useable UL PRBs - The RO does not overlap SSB |
| NEC | Proposal 7: • The relationship for RO in SBFD symbols and SS/PBCH block should be separately defined for option 2. |
| Panasonic | Proposal 5: For PRACH configuration Option 2, use the legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration. |
| Samsung | Proposal 9: For RACH configuration option 2, the legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration is used. |
| Nokia, Nokia Shanghai Bell | Proposal 6 RAN1 to study SSB-to-RO mapping rules for PRACH configuration Option 2 (regardless of whether Alt. 2-3 or 2-4 is adopted), such that for a set of ROs configured by the additional PRACH configuration (Set 1) that overlaps in time with another set of ROs configured by the legacy PRACH configuration (Set 2), ROs in the Set 1 are mapped to only the one or more SSB indices that are mapped to the Set 2. |
| Lenovo | Proposal 6: For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 2(i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), - For the legacy-ROs, including the ROs configured by legacy configuration in non-SBFD symbols and the ROs configured by the additional ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed. - For the additional ROs in SBFD symbols, separate SSB-RO mapping will be used. |
| ETRI | Proposal 8: Strive to a unified framework for RO validation and SSB-RO mapping rule for both options, for connected UEs. Proposal 17: Regarding Option 2, SSB-RO mapping is performed at non-SBFD symbols in the legacy association period firstly, and then additionally is done at SBFD symbols. |
| Google | Proposal 4: For additional ROs in SBFD symbols, the legacy SSB-RO mapping rule is used |
| Apple | Proposal 8: For RACH configuration Option 2, SSB to RO association is performed independently for ROs in the SBFD symbols. |
| NTT DOCOMO | Proposal 11: The SSB-to-RO mapping for valid ROs in SBFD symbols configured by the additional RACH configuration is separate from SSB-to-RO mapping of valid ROs configured by the legacy RACH configuration. - Legacy SSB-to-RO mapping rule (i.e., preamble domain first, then frequency domain, then time domain) is applied. - SSB-to-RO mapping parameters (e.g., ssb-perRACH-OccasionAndCB-PreamblesPerSSB, totalNumberOfRA-Preambles) configured in the additional RACH configuration are applied. |
| Qualcomm | Proposal 10: For both option-1 and option-2 of PRACH configuration, RAN1 to study the determination of the association period and association period pattern for the SBFD-ROs. |
| ITRI | Proposal 3: - For SBFD-aware UEs in the RRC CONNECTED state with RACH configuration Option 2: - A separate SSB-RO mapping should be used for the additional RACH configuration. - The legacy SSB-RO mapping rule could be reused. |
| WILUS | Proposal 5: For RACH configuration Option 2, ‐ We propose to have a separate/independent SSB index to RO mapping for new validated ROs in SBFD symbols from the SSB index to RO mapping for legacy ROs regardless of being configured as downlink or flexible by tdd-UL-DL-ConfigurationCommon. ‐ For validated ROs across SBFD symbols and non-SBFD symbols in the same slot or across slots, we propose to map the same SSB index to validated ROs across SBFD symbols and non-SBFD symbols in the same slot or across slots. |

#### RACH configuration table

| Company | Proposals |
| --- | --- |
| MediaTek | Proposal 3: Introduce a new parameter to provide additional slot numbers for ROs on SBFD symbols when using the existing random access configurations table for unpaired spectrum in FR2 (i.e., Table 6.3.3.2-4 in TS38.211) Proposal 4: For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) Proposal 5: Introduce a new parameter to provide additional subframe numbers for ROs on SBFD symbols when using the existing random access configurations table for unpaired spectrum in FR1 (i.e., Table 6.3.3.2-3 in TS38.211) |
| New H3C | Proposal 13: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, For FR1, At lease following Alt.1 is supported. - Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. |
| Huawei, HiSilicon | Proposal 9: For RACH configuration Option 2, for FR1, reuse random access configuration tables for paired spectrum for RACH in SBFD symbols; for FR2, introduce time offsets {0,1,2,3} to determine the slot number for ROs in SBFD symbols. |
| Spreadtrum | - For RACH configuration Option 2, using existing random access configurations table for unpaired spectrum without new parameter(s) to determine the slot number for additional ROs. |
| ZTE, Sanechips | Proposal 7: The existing random access configuration tables (i.e., Table 6.3.3.2-3 and Table 6.3.3.2-4 in TS38.211) can be reused without additional enhancement on the table itself. − No new parameter is introduced to determine the slot number for ROs in SBFD symbols. − For PRACH format 1, 2 in FR1, the start point of a configured RO can be adjusted to the start of SBFD symbols in the same radio frame. |
| Tejas | Proposal 6: Alt 1 (i.e., existing random access configurations table for unpaired spectrum in Table 6.3.3.2-3 in TS38.211) with time offset parameter can support all possible RO in SBFD symbols. |
| CMCC | Proposal 13: For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration: - For FR1, use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211) |
| LG Electronics | Proposal 8: For Option 2, support Alt 1 (i.e., use existing random access configurations table for unpaired spectrum). - Introduce a new parameter to configure an additional subframe/slot number. - i.e., the additional subframe/slot number could replace or combine with the configured subframe/slot number by the PRACH configuration index from existing random access configurations table for SBFD aware UE |
| China Telecom | Proposal 4: Postpone the discussion on which random access configurations table to be used until decision has been made with respect to whether the additional-ROs in non-SBFD symbols configured by additional RACH configuration is valid for SBFD-aware UEs for Option 2. |
| TCL | Proposal 3: For RACH configuration option 2, in the addition RACH configuration, consider an explicit indication of the sub-frame where SBFD operation is configured. |
| Korea Testing Laboratory | Proposal 1. For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211). |
| Ericsson | Proposal 7 The additional RACH configuration uses the existing PRACH configurations tables for unpaired spectrum for both FR1 and FR2, i.e., Table 6.3.3.2-4 and Table 6.3.3.2-3, respectively (Alt. 1). |
| vivo | Proposal 3: For Option 2 of random access configurations for FR1 and unpaired spectrum, use existing random access configurations table for paired spectrum/supplementary uplink. Proposal 4: For Option 2 of random access configurations for FR2 and unpaired spectrum, use existing random access configurations table for unpaired spectrum without new parameter(s). |
| Kookmin University | Proposal 6: For RACH configuration Option 2, new parameter(s) to determine the slot number for ROs in SBFD symbols is not introduced. |
| OPPO | Proposal 7: For RACH configuration Option 2 and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration: - For FR 1, use existing random access configurations table defined for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211); - For FR 2, use existing random access configurations table defined for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) |
| Xiaomi | Proposal 8: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, adopt alt 1: - Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - not introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - For FR1, adopt alt 1 and alt 2: - Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - not introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols - Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211) |
| CATT | Proposal 4: Discuss whether to enhance the existing random access configuration tables for unpaired spectrum for the following case: - PRACH format 0/3 with periodicity of larger than 10ms in TDD configurations where subframe 4 and 9 are UL subframes, e.g. DDDSUDDSUU and DDDDDDDSUU Proposal 5: For RACH configuration Option 2, it is not necessary to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. |
| Panasonic | Proposal 7: For random access configuration table for PRACH configuration Option 2, support Alt 1 with reusing subframe/slot offset prach-ConfigurationSOffset-IAB for specified for Rel.16 IAB. |
| Samsung | Proposal 4: For RACH configuration option 2 and interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration in FR1 or FR2: - Support Alt.1 (existing random access configurations table) with new/additional subframe shift/offset parameter to determine the subframe number for ROs in SBFD symbols. |
| Nokia, Nokia Shanghai Bell | Proposal 8. For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) with a bitmap indicating the slot numbers for ROs in SBFD symbols. - Details are FFS. - For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) with a bitmap indicating the subframe numbers for ROs in SBFD symbols. - Details are FFS. |
| Lenovo | Proposal 2: For option 2(i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), - for FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) (Alt 1) |
| Transsion Holdings | Proposal 4: For RACH configuration Option 2, Alt 1 for FR 1 should be specified. |
| InterDigital | Proposal 2. Support introducing time offset values to determine the slot/subframe number for ROs in SBFD symbols, for RACH configuration Option 2. |
| Apple | Proposal 5: For RACH configuration Option 2, FDD PRACH configuration table, i.e., Table 6.3.3.2-2, is used to indicate ROs in the SBFD symbols in FR1. Proposal 6: For RACH configuration Option 2, existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) in FR2 is used to indicate ROs in the SBFD symbols. |
| NTT DOCOMO | Proposal 8: For interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, support Alt 1 for FR1 and FR2, i.e., use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211 for FR1, and Table 6.3.3.2-4 in TS38.211 for FR2). - FFS for the necessity of enhancements to support PRACH format 1 and format 2 in SBFD symbols for FR1. - If the necessity is justified, slot/subframe offset parameters in the additional RACH configuration can be considered. |
| Qualcomm | Proposal 6: Support (Alt-1), as a common design for both FR1 and FR2, using existing random access configurations table for FR1 and FR2 unpaired spectrum and introducing new parameter(s) to additionally add or remove (i.e. invalidate) SBFD-ROs to control density of the valid ROs in SBFD symbols. |
| ITRI | Proposal 1: - For RACH configuration Option 2 in FR1, support Alt. 1 (reuse the existing random access configurations table for unpaired spectrum, i.e., Table 6.3.3.2-3 in TS38.211). Proposal 2: - For RACH configuration Option 2 in FR1 and FR2, support introducing new parameter(s) to determine the subframe number for ROs in SBFD symbols. - FFS: whether the new parameter(s) should indicate at least one slot/subframe index available for RO(s) in addition to, or replacing, the slot/subframe number indicated by the PRACH Configuration Index. |

#### PRACH power control

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 14: For PRACH transmission of SBFD-aware UEs, consider separate power control for different FDMed ROs of PRACH in SBFD symbols and PRACH in non-SBFD symbols, where separate parameter sets for different FDMed ROs in SBFD symbols are introduced, including target PRACH receive power and maximum transmission power. |
| Spreadtrum | - For RACH configuration Option 2, legacy RACH configuration and additional RACH configuration can have same or different PRACH power control parameters including P0, alpha and target power. |
| ZTE, Sanechips | Proposal 12: Support separate PRACH power control parameters configuration in SBFD symbols and non-SBFD symbols. − The separate configuration of PRACH power control parameters can be an absolute value of each power control parameter, or can be derived based on a relative value, e.g., a power offset. − The above mechanism should applicable to both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-4. |
| Tejas | Proposal 9: RACH parameters on Preamble Received Target Power, Maximum Preamble Transmission and Power Ramping Step can have different configuration values in SBFD symbols from that of non SBFD RACH configurations. Proposal 10: Separate PRACH power control parameters are required for RACH operation in RRC_IDLE/ RRC_INACTIVE mode and shall be sent in SIB. |
| CMCC | Proposal 14: For RACH configuration Option 2, no new PRACH power control parameters will be introduced for the additional RACH configuration, except the existing PRACH power control parameters (i.e., preambleReceivedTargetPower, powerRampingStep, preambleTransMax). |
| LG Electronics | Proposal 18: For Option 2, for additional RACH configuration, support separate PRACH power control parameters for ROs in SBFD symbol and ROs in non-SBFD symbol respectively. |
| China Telecom | Proposal 10: Support separate configuration of PPRACH,target,f,c for SBFD operation. |
| Korea Testing Laboratory | Proposal 6. For SBFD-aware UEs in RRC CONNECTED state and RACH configuration Option 2, support no new PRACH power control parameters will be introduced for the additional RACH configuration. |
| Ericsson | Proposal 4 Support separate power control for SBFD ROs for both single and dual RACH configurations. |
| vivo | Proposal 9：For Option 2 for RACH configuration, PCMAX,f,c(i), preambleReceivedTargetPower, preambleTransMax or powerRampingStep can be separately configured for ROs configured by additional RACH configuration. |
| OPPO | Proposal 11: For PRACH power control, support separate configuration of preambleReceivedTargetPower for SBFD symbol and non-SBFD symbol for both RACH configuration option 1 and option 2, FFS other parameters, e.g., powerRampingStep, preambleTransMax, etc. |
| CATT | Proposal 15: Support to configure separate power control parameters including preambleReceivedTargetPower, powerRampingStep and powerRampingStepHighPriority for SBFD symbols and non-SBFD symbols. |
| Fujitsu | Proposal 7: Support separate configurations/parameters for maximum PRACH transmission power and preambleReceivedTargetPower in SBFD and non-SBFD symbols to mitigate CLI. |
| NEC | Proposal 9: • Power control for PRACH enhancements on SBFD symbols should be considered. |
| Panasonic | Proposal 6: For PRACH configuration Option 2, support separate PRACH power control using the existing PRACH power control parameters (i.e., preambleReceivedTargetPower, powerRampingStep, preambleTransMax). |
| Hyundai | Proposal #2: - RAN1 should discuss how to define/use following parameters to adjust fairness between SBFD-aware UE and legacy UE: - Maximum number of msg1 transmissions - Transmission counter for msg1 - Power ramping step - Ramping counter |
| Nokia, Nokia Shanghai Bell | Proposal 9. RAN1 to considers a separate power configuration for Msg1 transmissions on SBFD symbols. |
| Lenovo | Proposal 8: Separate power control parameters are supported for PRACH transmission in SBFD symbols and non-SBFD symbols. |
| Transsion Holdings | Proposal 5: For PRACH power control, support separate configuration of preambleReceivedTargetPower for SBFD symbol and non-SBFD symbols. |
| InterDigital | Proposal 6. Support configuring different PRACH power control parameters for SBFD and non-SBFD ROs, also for FDM-ed SBFD ROs that are closer or farther from DL subband edges, in consideration of potential CLI caused by PRACH transmission. |
| NTT DOCOMO | Proposal 7: For RACH configuration option 2, power control related parameters can be included in the additional RACH configuration: - PRACH target received power parameter: preambleReceivedTargetPower) - RACH re-attempts related parameters: powerRampingStep, preambleTransMax |

### Summary

#### RACH resource

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

Companies discuss whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration for RACH configuration Option 2. Some companies think that all parameters can be supported, while others think some parameters may not be included and UE uses the corresponding parameters in the legacy RACH configuration. Besides, some companies propose to add more candidate values and new parameters. Considering this issue is not urgent at this stage, moderator suggests to defer the discussion in this meeting.

#### RO validation

In RAN1#116bis meeting, the following agreement was made.

Agreement For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives: - Alt 2-3: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. - FFS: The case where the additional-ROs partially overlap with non-SBFD symbols - Alt 2-4: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs. For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs.

Companies’ views on down-selection between Alt 2-3 and Alt 2-4 are summarized as below:

- Alt 2-3: New H3C, Huawei, HiSilicon, Ericsson, vivo, OPPO, Xiaomi, CATT, Fujitsu, NEC, Panasonic, Samsung, Lenovo, Transsion Holdings, Google, Apple, KT, NTT DOCOMO Qualcomm, CEWiT

- Alt 2-4: ZTE, Sanechips, LGE, China Telecom, Sony, Nokia, Nokia Shanghai Bell, InterDigital, Sharp, WILUS

- Both Alt 2-3 and Alt 2-4: CMCC

The arguments of not supporting Alt 2-4 include:

- Whether the addition-ROs in non-SBFD symbols is out of scope or not

- the handling of collision between additional-ROs and legacy-ROs in non-SBFD symbols, which may cause complicated SSB-RO mapping rule or gNB’s configuration restriction

  - Proponents of Alt 2-4: not a dedicated issue for Alt 2-4, and it also exits in SBFD symbols which is configured as flexible symbols by tdd-UL-DL-ConfigurationCommon in Alt 2-3

- cause UL resource fragmentation in non-SBFD symbols.

  - Proponents of Alt 2-4: no issue for {DU} subband configuration case

Moderator suggests initial proposal 1-3-1 to support Alt 2-3 since more companies prefer Alt 2-3.

Regarding the RO validation rule for additional-ROs in SBFD symbols configured by additional RACH configuration, most companies think the same conditions in RACH configuration Option 1 with Alt 1-1 should be applied to RACH configuration Option 2. Moderator suggests to discuss this issue after more progress for RACH configuration Option 1.

#### SSB-RO mapping

For RACH configuration option 2, companies’ views on the SSB-RO mapping rule for the additional-ROs configured by additional RACH configuration are summarized as below.

- Separate SSB-RO mapping between the additional-ROs configured by additional RACH configuration and the legacy-ROs configured by legacy RACH configuration:

  - Supported by: MediaTek, Spreadtrum, Huawei, HiSilicon, ZTE, Sanechips, CMCC, LGE, TCL, vivo, Kookmin University, OPPO, CATT, Sony, NEC, Panasonic, Samsung, Lenovo, ETRI, Apple, NTT DOCOMO, ITRI, WILUS

  - Joint SSB-RO mapping for additional ROs in SBFD symbols and additional ROs in non-SBFD symbols for Option 2 with Alt 2-4

    - Supported by: ZTE, Sanechips, CMCC

- Joint SSB-RO mapping between the additional-ROs configured by additional RACH configuration and the legacy-ROs configured by legacy RACH configuration:

  - Supported by: TCL

- for a set of ROs configured by the additional PRACH configuration (Set 1) that overlaps in time with another set of ROs configured by the legacy PRACH configuration (Set 2), ROs in the Set 1 are mapped to only the one or more SSB indices that are mapped to the Set 2:

  - Supported by: Nokia, NSB

Some companies also propose the detailed separate SSB-RO mapping rule for additional-ROs. Most companies think legacy SSB-RO mapping rule is enough for Option 2, [Huawei] suggests the SSB-RO mapping order rule for additional-ROs should be based on descending order of SSB indexes. [LGE] suggest allowing subset of SSBs or different start SSB index for additional ROs

Moderator suggests initial proposal 1-3-2 based on majority view.

#### RACH configuration table

Agreement (RAN1#117) Update the following agreement made in RAN1#116-bis meeting: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, adopt the following: - Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - For FR1, consider from the following alternatives: - Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. - Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211)

Regarding the further down-selection of random access configurations table for RACH configuration option 2, companies’ views are summarized as the following:

- FR1

  - Alt 1 using TDD table:

    - Supported by: MediaTek, NewH3C, LG Electronics, TCL, Panasonic, Samsung, Nokia, Nokia Shanghai Bell, InterDigital, Qualcomm, Spreadtrum, ZTE, Sanechips, Tejas, Xiaomi, Korea Testing Laboratory, Ericsson, OPPO, Transsion Holdings, NTT DOCOMO, ITRI

      - introduce new parameter(s) to determine the subframe number: MediaTek, LG Electronics, TCL, Panasonic, Samsung, Nokia, Nokia Shanghai Bell, InterDigital, Qualcomm

      - not introduce new parameter(s) to determine the subframe number: Spreadtrum, ZTE, Sanechips, Tejas, Xiaomi

  - Alt 2 using FDD/SUL table:

    - Supported by: Huawei, HiSilicon, CMCC, vivo, Xiaomi, Lenovo, Apple

- FR2

  - introduce new parameter(s) to determine the slot number:

    - Supported by: MediaTek, Huawei, HiSilicon, LG Electronics, Panasonic, Samsung, Nokia, Nokia Shanghai Bell, InterDigital, Qualcomm, ITRI

  - not introduce new parameter(s) to determine the slot number:

    - Supported by: Spreadtrum, ZTE, Sanechips, Tejas, vivo, Kookmin University, Xiaomi, CATT

Most companies propose to reuse the existing random access configuration table for TDD for both FR1 and FR2, but companies’ views on whether to introduce new parameters to determine the slot/subframe number are diverged, some companies analyze the inflexibility of existing TDD random access configuration table, including:

- FR1:

  - Fixed time location for PRACH format 1/2

  - PRACH format 0/3 with periodicity of larger than 10ms can be located only within subframe numbers 4 or 9 which are UL subframes in DDDSUDDSUU and DDDDDDDSUU

  - no PRACH configuration index that allows ROs only in subframe #0

- FR2:

  - have a vast SBFD ROs overhead when using some PRACH configuration index to configure ROs in non-UL slots

Moderator suggests initial proposal 1-3-3.

#### PRACH power control

For RACH configuration option 2, most companies support separate power control parameters for ROs in SBFD symbols and non-SBFD symbols. From moderator’s understanding, current agreements have already allowed Option 2 to configure separate power control parameters for additional-ROs, and it seems no further agreement is needed.

### 1st Round Proposals

#### Initial proposal 1-3-1

For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | Support |
| Spreadtrum | Support |
| ZTE | Do NOT support. The restriction of Alt 2-3 is unnecessary. As analyzed in our contribution, Alt 2-4 can provide larger PRACH capacity, better PRACH configuration flexibility, and most importantly much better PRACH performance due to less CLI. Regarding the arguments of not supporting Alt 2-4, our views are: - It is clearly within scope. The argument of using RO in non-SBFD symbols is out of scope contradicts with our decision of Option 1, for which we have agreed the SBFD UE can use the RO in non-SBFD symbols. If such argument is valid, one can even argue that configuring RO in non-SBFD symbols is also out of scope, which is apparently not the case. - Whether or not it will cause UL resource fragmentation in non-SBFD symbols is fully up to NW configuration. If needed, such resource fragmentation can be avoided as the figure shown in our contribution. - Alt 2-4 would not introduce any new issue. For collision between additional ROs and legacy ROs in non-SBFD symbols, it also exits in SBFD symbols which is configured as flexible symbols by tdd-UL-DL-ConfigurationCommon. For power control, it is also a common issue for both Option 1 and Alt 2-4 of Option 2. With above, we suggest the following proposal. For RACH configuration Option 2, support Alt 2-4 (the additional ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs) - For collision handling between additional ROs and legacy ROs in non-SBFD symbols, reuse the same rules defined for collision between additional ROs and legacy ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For power control of the additional ROs configured by additional RACH configuration, reuse the same mechanism defined for Option 1. |
| New H3C | OK |
| China Telecom | Not support. Regarding the argument points: Firstly, it is of-course within the scope, there is no limitation on resource configuration for SBFD RACH operation. The intention of introducing SBFD RACH operation is to introduce additional ROs for SBFD-aware UEs to improve PRACH performance. PRACH configuration Option 1 and Option 2 lead to two directions for RACH configuration, and we think the difference between them is not just flexibility, but whether to introduce additional RACH configuration for SBFD-aware UE. If Alt 2-3 is selected, the only difference between Option 1 and 2 is the flexibility of RO configuration in SBFD symbols, we think this is not aligned with the initial intention of Option 1 and Option 2. Secondly, collision between additional-ROs and legacy-ROs in non-SBFD symbols is not a dedicated issue for Alt 2-4. Anyway, we need to address this issue for collision in F symbols. |
| Tejas | We do not support the proposal. Additional RO created in non SBFD symbols can be used by SBFD aware UEs and this may reduce RACH latency. These RO are without any CLI and has better RACH detection performance. |
| Xiaomi | Support. |
| Interdigital | Do not support the proposal. This proposal is actually against Proposal 1-4-1 on PRACH repetition spanning across ROs in SBFD and non-SBFD symbols. Moreover, this proposal is restrictive and Alt 2-3 results in less RACH opportunities for SBFD-aware UEs. This proposal could only be applied in two specific cases: - Additional ROs using the same preamble format as legacy ROs: In this case, if the additional ROs are only overlapping in time with legacy ROs, and they are in different frequency resources, the additional ROs should be valid. In this case, the additional ROs should only be invalid if they overlap in both time and frequency with legacy ROs. - Additional ROs use different preamble formats as legacy ROs: In this case, the additional ROs that overlap in time with legacy ROs should be invalid. |
| Samsung | Support |
| Sharp | We think discussion is focused on cases with collision of legacy and additional ROs. However, the collision can be avoided by the network. For example, the network can allocate legacy ROs with short format in odd radio frame, additional ROs with long format in even radio frame. Furthermore, if enhancement to PRACH configuration as discussed in the Initial proposal 1-3-3, by shifting ROs or indicating ROs in more granular way, the network will be able to avoid such the collision of legacy and additional ROs. Therefore, we propose taking either - to discuss the initial proposal 1-3-3 first, or - to conclude that the network can avoid mapping additional ROs in non-SBFD symbols by its configuration. |
| LGE | As the discussion goes details, and even with moderator’s summary above, I don’t see the clear reasons why Alt 2-3 should be supported. Since collision handling on overlapping ROs on the same symbol and/or UL resource fragmentation should be also handled with different Alternatives, no additional specification impact and/or implementation complexity is no longer exist. Furthermore, Alt 2-4 can allow more UL opportunities for SBFD random access, load balancing and early detection associated with additional ROs in non-SBFD symbols. |
| QC | Support. |
| DOCOMO | Support. |
| Ericsson | Support. |
| ETRI | We support the Alt 2-4 because we still think that it does not need any restriction. We configure the option 2 (double config) to have extended set of ROs. In some sense, Alt 2-3 (restricted set of ROs) will be similar to Option 1 (single config). |
| Fujitsu | Support |
| Sony | Not support. Alt 2-4 would provide more RO capacity as per WI objective. |
| Panasonic | Support. |
| Nokia | The concerns related to enabling the additional ROs in non-SBFD symbols (Alt2-4) already exist for two other cases: For additional ROs configured in flexible symbols and for additional ROs crossing from SBFD symbol to non-SBFD symbol. Therefore, not supporting Alt 2-4 based only on these concerns is not justified. Besides, supporting Alt 2-4 increases the capacity of the UL ROs, which is one of the objectives of supporting SBFD operation. |
| OPPO | Support |
| ITRI | Support |

#### Initial proposal 1-3-2

For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | Support |
| Spreadtrum | Support |
| ZTE | Support |
| New H3C | OK |
| Tejas | Support the use of legacy SSB-RO mapping rule for additional RO configuration. |
| Xiaomi | Support. |
| InterDigital | Do not support the proposal. At least NW should be able to indicate to UEs on whether to reuse or use another (indicated) SSB-RO mapping in SBFD symbols that can have a different starting index or different ordering, so that duplicated mapping resources can be avoided as much as possible. |
| Samsung | Support |
| Sharp | Support |
| LGE | Support. |
| QC | Support. Similarly, we can add FFS on sets of SSBs and start SSB index. |
| WILUS | Support |
| DOCOMO | Support. |
| Ericsson | Support |
| ETRI | Support |
| Fujitsu | Support |
| Sony | Support |
| Panasonic | Support. |
| Nokia | We are open for discussion, but we think this proposal might create some issue in case RO spanning across SBFD and non-SBFD symbols is enabled. There exists scenarios that, at the time instance when the legacy UL ROs configured by the legacy PRACH configuration and the SBFD ROs configured by the additional PRACH configuration are overlapped, NW needs to enable more SSBs (at the same time) than that compared to the legacy. Indeed, as illustrated in the Figure, at the highlighted time instance, NW needs to enable more SSBs (all 4 SSBs) at the same time, instead of only 2 SSBs as per legacy (i.e., either {SSB#0, SSB#1} or {SSB#2, SSB#3}). The above observation leads to the following problem: • This either restricts the scheduler or requires enabling more beams at the same time compared to legacy. Indeed, NW needs to either very carefully select a combination of configuration to avoid such scenario or enable more beams at the same time. Therefore, RAN1 needs to study an SSB-to-RO mapping rule for PRACH configuration Option 2, such that for a set of ROs configured by the additional PRACH configuration (Set 1) that overlaps in time with another set of ROs configured by the legacy PRACH configuration (Set 2), ROs in the Set 1 are mapped to only the one or more SSB indices that are mapped to the Set 2, to avoid the aforementioned issue. Proposal For RACH configuration Option 2 the SSB-TO-RO mapping follows these three steps: Step 1: Apply the SSB-TO-RO mapping in legacy RACH configuration to the legacy ROs. Step 2: For a set of additional ROs configured by the additional RACH configuration (Set 1) that overlaps in time with another set of ROs configured by the legacy RACH configuration (set 2), ROs in set 1 are mapped to only the one or more SSB indices that are mapped to set 2. Step3: Apply the SSB-TO-RO mapping in additional RACH configuration to the remaining of the additional ROs. |
| NEC | Support |
| OPPO | Support |
| ITRI | Support |

#### Initial proposal 1-3-3

For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

  - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

- For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

  - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | Support |
| Spreadtrum | Support. For interpretation, we need more discussions. At least for the current proposed methods of interpretation, we found the following disadvantages and ambiguities. PRACH configuration period can be 10/20/40/80/160ms, and SBFD period is the same as TDD-UL-DL pattern period or sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon, the candidate values are much smaller than PRACH period. It is signalling wasting to configure RO time domain location by bitmap Another alternative is to configure time offset. However, the time offset is not a complete method. Because there can be several RO locations in a period, so how to determine the RO position still need more considerations, such as the every RO or each RO configured by RACH table index. |
| ZTE | Support For FR2, the current configuration is sufficient for all supported short PRACH formats. For PRACH format 1, 2 in FR1, the time domain position of current configuration is relatively fixed, and some enhancements are needed. |
| New H3C | OK |
| Tejas | Support |
| Xiaomi | For FR1, we think both alt 1 and alt 2 should be supported in order to accommodate UL subband configuration. SBFD operation can provide more available slots for PRACH transmission while current PRACH configuration table for unpaired spectrum has some restrictions. Considering we don’t need change anything about PRACH configuration table for unpaired spectrum and paired spectrum, supporting both alt 1 and alt 2 bring trivial specification impacts. |
| InterDigital | Support the proposal. Time offsets can be used in the context of TDD cycles, that is the time offset will indicate the starting point for the ROs in SBFD slots with regards to non-SBFD slots. |
| Samsung | Support |
| Sharp | Support |
| LGE | Support |
| QC | Support, a common solution for FR1/FR2 is preferred. |
| WILUS | Support |
| DOCOMO | Support. |
| Ericsson | Support |
| ETRI | Support |
| Sony | Support |
| Panasonic | Support. |
| Nokia | Support. Regarding Spreadtrum comments, we don’t think the PRACH configuration period will increase the signaling for the case of enabling/disabling additional ROs using a bitmap. In our contribution we present a bitmap solution that can cover all cases regardless of the PRACH configuration period using the same number of bits. Bitmap override the slot number for PCI “28” for FR2 unpaired spectrum. In the Figure we show a bitmap {0,1,0,1,1} that overrides the PCI “28” slot numbers, where in this example, only slot numbers 0 to 9 are present. With the override of the bitmap, the new valid slots are only 1,3,4,6,8 and 9, and all the rest of the slots (0,2,5,7) are disabled. Moreover, it is shown that the bitmap does not have to cover all the slot numbers since the slot pattern will repeat after a few slots. In this example, the slot pattern repeats after slot number 4, and consequently, the bitmap maps only the slots from “0” to “4” and then repeats from “5” to “9”. Regarding ZTE comment, we don’t think that the current PCI configuration is sufficient for FR2, unless we don’t consider an overhead of additional ROs as a problem. The following Table and Figure present some PRACH configurations with the common TDD pattern for the SBFD operation “DXXXU”. These configurations can only be enabled with the PCIs “28,58,88,111,143,172,201” for the different preamble formats “A1, A2, A3, B1, B4, C0, C2”. As shown using these PCIs, a huge SBFD ROs overhead is made. ROs overhead due to the limited configuration of FR2 PCI table Desired slot numbers (in green); PRACH configuration index (PCI) for PRACH format A1; Slots with overhead ROs (in red). 1,3,4,6,8,9; 28; 0,2,5,7. 3,4,8,9; 28; 0,1,2,5,6,7. 1,2,6,7; 28; 0,3,4,5,8,9. 1,3,6,8; 28; 0,2,4,5,7,9. 3,8; 28; 0,1,2,4,5,6,7,9. 1,6; 28; 0,2,3,4,5,7,8,9. ROs overhead (in red) due to the limited configuration of FR2 PCI table. |
| NEC | Support |
| OPPO | Support |
| ITRI | Support |

## Issue#1-4: PRACH transmission procedure (4-step RA)

### Submitted proposal

#### PRACH repetition

| Company | Proposals |
| --- | --- |
| MediaTek | Proposal 7: Support the use of valid ROs on SBFD symbols for PRACH repetitions. Proposal 8: A SBFD-aware UE forms RO sets for PRACH repetitions separately for SBFD and non-SBFD symbols. Proposal 9: Define new rules to allow/disallow transmission of PRACH repetitions on SBFD symbols. • Similar rules as the one presented in Proposal 6 can be adopted. |
| Huawei, HiSilicon | Proposal 10: At least PRACH repetition in SBFD symbols is supported. |
| Spreadtrum | - For SBFD aware UEs in RRC CONNECTED state, PRACH repetition is supported, - For both Option 1 Alt 1-1 and Option 2 RACH configurations, all ROs in a RO group should be either legacy valid RO or additional valid RO, and they cannot across legacy ROs and additional ROs. - The legacy RO group determination rules can be applied, including ROs in a RO group are consecutive in time, use same frequency resources, and are associated with a same SS/PBCH block index - To simplify discussion, for SBFD aware UE there are legacy RO groups and additional RO groups. - Legacy-RO groups: The RO groups that are valid for non-SBFD aware UEs. - Additional-RO groups: The valid RO groups that are not valid for legacy-ROs. |
| ZTE, Sanechips | Proposal 11: PRACH repetition is supported in SBFD RACH operation. − PRACH repetition in SBFD symbols only is supported; − PRACH repetition in non-SBFD symbols only is supported; − PRACH repetition across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols is supported. |
| Tejas | Proposal 12: RACH occasions across SBFD symbols and non SBFD symbols shall be used for RACH coverage enhancement. FFS on the how to repeat RACH across SBFD and non SBFD symbols. |
| CMCC | Proposal 15: For SBFD aware UEs, support the following: - PRACH repetition only in additional-ROs - PRACH repetition only in legacy-ROs - FFS PRACH repetition across additional-ROs and legacy-ROs. |
| LG Electronics | Proposal 2: RAN1 to allow PRACH repetition to be supported in SBFD random access operation for SBFD aware UEs under coverage limited conditions. Proposal 14: For SBFD aware UEs, PRACH repetition in SBFD symbols is supported for Option 1. - Configure RO group consisting of additional ROs in SBFD symbols. Proposal 15: For SBFD aware UEs, PRACH repetition in SBFD symbol configured by additional RACH configuration is supported for Option 2 if Alt. 2-3 is agreed. - Configure RO group consisting of additional ROs in SBFD symbols for additional RACH configuration. Proposal 16: For SBFD aware UEs, PRACH repetition across SBFD symbol and non-SBFD symbol configured by additional RACH configuration is supported for Option 2 if Alt 2-4. is agreed. - Configure RO group consisting of additional ROs in SBFD symbols and in non-SBFD symbol for additional RACH configuration. Proposal 19: Discuss whether/how to support UE PRACH transmission power switching between SBFD symbol and non-SBFD symbol based on the separate PRACH power control parameters for multiple PRACH transmission across SBFD symbol and non-SBFD symbol. |
| China Telecom | Proposal 1: Support PRACH repetitions for SBFD related random access operation. Proposal 2: At least PRACH repetition in additional-ROs is supported. - FFS PRACH repetition across additional-ROs and legacy-ROs. Note: legacy ROs means the valid ROs for non-SBFD aware UEs, additional-ROs means valid ROs for SBFD-aware UE while excluding legacy ROs. Proposal 3: For PRACH configuration Option 2, if Alt 2-4 is supported, support PRACH repetitions across SBFD symbols and non-SBFD symbols. |
| Korea Testing Laboratory | Proposal 3. For SBFD-aware UEs in the RRC CONNECTED state, support PRACH repetition in additional-ROs in SBFD symbols. Proposal 4. For SBFD-aware UEs in the RRC CONNECTED state, support PRACH repetition across additional-ROs in SBFD symbols and legacy-ROs in non-SBFD symbols with RACH configuration Option 1. |
| Ericsson | Proposal 6 SBFD random access supports PRACH preamble repetitions using only SBFD ROs. |
| vivo | Proposal 11: For RACH configuration Option 1 with alt 1-1, both ROs in SBFD symbols configured as downlink and legacy ROs can be configured within a RO group for PRACH repetition. Proposal 12: For RACH configuration Option 2, the following options can be considered for PRACH repetitions to determine RO resources within a RO group: • Option 1: RO resources from only a single RACH configuration, i.e., RO resources within a RO group is determined based on only legacy RACH configuration or only additional RACH configuration. • Option 2: RO resources from both legacy RACH configuration and additional RACH configuration. |
| OPPO | Proposal 9: PRACH repetition at least in SBFD symbols is supported. Proposal 10: If PRACH repetition in SBFD symbols is supported, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH repetition in SBFD symbols and non-SBFD symbols. Observation 2: Support PRACH repetition across SBFD symbols and non-SBFD symbols may lead to hypothesis detection issue at gNB side. |
| Xiaomi | Proposal 9: PRACH repetition on additional valid RO in SBFD slots should be supported. Proposal 10: For RO group determination in option 1 and option 2 ‒ The RO group should be separately determined for legacy ROs and additional ROs |
| CATT | Proposal 16: For RACH configuration Option 1, support PRACH repetition in SBFD symbols only. Proposal 17: For RACH configuration Option 2, support PRACH repetition in SBFD symbols. - FFS whether PRACH repetition can across SBFD symbols and non SBFD symbols. |
| Sony | Proposal 8: Support SBFD for PRACH repetitions. Proposal 9: A set of ROs for PRACH repetitions can contain SBFD RO and non-SBFD RO. |
| Fujitsu | Proposal 8: Support PRACH repetition in SBFD operation. Proposal 9: PRACH repetition across SBFD and non-SBFD is not supported at least for Option 1. |
| Panasonic | Proposal 10: For PRACH repetition in SBFD symbol, discuss how to configure RO group. |
| Samsung | Proposal 2: For SBFD-aware UEs, support Rel-18 PRACH repetition for random access in SBFD symbols with RACH configuration options 1 and 2. Proposal 3: For random access in SBFD symbols using Rel-18 PRACH repetition, further consider the following two options: Option 1: a RO set can only consist of SBFD RO(s) or only of non-SBFD RO(s). Option 2: a RO set can consist of SBFD RO(s) and/or non-SBFD RO(s). |
| Nokia, Nokia Shanghai Bell | Proposal 12. For SBFD aware UEs, PRACH with repetition is supported in SBFD symbols. PRACH repetitions can span across ROs in SBFD symbols and ROs in non-SBFDs symbols. |
| Lenovo | Proposal 7: PRACH repetitions across valid ROs in SBFD symbols and non-SBFD symbols should be supported. - FFS: how to determine the RO groups. |
| ETRI | Proposal 10: An RO group may have different frequency resource at the boundary of different symbol types, and FFS dropping conditions. |
| InterDigital | Proposal 7. Support PRACH repetition across SBFD symbols and non-SBFDs symbols. Proposal 12. Support PRACH repetition in SBFD symbols, for both contention-based and contention-free RACH occasions. |
| Google | Proposal 5: Support PRACH repetition in SBFD symbols. Proposal 6: Support PRACH repetition in additional-ROs in SBFD symbols |
| NTT DOCOMO | Proposal 16: Support PRACH repetitions in SBFD symbols. Proposal 17: Not support PRACH repetitions across SBFD symbols and non-SBFD symbols. - RO groups in SBFD symbols are determined from valid ROs in SBFD symbols. - RO groups in non-SBFD symbols are determined from valid ROs in non-SBFD symbols. |
| Qualcomm | Observation 11: PRACH repetition across SBFD-ROs and legacy-ROs can further reduce the latency of PRACH repetition and/or improve PRACH coverage. However, it requires further discussion how to handle PRACH repetition when SBFD-ROs are associated with different Power control parameters and procedure to determine the number of repetitions. Proposal 13: Rel-18 PRACH repetition can be reused at least to support PRACH repetition across SBFD-ROs. - FFS: PRACH repetition across SBFD-ROs and legacy-ROs. |

#### PRACH resource selection

| Company | Proposals |
| --- | --- |
| MediaTek | Proposal 6: Define new rules to allow/disallow PRACH transmission on SBFD symbols. The following rules can be considered as baseline. • Rule 1: UE can transmit PRACH on SBFD symbols only for certain PRACH configurations/formats.(e.g., PRACH formats with smaller bandwidth) • Rule 2: UE can transmit PRACH on SBFD symbols only if the PRACH transmit power is below a predefined transmit power threshold. • Rule 3: UE can transmit PRACH on SBFD symbols only if the PRACH occasion is below a predefined frequency offset form the centre of the UL-SB. |
| Huawei, HiSilicon | Proposal 11: A coverage limited UE with SSB RSRP smaller than a given threshold should select PRACH resource in non-SBFD symbols and a non-coverage-limited UE with SSB RSRP larger than a given threshold should select PRACH resource in SBFD symbols. Proposal 12: For PRACH resource selection in SBFD symbols for SBFD aware UEs, the UEs with SSB RSRP smaller than a given threshold should select FDM-ROs that are farther away from DL subband, and the UEs with SSB RSRP larger than a given threshold should select the FDM-RO closer to DL subband. Proposal 13: SBFD aware UEs can fall back to the PRACH resources in non-SBFD symbols under some conditions, e.g., when the PRACH transmission in SBFD symbols fails for a given number of times. |
| Spreadtrum | - For RACH resource selection, it should decide the RACH resource set configurations for SBFD early indication first, and two alternatives can be considered for RACH resource set for SBFD using RACH configuration Option 1 Alt 1-1 and option 2. - Alt1: only additional ROs can be used - Alt2: both additional/legacy ROs can be used |
| ZTE, Sanechips | Proposal 13: Support RO resource type switching within a same RACH configuration to initiate a RACH reattempt. Regarding power control mechanism of RACH reattempt after RO resource type switching, - A power offset parameter can be introduced to compensate the difference in previous power ramping for determining the value of PREAMBLE_RECEIVED_TARGET_POWER. - The above mechanism should applicable to both of RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-4. Proposal 14: If a valid RO in one symbol type is selected for the PRACH transmission, the subsequent UL transmissions during RACH procedure should be in the same symbol type. |
| CMCC | Proposal 16: For SBFD-aware UEs, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 1: Always prioritize the additional-ROs - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified conditions - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs Proposal 17: For SBFD aware UEs, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Freely select legacy-ROs or additional-ROs for each PRACH transmission re-attempt with/without certain conditions for the rest of random access procedure |
| LG Electronics | Proposal 6: For RACH configuration Option 1 and Option 2, RAN1 discusses the RO type (or a set of ROs) selection rule and/or selection timing (and switching rule/timing if provided). |
| Ericsson | Proposal 5 Support configuration of prioritization between SBFD and legacy ROs for both single and dual RACH configurations. |
| vivo | Proposal 10: Further discuss RO selection rule between legacy ROs and additional ROs for SBFD aware UEs. |
| OPPO | Proposal 12: For both RACH configuration Option 1 and Option 2, RAN1 further studies the selection/prioritization among available valid ROs (e.g., additional ROs configured in SBFD symbols and legacy ROs) for SBFD-aware UE. Proposal 13: At least for PRACH initial transmission, the prioritization of additional SBFD RO and legacy RO is up to gNB configuration or based on UE implementation. Proposal 14: RAN 1 further discusses whether the symbol type can be changed between PRACH initial transmission and the corresponding re-transmission, with the following options: - Option 1: The initial PRACH transmission and its re-transmissions are restricted to one same type (SBFD vs. non-SBFD) of symbols; - Option 2: UE can change the symbol type (SBFD vs. non-SBFD) between each of an initial PRACH transmission and its re-transmissions. Proposal 15: If UE can change the symbol type (SBFD vs. non-SBFD) between each of an initial PRACH transmission and its re-transmissions, RAN1 further studies how to handle the power ramping counter. |
| CATT | Proposal 11: For RACH configuration Option 1, following RO selection rules from legacy ROs and additional ROs can be further discussed. - Option 1: Prioritize additional ROs for the first set of RACH attempts and fallback to legacy ROs when fallback condition is met. - FFS additional criteria to select additional ROs - Option 2: Select a RO from additional ROs and legacy ROs in each RACH attempt. - FFS details on RO selection Proposal 12: For RACH configuration Option 2, legacy ROs in non-SBFD symbols configured by the legacy RACH configuration are also valid for SBFD aware UEs. Proposal 13: Support to prioritize additional ROs for the first set of RACH attempts and fallback to legacy ROs when fallback condition is met for RACH configuration Option 2. |
| NEC | Proposal 6: • If one SSB is associated with two RO/preamble separately on SBFD symbols and non-SBFD symbols, some rule should be defined, or some indication is needed for SBFD-aware UE to determine which RO to choose to perform random access Proposal 15: • For failed PRACH attempts in SBFD slots, the UE will wait for dedicated UL slots to re-attempt PRACH. |
| Panasonic | Proposal 9: Discuss whether/how to support PRACH resource selection between SBFD symbols and non-SBFD symbols. |
| Hyundai | Proposal #1: - For the sake of fairness between SBFD-aware UE and legacy UE, RAN1 should consider following alternatives as PRACH configuration: - Introducing separated configuration of RSRP threshold for SBFD-aware UEs - Dividing the preambles into subgroups that can be selected by SBFD-aware UEs and non SBFD-aware UEs respectively |
| Nokia, Nokia Shanghai Bell | Proposal 3 For Option 2, in case two RACH configurations with two different PRACH formats lengths, SBFD aware UEs selects the best PRACH format length configured by the legacy or the additional RACH configuration for the initial access based at least on SSB-RSRP threshold(s). Details FFS. |
| Transsion Holdings | Proposal 6: If UE can change the symbol type (SBFD vs. non-SBFD) between the initial PRACH transmission and re-transmission, how the power ramping counter is handled when the symbol type changes compared to the last PRACH (re)transmission needs to be discussed. |
| ETRI | Proposal 18: Regarding Option 2, in order to transmit a single preamble instance, a UE can access any RO from either RACH configuration. |
| Apple | Proposal 3: RACH re-attempt should be in the SBFD symbols as the first PRACH transmission. |
| KT | Proposal 5. Discuss whether SBFD-aware UEs can use both ROs in SBFD symbols and non-SBFD symbols in a single PRACH transmission. Proposal 6. Discuss whether the symbol type can be switched between PRACH transmission and the corresponding retransmission. |
| NTT DOCOMO | Proposal 12: At least for the first RACH attempt, following conditions can be considered for RO selection between SBFD and non-SBFD: - If DL-RSRP is above a RSRP threshold indicated by gNB, UE selects valid RO in SBFD symbols. Otherwise, UE selects valid RO in non-SBFD symbols. - If the selected SSB/CSI-RS index is within a set of SSB/CSI-RS indexes indicated by gNB, UE selects valid RO in SBFD symbols. Otherwise, UE selects valid RO in non-SBFD symbols. Proposal 13: For RACH re-attempts, further study whether to allow switching between ROs in SBFD symbols and ROs in non-SBFD symbols. - If switching between ROs in SBFD symbols and ROs in non-SBFD symbols is allowed, further study whether/how to define switching rule/condition. Proposal 14: If switching between ROs in SBFD symbols and ROs in non-SBFD symbols is not allowed, separate configuration of powerRampingStep and preambleTransMax for SBFD can be configured. Proposal 15: If switching between ROs in SBFD symbols and ROs in non-SBFD symbols is allowed, further study power ramping rule for following cases: - Case 1: The pervious RACH failure is in SBFD symbols and the subsequent RACH is in non-SBFD symbols. - Case 2: The pervious RACH failure is in non-SBFD symbols and the subsequent RACH is in SBFD symbols. |
| Qualcomm | Proposal 11: For the selection between SBFD-RO and legacy-RO for first contention based PRACH transmission (CBRA), RAN1 to discuss the followings: - gNB configuration/prioritization of SBFD-ROs over legacy-ROs to achieve certain load balancing and reducing the PRACH collision probability. - SBFD-awareness indication by preamble transmission in SBFD-RO or preamble partitioning. - UE implementation/preference to use legacy RACH procedure or SBFD-based RA. Proposal 12: For switching of PRACH transmission from SBFD-RO to legacy-RO, RAN1 to discuss the following: - When switching happens within the PRACH procedure (e.g. after certain number of failed PRACH attempts) - The number of switching within the PRACH procedures. - PRACH power ramping. - Whether applicable for PRACH configuration option 1 and/or PRACH configuration option 2. |
| ITRI | Proposal 4: - For RACH configuration Option 2, if configured, SBFD-aware UEs in the RRC CONNECTED state should comply with the additional RACH configuration for the RACH procedure to leverage the corresponding enhancements. |

### Summary

#### PRACH repetition

In RAN1#116, the following agreement was made.

Agreement For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols. - FFS PRACH repetition in SBFD symbols. - FFS PRACH repetition across SBFD symbols and non-SBFDs symbols.

In this meeting, most companies support PRACH repetition for SBFD-aware UEs to further improve the PRACH coverage. Regarding the determination of RO groups, companies’ views are summarized as the following.

- Option 1: PRACH repetition in additional-ROs and legacy-ROs separately:

  - Supported by: MediaTek, Huawei, HiSilicon, Spreadtrum, CMCC, LGE (Option 1, Option 2 with Alt 2-3), China Telecom, Ericsson, OPPO, Xiaomi, CATT, Fujitsu (Option 1), [Samsung], Google, NTT DOCOMO, Qualcomm, [vivo]

- Option 2: PRACH repetition across additional-ROs and legacy-ROs:

  - Supported by: ZTE, LGE (Option 2 with Alt 2-4), China Telecom (Option 2 with Alt 2-4), Korea Testing Laboratory, Sony, [Samsung], Nokia, NSB, Lenovo, InterDigital, [vivo], Tejas

  - FFS: CATT, Qualcomm

The reasons for supporting or not supporting PRACH repetition across additional-ROs and legacy-ROs are summarized as below:

- not support: more specification effort, different power control parameters, time-domain overlap between different PRACH occasion sets which may cause network cannot identify the number of repetitions and the PRACH occasion index of each PRACH transmission.

- support: reduce repetition latency, increase repetition number

  - Some companies think this can be supported in RACH configuration option 2 with alt 2-4 (if supported)

Moderator suggests Initial proposal 1-4-1.

#### PRACH resource selection

Both legacy-ROs and additional-ROs can be used for SBFD-aware UEs in RACH configuration option 1 and option 2. Regarding how to select the RO for PRACH initial attempt and PRACH re-attempt, companies’ views are summarized as the following:

- For PRACH initial attempt in one random access procedure:

  - Option 1: Pre-define or configure the additional-ROs to be prioritized over legacy-ROs

    - Ericsson, CATT (RACH configuration option 1 and option 2), NEC, Qualcomm, CMCC

  - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified conditions

    - MediaTek, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, NTT DOCOMO, CMCC

    - The conditions may include, e.g., PRACH configurations/formats, PRACH transmit power, DL RSRP, associated SSB/CSI-RS index, etc.

  - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs

    - CATT (RACH configuration option 1), Qualcomm, CMCC

- For PRACH re-attempt in one random access procedure:

  - Option 1: Always use ROs of the same type as the earlier PRACH transmission for the rest of the random access procedure

    - OPPO, Apple, CMCC

  - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure

    - Huawei, HiSilicon, CATT (RACH configuration option 2), NEC, Qualcomm, CMCC

  - Option 3: Freely select legacy-ROs or additional-ROs for each PRACH transmission re-attempt with/without certain conditions for the rest of random access procedure

    - ZTE, Sanechips, OPPO, Transsion Holdings, [NTT DOCOMO], CMCC

Moderator suggests initial proposal 1-4-2 and initial proposal 1-4-3.

### 1st Round Proposals

#### Initial proposal 1-4-1:

For SBFD aware UEs, support the following:

- PRACH repetition only in additional-ROs

- PRACH repetition only in legacy-ROs

- FFS PRACH repetition across additional-ROs and legacy-ROs.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | Support the proposal. We do not support PRACH repetition across additional ROs and legacy ROs but we are open to discuss. |
| Spreadtrum | We support the intention to support PRACH repetition for SBFD aware UE. But it seems not clear, suggest to use “preamble repetitions of one PRACH transmission”, if we understand correctly. For SBFD aware UEs, support the following: - PRACH repetition preamble repetitions of one PRACH transmission only in additional-ROs - PRACH repetition preamble repetitions of one PRACH transmission only in legacy-ROs - FFS PRACH repetition preamble repetitions of one PRACH transmission across additional-ROs and legacy-ROs. |
| ZTE | Support |
| New H3C | OK with SPRD’s modification |
| China Telecom | Support. We think PRACH repetitions should be supported since the main intention of introducing SBFD RACH operation is to increase PRACH performance which is highly aligned with the intention of introducing PRACH repetitions. We can’t find any reasons not to support PRACH repetition for SBFD RACH operation. Regarding Spreadtrum’s comment, we think PRACH repetitions is clear enough, or we can just use the description in current spec. as “PRACH transmission with preamble repetitions” |
| Tejas | Support the modified proposal of Spreadtrum. |
| Xiaomi | Support. |
| InterDigital | Support the proposal. |
| Samsung | Support |
| Sharp | OK |
| LGE | Depending on RACH configuration Option 1, Option 2 Alt 2-3, or Option 2 Alt 2-4, Additional-ROs could mean different valid ROs. For example, ROs in SBFD symbol and ROs in non-SBFD symbols are configured in additional-ROs in Option 2 Alt 2-4 while ROs in SBFD symbol are configured in Option 2 Alt 2-3 and Option 1. In that sense, clarification is required as well in FFS. For RACH configuration Option 2, we are not sure whether SBFD aware UE is allowed to form a new RO group consisting of both additional-ROs and legacy-ROs configured by additional RACH configuration and legacy RACH configuration, respectively. FFS probably is meant to be PRACH repetition across additional-ROs and legacy-ROs configured by Option 1 or by additional RACH configuration in Option 2 Alt 2-4 if supported. For Option 2 Alt 2-3, case is not exist in FFS. |
| QC | Support. Our understanding this is general agreement for both option-1 and option-2. |
| DOCOMO | Support. We don’t support PRACH repetition across additional-ROs and legacy-ROs, due to separate SSB-to-RO mapping and issue of gNB detection. |
| Ericsson | Support but do not think the FFS is needed. Any cross RO type repetitions should furthermore be limited to Option 1, since for Option 2, a different preamble and even a different number of preamble indices may be available to the UE, making it all but impossible for the gNB to combine across RO types. |
| ETRI | Support, and spreadtrum’s revision seems clear. |
| Fujitsu | Support. We prefer not to support FFS case. |
| Sony | Support. We share similar views with LG, repetition across legacy and additional ROs is only available in Option 1 if we do not agree to Configuration Option 2 Alt 2-4. |
| Panasonic | Support. |
| Nokia | Support. |
| NEC | Agree with principal but agree with LG in that it is better for us to figure alt 2-3 and alt 2-4 before tackling this issue |

#### Initial proposal 1-4-2:

For SBFD-aware UEs, consider the following options for initial PRACH transmission attempt in one random access procedure:

- Option 1: Pre-define or configure the additional-ROs to be prioritized over legacy-ROs

- Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified conditions

- Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | Support. But we want to make it open that different options may apply for different RACH configuration Options at this point. |
| Spreadtrum | As we commented in proposal 1-1-3. Before we discuss RACH resource selection, the way to configure RACH resource set for RA in SBFD can be decided first. |
| ZTE | Support. But, it seems Option 1 is a special case of Option 2. |
| China Telecom | We think this issue needs to be addressed. Share the similar view with ZTE that Option 1 is covered by Option 2. |
| Tejas | We support option 2 or option 3. Option 1 is subset of option 2. There should be an option (option 4) to select both legacy RO and additional ROs based on certain specified conditions. |
| Xiaomi | Support. We prefer the simplest mechanism, i.e., option 3. |
| InterDigital | We support the proposal in general. The option 1 and Option 2 could be configurable by gNB, and Option 3 could be selected in the absence of indications/configurations from gNB. |
| Samsung | We prefer network-side configuration of the desired behavior, i.e., network configures if additional ROs or legacy ROs are selected by the UE (Option 1). We also think that at least RSRP thresholds are necessary as part of the RO selection (Option 2). We fail to see the motivation for Option 3. |
| Sharp | OK. We prefer Option 1. |
| LGE | As the first discussion, this initial proposal seems to contain many things mixed up such that clarification should be required. First, depending on the RACH configuration Options of Option 1 and Option 2, the preferred option for initial PRACH transmission attempt could be different since initial attempt could impact on the following PRACH re-attempts differently between Option 1 and Option 2. |
| QC | Support. We would like to highlight that different options may be selected for PRACH configuration option-1 and option-2 as the motivation is different for each design option. |
| DOCOMO | From NW operation perspective, we think load balancing in SBFD symbols and non-SBFD symbols controlled by gNB would be beneficial. Therefore, RO type selection based on condition configured by gNB is preferred. We suggest to update option 2 to allow gNB configured conditions. For SBFD-aware UEs, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 1: Pre-define or configure the additional-ROs to be prioritized over legacy-ROs - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs |
| Ericsson | Support. We prefer Option 1 but also think that the detailed implementation should be discussed in RAN2. |
| ETRI | We support Option 3. |
| Sony | Support to discuss these options. |
| Panasonic | Support. |
| Nokia | Support. |
| NEC | Deprioritize option 3. Fine with rest |
| OPPO | Option 1 should be network configure the prioritization between additional RO and legacy RO for load balancing, but now option 1 close the door for legacy RO prioritize over additional RO. |

#### Initial proposal 1-4-3:

For SBFD aware UEs, consider the following options for PRACH transmission re-attempt in one random access procedure:

- Option 1: Always use ROs of the same type as the earlier PRACH transmission for the rest of the random access procedure

- Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure

- Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | For subsequent RACH attempts, we think we should separately discuss for RACH configuration Option 1 and Option 2. For example for RACH configuration Option 2, we do not think Option 3 should be pursued, which is not aligned with current RACH procedure defined in RAN2. |
| Spreadtrum | As we commented in proposal 1-1-3. Before we discuss RACH resource selection, the way to configure RACH resource set for RA in SBFD can be decided first. |
| ZTE | Based on existing RAN2 framework, the RACH configuration configuring an RO for a re-attempt should be the same as the RACH configuration configuring an RO for initial PRACH transmission. Take PRACH repetition in Rel-18 as an example, additional RACH configuration is supported while a re-attempt is NOT allowed to across ROs configured by different RACH configurations So, Option 3 (not our preference) should be updated as follows: - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure, where the legacy-ROs and additional-ROs are configured by one RACH configuration. |
| Tejas | Support the proposal. We prefer option 2. |
| Xiaomi | As comment by other companies, it is more proper to leave it to RAN2 on how to select ROs for re-attempt. |
| InterDigital | Support the proposal. |
| Samsung | Support in principle. |
| Sharp | For RACH configuration Option 1, we prefer Option 3 to obtain latency reduction gain. For RACH configuration Option 2, Option 2 is preferred. |
| LGE | Same comments as in the initial proposal 1-4-2. For RACH configuration Option 2, in accordance with the RACH configurations in the current specification, the switch between two RO sets (a set of additional-ROs and a set of legacy-ROs) by additional RACH configuration and legacy RACH configuration may require specification impact. Since different options could be preferred in re-attempts depending on RACH configuration options, discussion should be split for each RACH configuration option. |
| QC | Support from study point of view. However, option-1 is very restrictive and could lead to large latency. Also, we would like to highlight that different options may be selected for PRACH configuration option-1 and option-2 as the motivation is different for each design option. |
| DOCOMO | Support. We are open to discuss the three options. Not sure RAN1 can make the decision or we need to leave it to RAN2? |
| Ericsson | In our opinion RAN1 should leave this for RAN2 to decide. |
| ETRI | We support Option 3. We think that UE can access to any RO that is shown by configurations either Option 1 or option 2. |
| Fujitsu | Support |
| Sony | Support the proposal. |
| Panasonic | Support. |
| Nokia | Support. |
| NEC | Support in principle |

## Issue#1-5: Msg2/Msg3/Msg4 PDCCH (4-step RA)

    - Submitted proposal

| Company | Proposals |
| --- | --- |
| MediaTek | Proposal 10: Define new rules for Msg3 transmission on SBFD symbols. • Similar rules as the one presented in Proposal 6 can be adopted. Proposal 11: Support separate frequency domain allocations for Msg3 repetitions and/or frequency hopping on non-SBFD symbols. Proposal 12: Define new frequency domain validation rules for Msg3 repetitions on SBFD symbols. • Msg3 allocation may be considered as invalid if it overlaps (partially or fully) with DL subband(s) |
| Huawei, HiSilicon | Observation 5: Solutions for PDSCH and PUSCH discussed in AI 9.3.1 can be reused for Msg2/Msg4 PDSCH and Msg3 PUSCH respectively with necessary modification. Observation 6: Separate common PUCCH resource sets on SBFD symbols and non-SBFD symbols and additional frequency offset can be considered for Msg4 HARQ-ACK PUCCH transmission. |
| Spreadtrum | - For MSG3 (re)transmission, support separate FH offsets for PUSCH transmissions in SBFD symbols and non-SBFD symbols respectively. - For MSG3 (re)transmission in SBFD symbols, the NUL,hop Hopping Bits still depend on size of UL initial BWP, LSB bits is used according to the size of UL usable PRBs |
| CMCC | Proposal 18: The interpretation of frequency domain resource allocation in UL grant provided by RAR and the frequency offset of the second hop of Msg3 PUSCH provided in Table 8.3-1 in TS 38.213 should be determined based on size of SBFD UL usable PRBs if Msg3 is transmitted in SBFD symbols. Proposal 19: Separate power control parameters can be configured for Msg3 transmission in SBFD symbols and non-SBFD symbols. - FFS which parameters can be separately configured, e.g., msg3-DeltaPreamble and msg3-Alpha. |
| LG Electronics | Proposal 20: RAN1 to need to keep the discussion on RA specific enhancements for SBFD aware UE to evaluate benefits for Msg2/Msg3/Msg4 related transmission/reception in SBFD symbols. - Enhancements on the interpretation on the frequency domain resource assignment field and/or frequency hopping for Msg 3 PUSCH (re)transmission within UL usable PRB in SBFD symbol. - Enhancements on the PUCCH resource sets before dedicated PUCCH resource configuration in time domain and/or frequency domain (e.g., frequency hopping) for Msg4 HARQ-ACK PUCCH transmission within UL usable PRB in SBFD symbol. |
| TCL | Proposal 5: For Msg3 in SBFD symbols consider the following separate parameters: - Separate FDRA for Msg3 PUSCH in SBFD and non-SBFD symbols. - Separate FH for Msg3, in SBFD and non-SBFD symbols. |
| vivo | Proposal 13: It can be discussed whether a single configuration or separate configurations are used to determine Msg.3 PUSCH transmission power. Proposal 14: For frequency hopping for Msg.3 PUSCH, frequency offset for second hop of PUSCH transmission within the usable UL subband can be considered. |
| OPPO | Proposal 16: For Msg3 (re)transmission with frequency hopping in SBFD symbols, the frequency offset for the second hop and/or the number of bits of NUL,hop are determined based on the UL subband size (the size of UL usable PRBs). |
| Xiaomi | Proposal 11: RAN1 needs to clarify that only the RBs contained in DL subband are available RBs for PDSCH in SBFD symbols. Proposal 12: If inter-slot frequency hopping is enabled for Msg3 PUSCH, further study the mechanisms to guarantee hop located in SBFD slot does not exceed UL subband. The following aspects could be considered as starting point: - FH offset applied to SBFD symbols and non-SBFD symbols with updating FH formula, e.g., mod operation is based on total number of UL RBs in SBFD symbols and non-SBFD symbols, respectively. - The frequency hopping pattern in time domain is determined per slot type or across different slot types. Proposal 13: If intra-slot frequency hopping is enabled for Msg3 PUSCH or MsgA-PUSCH, further study the mechanisms to guarantee hop located in SBFD symbols does not exceed UL subband. The following can be considered as starting point: - Updating the FH formula by replacing BWP size with UL subband size for mod operation. |
| CATT | Proposal 18: Reuse the enhancements on DL receptions in non RACH procedure for Msg2 and Msg4 PDSCH if needed. Proposal 19: Discuss whether/how to solve the issue in case the RBs within UL subband cannot be indicated by the RAR UL grant or the fallback DCI. Proposal 20: The enhancements on PUSCH transmission in non RACH procedure can be reused for Msg3 PUSCH with or without frequency hopping. Proposal 21: Update the definition of available slot counting by taking the subband frequency location into account in SBFD symbols for Msg3 repetition. |
| NEC | Proposal 8: For Msg3 PUSCH enhancements, the following aspects can be considered. • PUSCH frequency hopping offset can be based on the UL subband size. • PUSCH repetition transmission across SBFD symbols and non-SBFD symbols |
| Samsung | Proposal 12: The existing RACH Msg.2 (RAR) reception procedure is reused for random access in SBFD symbols. Proposal 13: For SBFD-aware UEs, support enhancements to UL frequency resource allocation and frequency hopping behavior for transmission of RACH Msg.3 (PUSCH) and PUCCH A/N associated with RACH Msg.4 (PDSCH). |
| ETRI | Proposal 19: Strive to design a unified design for both connected and non-connected modes provided that non-connected mode supports SBFD operations, and remove bracket for Msg4. Proposal 5: At least msg2/3/4 in SBFD symbols are supported for non-connected UEs. |
| Sharp | Proposal 7: Enhancement to DCI format 1_0 scheduled in CSS is not pursued in Rel-19. Proposal 8: For msg3 PUSCH, in frequency hopping offset determination, N_BWP^size is set to the value of the size of the UL subband. Proposal 9: Msg3 PUSCH repetition is supported in UL subbands. |
| Apple | Proposal 10: Msg3 PUSCH and its retransmission can be scheduled in the SBFD sub-band. |
| NTT DOCOMO | Proposal 18: For Msg 2 and Msg 4 PDSCH reception in DL subband(s), wait for progress in AI 9.3.1. Proposal 19: For Msg3 PUSCH and Msg4 HARQ-ACK PUCCH frequency resource allocation, frequency hopping and power control, wait for progress in AI 9.3.1. Proposal 20: For CBRA, gNB can identify UE’s SBFD support during random access. - The identification of SBFD support is realized by PRACH detection on valid RO in SBFD symbols. - If UE transmits PRACH transmission on valid RO in SBFD symbols, gNB can identify that the UE is capable of SBFD and the UE can use SBFD symbols for Msg 3 repetitions. Otherwise, UE should determine Msg 3 PUSCH repetition slots same as legacy since gNB cannot identify UE’s SBFD support. |
| Qualcomm | Proposal 14: The reception of MSG2/4 in SBFD symbols should follow the same design approach for PDSCH reception (Type 1 FDRA) in SBFD symbols in AI 9.3.1. Observation 12: The frequency offsets of the second hop Msg3/MsgA PUSCH depends on the size of the UL BWP which may impose some limit for msg3 frequency hopping in SBFD symbols. Proposal 15: RAN1 to discuss how to enable efficient Msg3 PUSCH frequency hopping in the UL-subband of SBFD symbols. Proposal 16: RAN1 to discuss the design objectives and motivations for separate MSG3 power control parameters in SBFD and non-SBFD symbols including separate Po. Proposal 17: MSG3 repetition across SBFD and/or non-SBFD symbols should follow the same design approach for PUSCH repetition Type-A in AI 9.3.1. |

### Summary

In RAN1#116 meeting, the following agreement was made.

Agreement For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects: - Msg2[/Msg4 PDSCH] reception in DL subband(s) - Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping - Msg3 repetition - Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control - FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE Note: Strive to make progress in accordance to the discussion in AI 9.3.1.

Companies provide some further solutions in this meeting and most of them prefer to follow the progress in AI 9.3.1, considering the situation, moderator suggests to defer the discussion on this issue after more progress is made in AI 9.3.1.

## Issue#1-6: 2-step RA enhancement

    - Submitted proposal

| Company | Proposals |
| --- | --- |
| New H3C | Proposal 1: MsgA RO and MsgA PUSCH for Type-2 random access procedure is supported to be configured in semi-static SBFD resource. |
| Spreadtrum | - It can support 2-step RACH for random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode. |
| ZTE, Sanechips | Proposal 16: About SBFD random access operation in RRC CONNECTED mode, the discussion on 2-step RA type should be deprioritized. |
| LG Electronics | Proposal 1: RAN1 to allow 2-step RACH to be supported in SBFD symbols for SBFD aware UEs to fit into the latency limited conditions. |
| TCL | Proposal 11: RAN1 to consider the re-interpretation of the following parameters for the configuration of 2 step RACH in SBFD symbols. - msgA-FrequencyStart - frequencyStartMsgA-PUSCH, - nrofMsgA-PO-FDM, - msgA-IntraSlotFrequencyHopping, - msgA-DMRS-Config Proposal 12: RAN1 to define additional mapping rules, on top of the existing rules, for mapping ROs and POs in 2-step RACH within SBFD symbols. Proposal 13: For the configuration of 2-step RACH in SBFD and non-SBFD symbols, RAN1 to consider the following two options. - Option 1: Use one single 2-step RACH configuration, based on the existing parameters of the 2-step RACH configuration to - Option 2: Use two separate 2-step RACH configurations, including one legacy 2 step RACH configuration and one additional 2-step RACH configuration. |
| Xiaomi | Proposal 1: For SBFD aware UE in RRC CONNECTED state, support Type-2 random access procedure (2-step RACH) in SBFD symbols. Proposal 14:For PUSCH occasions in SBFD symbols: - For the legacy POs in option 1, no enhancements for the PO validation rule are needed - For the additional POs in option 1, they are valid if at least - Not overlap in time and frequency with any valid RO - Frequency resource of the RO is fully within UL usable PRBs, and - Not overlapped with SSB (Ngap can be 0 for all preamble SCS), and - Starts at least X>=0 symbols after a last downlink symbol, and - Value of X depends on required transition time - Not across SBFD symbols and non-SBFD symbols within a slot or across slots - For the additional POs in SBFD symbols configured by additional MsgA-PUSCH configuration, they are valid if at least: - Not overlap in time and frequency with any valid RO - Frequency resource of the RO is fully within UL usable PRBs, and - Not overlapped with SSB (Ngap can be 0 for all preamble SCS), and - Starts at least X>=0 symbols after a last downlink symbol, and - Value of X depends on required transition time |
| NEC | Proposal 11: • Both 4-step RACH and 2-step RACH can be supported on SBFD symbols. Proposal 12: - The following aspects should be considered for 2-step RACH enhancements on SBFD symbols: - PO configuration and valid determination in the UL subband of SBFD symbols - The RO mapping relationship with PRU in SBFD symbols - Power control for PUSCH for MsgA |
| Samsung | Proposal 1: For SBFD-aware UEs, support the Type-2 random access procedure in SBFD symbols. |
| Lenovo | Proposal 10: The discussion on whether/how to support type 2 random access procedure in SBFD symbol could be postponed. |
| InterDigital | Proposal 8. Support 2-step RACH in SBFD ROs. |
| Apple | Proposal 9: Support both 4-step RACH and 2-step RACH procedure in SBFD sub-band. |
| NTT DOCOMO | Proposal 21: Support random access operation in SBFD symbols for Type-2 RACH. Proposal 22: Support unified design for PRACH in SBFD symbols for Type-1 RACH and Type-2 RACH. Proposal 23: For Msg A PUSCH in SBFD symbols, further study Msg A PUSCH configuration, PO validation, RO-to-PO mapping, etc. |
| Qualcomm | Proposal 18: RAN1 to discuss the following two design options for msgA PRACH/PUSCH configurations for SBFD random access operation. - Single msgA PRACH /PUSCH configuration for both legacy and SBFD-aware UE with ROs/POs in both UL and SBFD symbols. - SBFD-aware UEs are configured with additional msgA PRACH/PUSCH configuration. Proposal 19: RAN1 to discuss the determination of valid PUSCH occasions (PO) in the uplink subband of SBFD symbols. |

### Summary

In RAN1#116 meeting, the following agreement was made.

Agreement For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH)

Companies’ views on whether to support 2-step RA are summarized as below:

- Support: New H3C, Spreadtrum, LGE, TCL, InterDigital, Samsung, Xiaomi, NEC, Apple, NTT DOCOMO, Qualcomm

- Not support or deprioritized: ZTE, Sanechips, Lenovo

More companies prefer to support 2-step RA in SBFD symbols, but there are still some companies prefer to deprioritize it. The reasons for deprioritizing 2-step RACH include: the justification of 2-step RA is not clear since the 2-step RA is applied in good RSRP condition, 2-step RACH is optional features, PRACH repetitions is not supported in 2-step RACH which means 2-step RACH cannot enhance uplink coverage.  Moderator suggests to defer the discussion until the basic design principle is decided.

# Issue#2: Specific issues for IDLE/INACTIVE mode

## Issue#2-1: UL/DL usable PRBs

### Submitted proposal

| Company | Proposals |
| --- | --- |
| ZTE, Sanechips | Proposal 17: For avoiding impact the reception of cell common transmissions by legacy UEs, the following options can be considered for determining the UL usable PRBs for SBFD RACH operation in RRC IDLE/INACTIVE mode, − Option 1: A separate initial BWP is configured to SBFD UEs for avoiding overlapping between UL subband and CORESET#0. − Option 2: If the UL subband overlaps with some frequency domain resources of the CORESET#0, puncturing based solution is supported for DL cell common transmissions. |
| CMCC | Proposal 2: For SBFD-aware UEs in RRC_IDLE/INACTIVE mode, UL subband frequency resources within initial UL BWP are called UL usable PRBs and DL subband(s) frequency resources within initial DL BWP are called DL usable PRBs. |

### Summary

In AI 9.3.1, the definition of usable RPBs was agreed as the following for the discussion of UE’s UL and DL frequency resources in SBFD symbols in RRC_CONNECTED mode.

Agreement For discussion purpose, UL subband frequency resources within active UL BWP are called UL usable PRBs and DL subband(s) frequency resources within active DL BWP are called DL usable PRBs. For determining UL/DL usable PRBs, consider the following options. - Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols. - Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

For RRC_IDLE/INACTIVE mode, initial UL/DL BWP are used instead of active UL/DL BWP, and two companies [ZTE, CMCC] propose to clarify the definition of UL/DL usable PRBs for RRC_IDLE/INACTIVE mode. Moderator suggests initial proposal 2-1-1.

### 1st Round Proposals

#### Initial proposal 2-1-1:

For SBFD-aware UEs in RRC_IDLE/INACTIVE mode, UL subband frequency resources within initial UL BWP are called UL usable PRBs and DL subband(s) frequency resources within initial DL BWP are called DL usable PRBs.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | Support |
| Spreadtrum | We are fine to have aligned understanding towards UL usable PRB and DL usable PRB for RRC_IDLE/INACTIVE mode. However, for DL usable PRBs, if CORESET 0 is configured, CORESET 0 can be used, otherwise initial DL BWP can be used. Another issue is for redcap UE, this initial UL/DL BWP is RedCap-specific initial DL/UL BWP or initial DL/UL BWP configured by initialDownlinkBWP/initialUplinkBWP. |
| ZTE | Support |
| New H3C | OK |
| Tejas | Can we modify as follows: For SBFD-aware UEs in RRC_IDLE/INACTIVE mode, UL subband frequency resources within initial UL BWP are called initial UL usable PRBs and DL subband(s) frequency resources within initial DL BWP are called initial DL usable PRBs. |
| Xiaomi | Support. |
| InterDigital | Support the proposal. |
| Samsung | Support in principle. |
| Sharp | Support |
| LGE | Support |
| QC | We are okay with the proposal, but we would like to bring-up some issue. Usually, the initial UL/DL BWP is small (CORESET#0) and have aligned center frequency. In such scenario, as explained in figure below, there is no usable UL PRBs in SBFD symbols. Then, gNB needs to configure a wide UL/DL initial BPWs to accommodate the UL/DL subbands. This will forfeit the UE power savings aspects. This is happening due to the limitation of aligned centre frequency between the UL and DL BWP. If that restriction is lifted for narrow UL/DL BPW for SBFD-aware only, then SBFD-aware UE can transmit or receive in the SBFD symbols. SBFD-aware operation with narrow UL/DL BPW and unaligned center frequency |
| DOCOMO | Support. |
| Ericsson | We don’t think this is needed. In IDLE/INACTIVE the configured BWP is the initial BWP and hence this is already agreed in 9.3.1. |
| ETRI | Support |
| Sony | Support |
| Nokia | We are open to discuss. |
| NEC | Support in principle |

## Issue#2-2: Early indication

### Submitted proposal

| Company | Proposals |
| --- | --- |
| Spreadtrum | - SBFD early indication is supported for MSG3 and MSG4 HARQ-ACK PUCCH scheduling. It can use RACH resource set configuration as a baseline. |
| ZTE, Sanechips | Proposal 5: For RACH configuration Option 1 with Alt 1-1, for UE type identification in legacy RO, the following options can be further considered, - Option 1: Distinguish different types of UE via preamble partition. - Option 2: The UE is regarded as non-SBFD aware UE. |
| CMCC | Proposal 3: Discuss whether to support preamble partitioning on legacy-ROs for early indication of SBFD-aware UEs. |
| Ericsson | Proposal 13 The selected PRACH RO (SBFD or legacy) determines whether the UE follows the legacy or an SBFD-specific RACH procedure for Msg2, 3, 4. Proposal 14 RAN1 to leave to RAN2 to decide if a new MAC CE should be introduced for identification of SBFD-aware UEs in Msg3. |
| CATT | Proposal 14: Do not support early identification for SBFD aware UEs on legacy ROs configured by legacy RACH configurations. |
| Sony | Proposal 10: The following methods are used to distinguish an SBFD UE from a legacy UE at the gNB: - PRACH from SBFD slot comes from SBFD UE - Distinguished using different RO, i.e., PRACH from RO belonging to the additional (SBFD) PRACH configuration in PRACH configuration Option 2, comes from SBFD UE, whilst PRACH from RO belonging to legacy PRACH configuration comes from legacy UE. - If the SBFD UE and legacy UE share an RO in an UL slot, preamble partitioning is used on these ROs to distinguish between SBFD UEs and legacy UEs. |
| Panasonic | Proposal 8: Additional RO can be used for the early indication. Separate preamble in legacy RO for early indication would not be required in this case. |
| Samsung | Proposal 10: For SBFD-aware UEs in RRC_IDLE mode, support an Early Indication mechanism. Proposal 11: For Early Indication by SBFD-aware UEs in RRC_IDLE mode, further consider the following 2 options: Option 1: L1-based Early Indication is supported. Option 2: L3-based Early Indication is supported. |
| Nokia, Nokia Shanghai Bell | Proposal 11. A SBFD-aware UE indicates its capability, if needed, by selecting RO from a set of ROs in SBFD symbols. |
| InterDigital | Proposal 9. Support early indication for SBFD-aware UE in RRC-Idle/Inactive modes to indicate UE’s support of SBFD operation as part of random-access, for example, via - SBFD-specific preamble selection, or - PRACH transmission in SBFD ROs, or - Indication as part of Msg3. |
| Google | Proposal 7: A UE in RRC-Idle/Inactive mode can indicate its support of SBFD by selecting an RO located in SBFD symbols or by explicit indication in Msg3. |
| Qualcomm | Proposal 22: RAN1 to discuss SBFD-awareness indication for RRC Idle/Inactive UE by preamble partitioning and/or SBFD-dedicated ROs. |

### Summary

Companies’ views on whether to support early indication for RRC_IDLE/INATCIVE SBFD-aware UE are summarized as below:

- Opt 1: Early indication by selecting additional-ROs:

  - Supported by: Spreadtrum, Ericsson, sony, [Samsung], Nokia, Nokia Shanghai Bell, InterDigital, Google, Qualcomm

- Opt 2: Early indication by preamble partitioning on legacy-ROs:

  - Supported by:  sony, [Samsung], InterDigital, Qualcomm, [CMCC], ZTE

  - Not supported by: CATT

- Opt 3: Early indication by Msg 3:

  - Supported by: Ericsson, Samsung, InterDigital, Google

For Opt 1, it is naturally supported when SBFD-ware UEs transmit PRACH on additional-ROs, it seems no additional agreement is needed.

For Opt 2, companies’ views are diverged, because preamble partitioning will reduce the RACH capacity of non-SBFD aware UEs.

For Opt 3, moderator suggests to discuss it in RAN2 similar to other topics in previous releases, e.g., redcap.

Moderator suggests initial proposal 2-2-1.

### 1st Round Proposals

#### Initial proposal 2-2-1:

Discuss whether to support preamble partitioning on legacy-ROs for early indication of SBFD-aware UEs.

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| CATT | We do not support preamble partitioning on legacy ROs for early indication of SBFD-aware UEs, but we are fine to discuss. |
| ZTE | Support preamble partitioning on legacy-ROs for early indication of SBFD-aware UEs. This allows NW to know the UE capability earlier so that can schedule subsequent transmissions/receptions in SBFD symbols. |
| Tejas | Msg 3 is a better solution for early indication. We can further discuss on partitioning the preamble. |
| Xiaomi | The only benefits to support early indication of SBFD-aware UE on legacy ROs is to reduce latency on msg3/msgB transmission. We don’t think this issue is essential as the overall delay of random access is already significantly reduced by additional RO. |
| InterDigital | We are fine to discuss early indication methods. We suggest adding an FFS: other techniques for early indication of SBFD-aware UEs. |
| Samsung | Do not support. Implicitly, additional ROs already provide a Msg.1 based EI mechanism. However, RACH config option 2 may not be always configured/enabled by the network. Similarly, using RACH config option 1, depending on upcoming agreements on RO selection, the UE may not always select the additional ROs. So Msg.3 based EI is also needed. We have concerns about the complexity introduced by preamble partitioning. Already for several NR features in earlier releases, there were such concerns. |
| Sharp | Support |
| LGE | We are open to discuss. However, we are aware that preamble partitioning may impact on random access capability of legacy UE with limited number of preambles. |
| QC | Support |
| DOCOMO | We prefer to not support preamble partitioning on legacy ROs. Early indication by additional-ROs is enough. |
| Ericsson | Not sure what the intention with this agreement is: to discuss more in future contributions or to use online time to discuss? Either way, this agreement seems unnecessary. We should have a discussion where the outcome/agreement is that either we do support or do not support preamble partitioning. |
| ETRI | We support preamble partitioning, and its potential fragmentation can be relieved by network configuration. |
| Fujitsu | We support to discuss and also support to utilize preamble partitioning mechanism for early indication. |
| Sony | Support. This beneficial for Configuration Option 1. This is not needed for Configuration Option 2. |
| Panasonic | We do not prefer to support preamble partitioning on legacy-ROs for early indication, but we are OK to discuss it. |
| Nokia | We prefer to not support preamble partitioning. |
| NEC | Support in principle, need clarifications on legacy RO partitioning |

## Issue#2-3: Msg4 HARQ-ACK PUCCH (4-step RA)

    - Submitted proposal

| Company | Proposals |
| --- | --- |
| MediaTek | Proposal 13: Support separate FDRA for SBFD and non-SBFD symbols when frequency hopping enabled for PUCCH in response to Msg4. |
| CMCC | Proposal 20: The interpretation of PRB locations of first hop and second hop for Msg4 HARQ-ACK feedback on PUCCH in SBFD symbols should be determined based on size of SBFD UL usable PRBs. Proposal 21: Separate power control parameters can be configured for Msg4 HARQ-ACK feedback on PUCCH transmission in SBFD symbols and non-SBFD symbols. - FFS which parameters can be separately configured, e.g., p0-nominal. |
| LG Electronics | Proposal 20: RAN1 to need to keep the discussion on RA specific enhancements for SBFD aware UE to evaluate benefits for Msg2/Msg3/Msg4 related transmission/reception in SBFD symbols. - Enhancements on the interpretation on the frequency domain resource assignment field and/or frequency hopping for Msg 3 PUSCH (re)transmission within UL usable PRB in SBFD symbol. - Enhancements on the PUCCH resource sets before dedicated PUCCH resource configuration in time domain and/or frequency domain (e.g., frequency hopping) for Msg4 HARQ-ACK PUCCH transmission within UL usable PRB in SBFD symbol. |
| TCL | Proposal 6: Define a new table for "cell-specific PUCCH resource common" that can be used for the transmission of HARQ-ACK for Msg4/MsgB in SBFD symbols. Proposal 7: To define a new table for the cell-specific PUCCH used for HARQ-ACK transmission of Msg4/MsgB in SBFD symbols, the following options can be considered: - Option 1: Consider the starting RB of the UL subband as a starting point and keep the existing RB offset values in table 9.2.1-1 in TS 38.213. - Option 2: Introduce new RB offset values in a new table to align and position the cell –specific PUCCH within the bandwidth of the UL subband. Proposal 8: For cell-specific PUCCH in SBFD symbols, consider a separate intra-slot frequency hopping (intra-SlotFH) configuration in PUCCH-configCommon. |
| vivo | Proposal 15: It can be discussed whether a single configuration or separate configurations are used for PUCCH carrying HARQ-ACK for Msg. 4 reception between SBFD symbols and non-SBFD symbols. Proposal 16: For PUCCH carrying HARQ-ACK for Msg. 4 and with frequency hopping, PRB index is derived by the usable UL subband can be considered. |
| OPPO | Proposal 17: For PUCCH transmission of HARQ-ACK for Msg4 in SBFD symbols, re-interpret the PUCCH PRB offset with respective to the lowest PRB of the UL subband (UL usable PRBs) and re-interpret the PUCCH frequency resources with respective to the UL subband size (the size of UL usable PRBs). |
| Xiaomi | Proposal 16:For the PUCCH resource carrying HARQ-ACK bit for Msg4/MsgB in SBFD symbols, it is determined within UL subband instead of initial UL BWP. The following modification for first PRB index calculation can be considered: - The PRB index of the PUCCH transmission in the first hop is calculated as RBBWPoffset+rPUCCHNCS. - The PRB index of the PUCCH transmission in the second hop is calculated as NUL, subbandsize-1-RBBWPoffset-rPUCCHNCS. |
| CATT | Proposal 22: Consider enhancements on PUCCH for Msg4 in RACH procedure to ensure PUCCH transmission in UL subband in SBFD symbols. |
| NEC | Proposal 10: • PUCCH carrying HARQ-ACK for Msg. 4 should be within the UL subband of SBFD symbols. |
| Samsung | Proposal 13: For SBFD-aware UEs, support enhancements to UL frequency resource allocation and frequency hopping behavior for transmission of RACH Msg.3 (PUSCH) and PUCCH A/N associated with RACH Msg.4 (PDSCH). |
| Sharp | Proposal 10: For msg4 HARQ-ACK PUCCH, in frequency resource offset determination, NBWPsize is set to the value of the size of the UL subband. |
| Qualcomm | Proposal 23: RAN1 to discuss how to enable efficient PUCCH frequency hopping in SBFD symbols for RRC idle/inactive UE. |

### Summary

To enable Msg 4 HARQ PUCCH transmission in SBFD symbols, the following two issues need to be resolved:

- Single or separate PUCCH configuration for SBFD symbols and non-SBFD symbols

- Whether/How to modify the frequency hopping formulation based on UL usable PRBs

Considering the similar issues for PUCCH in RRC_CONNECTED mode are under discussion in AI 9.3.1, moderator suggests to follow the related discussion progress in AI 9.3.1 and strive for a common solution.

## Issue#2-4: Others

### Submitted proposal

| Company | Proposals |
| --- | --- |
| New H3C | Proposal 14: UE is scheduled to transmit SDT PUSCH in SBFD time-frequency resource in inactive mode. |
| TCL | Proposal 10: RAN1 to study Paging in the SBFD symbols. |
| vivo | Proposal 17：For RA in RRC_IDLE/INACTIVE mode, at least SBFD time/frequency resources should be provided to UEs with SBFD capability by SIB. |
| Xiaomi | Observation 4: SBFD configuration broadcast via SIB1 is needed for RA in RRC_IDLE/INACTIVE state. |
| NEC | Proposal 13: - The aspects below should be considered for SDT enhancements on SBFD symbols: - PUSCH allocation/configuration in the UL subband of SBFD symbols - Valid PO determination on SBFD symbols - The PO mapping relationship with SSB for PO in SBFD symbols Proposal 14: - Study paging enhancement on SBFD symbols. |
| Lenovo | Proposal 11: RAN1 to discuss solutions to limit impact of inter-UE CLI that may occur due to RA operations in SBFD UL subband. |
| ETRI | Proposal 2: SIB1 should be enhanced to deliver RACH configurations and SBFD patterns. |
| InterDigital | Proposal 10. In cell selection or initial access, support indicating cells’ support on SBFD operation (e.g., in SIB1) to be used for cell ranking by SBFD-aware UEs. Proposal 11. Support prioritization rules for selecting the cells that support SBFD operation based on RSRP or potential CLI, during initial access or cell selection procedures for SBFD-aware UEs in RRC-IDLE/INACTIVE modes. |
| Sharp | Proposal 1: Location indication of SBFD subbands should be supported in SIB1. |

### Summary

Some companies propose to indicate the SBFD subband configuration by SIB1 for RRC_IDLE/IANCTIVE UE, moderator thinks this can be discussed in AI 9.3.1.

Regarding other proposals, e.g., SDT on PUSCH in SBFD symbols, paging enhancement, cell selection enhancement, moderator suggests to defer the discussion.

# Issue#3: Specific issues for CONNECTED mode

## Issue#3-1: RA trigger events

    - Submitted proposal

| Company | Proposals |
| --- | --- |
| ZTE, Sanechips | Proposal 15: RAN1 can focus on the potential enhancement to support SBFD random access operation triggered by PDCCH order. - Other triggering events, which may have no RAN1 impacts, can be discussed in RAN2. |
| Ericsson | Observation 8 RAN1 to leave to RAN2 to decide on triggering events for SBFD random access. |
| Xiaomi | Proposal 15: The enhancements considered for 4-step CBRA and 2-step CBRA should also be applied to CFRA with 4-step RA type and CFRA with 2-step RA type. |
| NEC | Proposal 1: - PRACH transmission in SBFD symbols during RRC_CONECTED mode should be supported at least for the following use case: - Handover procedure - Beam failure recovery - PDCCH order - UL data arrival with no availability of SR resources - SR failure |
| Sharp | Proposal 11: Support indicating which legacy or additional ROs should be used in a PDCCH order. |
| Qualcomm | Proposal 1: RAN1 to further discuss which triggering events to be supported for CBRA and CFRA in SBFD symbols. e.g. BFR, SR failure, handover/mobility, SI request, etc. |

### Summary

Some companies discussed where to handle the discussion of RA triggering events, e.g., in RAN1 or RAN2. There are the following views.

- Leave to RAN2 to decide on triggering events for SBFD random access.

- RAN1 to further discuss which triggering events to be supported for CBRA and CFRA in SBFD symbols.

- RAN1 can focus on the potential enhancement to support SBFD random access operation triggered by PDCCH order.

Moderator suggests to defer the discussion in this meeting.

# Proposals for online discussion

#### Updated proposal 1-1-1a:

Extend the previous agreements for UEs in RRC_CONNECTED mode to UEs in RRC_IDLE/INACTIVE mode.

#### Updated proposal 1-1-2a:

For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following:

- For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon.

- For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

#### Initial proposal 1-2-2:

Regarding RO validation for RACH configuration Option 1 with Alt 1-1, an RO across SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon is treated the same as an RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon.

#### Updated proposal 1-2-1a:

For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, down-select from the following alternatives.

- Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation.

- Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- Alt 2-2: Use a fixed value (e.g., 0) for the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- Alt 2-3: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs).

- Alt 2-4: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRB is msg1-FrequencyStart*scaling factor, where the scaling factor=UL usable PRB sizeUL BWP size.

- Note: Other alternatives are not precluded

#### Updated proposal 1-2-3:

For RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, support the following conditions on top of what was agreed in RAN1#116bis for RO validation.

- Condition#1: A valid RO starts at least Ngap symbols after a last downlink non-SBFD symbol.

- Condition#2: A valid RO starts at least Ngap symbols after the SSB.

- Note: The Ngap here is the same as the Ngap in the current specification

#### Initial proposal 1-2-4:

For RACH configuration Option 1 with Alt 1-1, the legacy SSB-RO mapping rule is reused for additional-ROs.

#### Initial proposal 1-3-1

For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

#### Initial proposal 1-3-2

For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration.

#### Initial proposal 1-3-3

For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

  - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

- For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

  - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

# Contact person

Please provide/update the information of the contact person in the following table to facilitate the discussions.

| Company | Name | Email address |
| --- | --- | --- |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| Sony | Shin Horng Wong | shinhorng.wong@sony.cm |
| ETRI | Cheulsoon Kim | cs.kim@etri.re.kr |
| IDC | Jonghyun Park | jonghyun.park@interdigital.com |
| TCL | Shahid Jan | shahid.jan@tcl.com |
| Google | Abdellatif Salah Kaopeng Chou | asalah@google.com nevillechou@google.com |
| SK Telecom | Doohee Kim | doohee.kim@sk.com |
| CATT | Yanping Xing | xingyanping@catt.cn |
| Ericsson | Magnus Åström | magnus.astrom@ericsson.com |
| Ericsson | Ratheesh Kumar Mungara | ratheesh.kumar.mungara@ericsson.com |
| Ericsson | Narendar Madhavan | narendar.madhavan@ericsson.com |
| NEC | Frank Zhang Pravjyot Deogun | Zhang_bohang@nec.cn Pravjyot.Deogun@EMEA.NEC.COM |
| Qualcomm | Muhammad | mabdelgh@qti.qualcomm.com |
| Fujitsu | Taewoo LEE | lee.taewoo@fujitsu.com |
| Panasonic | Tomoya Nunome | nunome.tomoya@jp.panasonic.com |
| Panasonic | Hidetoshi Suzuki | suzuki.hidetoshi@jp.panasonic.com |
| Spreadtrum | Huan Zhou | Huan.zhou@unisoc.com |
| ZTE | Xianghui Han | han.xianghui@zte.com.cn |
| Tejas Networks | Abhijith B G | abhijithb@tejasnetworks.com |
| Sharp | Tomoki Yoshimura | yoshimurat@sharplabs.com |
| Nokia | Karim kasan | karim.kasan@nokia.com |
| DOCOMO | Qiping Pi | piqp@docomolabs-beijing.com.cn |
| DOCOMO | Hiroki Harada | hiroki.harada.sv@nttdocomo.com |
| Samsung | Marian Rudolf | m.rudolf@samsung.com |
| Lenovo | Yuantao Zhang | zhangyt18@lenovo.com |
| LG | Minwoo song | Minwoo1.song@lge.com |
| LG | Yujin Noh | Yujin.noh@lge.com |

# References

- RP-234035  New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)  CMCC (Moderator, RAN1 VC)

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) Huawei

- R1-2405816	Discussion on SBFD Random Access Operation	MediaTek Inc.

- R1-2405832	Discussion for SBFD random access operation	New H3C Technologies Co., Ltd.

- R1-2405848	On subband full duplex random access operation	Huawei, HiSilicon

- R1-2405908	Discussion on SBFD random access operation	Spreadtrum Communications

- R1-2405934	Discussion on SBFD random access operation	ZTE Corporation, Sanechips

- R1-2405941	Discussion on SBFD Random Access Operation	Tejas Networks Limited

- R1-2405985	Discussion on SBFD random access operation	CMCC

- R1-2406060	Discussion on SBFD random access operation	LG Electronics

- R1-2406088	Discussion on SBFD random access operation	China Telecom

- R1-2406103	Discussion on SBFD random access operation 	TCL

- R1-2406106	Discussion on SBFD random access operation	Korea Testing Laboratory

- R1-2406135	SBFD Random access operation	Ericsson

- R1-2406182	Discussion on random access for Rel-19 SBFD	vivo

- R1-2406210	Discussion on SBFD random access operation	Kookmin University

- R1-2406238	Discussion on SBFD random access operation	OPPO

- R1-2406284	Discussion on SBFD random access operation	Xiaomi

- R1-2406368	Discussion on SBFD random access operation	CATT

- R1-2406471	SBFD PRACH Operations	Sony

- R1-2406514	Discussion on SBFD random access operation	Fujitsu

- R1-2406562	Discussion on random access for SBFD	NEC

- R1-2406569	Discussion on SBFD random access operation	Panasonic

- R1-2406575	Discussion on SBFD random access operation	Hyundai Motor Company

- R1-2406650	Random access on SBFD resources	Samsung

- R1-2406685	SBFD random access operation	Nokia, Nokia Shanghai Bell

- R1-2406688	SBFD random access operation	Lenovo

- R1-2406703	Discussion on SBFD random access operation	Transsion Holdings

- R1-2406726	SBFD random access operation	ETRI

- R1-2406750	On SBFD random access operation	InterDigital, Inc.

- R1-2406781	On SBFD random access operation	Google Ireland Limited

- R1-2406801	SBFD random access aspects	Sharp

- R1-2406836	Views on SBFD random access operation	Apple

- R1-2406886	Discussion on SBFD Random Access operation	KT Corp.

- R1-2406930	Discussion on SBFD random access operation	NTT DOCOMO, INC.

- R1-2407029	SBFD Random Access Operation	Qualcomm Incorporated

- R1-2407068	Discussion on SBFD random access operation for SBFD aware UEs in RRC CONNECTED state	ITRI

- R1-2407085	Discussion on SBFD random access operation	CEWiT

- R1-2407159	Discussion on SBFD random access operation	WILUS Inc.

# Previous agreements

## RAN1#116

Working assumption:

For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

Conclusion

No new PRACH format is introduced in Rel-19 duplex WI.

Agreement

For random access operation for SBFD-aware UEs in RRC CONNECTED state, at least consider the following options:

- Option 1: Use one single RACH configuration with possible enhancement

  - The ROs within UL subband in SBFD symbols can be valid for SBFD-aware UE

  - FFS: Further details

- Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

  - The ROs within UL subband in SBFD symbols configured by the additional RACH configuration can be valid for SBFD-aware UE

  - FFS: Further details

Agreement

For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols.

- FFS Type-2 random access procedure (2-step RACH)

Conclusion

If PRACH is allowed in SBFD symbols for SBFD-aware UEs in RRC_IDLE/INACTIVE mode, RAN1 observed the following:

- The benefits include at least one or more of the following:

  - reduced random access latency

  - reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots

  - improved coverage of PRACH with sparse UL resources

  - increased cell range of PRACH with sparse UL resources

- PRACH transmissions in UL subband in SBFD symbols may cause UE-to-UE CLI (similar to the case of RRC connected mode UEs) for some deployment scenarios. Initial studies based on two companies’ evaluation results, the DL performance degradation due to UE-to-UE CLI caused by PRACH transmission in SBFD symbols is not significant for indoor office scenario and Urban Macro scenario.

Agreement

For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols.

- FFS PRACH repetition in SBFD symbols.

- FFS PRACH repetition across SBFD symbols and non-SBFDs symbols.

Agreement

For SBFD-aware UEs in RRC CONNECTED state, further study the following two options:

- Option 1: a valid RO can only be on SBFD symbols or on non-SBFD symbols

  - a configured RO across SBFD and non-SBFD symbols in the same slot or across slots is invalid

- Option 2: a valid RO can be across SBFD and non-SBFD symbols in the same slot or across slots

RAN1 to leverage the study in Rel-18 as baseline.

Agreement

For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects:

- Msg2[/Msg4 PDSCH] reception in DL subband(s)

- Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping

- Msg3 repetition

- Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control

- FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE

Note: Strive to make progress in accordance to the discussion in AI 9.3.1.

## RAN1#116-bis

Agreement

Confirm the working assumption:

Working assumption:

For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, consider the following alternatives to derive the time and frequency resources of the configured ROs in SBFD symbols.

- Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon).

  - FFS the details

- FFS: Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s).

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, no separate prach-ConfigurationIndex to be configured in this option.

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, use existing random access configurations tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2 in TS38.211).

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

Agreement

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives:

- Alt 2-3:

  - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs.

  - FFS: The case where the additional-ROs partially overlap with non-SBFD symbols

- Alt 2-4:

  - The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs.

For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs.

Agreement

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration),

- For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed.

- For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used

Agreement

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, consider from the following alternatives:

  - Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

    - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

  - Alt 3: Introduce new entries on top of existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

- For FR1, consider from the following alternatives:

  - Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

    - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

  - Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211)

  - Alt 3: Introduce new entries on top of existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state,

- no enhancements for the RO validation rule for the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any).

  - FFS: the ROs in non-SBFD symbols that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs.

  - FFS: It’s up to network configuration to ensure the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, which are valid for non-SBFD aware UEs based on legacy RO validation rule, are also valid for SBFD aware UEs (i.e., the configured ROs in SBFD symbols, if configured as flexible by tdd-UL-DL-ConfigurationCommon, are within the UL usable PRBs)

- the RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon is valid if at least:

  - Time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB

  - FFS: Other condition.

Note: For the case that all the SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, there is no restriction that all the configured ROs in SBFD symbols should be within the UL usable PRBs.

## RAN1#117

Agreement

A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default.

A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE:

- The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH.

- UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols

- There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH.

- Other assumptions are not precluded.

NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB.

Agreement

Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

Agreement

Confirm the following working assumption.

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

Conclusion

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1:

- The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs.

- It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon.

  - If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs

Agreement

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, consider whether additional condition(s) on top of what was agreed in RAN1#116bis for RO validation are necessary. For example,

- Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol.

- Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB.

- Condition#3: A valid RO does not precede a SSB in the PRACH slot.

Agreement

Update the following agreement made in RAN1#116-bis meeting:

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, adopt the following:

  - Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

    - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

- For FR1, consider from the following alternatives:

  - Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

    - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

  - Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211)