3GPP TSG RAN WG1 #118	        		              R1-240xxxx

Maastricht, Netherlands, August 19th – 23th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 editorial corrections for TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core, NR_MC_enh-Core, NR_pos_enh2-Core, BWP_wor-Core |  |  |  |  |  | Date: |  |  | 2024-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Misaligned parameter names for multi-panel transmission with TS 38.331 v18.2.0 in Clauses 7.1.1, 7.2.1, and 7.7.1. - Undefined higher-layer parameter for SRS transmission with frequency hopping in Clause 7.3.1. - Typo in using PCMAX,f,c,k(i) instead of PCMAX,f,c,k(i), and typo in missing “second” for the configured maximum output power provided together with the second PHR in Clause 7.7.1. - Missing parentheses in indexes of the pseudo-code in Clause 9.1.3.1. - Redundant/ambiguous text in the pseudo-code in Clause 9.1.3.1. - Missing support for DCI format 1_3 in Clause 9.1.5. - Misaligned parameter name for apply-IndicatedTCIState with TS 38.331 v18.2.0 in Clauses 9.2.1, 9.2.2, 9.2.6, and 10.1. - Incorrect/non-existing parameter UCI-OnPUSCH-DCI-0-3 in Clause 9.3. - Missing parameter SearchSpaceExt-v1800 in the search space sets providing USS sets in Clause 10.1. - Unclear that the search space set dropping procedure is for search space sets with PDCCH candidates and non-overlapping CCEs counted on the primary cell in Clause 10.1. - Miscellaneous corrections or alignment of RRC parameter names in Clauses 5 and 7.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align parameter names with TS 38.331 v18.2.0 in Clauses 7.1.1, 7.2.1, and 7.7.1. - Capture the higher-layer parameter for SRS transmission with frequency hopping in Clause 7.3.1. - Change PCMAX,f,c,k(i) to PCMAX,f,c,k(i), and add “second” for the configured maximum output power provided together with the second PHR in Clause 7.7.1. - Add the missing parentheses in the indexes of the pseudo-code in Clause 9.1.3.1. - Remove redundant/ambiguous text in pseudo-code in Clause 9.1.3.1. - Clarify that the non-scheduled cells can be more than one in Clause 9.1.5. - Change apply-IndicatedTCIState to applyIndicatedTCI-State in Clauses 9.2.1, 9.2.2, 9.2.6, and 10.1. - Remove parameter UCI-OnPUSCH-DCI-0-3 and clarify that UCI-onPUSCH determines UCI multiplexing for PUSCH scheduled by DCI format 0_3 in Clause 9.3. - Add parameter SearchSpaceExt-v1800 in the search space sets providing USS sets in Clause 10.1. - Clarify that the search space set dropping procedure is for search space sets with PDCCH candidates and non-overlapping CCEs counted on the primary cell in Clause 10.1. - Correction/alignment of RRC parameter name in Clauses 5 and 7.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous/incorrect/incomplete specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5, 7.1.1, 7.2.1, 7.3.1, 7.4, 7.7.1, 9.1.3.1, 9.1.5, 9.2.1, 9.2.2, 9.2.6, 9.3, 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text is omitted ***

# 5	Radio link monitoring

The downlink radio link quality of the primary cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP, as described in clause 12, on the primary cell unless the UE indicates a capability bwpOperationMeasWithoutInterrupt [18, TS 38.306]. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in clause 13, the UE is expected to perform RLM using the associated SS/PBCH block when the associated SS/PBCH block index is provided by RadioLinkMonitoringRS.

If the UE is configured with a SCG, as described in [12, TS 38.331], and the parameter rlf-TimersAndConstants is provided by higher layers and is not set to release, the downlink radio link quality of the PSCell of the SCG is monitored by the UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the PSCell unless the UE indicates a capability bwpOperationMeasWithoutInterrupt [18, TS 38.306].

*** Unchanged text is omitted ***

### 7.1.1	UE behaviour

If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l

-	if the UE is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and is configured with multipanelSchemeSDM or multipanelSchemeSFN or sTx-2Panel, and the UE determines to apply both the first TCI-State or TCI-UL-State and the second TCI-State or TCI-UL-State in PUSCH transmission occasion i, the UE determines the PUSCH transmission power PPUSCH,b,f,c, k(i,j,qd,l) for the k-th indicated TCI-State or TCI-UL-State as

PPUSCH,b,f,c,ki,j,qd,l=minPCMAX,f,c,kiPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l   [dBm]

-	else, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as

PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ciPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l   [dBm]

*** Unchanged text is omitted ***

### 7.2.1	UE behaviour

If a UE transmits a PUCCH on active UL BWP b of carrier f in the primary cell c using PUCCH power control adjustment state with index l

-	if the UE is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and is configured with multipanelSFN-Scheme, and the UE determines to apply both the first TCI-State or TCI-UL-State and the second TCI-State or TCI-UL-State in PUCCH transmission occasion i, the UE determines the PUCCH transmission power PPUCCH,b,f,c, k(i,j,qd,l) for the k-th indicated TCI-State or TCI-UL-State as

PPUCCH,b,f,c,ki,qu,qd,l=minPCMAX,f,c,kiPO_PUCCH,b,f,cj+10log102μ∙MRB,b,f,cPUCCHi+PLb,f,cqd+∆F_PUCCHF+∆TF,b,f,ci+gb,f,ci,l   [dBm]

-	else, the UE determines the PUCCH transmission power PPUCCH,b,f,c(i,qu,qd,l) in PUCCH transmission occasion i as

PPUCCH,b,f,ci,qu,qd,l=minPCMAX,f,ciPO_PUCCH,b,f,cqu+10log102μ∙MRB,b,f,cPUCCHi+PLb,f,cqd+∆F_PUCCHF+∆TF,b,f,ci+gb,f,ci,l   [dBm]

*** Unchanged text is omitted ***

### 7.3.1	UE behaviour

*** Unchanged text is omitted ***

If a UE transmits SRS based on a configuration by SRS-PosResourceSet in SRS-PosRRC-InactiveValidityAreaConfig in RRC_INACTIVE state [12, TS 38.331], the active UL BWP b refers to the BWP provided by bwp in SRS-PosRRC-InactiveValidityAreaConfig. If the UE is not provided pathlossReferenceRS-Pos in SRS-PosResourceSet, or if the UE is provided pathlossReferenceRS-Pos in SRS-PosResourceSet and the UE cannot accurately measure the pathloss RS provided in pathlossReferenceRS-Pos, the UE calculates PLb,f,c(qd) using an RS resource from an SS/PBCH block with same index as the one the UE used to obtain MIB; otherwise, the UE uses the RS indicated by pathlossReferenceRS-Pos to calculate PLb,f,c(qd).

If a RedCap UE transmits SRS with frequency hopping outside the active UL BWP of carrier f of serving cell c in RRC_CONNECTED state based on a configuration by SRS-PosResourceSet, the active UL BWP b refers to the BWP provided by bwp in SRS-PosTx-Hopping.

If a RedCap UE transmits SRS with frequency hopping outside the initial UL BWP of carrier f of serving cell c in RRC_INACTIVE state based on a configuration by SRS-PosResourceSet, the active UL BWP b refers to the BWP provided by bwp in SRS-PosTx-Hopping.

*** Unchanged text is omitted ***

## 7.4	Physical random access channel

*** Unchanged text is omitted ***

If a PRACH transmission from a UE is in response to a detection of a PDCCH order by the UE that triggers a contention-free random access procedure and depending on the DL RS that the DM-RS of the PDCCH order is quasi-collocated with as described in clause 10.1

-	when the PRACH association indicator is not present in the PDCCH order, or

-	when the cell indicator field in the PDCCH order is not present or has value 0, or

-	when a value of a PRACH association indicator field in the PDCCH order is 0 if the UE is not provided SSB-MTC-AdditionalPCI, or

-	when the PRACH association indicator field in the PDCCH order indicates a physCellId associated with the cell of the PDCCH order reception,

or depending on an indicated SS/PBCH block

-	when the PRACH transmission is on a non-serving cell indicated by the cell indicator field in the PDCCH order, or

-	when a value of a PRACH association indicator field in the PDCCH order is 1 if the UE is not provided SSB-MTC-AdditionalPCI, or

-	when the PRACH association indicator field in the PDCCH order indicates a physCellId that is different than the physCellId associated with the cell of the PDCCH order reception,

referenceSignalPower is provided by a corresponding ss-PBCH-BlockPower.

*** Unchanged text is omitted ***

### 7.7.1	Type 1 PH report

If a UE determines that a Type 1 power headroom report for an activated serving cell is based on an actual PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c,

-	if for the active UL BWP b of carrier f of serving cell c, the UE is provided

-	twoPHRMode,

-	two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook',

-	dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and

-	multipanelSchemeSDM or multipanelSchemeSFN

the UE computes the Type 1 power headroom report associated with the k-th TCI-State or TCI-UL-State as

PHtype1,b,f,c,ki,j,qd,l= PCMAX,f,c,k(i)-PO_PUSCH,b,f,cj+10log102μ⋅MRB,b,f,cPUSCH(i)+αb,f,cj⋅PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l [dB]

-	else, the UE computes the Type 1 power headroom report as

PHtype1,b,f,ci,j,qd,l= PCMAX,f,ci-PO_PUSCH,b,f,cj+10log102μ⋅MRB,b,f,cPUSCH(i)+αb,f,cj⋅PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l [dB]

where PCMAX,f,c(i), PCMAX,f,c,k(i)PCMAX,f,c,k(i), PO_PUSCH,b,f,c(j), MRB,b,f,cPUSCH(i), αb,f,cj, PLb,f,c(qd), ∆TF,b,f,ci and fb,f,ci,l are defined in clause 7.1.1.

If a UE is configured with multiple cells for PUSCH transmissions, where a SCS configuration μ1 on active UL BWP b1 of carrier f1 of serving cell c1 is smaller than a SCS configuration μ2 on active UL BWP b2 of carrier f2 of serving cell c2, and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP b1 that overlaps with multiple slots on active UL BWP b2, the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the multiple slots on active UL BWP b2 that fully overlaps with the slot on active UL BWP b1. If a UE is configured with multiple cells for PUSCH transmissions, where a same SCS configuration on active UL BWP b1 of carrier f1 of serving cell c1 and active UL BWP b2 of carrier f2 of serving cell c2, and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP b1, the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the slot on active UL BWP b2 that overlaps with the slot on active UL BWP b1.

If a UE is configured with multiple cells for PUSCH transmissions and provides a Type 1 power headroom report in a PUSCH transmission with PUSCH repetition Type B having a nominal repetition that spans multiple slots on active UL BWP b1 and overlaps with one or more slots on active UL BWP b2, the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the one or more slots on active UL BWP b2 that overlaps with the multiple slots of the nominal repetition on active UL BWP b1.

For a UE configured with EN-DC/NE-DC and capable of dynamic power sharing, if E-UTRA Dual Connectivity PHR [14, TS 36.321] is triggered, the UE provides power headroom of the first PUSCH, if any, on the determined NR slot as described in clause 7.7.

If a UE is configured with multiple cells for PUSCH transmissions, the UE does not consider for computation of a Type 1 power headroom report in a first PUSCH transmission that includes an initial transmission of transport block on active UL BWP b1 of carrier f1 of serving cell c1, a second PUSCH transmission on active UL BWP b2 of carrier f2 of serving cell c2 that overlaps with the first PUSCH transmission if

-	the second PUSCH transmission is scheduled by a DCI format in a PDCCH received in a second PDCCH monitoring occasion, and

-	the second PDCCH monitoring occasion is after a first PDCCH monitoring occasion where the UE detects the earliest DCI format scheduling an initial transmission of a transport block after a power headroom report was triggered

or

-	the second PUSCH transmission is after the first uplink symbol of the first PUSCH transmission minus T'proc,2=Tproc,2 where Tproc,2 is determined according to [6, TS 38.214] assuming d2,1=1, d2,2=0, and with μDL corresponding to the subcarrier spacing of the active downlink BWP of the scheduling cell for a configured grant if the first PUSCH transmission is on a configured grant after a power headroom report was triggered.

If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c,

-	if for the active UL BWP b of carrier f of serving cell c, the UE is provided

-	twoPHRMode,

-	two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook',

-	dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and

-	multipanelSchemeSDM or multipanelSchemeSFN

the UE computes the Type 1 power headroom report associated with the k-th TCI-State or TCI-UL-State as

PHtype1,b,f,c,ki,j,qd,l= PCMAX,f,c,k(i)-PO_PUSCH,b,f,cj+αb,f,cj⋅PLb,f,cqd+fb,f,ci,l [dB]

-	else, the UE computes the Type 1 power headroom report as

PHtype1,b,f,ci,j,qd,l= PCMAX,f,c(i)-PO_PUSCH,b,f,cj+αb,f,cj⋅PLb,f,cqd+fb,f,ci,l [dB]

where PCMAX,f,c(i) and PCMAX,f,c,k(i) are computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2], [8-3, TS 38.101-3] and [8-5, TS 38.101-5]. The remaining parameters are defined in clause 7.1.1 and, if ul-powerControl is not provided, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0, and l=0. If ul-powerControl is provided and the UE is indicated one TCI-State or TCI-UL-State, PO_PUSCH,b,f,c(j), αb,f,cj and l are obtained by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State, PLb,f,c(qd) is obtained by PL-RS associated with the indicated TCI-State or TCI-UL-State. If ul-powerControl is provided and the UE is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, PO_PUSCH,b,f,c(j), αb,f,cj and l for PHtype1,b,f,c,ki,j,qd,l are obtained by p0AlphaSetforPUSCH associated with the k-th indicated TCI-State or TCI-UL-State, PLb,f,c(qd) is obtained by PL-RS associated with the k-th indicated TCI-State or TCI-UL-State.

*** Unchanged text is omitted ***

If a UE is provided, for active UL BWP b of carrier f of serving cell c,

-	twoPHRMode,

-	two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook',

-	dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and

-	multipanelSchemeSDM or multipanelSchemeSFN

the UE provides

-	a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State for an actual PUSCH transmission using a spatial domain filter corresponding only to the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State for a reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the second TCI-State or TCI-UL-State

-	a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State for an actual PUSCH transmission using a spatial domain filter corresponding only to the second TCI-State or TCI-UL-State, and a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State for a reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the first TCI-State or TCI-UL-State

-	a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State, for an actual PUSCH transmission using a spatial domain filter corresponding to the first TCI-State or TCI-UL-State and using a spatial domain filter corresponding to the second TCI-State or TCI-UL-State.

-	a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State for a reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State for another reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the second TCI-State or TCI-UL-State

*** Unchanged text is omitted ***

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

*** Unchanged text is omitted ***

while m<M

c=0

if harq-ACK-SpatialBundlingPUCCH is not provided,

while c<NcellsDL

if PDCCH monitoring occasion m is before an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn

c=c+1;

else

if there is a PDSCH reception on serving cell c that is scheduled by a DCI format scheduling more than one PDSCHs that provide respective more than one transport blocks with enabled HARQ-ACK information on respective more than one serving cells, where the DCI format is associated with a PDCCH reception in PDCCH monitoring occasion m and c is the smallest serving cell index among the more than one serving cells

if VC-DAI,c,mDL≤Vtemp

j=j+1;

end if

Vtemp=VC-DAI,c,mDL;

if VT-DAI,mDL=∅

Vtemp,2=VC-DAI,c,mDL;

else

Vtemp,2=VT-DAI,mDL;

end if

cnt=0;

mc=0;

while mc<Ncells,setDL,max

if the UE is scheduled PDSCH reception on serving cell mc, if any, from the more than one serving cells

if maxNrofCodeWordsScheduledByDCI is 2 for serving cell mc

oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+cntACK = HARQ-ACK information bit corresponding to the first transport block of this cell

oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+1+cntACK = HARQ-ACK information bit corresponding to the second transport block of this cell

cnt=cnt+2;

else

oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+cntACK = HARQ-ACK information bit corresponding to the transport block of this cell

cnt=cnt+1;

end if

end if

mc=mc+1;

end while

while cnt< NsetsTB,max

oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1NsetsTB,max⋅VC-DAI,c,mDL-1+cntACK= NACK;

cnt=cnt+1;

end while

Vs=Vs∪NsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1, …, NsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+NsetsTB,max-1;

end if

c=c+1;

end if

end while

else

while c<NcellsDL

if PDCCH monitoring occasion m is before an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn

c=c+1;

else

if there is a PDSCH reception on serving cell c that is scheduled by a DCI format scheduling more than one PDSCHs that provide respective more than one transport blocks with enabled HARQ-ACK information on respective more than one serving cells, where the DCI format is associated with a PDCCH reception in PDCCH monitoring occasion m and c is the smallest serving cell index among the more than one serving cells

if VC-DAI,c,mDL≤Vtemp

j=j+1;

end if

Vtemp=VC-DAI,c,mDL;

if VT-DAI,mDL=∅

Vtemp,2=VC-DAI,c,mDL;

else

Vtemp,2=VT-DAI,mDL;

end if

cnt=0;

mc=0;

while mc<Ncells,setDL,max

if the UE is scheduled PDSCH reception for transport blocks with enabled HARQ-ACK information on serving cell mc, if any, from the more than one serving cells

if maxNrofCodeWordsScheduledByDCI is 2 for serving cell mc

if the PDSCH reception provides two transport blocks

oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1Ncells,setDL,max⋅VC-DAI,c,mDL-1+cntACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell

else

oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1Ncells,setDL,max⋅VC-DAI,c,mDL-1+cntACK = HARQ-ACK information bit corresponding to the first transport block of this cell

end if

else

oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1Ncells,setDL,max⋅VC-DAI,c,mDL-1+cntACK= HARQ-ACK information bit of this cell

end if

cnt=cnt+1;

end if

mc=mc+1;

end while

while cnt< Ncells,setDL,max

oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1Ncells,setDL,max⋅VC-DAI,c,mDL-1+cntACK= NACK;

cnt=cnt+1;

end while

Vs=Vs∪Ncells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1, Ncells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1+1…, Ncells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1+Ncells,setDL,max-1;

end if

c=c+1;

end if

end while

end if

m=m+1;

end while

*** Unchanged text omitted ***

### 9.1.5	HARQ-ACK codebook retransmission

With reference to slots of PUCCH transmissions on the primary cell and for Type-1 or Type-2 HARQ-ACK codebooks, a UE that transmitted or would transmit a PUCCH or a PUSCH with a first HARQ-ACK codebook in slot m can be indicated by a DCI format with CRC scrambled by a C-RNTI or a MCS-C-RNTI that does not schedule a PDSCH reception [4, TS 38.212] on one or more serving cells and is received in a PDCCH ending in slot n, to transmit a PUCCH with the first HARQ-ACK codebook in slot n+k, where slot n+k is after slot m. The UE determines k and a resource for the PUCCH transmission as described in clauses 9.2.3 and 9.2.5. If the UE is provided a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern, the UE further determines a corresponding cell based on the periodic cell switching pattern as described in clause 9.A.

*** Unchanged text omitted ***

### 9.2.1	PUCCH Resource Sets

*** Unchanged text omitted ***

A PUCCH resource includes the following parameters:

-	a PUCCH resource index provided by pucch-ResourceId

-	an index of the first PRB prior to frequency hopping or for no frequency hopping by startingPRB, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an index of the first PRB after frequency hopping by secondHopPRB, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an indication for intra-slot frequency hopping by intraSlotFrequencyHopping, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an index of a first interlace by interlace0, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	if provided, an index of a second interlace by interlace1, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an index of an RB set by rb-SetIndex, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an indication for applying one or both of TCI-State or TCI-UL-State by applyIndicatedTCI-State, if provided

-	a configuration for a PUCCH format provided by format

*** Unchanged text omitted ***

### 9.2.2	PUCCH Formats for UCI transmission

*** Unchanged text omitted ***

A spatial setting for a PUCCH transmission by a UE is provided by

-	an indicated TCI-State or TCI-UL-State, if provided, as described in [6, TS 38.214];

-	PUCCH-SpatialRelationInfo if the UE is configured with a single value for pucch-SpatialRelationInfoId;

-	as described in [11, TS 38.321], if the UE is provided multiple values for PUCCH-SpatialRelationInfo. The UE applies corresponding actions in [11, TS 38.321] and a corresponding setting for a spatial domain filter to transmit PUCCH in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information with ACK value corresponding to a PDSCH reception providing the PUCCH-SpatialRelationInfo, each slot consists of Nsymbslot symbols as defined in [4, TS 38.211], and μ is the SCS configuration for the PUCCH

-	If PUCCH-SpatialRelationInfo or the indicated TCI-UL-State provides ssb-Index, the UE transmits the PUCCH using a same spatial domain filter as for a reception of a SS/PBCH block with index provided by ssb-Index for a same serving cell or, if servingCellId is provided, for a serving cell indicated by servingCellId

-	else if PUCCH-SpatialRelationInfo or the indicated TCI-UL-State provides csi-RS-Index, or the indicated TCI-State provides csi-rs configured with qcl-Type set to 'typeD', the UE transmits the PUCCH using a same spatial domain filter as for a reception of a CSI-RS with resource index provided by csi-RS-Index or csi-rs for a same serving cell or, if servingCellId or cell is provided, for a serving cell indicated by servingCellId or cell

-	else PUCCH-SpatialRelationInfo or the indicated TCI-UL-State provides srs, the UE transmits the PUCCH using a same spatial domain filter as for a transmission of an SRS with resource index provided by resource for a same serving cell and/or active UL BWP or, if servingCellId and/or uplinkBWP are provided, for a serving cell indicated by servingCellId and/or for an UL BWP indicated by uplinkBWP

-	an indicated applyIndicatedTCI-State, if provided

-	if applyIndicatedTCI-State = 'first', the UE transmits a PUCCH using a spatial domain filter corresponding to a first TCI-State or TCI-UL-State

-	if applyIndicatedTCI-State = 'second', the UE transmits a PUCCH using a spatial domain filter corresponding to second TCI-State or TCI-UL-State

-	if applyIndicatedTCI-State = 'both', the UE transmits a PUCCH using respective first and second spatial domain filters corresponding to the first and the second TCI-State or TCI-UL-State

If the UE

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on an active DL BWP of a serving cell, and

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on the active DL BWP of the serving cells,

the first and second TCI-State or TCI-UL-State are specific to the first and second CORESETs, respectively.

*** Unchanged text omitted ***

### 9.2.6	PUCCH repetition procedure

*** Unchanged text omitted ***

For NPUCCHrepeat>1,

-	the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots

-	if the UE is provided multipanelSFN-Scheme and applyIndicatedTCI-State = 'both', a repetition of the PUCCH transmission simultaneously uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State

-	a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same number of consecutive symbols, as provided by nrofSymbols

-	a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same first symbol, as provided by startingSymbolIndex if subslotLengthForPUCCH is not provided; otherwise mod(startingSymbolIndex, subslotLengthForPUCCH)

-	the UE is configured by interslotFrequencyHopping whether or not to perform frequency hopping for repetitions of the PUCCH transmission in different slots

*** Unchanged text omitted ***

The UE repeats the above procedure until there is no PUCCH overlapping with any PUCCH with repetitions in the slot.

When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes

-	first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clauses 7 and 7.2.1, or

-	first and second TCI-State or TCI-UL-State and applyIndicatedTCI-State = 'both', and the PUCCH resource does not include multipanelSFN-Scheme

the UE

-	uses the first and second spatial settings or the first and second indicated TCI-State or TCI-UL-State, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2,

-	alternates between the first and second spatial settings or between the first and second indicated TCI-State or TCI-UL-State, or between the first and second sets of power control parameters, respectively, per NPUCCHswitch repetitions of the PUCCH transmission, where NPUCCHswitch=1 if mappingPattern = 'cyclicMapping'; else, NPUCCHswitch=2.

*** Unchanged text omitted ***

## 9.3	UCI reporting in physical uplink shared channel

*** Unchanged text omitted ***

If a DCI format that includes a beta_offset indicator field with one bit or two bits, as configured by UCI-OnPUSCH for DCI format 0_1/0_3 or UCI-OnPUSCH-DCI-0-2 for DCI format 0_2, schedules the PUSCH transmission from the UE, the UE is provided by each of {betaOffsetACK-Index1, betaOffsetACK-Index2, betaOffsetACK-Index3}, the {first, second, third} values provided by betaOffsetsCrossPri0, or betaOffsetsCrossPri0DCI-0-2, and the {first, second, third} values provided by betaOffsetsCrossPri1, or betaOffsetsCrossPri1DCI-0-2, a set of two or four IoffsetHARQ-ACK, IoffsetHARQ-ACK,0, and IoffsetHARQ-ACK,1  indexes from Table 9.3-1 for multiplexing HARQ-ACK information in the PUSCH transmission and by each of {betaOffsetCSI-Part1-Index1, betaOffsetCSI-Part1-Index2} a set of two or four IoffsetCSI-1 indexes, and by each of {betaOffsetCSI-Part2-Index1, betaOffsetCSI-Part2-Index2} a set of two or four IoffsetCSI-2 indexes from Table  9.3-2, respectively, for multiplexing Part 1 CSI reports and Part 2 CSI reports, respectively, in the PUSCH transmission. The beta_offset indicator field indicates a IoffsetHARQ-ACK value and/or a IoffsetHARQ-ACK,0 value, and/or a IoffsetHARQ-ACK,1 value, a IoffsetCSI-1 value and a IoffsetCSI-2 value from the respective sets of values, with the mapping defined in Table 9.3-3 and in Table 9.3-3A. If the PUSCH transmission has priority 0 or priority 1, and the UE is provided uci-MuxWithDiffPrio, and the UE multiplexes HARQ-ACK information of priority 1 or priority 0 in the PUSCH, the UE applies the {first, second, third} values provided by betaOffsetsCrossPri1 = 'dynamic' for DCI format 0_1/0_3, betaOffsetsCrossPri1DCI-0-2= 'dynamic' for DCI format 0_2, or applies the {first, second, third} values provided by betaOffsetsCrossPri0 = 'dynamic' for DCI format 0_1/0_3, betaOffsetsCrossPri0DCI-0-2= 'dynamic' for DCI format 0_2.

*** Unchanged text omitted ***

## 10.1	UE procedure for determining physical downlink control channel assignment

*** Unchanged text omitted ***

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set on the primary cell of the MCG configured by

-	pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI, or

-	searchSpaceZero by providing searchSpaceID=0 for searchSpaceMCCH or searchSpaceMTCH for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI for broadcast, or

-	searchSpaceZero by providing searchSpaceID=0 for searchspaceMulticastMCCH for a DCI format 4_0 with CRC scrambled by a Multicast MCCH-RNTI, or by searchSpaceMulticastMTCH for a DCI format 4_1 with CRC scrambled by a G-RNTI for multicast in RRC_INACTIVE state

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0B-PDCCH CSS set configured by

-	searchSpaceMCCH and searchSpaceMTCH for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI for broadcast, on the primary cell of the MCG

-	searchspaceMulticastMCCH for a DCI format 4_0 with CRC scrambled by a Multicast MCCH-RNTI, or by searchSpaceMulticastMTCH for a DCI format 4_1 with CRC scrambled by a G-RNTI for PDCCH receptions in RRC_INACTIVE state

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-	a Type1A-PDCCH CSS set configured by sdt-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a C-RNTI or a CS-RNTI on the primary cell

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type2A-PDCCH CSS set configured by pei-SearchSpace in pei-ConfigBWP for a DCI format 2_7 with CRC scrambled by a PEI-RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by

-	SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI, or cellDTRX-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI, or

-	SearchSpace in pdcch-ConfigMulticast for DCI formats with CRC scrambled by G-RNTI, or G-CS-RNTI, or

-	searchSpaceMCCH and searchSpaceMTCH on a secondary cell for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI for broadcast, and

-	a USS set configured by

-	SearchSpace or by SearchSpaceExt-v1800 in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI, or NCR-RNTI

*** Unchanged text omitted ***

For a CORESET with index 0,

-	if the UE is provided TCI-State and followUnifiedTCI-State for the CORESET, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with the reference signals provided by the indicated TCI-State [6, TS 38.214]

-	else if the UE is provided dl-OrJointTCI-StateList and is indicated a first TCI-State and a second TCI-State, and applyIndicatedTCI-State for the CORESET

-	if the CORESET is associated with a Type 0/0A/2-PDCCH CSS set that has search space set index 0

-	if applyIndicatedTCI-State = 'first', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State,

-	if applyIndicatedTCI-State = 'second', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the second TCI-State,

-	if applyIndicatedTCI-State = 'none', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any

-	else

-	if applyIndicatedTCI-State = 'first', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State,

-	if applyIndicatedTCI-State = 'second', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the second TCI-State,

-	if applyIndicatedTCI-State = 'both', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first and the second TCI-State,

-	if applyIndicatedTCI-State = 'none', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET.

-	else, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with

-	the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or

-	the one or more DL RS configured by a TCI state provided by CandidateTCI-State, where the TCI state is indicated by an LTM Cell Switch Command MAC CE that triggers a RACH-less or RACH-based LTM cell switch, if any, or

-	a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure, or a SS/PBCH block the UE identified during a most recent configured grant PUSCH transmission as described in clause 19 or 22.1.

*** Unchanged text omitted ***

If a UE is provided dl-OrJointTCI-StateList and is indicated a first TCI-State and a second TCI-State, and is provided applyIndicatedTCI-State for a CORESET, other than a CORESET with index 0,

-	if the CORESET is associated only with USS sets and/or Type3-PDCCH CSS sets

-	if applyIndicatedTCI-State = 'first', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State

-	if applyIndicatedTCI-State = 'second', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the second TCI-State

-	if applyIndicatedTCI-State = 'both', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State and the second TCI-State

-	if the CORESET is associated at least with CSS sets other than Type3-PDCCH CSS sets,

-	if applyIndicatedTCI-State = 'first', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State

-	if applyIndicatedTCI-State = 'second', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the second TCI-State

-	if applyIndicatedTCI-State = 'both', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the reference signals provided by the first TCI-State and the second TCI-State

-	if applyIndicatedTCI-State = 'none', the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by a TCI state indicated by a MAC CE activation command for the CORESET

*** Unchanged text omitted ***

In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,Xs,μ and CPDCCHmax,Xs,μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,Xs,μ and CPDCCHtotal,Xs,μ respectively.

For all search space sets that a UE monitors PDCCH on the primary cell within a slot n, or within a group of Xs slots for a corresponding combination Xs,Ys, or within a span in slot n, denote by Scss a set of CSS sets, except for CSS sets provided by searchSpaceMCCH, searchSpaceMTCH, searchSpaceMulticastMCCH, searchSpaceMulticastMTCH or by SearchSpace in pdcch-ConfigMulticast for DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI, with cardinality of Icss and by Suss a set of USS sets and CSS sets provided by searchSpaceMCCH, searchSpaceMTCH or by SearchSpace in pdcch-ConfigMulticast for DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI with cardinality of Juss with PDCCH candidates and non-overlapping CCEs counted on the primary cell. The location of search space sets sj, 0≤j<Juss, in Suss is according to an ascending order of the search space set index.

Denote by MScss(i)(L), 0≤i<Icss, the number of counted PDCCH candidates for monitoring for CSS set Scss(i) and by MSuss(j)(L), 0≤j<Juss, the number of counted PDCCH candidates for monitoring for search space set Suss(j). If a UE indicates numBD-twoPDCCH-r17 with value of 3 and is provided searchSpaceLinkingId with same value for search space sets sj and si, with si<sj, set MScss(j)(L)=2∙MScss(j)(L) if si and sj are CSS sets or set MSuss(j)(L)=2∙MSuss(j)(L) if si and sj are USS sets.

*** Unchanged text omitted ***