3GPP TSG RAN WG1 #120                             	                                     	               R1-2500355

Athens, Greece, February 17th – 21st, 2025

Source: 	vivo

Title:	Adaptation of common signal/channel transmissions

Agenda Item:	9.5.3

Document for:	Discussion and Decision

- Introduction

In RAN#105, R19 NES WI was revised [1]. The objectives of the R19 NES work item are shown as follows. In this contribution, the views on the adaptation of common signal will be provided.

Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g., adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

- Discussion

Adaptation of common signal/channel has already been discussed in R18 NES SI [2]. In this section, SSB adaptation, PRACH adaptation, and paging adaptation and RRC parameters will be discussed respectively.

  - Adaptation of SSB in time domain

Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) Agreement The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 FFS: Whether the value of N can be implicitly determined using a timer

For adaptation of SSB configured for the SCell, one remaining case is still under discussion, i.e., adaptation for CD-SSB (B1). In our opinion, this case should not be supported since the Rel-19 NES-capable UE’s SCell may be a legacy UE’s PCell. A legacy UE assumes a fixed SSB periodicity and cannot accommodate or recognize the CD-SSB adaptation periodicity.

Proposal 1: For Rel-19 NES-capable UE’s SCell, adaptation for CD-SSB (B1) is not supported.

It has been agreed SSB periodicity adaptation for not CD-SSB on Rel-19 NES-capable UE’s SCell should be supported. However, the signaling mechanism for this adaptation remains undetermined.

During the RAN1 119 meeting, companies propose to use group-common DCI to indicate the SSB periodicity adaptation. In our opinion, it is unnecessary to use group-common DCI to provide the SSB burst periodicity information to all UEs. Because, to our best understanding, the valid motivation of SSB burst periodicity adaptation is to facilitate faster measurement for a given UEs, e.g., if a UE with high mobility needs denser SSB to perform L1 measurement, gNB will reduce the SSB periodicity using the SSB adaptation mechanism. This behavior is UE specific, thus group-common DCI is not necessary.

In our opinion, MAC CE should be supported to indicate the SSB adaptation, the benefit to use MAC CE is to reduce the DCI monitoring complexity and reduce specification effort. In AI 9.5.1, MAC CE based signaling has been supported to indicate the OD-SSB activation, and this mechanism can potentially be reused to indicate SSB burst periodicity adaptation in the time domain.

Proposal 2: For adaptation of SSB burst periodicity, the MAC-CE based signaling used for on-demand SSB can be reused.

When adapting the SSB burst periodicity in time domain via a MAC CE, the starting time for the adapted SSB periodicity should be determined. In our opinion, the definition of instant A in AI 9.5.1 can be reused, and the following proposal is made.

Proposal 3: The new SSB burst period is applied from the first SSB burst whose first actually transmitted SSB is at least T slots after the UE receives the MAC CE signalling.

- The value of T reuses that in AI 9.5.1.

In the last RAN1 meeting, some company points out that when adapting the SSB burst periodicity in time domain via a MAC CE, mismatches between SSB transmission and the SMTC window without SMTC window adaption occur, which potentially degrades UE measurement accuracy[3]. However, the gNB can ensure the SSB can drop in SMTC window by implementation. For example, the gNB transmits the indication signalling T slots before SMTC window.

Observation 1: When adapting the SSB burst periodicity in time domain via a MAC CE, the gNB can ensure the adapted SSB can drop in SMTC window by implementation.

  - Adaptation of PRACH in time domain

- Additional RO configuration

Agreement For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

It has been agreed that the additional PRACH resources are configured based on at least a PRACH configuration index which is same as or different from the PRACH configuration index for the legacy resources. Some potential enhancements are discussed when the PRACH configuration for additional PRACH resources is the same as or different from that for the legacy resources. We think the possible reason to introduce additional parameters for additional RACH resources is to provide flexibility for gNB to avoid overlapping between legacy ROs and additional ROs.

However, from our perspective, it is up to gNB how to configure the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs. The gNB can avoid the overlap case of additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs if this case is not expected. For example, in current TS 38.211 Table 6.3.3.2-3, when the legacy PRACH configuration index is 0, and the additional PRACH configuration index is 15, the time domain resources of legacy ROs and additional ROs are non-overlapped. Furthermore, it has been agreed that gNB can also configure separate frequency domain resource for the additional PRACH resources from the legacy resources. Therefore, PRACH resource configuration based on current PRACH configuration table and frequency domain parameter can provide sufficient flexibility and these enhancements in time domain are not necessary.

Observation 2: Existing PRACH configuration table and frequency domain parameter without any other enhancements in time domain can provide sufficient flexibility for gNB to configure additional PRACH resources.

Proposal 4: For Alt-1 or Alt-2, no additional mechanism for determining the additional PRACH resources in time domain (Opt 1-5) is supported.

- DCI-based adaptation for additional PRACH resources

Agreement Confirm the following working assumption from RAN1#118bis. Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - [DCI payload size = 1 bit] - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources - FFS: details Working Assumption For DCI-based adaptation for additional PRACH resources, - Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI - Option 1: Use reserved bits in the DCI format - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format - Option 2: Use Bits 5-8 within the Short Message (from upper layers) - Note: Availability should be confirmed by checking with RAN2. - Option 3: Use bits available for both Option 1 and Option 2

It has been agreed that the paging DCI (DCI format 1_0 with CRC scrambled by P-RNTI) is adopted to carry the adaptation indication for additional PRACH resources for UEs in idle/inactive and connected mode. During the RAN1 119 meeting, the detail DCI bits used for the adaptation for additional PRACH resources has three options for further study in which the reserved codepoint of Short Message indicator or the reserved bits in Short Message can be used. Before under-select the options for detail bits, the content for the paging DCI used to adaptation indication for additional PRACH resources should be firstly discussed. From our view, there are two alternatives can be considered:

- Alt-1: paging DCI contains activation or deactivation information.

- Alt-2: paging DCI contains activation information, and the length of validity time duration for availability is configured by RRC.

For Alt-1, 1 bit is enough. For example, “0” indicates activation, “1” indicates deactivation. For example, when the 5th bit in Short Message indicates “1”, the additional PRACH resources are activated, when the 5th bit in Short Message indicates “0”, the additional PRACH resources are deactivated. However, when a UE misses the deactivation signaling, the UE may transmit PRACH on the disabled additional PRACH resource which would cause interferences to other transmission and always result in RACH failure. To avoid this issue, the acknowledge for deactivation signaling is needed.

For Alt-2, one codepoint for the adaptation indication is sufficient, e.g., using the Short Message indicator “00”. When a UE receives a paging DCI with the Short Message indicator indicating “00”, the UE assumes the additional PRACH resources are available for a given time period starting from a time instance after the reception time of activation indication. The duration of the time period is defined as N unit times, where a unit time denotes the smallest unit of validity time of additional PRACH resource and N is the number of unit time configured by higher layer signaling. The time instance is determined based on the first unit time after the reception time of activation indication. If another activation signaling is detected within the availability time, i.e., the time distance between two activation DCIs is smaller than N unit times, the validity time duration associated with the two DCIs are overlapping and UE would assume additional PRACH resources are available for the union of the two durations. An example is illustrated in Figure 1.

In this way, there is no need to indicate deactivation with Alt. 2. Compared with Alt-1, there is no issue even if UE misses the paging DCI for PRACH adaptation. Therefore, Alt-2 is preferred for PRACH adaptation.

Proposal 5: For DCI-based adaptation for additional PRACH resources, Paging DCI contains activation information for additional PRACH resource, and the length of validity time duration for availability is configured by higher layer signaling.

- The reserved codepoint of Short Message indicator in the DCI format can be used for adaptation indication.

Proposal 6: When a UE receives an activation signaling, the UE assumes the additional PRACH resources are available for N unit times starting from the first unit time after the reception time of activation indication, where N is configured by higher layer signaling.

Furthermore, the unit time of the validity time need to be defined. For additional PRACH resources, following options can be considered for the granularity of unit time :

- Option 1: RO level

- Option 2: PRACH configuration period level

- Option 3: SSB-to-RO mapping cycle level

- Option 4: PRACH association period level

- Option 5: PRACH association pattern period level

- Option 6: SFN level

- Option 7: Paging cycle level

- Option 8: Modification period level

For option 1, if the unit time is RO level, the RRC signaling overhead is larger since there may be multiple ROs in a PRACH configuration period. For option 2/6, if the unit time is an additional PRACH configuration period or SFN level, when the SSB-RO mapping cycle is larger than an additional PRACH configuration period or multiple SFN units, non-uniform design is performed in different SSBs. However, this issue does not exist for option 3/4/5/7/8. For option 3, the SSB-to RO mapping cycle level is changing with the number of the valid ROs. For option 7 and option 8, the paging cycle and modification period are too long to make the DCI indication effective.

Comparing the option 4 and 5, PRACH association period is more flexible with finer granularity. So, option 4(PRACH association period level) is preferred as the unit time.

Figure 1 the starting time and ending time of the validity time

Another issue is the DCI processing time when determining the starting time after receiving the DCI. In our view, the DCI processing time can be defined similarly as that of the SPS release indication. For example, a UE is expected to determine the first time-unit after N symbols from the last symbol of a PDCCH providing the additional PRACH resource indication. For UE capability 2, N=5 for μ=0, N=5.5 for μ=1, and N=11 for μ=2, otherwise, N=10 for μ=0, N=12 for μ=1, N=22 for μ=2, N=25 for μ=3, N=100 for μ=5, and N=200 for μ=6, wherein μ corresponds to the SCS configuration of the PDCCH activating or deactivating the additional PRACH resource indication.

Proposal 7: A unit time of the validity time duration is a PRACH association period.

Proposal 8: The starting time of the validity time of additional PRACH resources is the first unit time (a PRACH association period) after UE receives and processes the activation DCI, where the DCI processing time can be defined similarly as that of the SPS release indication.

3）Prioritize the selection of additional PRACH resources

Agreement For the adaptation mechanisms of PRACH in time-domain - Support at least PRACH adaptation provided by Gnb without UE trigger - FFS: PRACH adaptation with UE trigger - Note: UE trigger means UE requests adaptation of PRACH - Study at least the following, - Dynamic signaling and/or semi-static signaling of PRACH adaptation - List options for Candidate channel types - Adaptation of PRACH transmission according to certain condition - Applicability to idle/inactive and/or connected mode Ues Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 Ues)

Since Rel-19 NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs, Rel-19 NES-capable UEs shall prioritize the selection of additional RO when there are both activated legacy RO and additional RO so that the congestion level of legacy RO would not be increased heavily. Furthermore, when the number of RACH failures exceeds a threshold in additional RO, the UE shall switch to legacy RO.

Proposal 913: To avoid the negative impact on legacy UE, the Rel-19 NES-capable UE should prioritize the selection of additional PRACH resources if applicable. When the number of RACH failures exceeds a threshold in additional PRACH resources, the UE shall switch to legacy PRACH resources.

  - Adaptation of paging occasions in the time domain

Agreement @RAN2#127bis 1. Select option-b as baseline for R19 NES paging enhancement. 2. R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) 3. Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. 4. Legacy UE is not barred. 5. Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration. Agreement @RAN2#128 Introduce value for Ns=8. FFS on 16. Introduce value for N= T/32. FFS on T/64, T/128, T/256.

It has been agreed that the specification details for PO/PF determination and paging-related configurations or procedures will be handled by RAN2, and RAN1 can discuss the RACH congestion caused by paging concentration. In RAN2, the option b (extend the values of N to have increased interval between PFs (e.g., T/32,) and compensating decrease in number of PFs by increasing POs per PF.) is agreed as the baseline for R19 NES paging enhancement. In this case, the possibility of RACH failure would be increased. More specifically, the network may page plenty of UEs in the confined paging occasions, after UEs receive the paging message, the UEs trigger the random-access procedure within a short period which would increase the RO congestion heavily. For example, as illustrated in Figure 2, aggregated POs may be configured within the PF of Rel-19, in this case, if the number of PRACH resources is not increased, there would be more UEs initiating access in the limited RO, especially when the PRACH period is large. Therefore, the PRACH resource congestion issue caused by paging concentration should be resolved.

Figure 2 The paging occasion and PRACH occasion for additional paging occasion case

Proposal 10: Study PRACH resource congestion issue caused by paging concentration.

- RRC parameter

Since the prach-ConfigurationIndex and msg1-FrequencyStart have been agreed to be supported for additional PRACH resources. The following parameters related to RRC parameters are listed.

Proposal 11: For PRACH adaptation, the RRC parameters are listed in Table 1.

Table 1 RRC parameters for PRACH adaptation

| Sub-feature group | RAN1 specification | RAN2 Parent IE | New or existing? | Parameter name in the text | Description | Value range | Per (UE, cell, TRP, …) | Required for initial access or IDLE/INACTIVE | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRACH adaptation | 38211 | RACH-ConfigGeneric | new | msg1-FrequencyStart-additionalPRACH | 1, Indicate the offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0 for additional PRACH resource. 2, When msg1-FrequencyStart-additionalPRACH is present, the msg1-FrequencyStart is also valid for legacy PRACH resource. | INTEGER (0..maxNrofPhysicalResourceBlocks-1), | Per cell | yes | Agreement At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH. Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format. |
| PRACH adaptation | 38211 | RACH-ConfigGeneric | new | prach-ConfigurationIndex-additionalPRACH | 1,Indicate the configuration index for additional PRACH resources. 2, When prach-ConfigurationIndex-additionalPRACH is present, the prach-ConfigurationIndex is also valid for legacy PRACH resource. | INTEGER (0..255) | Per cell | yes | Agreement For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format. |

- Conclusion

In this contribution, the following observations and proposals are summarized as follows:

Observation 1: When adapting the SSB burst periodicity in time domain via a MAC CE, the gNB can ensure the adapted SSB can drop in SMTC window by implementation.

Observation 2: Existing PRACH configuration table and frequency domain parameter without any other enhancements in time domain can provide sufficient flexibility for gNB to configure additional PRACH resources.

Proposal 1: For Rel-19 NES-capable UE’s SCell, adaptation for CD-SSB (B1) is not supported.

Proposal 2: For adaptation of SSB burst periodicity, the MAC-CE based signaling used for on-demand SSB can be reused.

Proposal 3: The new SSB burst period is applied from the first SSB burst whose first actually transmitted SSB is at least T slots after the UE receives the MAC CE signalling.

- The value of T reuses that in AI 9.5.1.

Proposal 4: For Alt-1 or Alt-2, no additional mechanism for determining the additional PRACH resources in time domain (Opt 1-5) is supported.

Proposal 5: For DCI-based adaptation for additional PRACH resources, Paging DCI contains activation information for additional PRACH resource, and the length of validity time duration for availability is configured by higher layer signaling.

- The reserved codepoint of Short Message indicator in the DCI format can be used for adaptation indication.

Proposal 6: When a UE receives an activation signaling, the UE assumes the additional PRACH resources are available for N unit times starting from the first unit time after the reception time of activation indication, where N is configured by higher layer signaling.

Proposal 7: A unit time of the validity time duration is a PRACH association period.

Proposal 8: The starting time of the validity time of additional PRACH resources is the first unit time (a PRACH association period) after UE receives and processes the activation DCI, where the DCI processing time can be defined similarly as that of the SPS release indication.

Proposal 9: To avoid the negative impact on legacy UE, the Rel-19 NES-capable UE should prioritize the selection of additional PRACH resources if applicable. When the number of RACH failures exceeds a threshold in additional PRACH resources, the UE shall switch to legacy PRACH resources.

Proposal 10: Study PRACH resource congestion issue caused by paging concentration.

Proposal 11: For PRACH adaptation, the RRC parameters are listed in Table 1.

# Reference

- 3GPP TSG RAN Meeting #105, Melbourne, Australia, Sep 9-12 2024, Revised WID for Enhancements of network energy savings for NR, RP-242354.

- TR 38.864, Study on network energy savings for NR, Release 18.

- R1-2409810, On adaptation of common signal/channel for NES enhancements, Apple, 3GPP TSG RAN WG1 #119, Orlando, US, Nov 18th – 22nd, 2024