3GPP TSG RAN WG1 #120		                                           R1-2500437

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.6.1

Source: 	OPPO

Title:	Signal design for LP-WUS and LP-SS

Document for:	Discussion & Decision

Introduction

In RAN#102 meeting, the WI [1] of LP-WUS was agreed, and it includes the following aspects that related to signal design for LP-WUS and LP-SS.

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

In this contribution, we further discuss the design details for LP-WUS and LP-SS.

LP-WUS signal

Waveform design of OOK-1 and OOK-4

Overlaid OFDM sequences design

During the SI phase of LP-WUS, there has been extensive discussion on the waveform generation. In RAN#102 meeting, approved WID has included OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol. The generation of OOK-1 and OOK-4 is as in the TR.

For OOK-1, only one OOK symbol can be transmitted per OFDM symbol, and if different data rates should be supported, it needs to be achieved by adjusting the SCS. For OOK-4, M OOK symbols can be transmitted per OFDM symbol, and different data rates can be supported by adjusting the value of M and/or the SCS. In our view, OOK-1 can be assumed as a special case of OOK-4, i.e., OOK-4 can fallback to OOK-1 when M is equal to 1. In the actual transmission of LP-WUS signals, the performance of LP-WUS is different due to the different channel quality.

OOK-1 and OOK-4 have been captured in TR 38.869 as following.

OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are - OOK=1 means all SCs are modulated - OOK=0 means all SCs are zero power (from base-band point of view)

OOK-4: Transform M-bit OOK in time domain - N SCs of OOK-4 are generated by a transformation (DFT/Least square) - N’ samples are generated from M-bits - signal modification may or may NOT be used - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’ - N’ can be the same as K

In our view, for OOK-4 with M = 1, there are two cases that ‘OOK = 1’ or ‘OOK = 0’ within one OFDM symbol. When ‘OOK = 0’, the waveform is same as OOK-1, it also means all SCs are zero power. While ‘OOK = 1’, according to the waveform generation process of OOK-4, the bit “1” will map to one sequence with the length equal to N, then all SCs of LP-WUS are modulated by the sequence. It means all SCs are modulated. The sequence mapped from “OOK = 1” could also be prestored at gNB, in this way, gNB also don’t need to do DFT when generating the waveform of LP-WUS.

The waveform of OOK-4 is similar as that of OOK-1 when M = 1. And following Working Assumption has been reached in RAN1#118bis meeting.

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1. Note: Concerns were raised by Ericsson and CATT that the above is not beneficial in terms of implementation complexity

Regardless OOK-1 or OOK-4 M=1, overlaid OFDM sequence (i.e. ZC sequence) would be mapped to the REs used to transmit LP-WUS signal if the OOK symbol within the OFDM symbol is ‘OOK-on’. Otherwise, no overlaid OFDM sequence is applies within the OFDM symbol. For all the values of M, UE has the same behavior when detecting the LP-WUS, and gNB also has the similar waveform generation process.

- Confirm the working assumption. Support option 1-1 for OOK-1 and OOK-4 M=1.

Regarding on the sequence length of overlaid OFDM sequence, following agreements had been made in RAN1#119 meeting.

Agreement For the overlaid OFDM sequence in time domain, the sequence is generated based on , - M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequence is generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) ,, - M=2, 4, is down-selected from the following: - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any. - Alt2:is given by the largest prime number such that, is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is. For example,, - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s) For example,, where, - FFS the value of G, with G>0. - FFS value(s) of root q and/or CS for generating the candidate sequences if applied - FFS on whether changes are needed to handle intra-/inter-cell interference - Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above. - Above overrides any previous RAN1 agreement / working assumption Agreement At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length from the following: - Alt1:, with - FFS how to map the generated overlaid OFDM sequence to the frequency domain - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included) Agreement CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling: - FFS: the set of values of CS and root for configuration - FFS: details of the RRC signaling

Considering the overlaid OFDM sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing, regardless of the value of M, it is better to have the same design of overlaid OFDM sequence.

In the case of M=1, for the overlaid OFDM sequence in time domain, the sequence is generated based on

,

is given by the largest prime number such that, is the overlaid OFDM sequence length.

In the case of M=2,4, there are two alternatives about , one has the same regulation as M=1, another introducing the zeros before and/or after ZC sequence Xq.

Figure 1: Illustration of alt1 and alt2 for M=2,4

For Alt 2, there are inserting zeros before and/or after the sequence Xq, it means there are more ‘low level envelope’ durations. Regarding on the overlaid OFDM sequence(s) for LP-SS. Following agreement has been reached in RAN1#118bis meeting.

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

OOK based receiver could only do the correlation detection based on the envelope of LP-SS. For the same LP-SS binary sequence, alt 1 has higher autocorrelation peak.

- Considering overlaid OFDM sequence(s) has been supported for LP-SS, and LP-SS would reuse the overlaid OFDM sequence(s) specified for LP-WUS. In case of M=2,4, Alt1 may result in higher autocorrelation peak than alt2.

- LP-WUS support the unified design of BZC for different M values and less impact on LP-SS detection performance. LP-WUS support Alt1 for M=2,4, i.e. BZC is given by the largest prime number such that BZC<LZC.

As for the sequence length of overlaid OFDM sequence LZC, there are two alternatives for M>1. In our views, it is natural to use LZC = 12*X/1 for M=1. We can also have unified design of LZC for different M values. On the other hand, if LZC = 12*X/M, after the signal generation and modification of waveform generation, the signal length in time domain is equal to 12*X, i.e. the number of REs of LP-WUS/LP-SS, it can be mapped to 12*X subcarriers directly. However, Alt1 means LZC<12*X (X=11 according to the previous agreement), it would not be mapped to 12*X subcarriers directly.

- It is natural to use LZC = 12*X/M for M=1.

- RAN1 support the unified design of LZC for different M values and less impact on signal mapping to the REs. LP-WUS support Alt2 for M>1, i.e. LZC =12*X/M.

M value for LP-WUS

Regarding the value of M for OOK-4, following agreement were reached in RAN1#118.

Agreement Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS. Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4.

In our views, the time domain resources required for M=4 are half of those for M=2. Thus, if the channel quality is good and the LP-WUS transmission performance is met, gNB could configure M=4 to reduce system resource overhead. On the other hands, M=4 for 15kHz SCS has been supported, it is not necessary to limit the value of M for different SCS. gNB could configure all supported value of M independently regardless of the SCS.

- Confirm the working assumption. Support M=4 for 30KHz SCS.

How to determine the SCS of LP-WUS

Figure 2 Duration of LP-WUS

The duration of LP-WUS maybe consist of multiple slots, e.g., 3 slots. From gNB perspective, when generating LP-WUS signal in every CP-OFDM symbol, the SCS of a CP-OFDM symbol used for LP-WUS can be the same as the SCS(s) used for other NR transmissions in the same CP-OFDM symbol. And in the whole duration of LP-WUS, it is better the SCS used for all CP-OFDM symbols used for LP-WUS could be fixed.

From UE perspective, it needs to determine the SCS used for LP-WUS. If UE not know the SCS of LP-WUS, it could not determine the length of one chip, and it is hard to perform OFDM sequence correlation detection. gNB can explicitly configure the SCS used for LP-WUS, or UE can determine the SCS used for LP-WUS based on reference signal, e.g. SSB.

LP-WUS SCS had been discussed with following agreements.

Agreement Support both OOK-1 and OOK-4 for LP-WUS. - FFS how OOK-1 and OOK-4 are specified - For OOK-4, M<=4, FFS supported values - The SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol - FFS different SCS. Agreement For the SCS used for LP-WUS and LP-SS, further discuss the following options: - The single SCS is configured by gNB - The single SCS is determined by pre-defined rule Agreement For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier - The single SCS for LP-WUS/LP-SS is same as the associated CD-SSB - FFS: Case where associated CD-SSB and initial DL BWP have different SCSs - FFS: Which initial BWP for RedCap

For RRC idle/ inactive states, there few FFS cases when SSB may have different SCSs. For the case where associated CD-SSB and initial DL BWP have different SCSs, it should be notedly that the legacy NR leave this issue to real resource allocation, e.g., reserving guard resources. We consider this approach can be used for LP-WUS/SS naturally.

We had identified the main cases for the RRC idle/ inactive states. For the CONNECTED state, the active BWP was considered to be possible with different SCS as for CD-SSB. Thus, it was proposal to make the SCS configurable CONNECTED state. However, we did not see the major different to issue in the above RRC idle/inactive states. If no other strong impact, we prefer to have same solution.

- For all RRC states, regardless associated CD-SSB and initial DL BWP have the same SCS or not, when the associated CD-SSB and LP-WUS are on the same cell

- Single SCS for LP-WUS/LP-SS is same as the associated CD-SSB.

Information carried by LP-WUS

Function of the LP-WUS signal

The main function of the LP-WUS signal is to wake up the MR for signal reception. In the IDLE/INACTIVE mode, the LP-WUS is mainly used for paging indication, it can wake up the target UE(s)’s MR to receive the paging message. The MR will initiate the RACH process after woken up. In the CONNECTED mode, the LP-WUS is mainly used to wake up the target UE(s)’s MR for PDCCH reception. Considering the design of LP-WUS needs to be applicable to both IDLE/INACTIVE and CONNECTED modes, it can carry wake-up indication information to wake up the target UE(s). The difference between the IDLE/INACTIVE and CONNECTED modes lies in the operations performed by the MR after the UE(s) is woken up.

- The content of LP-WUS should include the wake-up indication information.

  - In IDLE/INACTIVE mode, it could be used to indicate which UE(s) need to wake up the MR for RACH process, including the paging message reception.

  - In CONNECTED mode, it could be used to indicate which UE(s) need to wake up the MR for PDCCH monitoring.

For UE in the IDLE/INACTIVE mode, in addition to the wake-up indication, there are other information that needs to be received, such as cell information which mainly used for cell reselection, and UE can change the serving cell by receiving the cell information. Due to the worse oscillator performance of the UE’s LR, the UE needs to monitor the LP-SS to synchronize, and the LP-SS can also be used to assist the UE in RRM measurement. We think that when monitoring LP-WUS, the UE needs to monitor LP-SS to obtain synchronization and process the RRM measurement for the serving cell. Then, LP-SS is suitable to carry the cell information and serve the measurement purpose.

Regarding SI change and ETWS/CMAS information, they are currently included in short message in paging PDCCH. LP-WUS can wake up the MR to receive the paging PDCCH, it is not necessary to carry the SI change and ETWS/CMAS information in LP-WUS.

As in the IDLE/INACTIVE mode, the main purpose of LP-WUS is to wake up the UE’s MR for the RACH process, and other information beyond the wake-up indication could be indicated through other signals. Therefore, we have not observed a strong demand for LP-WUS to carry additional information (e.g., cell information, SI change and ETWS/CMAS information, tracking area information, RAN area information, etc.) beyond the wake-up indication.

- In IDLE/INACTIVE mode, it is not necessary for LP-WUS to carry additional information (e.g., cell information, SI change and ETWS/CMAS information, tracking area information, RAN area information, etc.)  beyond the wake-up indication. Other information beyond the wake-up indication could be indicated through other signals (e.g., LP-SS signal, paging PDCCH).

- The content of LP-WUS should include the wake-up indication information, additional information (e.g., cell information, SI change and ETWS/CMAS information, tracking area information, RAN area information, etc.) is not necessary to be carried in LP-WUS.

LP-WUS information to trigger PDCCH monitoring

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, following agreements were reached in RAN1#118 meeting.

Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA

During RAN1#119 meeting, some details were discussed as following.

For future meetings: Companies are encouraged to consider the following for future discussions Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, - Option 1: A bitmap with each bit corresponding to [one or more] UEs - 1A No CRC - Per bit coding, e.g., one bit is mapped to one of 2 codeword/sequences - The maximum number of codewords targeting per bit position in the bitmap for a UE is 1 - FFS coded bit length - Single MO - 1B No CRC - Bitmap-level coding, e.g., one bitmap of L bits is mapped to one of 2L codeword/sequences - The maximum number of codewords/sequences targeting for a UE is 2L-1 - Single MO - 1C With CRC - FFS CRC length - Single MO - Option 2: A codepoint value corresponding to [one or more] UEs - 2A only 1-1 mapping from codepoint to UE - The maximum number of codepoints targeting per MO for a UE is 1 - FFS the maximum number of MOs to be detected by a UE - 2B 1-to-1, 1-to-all mapping from codepoint to UEs - The maximum number of codepoints targeting per MO for a UE is up to 2 - FFS the maximum number of MOs to be detected by a UE - 2C 1-to-X mapping from codepoint to UEs, - The maximum number of codepoints targeting per MO for a UE is up to - FFS the maximum number of MOs to be detected by a UE - Note: L is the number of UEs associated monitoring the same WUS

For the information of LP-WUS, it is better to have the unified design for different RRC states. The option 2 would be sufficient for CONNECTED state UEs. If UE needs to detect larger number of codepoints, it would introduce additional complexity. And the power saving gain will be reduced. There should be some upper limit like 2 or 3. In that sense the Option 2B can be the tradeoff.

- Support option 2B: 1-to-1, 1-to-all mapping from codepoint to UEs

Information carried by overlaid OFDM sequence

Figure 3: ‘OOK symbol’ vs ‘OOK symbol w/ overlaid OFDM sequence(s)’

As shown in Figure 3, when transmitting OOK symbols, OFDM sequences could be overlaid over the OOK symbols.

The number of candidate OFDM sequences increases exponentially with the number of bits carried per OFDM sequence. Regarding the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip. Following agreement were reached.

Agreement Regarding the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, down-select from the below: - 4 - 8 - 16 - 32 - 64 for M not larger than 2 - FFS whether the same or different set of candidate overlaid sequences are used for different cells Agreement Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 4 candidates overlaid sequences for M=4

The complexity of sequence correlation detection by LR is higher, and the performance of sequence correlation detection would be worse when the number of candidate sequences is larger. In order to reduce the complexity of LR when performing sequence correlation detection, it is better to limit the number of candidate OFDM sequences.

For example, dividing the LP-WUS information into multiple segments, and each segment of indication information could be mapped to an OFDM sequence. For example, K OFDM sequences will be overlaid over OOK symbols on the time-domain resources of LP-WUS transmission when the indication information is divided into K segments, and the number of candidate OFDM sequences could be reduced from 2L to 2L/K (L is the number of entire information bits carried by OFDM sequences).

- Larger number of candidates overlaid OFDM sequences will increase the complexity of sequence correlation detection, and decrease the performance of sequence correlation detection.

Considering the Manchester encoding, one bit would be mapped to two bits. The number of OOK-on symbols is enough for carried the overlaid OFDM sequences. In our views, the small M values (e.g. 1,2) are aimed to lower SINR, in these cases, increasing the number of overlaid OFDM sequences would also impact the sequence detection performance. We think 4 candidate overlaid sequences is enough regardless of the values of M.

- Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell:

- support maximum 4 candidates overlaid sequences for M=1,2.

Regarding the overlaid OFDM sequence(s) of LP-WUS, following agreement were reached.

Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

Agreement Update the existing agreement as below In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS.

Considering gNB generates the OOK waveform per M OOK symbol, i.e. per OFDM symbol. gNB could also determine the overlaid OFDM sequence based on the OOK bits transmitted within the same OFDM symbol. In this way, gNB only need to determine the OFDM sequence once time when generating the M OOK symbol within one OFDM symbol. If there are same OFDM sequence within one OFDM symbol, UE could determine how to do the correlation detection.

Figure 3: Generate the overlaid OFDM sequence based the OOK bit transmitted within the OFDM symbol

For OOK-4 with M=1 and 2, the number of OOK-on waveform per OFDM symbol is at most 1. And for OOK-4 with M = 4, the number of OOK-on waveform per OFDM symbol is 2, thus the overlaid OFDM sequence can be transmitted with 2 repetition. In this case, OFDM based receiver only needs to detect one overlaid OFDM sequence within one OFDM symbol.

- For the information bits carried by the overlaid OFDM sequence(s):

- gNB determines the overlaid OFDM sequence(s) based on the OOK bit(s) transmitted within the OFDM symbol. In this way, it does not need to separately determine the bits mapping to the overlaid OFDM sequence when generating the M OOK waveform per OFDM duration.

- For OOK-4 with M=1 and 2, one sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbols.

- For OOK-4 with M=4, one sequence is selected from multiple candidates overlaid OFDM sequences on two OOK ‘ON’ symbols within one OFDM symbol, i.e. the overlaid OFDM sequence mapped from OOK bits within the OFDM symbol could be transmitted with repetition.

LP-WUS information for OOK to meet target requirements

We have reached the agreement to support Manchester encoding for LP-WUS. During RAN1#118 meeting, there are some discussion about the other coding schemes, and following agreement was reached.

Agreement Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements: - adding CRC - channel coding other than Manchester coding, including rate matching if any - mapped to a binary sequence/codeword, include truncation if any - repetition, including rate matching if any - Note: combination of options above is not precluded - Note: combination of Manchester coding with above is assumed FFS how LP-WUS is transmitted within a MO/MOs

At the same time, we have made following agreement and working assumption in 9.6.2 about the number of codepoints.

Agreement At least the following codepoints are supported for LP-WUS: - One codepoint corresponding to each of the subgroups that can be indicated by LP-WUS - One codepoint corresponding to all the subgroups that can be indicated by LP-WUS - FFS: Additional codepoints

Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

If the maximum number of subgroups per PO supported in Rel-19 is 32, considering one LP-WUS could indicate anyone subgroups. Maybe the number of codepoint is at least 32+1 = 33. Thus, the number of bits for one codepoint is at least 6. Even without any other process than Manchester coding, the information bits need to transmit via OOK symbol is 12. We suggest the time duration of one LP-WUS is no longer than one slot.

For M=1, after the Manchester coding, it needs to transmit at least 12 OOK symbols, i.e 12 OFDM symbols. In our views, following alternatives would increase the overhead significantly, which cause the time duration of one LP-WUS larger than one slot.

- adding CRC

- channel coding other than Manchester coding, including rate matching if any

- repetition, including rate matching if any

In order to limit the overhead, maybe it is better to map the codepoint to a binary sequence/codeword, e.g. 6 bits codepoint => 7 bits binary sequence => 14 OOK symbols, i.e. at most 14 OFDM symbols.

- The time duration of one LP-WUS is better no longer than one slot. Support mapping codepoint to a binary sequence before Manchester coding.

LP-SS signal

LP-SS modulation, coding and sequences

For low power signal tracking, the LP-SS may mainly carry the synchronization related information. That information can be modulated in the OOK symbols. However, the complex sequences cannot be applied, due to the lower capability of WUR. The suitable candidates can be real sequences like Gold or m sequences.

The exact real sequence is under evaluation, we also had agreements in RAN1 #117 for the determination of sequences on LP-SS.

Agreement For the LP-SS sequence used in a cell, - Option 1: the information necessary for determining the sequence is explicitly configured - FFS: Additional support of determining the sequence by predefined rule without configuration

Regarding the sequence selection, it was agreed to support Option 1 for binary LP-SS sequences for the ‘ON-OFF’ pattern.

Further, we can consider simple way to configure the sequence. The binary sequence can be associated with the cell ID for reducing neighboring cell interference and identification. The PCI have larger number of IDs than the current agreed number (up to 16) of LP-SS sequences. A mapping schemes can be introduced between cell ID and LP-SS sequences.

- LP-SS uses a binary sequence associated to the cell ID by configuration. FFS: mapping schemes between cell ID and LP-SS sequences.

Evaluation of LP-SS

Following agreement was reached during RAN1#118bis and RAN1#119 meeting for evaluating LP-SS.

Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4 - Other values of SNR, T, M are up to company report. - Additionally, companies can submit results on SINR with details on how the interference was modelled. - Assume one-shot estimation - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report. - As a starting point, the time error is based on 5-10ppm residual frequency error. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified - The frequency error: up to company report - Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS. - Channel: AWGN and TDL-C 300ns is assumed. - Consider cross-correlation for 4 binary sequences under time and frequency error - Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption Agreement To determine the binary sequences for LP-SS, for each M value, down-select the sequence length L from the corresponding candidate values: - M=1, L= {4,6,8} - M=2, L= {8,12,16,24} - M=4, L= {16,24,32,56} - FFS whether one or more than one L value (s) applied to each of M values M=1,2,4 - FFS the L values applied to other applicable SCS(s) - FFS the L values if other M values in addition to M=1,2,4 are supported. - FFS whether the time estimation averaged across multiple LP-SS occasions is applied - Note 1: with sequence length L, it includes Manchester coding (if supported for LP-SS) - Note 2: This doesn’t preclude any of three agreed sequence types Above applies at least for both 15kHz and 30kHz Agreement To determine the binary sequences for LP-SS, consider the following for performance comparison: - Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority)) - Assume the sliding window size is at least [-25us, 25us]. - RRM measurement accuracy X: the achieved measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s - Assume X= 3.5dB shall be satisfied for LP-RSRP and LP-RSRQ, based on Y=1, 4 and Z=1.28s for comparison - Cross correlation: maximum cross-cor value/ maximum auto-cor value FS how to use cross-correlation for comparison - Note: refer to RAN4 RRM assumption for the inter-cell interference Agreement For the length L of LP-SS binary sequence, limit the selection to the following: - M=1, L= {4, 6, 8} - M=2, L= {8, 12, 16} - M=4, L= {16, 32}

Base on the agreed evaluation assumption, we further evaluate the LP-SS with different parameters.

In evaluating the performance of LP-SS, the impact of CP should also be considered. For OOK-1, there is only one OOK symbol within one OFDM symbol, it means every OOK symbol has a CP. However, for OOK-4 with M=2 and M=4, there is more than one OOK symbols within one OFDM symbol, only the 1st OOK symbol has the CP. The actual LP-SS signal transmitted of gNB is as following.

- The impact of CP duration should be considered when design and evaluate the LP-SS.

We evaluate the performance of different LP-SS sequence under the following assumption.

| Parameters | Value |
| --- | --- |
| Carrier Frequency | 2.4GHz |
| SCS | 30kHz |
| SNR | -3dB |
| Target | T = 5us for OOK-1 T = 2us for OOK-4 with M=2 T = 1us for OOK-4 with M=4 |
| Frequency error | 20ppm |
| Sampling rate | 7.68MHz |
| Channel | TDL-C 300ns |

Other assumption and cross-checking results could be found in the attachment (a separate excel sheet).

A set of binary sequences were generated for that evaluation, to give some preliminary assessment on the needed sequence and time resource.

- M = 1 @ L=4

For L=4, Based on the performance of cross correlation and auto correlation, we suggest following LP-SS sequence if L=4 for M=1.

| Binary sequence | T / us |
| --- | --- |
| 0 0 1 1 | 4.43 |
| 0 1 0 1 | 4.94 |
| 0 1 1 0 | 12.89 |
| 1 0 0 1 | 1.56 |

- For L=4 @M=1, considering balance between “1” and “0” of the binary sequence, only 3 binary sequence could meet the requirement of timing estimation error for SNR=-3dB.

- M = 1 @ L=6

For L=6, Based on the performance of cross correlation and auto correlation, we suggest following LP-SS sequence if L=6 for M=1.

| Binary sequence | T / us |
| --- | --- |
| 1 0 0 1 0 1 | 1.43 |
| 1 0 1 0 0 1 | 1.43 |
| 1 0 0 0 1 1 | 1.69 |
| 1 1 0 0 0 1 | 1.82 |

- For L=6 @M=1, considering balance between “1” and “0” of the binary sequence, timing estimation error could smaller than 2us for SNR=-3dB.

- M = 1 @ L=8

For L=8, Based on the performance of cross correlation and auto correlation, we suggest following LP-SS sequence if L=8 for M=1.

| Binary sequence | T / us |
| --- | --- |
| 1 0 0 1 0 1 0 1 | 1.56 |
| 1 0 1 0 0 1 0 1 | 1.56 |
| 1 0 1 0 1 0 0 1 | 1.56 |
| 1 0 0 0 1 1 0 1 | 1.70 |

- For L=8 @M=1, considering balance between “1” and “0” of the binary sequence, timing estimation error could smaller than 2us for SNR=-3dB.

- M = 2 @ L=8

For L=8, Based on the performance of cross correlation and auto correlation, we suggest following LP-SS sequence if L=8 for M=2.

| Binary sequence | T / us |
| --- | --- |
| 1 1 0 0 0 0 1 1 | 1.43 |
| 1 0 0 0 1 0 1 1 | 1.95 |
| 1 0 0 1 0 0 1 1 | 1.95 |
| 1 0 1 0 0 0 1 1 | 1.95 |

- For L=8 @M=2, considering balance between “1” and “0” of the binary sequence, timing estimation error could smaller than 2us for SNR=-3dB.

- M = 2 @ L=12

For L=12, Based on the performance of cross correlation and auto correlation, we suggest following LP-SS sequence if L=12 for M=2.

| Binary sequence | T / us |
| --- | --- |
| 1 1 0 0 0 0 1 1 0 0 1 1 | 1.56 |
| 1 1 0 0 1 1 0 0 0 0 1 1 | 1.56 |
| 1 1 0 0 0 0 0 0 1 1 1 1 | 1.69 |
| 1 1 1 1 0 0 0 0 0 0 1 1 | 1.69 |

- For L=12 @M=2, considering balance between “1” and “0” of the binary sequence, timing estimation error could smaller than 2us for SNR=-3dB.

- M = 2 @ L=16

For L=16, Based on the performance of cross correlation and auto correlation, we suggest following LP-SS sequence if L=16 for M=2.

| Binary sequence | T / us |
| --- | --- |
| 1 1 0 0 1 1 0 0 0 0 1 1 0 0 1 1 | 1.56 |
| 1 1 0 0 0 0 1 1 0 0 1 1 0 0 1 1 | 1.56 |
| 1 1 0 0 1 1 0 0 1 1 0 0 0 0 1 1 | 1.56 |
| 1 1 0 0 0 0 0 0 1 1 0 0 1 1 1 1 | 1.69 |

- For L=16 @M=2, considering balance between “1” and “0” of the binary sequence, timing estimation error could smaller than 2us for SNR=-3dB.

- M = 4 @ L=16

For L=16, Based on the performance of cross correlation and auto correlation, we suggest following LP-SS sequence if L=16 for M=4.

| Binary sequence | T / us |
| --- | --- |
| 1 1 1 1 0 0 0 0 0 0 0 0 1 1 1 1 | 1.56 |
| 1 0 1 1 0 0 0 0 0 1 0 0 1 1 1 1 | 1.82 |
| 1 1 0 1 0 1 0 0 1 0 0 0 1 0 1 1 | 1.82 |
| 1 0 0 1 0 0 0 0 0 1 1 0 1 1 1 1 | 1.82 |

- For L=16 @M=4, considering balance between “1” and “0” of the binary sequence, binary sequence could not meet the requirement of timing estimation error for SNR=-3dB.

- M = 4 @ L=32

For L=32, Based on the performance of cross correlation and auto correlation, we suggest following LP-SS sequence if L=32 for M=4.

| Binary sequence | T / us |
| --- | --- |
| 1 1 1 1 0 0 0 0 0 0 0 0 1 1 1 1 0 0 0 0 1 1 1 1 1 1 1 1 0 0 0 0 | 1.43 |
| 1 1 1 1 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 1 1 1 1 | 1.43 |
| 1 1 1 1 0 0 0 0 0 1 1 0 1 0 0 1 1 1 1 1 0 0 0 0 0 1 1 0 1 0 0 1 | 1.88 |
| 1 1 0 1 0 0 0 0 0 0 1 0 1 1 1 1 1 1 0 1 0 0 0 0 0 0 1 0 1 1 1 1 | 1.82 |

- For L=32 @M=4, considering balance between “1” and “0” of the binary sequence, binary sequence could not meet the requirement of timing estimation error for SNR=-3dB.

- For M=4, the values for L to be selection could not meet the requirement of timing estimation error for SNR=-3dB.

LP-SS in time and frequency

As the LP-SS is for assisting LP-WUS, the closest or same frequency location should be assumed. And the same occupation of bandwidth should be used.

For LP-WUS signal bandwidth 11RBs in 30KHz SCS as example, ideally the LP-SS should be with same SCS and number of RBs.  Further as also been provided by LP-WUS, same occupied bandwidth even for different SCSs would also help for reducing LR receiver complexity.

In time domain the periodicity is considered at least 320ms for LP-SS transmission. In the LP-WUS, we prefer the simple schemes with only the wake-up information and necessary coding. For the low power waveform receiving, preamble may not be introduced. However, some other signal can serve for the training purpose. Consider the LP-SS, shorter periodicity like 80ms or 160ms can be adopted.

Figure 10. TDM based multiplexing for LP-WUS and LP-SS.

The wake-up schemes support different receivers. The overlaid sequences of WUS would depend on the measurement of PSS/SSS. Thus, the LP-WUS may be configured with fixed relative location in frequency with SSB. If the LP-WUS and LP-SS share the same frequency location, SSB location should be associated with LP-SS.

Figure 11. Time and frequency location between SSB and LP-WUS/SS

In time domain, the LP-SS should be periodically transmitted. The LP-SS transmission should consider the multi-beam operations. A set of LP-SSs can be transmitted in a period. Each LP-SS may be associated with a beam.

- LP-WUS and LP-SS share the same frequency location, SSB location should be associated with LP-WUS/LP-SS.

- Consider shorter periodicity like 80ms or 160ms for LP-SS.

- Multiple LP-SSs can be transmitted in a period. Each LP-SS can be associated with a beam/SSB.

Necessity of Additional synchronization signal

LP-SS can provide synchronization. The necessity of additional synchronization signal highly depends on whether LP-SS alone is sufficient for synchronization requirement for LP-WUS detection. There are some discussions on it for additional synchronization signal.

Our views in brief:

- When the periodicity of LP-SS is small, LP-SS alone is sufficient for synchronization requirement for LP-WUS detection. It is no need to consider additional synchronization signal.

- When the periodicity of LP-SS is large and LP-SS alone is not sufficient for synchronization requirement in some cases,

  - If consider additional synchronization signal(s) between every two LP-SS, and minimize the difference between sequence design for periodic LP-SS, the overhead may be similar to reducing the LP-SS periodicity. Then, it is better to reduce the LP-SS periodicity.

  - If consider the additional synchronization signal as the preamble of LP-WUS, maybe multiple LP-WUS needs to transmit between two LP-SS, it not clear whether it can decrease the overhead compared with reducing the periodicity of LP-SS

  - In this phase of WI, it is better to focus on the design of LP-WUS and LP-SS.

During RAN1#119 meeting, following agreements were reached.

Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different. Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present

Timing error for different combinations of LP-SS periodicity, residual frequency error Fr, and residual timing error Te has been captured in FL’s summary as below.

- Table 4r1 Timing error based on combinations of Fe, Tr and LP-SS periodicity

| LP-SS periodicity | Fe=5 ppm for OOK-WUR |  |  | Fe=10 ppm for OOK-WUR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Tr=0.5,1,2, 5 us | Tr=0.5,1,2 us | Tr=0.5,1 us | Tr=0.5,1,2, 5 us | Tr=0.5,1,2 us | Tr=0.5,1 us |
| Time error tolerance Te in TR 38.869 | 5 (OOK-1) | 3 (OOK4, M=2) | 1(OOK4, M=4) | 5 (OOK-1) | 3 (OOK4, M=2) | 1(OOK4, M=4) |
| 320 ms | 2.1, 2.6, 3.6, 6.6us | 2.1, 2.6, 3.6 | 2.1,2.6 | 3.7, 4.2, 5.2, 8.2us | 3.7, 4.2, 5.2 us | 3.7, 4.2us |
| 160 ms | 1.3us,1.8us, 2.8us,5.8us | 1.3us,1.8us,2.8us | 1.3us-1.8us | 2.1us, 2.6us,3.6us,6.6us | 2.1us, 2.6us,3.6us | 2.1, 2.6us |
| 80 ms | 0.9us, 1.4us, 2.4us,5.4us | 0.9us,1.4us,2.4us | 0.9us-1.4us | 1.3us,1.8us,2.8us,5.8us | 1.3us,1.8us,2.8us | 1.3,1.8us |

Regarding on the M value for LP-WUS and LP-SS. If the M values for LP-WUS and LP-SS can be configured to be different. In cased of larger M value for LP-SS with smaller M value for LP-WUS, it is no need to reduce the periodicity of LP-SS in some cases. However, in case of larger M value for LP-WUS with smaller M value for LP-SS, it maybe needs to reduce the periodicity of LP-SS.

- For LP-WUS with M=1

- Fe=5ppm:

If LP-SS could only be configured with same M value, i.e. M=1, LP-SS design should limit the Tr<5us, in this case, no need to support 80ms, 160ms periodicity for LP-WUS with M=1.

If LP-SS could be configured with larger M values, i.e. M=2, 4, maximum timing error of LP-SS is 3.6us<5us for 320ms periodicity. No need to support 80ms, 160ms periodicity for LP-WUS with M=1.

- Fe=10ppm:

If LP-SS could only be configured with same M value, i.e. M=1, LP-SS design should limit the Tr<5us. In this case, when Tr=0.5,1us, no need to support 80ms, 160ms periodicity for LP-WUS with M=1. When Tr = 2us, at least need to consider 160ms periodicity for LP-SS.

If LP-SS could be configured with larger M values, i.e. M=2, 4, when Tr<2us, maximum timing error of LP-SS is 4.2us<5us for 320ms periodicity. No need to support 80ms, 160ms periodicity for LP-WUS with M=1.When Tr=2us, at least need to consider 160ms periodicity for LP-SS.

- For LP-WUS with M=2, when Fe=5ppm

- If LP-SS could only be configured with same M value, i.e. M=2

When Tr = 2us, the timing error of LP-SS is 3.6us>3us for 320 ms periodicity. At least need to consider 160ms periodicity for LP-SS.

When Tr < 2us, i.e. Tr=0.5us,1us, the maximum timing error of LP-SS is 2.6us<3us for 320ms periodicity. No need to support 80ms, 160ms periodicity for LP-WUS with M=1.

- If LP-SS could be configured with larger M value, i.e. M=4, maximum timing error of LP-SS is 2.6us<3us for 320ms periodicity. No need to support 80ms, 160ms periodicity for LP-WUS with M=2.

- If LP-SS could be configured with smaller M value, i.e. M=1,

When Tr = 0.5us,1us, the maximum timing error of LP-SS is 2.6us<3us for 320ms periodicity. No need to support 80ms, 160ms periodicity for LP-WUS with M=1.

When Tr = 2us, the timing error of LP-SS is 3.6us>3us for 320 ms periodicity. At least need to consider 160ms periodicity for LP-SS.

When Tr = 5us, LP-SS with 80ms periodicity still could not meet the performance requirement.

- For LP-WUS with M=2, when Fe=10ppm

- If LP-SS could only be configured with same M value, i.e. M=2, LP-SS with 320ms periodicity could not meet the performance requirement.

When Tr < 2us, i.e. Tr=0.5us,1us, at least need to consider 160ms periodicity for LP-SS.

When Tr = 2us, only LP-SS with 80ms periodicity could meet the performance requirement.

- If LP-SS could be configured with larger M value, i.e. M=4, at least need to consider 160ms periodicity for LP-SS.

- If LP-SS could be configured with smaller M value, i.e. M=1,

When Tr = 0.5us,1us, at least need to consider 160ms periodicity for LP-SS.

When Tr = 2us, only LP-SS with 80ms periodicity could meet the performance requirement.

When Tr = 5us, LP-SS with 80ms periodicity still could not meet the performance requirement.

- For LP-WUS with M=4,

- when Fe=5ppm:

Only LP-SS periodicity = 80ms under Tr=us could meet the performance requirement.

- when Fe=10ppm:

All the LP-SS design could not meet the performance requirement.

- Support Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

- Considering timing error requirement for M=4, support option 1A: No additional sync signal, LP-SS periodicity =320ms, and additional 80ms periodicity.

Conclusion

In this contribution, we discussed the signal design for LP-WUS and LP-SS.  Observations and proposals are summarized as following.

- LP-WUS

- Confirm the working assumption. Support option 1-1 for OOK-1 and OOK-4 M=1.

- Observation 1:	Considering overlaid OFDM sequence(s) has been supported for LP-SS, and LP-SS would reuse the overlaid OFDM sequence(s) specified for LP-WUS. In case of M=2,4, Alt1 may result in higher autocorrelation peak than alt2.

- LP-WUS support the unified design of BZC for different M values and less impact on LP-SS detection performance. LP-WUS support Alt1 for M=2,4, i.e. BZC is given by the largest prime number such that BZC<LZC.

- It is natural to use LZC = 12*X/M for M=1.

- RAN1 support the unified design of LZC for different M values and less impact on signal mapping to the REs. LP-WUS support Alt2 for M>1, i.e. LZC =12*X/M.

- Confirm the working assumption. Support M=4 for 30KHz SCS.

- For all RRC states, regardless associated CD-SSB and initial DL BWP have the same SCS or not, when the associated CD-SSB and LP-WUS are on the same cell

- Single SCS for LP-WUS/LP-SS is same as the associated CD-SSB.

- The content of LP-WUS should include the wake-up indication information.

  - In IDLE/INACTIVE mode, it could be used to indicate which UE(s) need to wake up the MR for RACH process, including the paging message reception.

  - In CONNECTED mode, it could be used to indicate which UE(s) need to wake up the MR for PDCCH monitoring.

- In IDLE/INACTIVE mode, it is not necessary for LP-WUS to carry additional information (e.g., cell information, SI change and ETWS/CMAS information, tracking area information, RAN area information, etc.)  beyond the wake-up indication. Other information beyond the wake-up indication could be indicated through other signals (e.g., LP-SS signal, paging PDCCH).

- The content of LP-WUS should include the wake-up indication information, additional information (e.g., cell information, SI change and ETWS/CMAS information, tracking area information, RAN area information, etc.) is not necessary to be carried in LP-WUS.

- Support option 2B: 1-to-1, 1-to-all mapping from codepoint to UEs

- Larger number of candidate overlaid OFDM sequences will increase the complexity of sequence correlation detection, and decrease the performance of sequence correlation detection.

- Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell:

- support maximum 4 candidates overlaid sequences for M=1,2.

- For the information bits carried by the overlaid OFDM sequence(s):

- gNB determines the overlaid OFDM sequence(s) based on the OOK bit(s) transmitted within the OFDM symbol. In this way, it does not need to separately determine the bits mapping to the overlaid OFDM sequence when generating the M OOK waveform per OFDM duration.

- For OOK-4 with M=1 and 2, one sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbols.

- For OOK-4 with M=4, one sequence is selected from multiple candidates overlaid OFDM sequences on two OOK ‘ON’ symbols within one OFDM symbol, i.e. the overlaid OFDM sequence mapped from OOK bits within the OFDM symbol could be transmitted with repetition.

- The time duration of one LP-WUS is better no longer than one slot. Support mapping codepoint to a binary sequence before Manchester coding.

- LP-SS signal

- LP-SS uses a binary sequence associated to the cell ID by configuration. FFS: mapping schemes between cell ID and LP-SS sequences.

- The impact of CP duration should be considered when design and evaluate the LP-SS.

- For L=4 @M=1, considering balance between “1” and “0” of the binary sequence, only 3 binary sequence could meet the requirement of timing estimation error for SNR=-3dB.

- For L=6 @M=1, considering balance between “1” and “0” of the binary sequence, timing estimation error could smaller than 2us for SNR=-3dB.

- For L=8 @M=1, considering balance between “1” and “0” of the binary sequence, timing estimation error could smaller than 2us for SNR=-3dB.

- For L=8 @M=2, considering balance between “1” and “0” of the binary sequence, timing estimation error could smaller than 2us for SNR=-3dB.

- For L=12 @M=2, considering balance between “1” and “0” of the binary sequence, timing estimation error could smaller than 2us for SNR=-3dB.

- For L=16 @M=2, considering balance between “1” and “0” of the binary sequence, timing estimation error could smaller than 2us for SNR=-3dB.

- For L=16 @M=4, considering balance between “1” and “0” of the binary sequence, binary sequence could not meet the requirement of timing estimation error for SNR=-3dB.

- For L=32 @M=4, considering balance between “1” and “0” of the binary sequence, binary sequence could not meet the requirement of timing estimation error for SNR=-3dB.

- For M=4, the values for L to be selection could not meet the requirement of timing estimation error for SNR=-3dB.

- LP-WUS and LP-SS share the same frequency location, SSB location should be associated with LP-WUS/LP-SS.

- Consider shorter periodicity like 80ms or 160ms for LP-SS.

- Multiple LP-SSs can be transmitted in a period. Each LP-SS can be associated with a beam/SSB.

- Necessity of Additional synchronization signal

- For LP-WUS with M=1

- Fe=5ppm:

If LP-SS could only be configured with same M value, i.e. M=1, LP-SS design should limit the Tr<5us, in this case, no need to support 80ms, 160ms periodicity for LP-WUS with M=1.

If LP-SS could be configured with larger M values, i.e. M=2, 4, maximum timing error of LP-SS is 3.6us<5us for 320ms periodicity. No need to support 80ms, 160ms periodicity for LP-WUS with M=1.

- Fe=10ppm:

If LP-SS could only be configured with same M value, i.e. M=1, LP-SS design should limit the Tr<5us. In this case, when Tr=0.5,1us, no need to support 80ms, 160ms periodicity for LP-WUS with M=1. When Tr = 2us, at least need to consider 160ms periodicity for LP-SS.

If LP-SS could be configured with larger M values, i.e. M=2, 4, when Tr<2us, maximum timing error of LP-SS is 4.2us<5us for 320ms periodicity. No need to support 80ms, 160ms periodicity for LP-WUS with M=1.When Tr=2us, at least need to consider 160ms periodicity for LP-SS.

- For LP-WUS with M=2, when Fe=5ppm

- If LP-SS could only be configured with same M value, i.e. M=2

When Tr = 2us, the timing error of LP-SS is 3.6us>3us for 320 ms periodicity. At least need to consider 160ms periodicity for LP-SS.

When Tr < 2us, i.e. Tr=0.5us,1us, the maximum timing error of LP-SS is 2.6us<3us for 320ms periodicity. No need to support 80ms, 160ms periodicity for LP-WUS with M=1.

- If LP-SS could be configured with larger M value, i.e. M=4, maximum timing error of LP-SS is 2.6us<3us for 320ms periodicity. No need to support 80ms, 160ms periodicity for LP-WUS with M=2.

- If LP-SS could be configured with smaller M value, i.e. M=1,

When Tr = 0.5us,1us, the maximum timing error of LP-SS is 2.6us<3us for 320ms periodicity. No need to support 80ms, 160ms periodicity for LP-WUS with M=1.

When Tr = 2us, the timing error of LP-SS is 3.6us>3us for 320 ms periodicity. At least need to consider 160ms periodicity for LP-SS.

When Tr = 5us, LP-SS with 80ms periodicity still could not meet the performance requirement.

- For LP-WUS with M=2, when Fe=10ppm

- If LP-SS could only be configured with same M value, i.e. M=2, LP-SS with 320ms periodicity could not meet the performance requirement.

When Tr < 2us, i.e. Tr=0.5us,1us, at least need to consider 160ms periodicity for LP-SS.

When Tr = 2us, only LP-SS with 80ms periodicity could meet the performance requirement.

- If LP-SS could be configured with larger M value, i.e. M=4, at least need to consider 160ms periodicity for LP-SS.

- If LP-SS could be configured with smaller M value, i.e. M=1,

When Tr = 0.5us,1us, at least need to consider 160ms periodicity for LP-SS.

When Tr = 2us, only LP-SS with 80ms periodicity could meet the performance requirement.

When Tr = 5us, LP-SS with 80ms periodicity still could not meet the performance requirement.

- For LP-WUS with M=4,

- when Fe=5ppm:

Only LP-SS periodicity = 80ms under Tr=us could meet the performance requirement.

- when Fe=10ppm:

All the LP-SS design could not meet the performance requirement.

- Support Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

- Proposal 16: Considering timing error requirement for M=4, support option 1A: No additional sync signal, LP-SS periodicity =320ms, and additional 80ms periodicity.

Reference

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo

- 3GPP TR 38.869 V2.0.0, Study on low-power wake up signal and receiver for NR.

Cross-checking results