3GPP TSG RAN WG1 #120		R1-2500544

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.1

Source:	Google

Title:	Discussion on PMI prediction during cell DTX

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide some discussion on PMI prediction during cell DTX.

# Discussion

Currently it is defined that UE receives the CSI-RS for CSI acquisition during cell DTX active period(s), and UE does not receive the CSI-RS for CSI acquisition during cell DTX inactive period(s). For a CSI report, it is defined that UE transmits the CSI report after receiving at least one transmission occasion of each CSI-RS resource as follows:

For the CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI', the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell with cell DTX activated [10, TS 38.321] for channel measurement and/or interference measurement in active periods of cell DTX of the serving cell no later than CSI reference resource, and the UE drops the CSI report otherwise.

For PMI prediction, UE needs to measure Kp transmission occasions for periodic/semi-persistent CSI-RS resources to predict a CSI, which is defined as follows:

For a CSI-ReportConfig configured with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least one aperiodic or Kp periodic or semipersistent consecutive CSI-RS transmission occasions for each CSI-RS resource in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise. The value of Kp∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6.

Therefore, for PMI prediction, when cell DTX is activated, for periodic/semi-persistent CSI-RS based operation, UE should not be required to transmit a CSI report if it does not receive Kp consecutive transmission occasions of each CSI-RS resource during cell DTX active period(s). In that case, UE drops the CSI report.

Proposal 1: Endorse the CR R1-2500543 to clarify that UE transmits a CSI report after it receives Kp consecutive transmission occasions of each periodic or semi-persistent CSI-RS resource during cell DTX active period(s).

# Conclusion

In this contribution, we provided discussion on PMI prediction during cell DTX. Based on the discussion, the following proposals are provided.

Proposal 1: Endorse the CR R1-2500543 to clarify that UE transmits a CSI report after it receives Kp consecutive transmission occasions of each periodic or semi-persistent CSI-RS resource during cell DTX active period(s).